3GPP TSG-RAN WG4 Meeting #117	R4-2520255

# Dallas, USA, Nov. 17-21, 2025

Source: 	Huawei, HiSilicon

Title: 	More on CA_n5-n8 UE RF

Agenda Item:	5.9.3

Document for:	Approval

# Introduction

This contribution addresses raised concern in terms of UE RF for dual UL CA of CA_n5A-n8A.

# Discussion

One of the root causes to make us reach impasse in RF domain is that the idea captured in [1] accompanies RF Front End (RFFE) switching, e.g., if the UE wants to monitor/measure some key DL information scheduled or configured by NW via PCell (n5), even if the CA configuration stays at dual UL CA.

One paper [2] showed that this is inevitable due to the fact that isolation between n8 UL and n5 DL is not obtained by one common RF component.

Observation 1: From UE RF perspective, one of the root causes to make us reach impasse would be that the idea captured in [R4-2507910] accompanies RF Front End (RFFE) switching, e.g., if the UE wants to monitor/measure some key DL information scheduled or configured by NW via n5 DL even if the CA configuration stays at dual UL CA.

Another aspect is that some companies think that the switching behaviours should be clearly defined in the spec specifically if an associated UE capability is introduced. Some other think that standardization does not always specify all the details or corner cases.

Observation 2: Another issue associated with Observation 1 is that some think that if a new UE capability is defined, then, UE behaviours should be clearly defined, while some other think that standardization does not always specify all the details or corner cases.

Although everyone may not be happy, one way forward could be capturing only the essential switching mechanisms into 38.101-1, while the rest is left to UE and network implementation. The t-doc of [2] actually provided a specific proposal in it as below.

Figure 1: a proposal from [2]

Proposal 1: Discuss the below NOTE to be added to dual UL CA of CA_n5A-n8A as an alternative which takes into account the proposal in R4-2504116.

The UL configuration is supported for UEs that support [supportOfnon-ConcurrentInterbandULDL-r19] capability; During RRC configuration of UL CA_n5A-n8A with DL CA_n5A-n8A, UL transmission via n8 is not scheduled to the UEs in slots where DL reception via n5 is scheduled or configured to the UEs, or DL reception via n5 is not scheduled to the UEs in slots where UL transmission via n8 is scheduled to the UEs. The UEs are allowed to use switching period of 35 or 140 us depending on the capability, right before the start of n5 DL when switching from n8 UL or right before the start of the first n8 UL when switching from n5DL.

We, however, also received the comment that the proponent of [R4-2504116] believes that 3GPP shouldn’t introduce a new UE capability if UE behaviors are not defined probably in the same level of details introduced into other switching features. If this is the last piece to introduce UL CA_n5A-n8A, it would worth discussing whether the new UE capability is introduced or not.

Observation 4: According to offline discussion, some believe that 3GPP shouldn’t introduce a new UE capability if UE behaviors are not defined probably in the same level of granularity introduced into other switching features. If this is the last piece to introduce UL CA_n5A-n8A, it would worth discussing whether the new UE capability is introduced or not.

Here we share our original intention on why the new UE capability is needed. The necessity mainly comes from the fact that simultaneous Rx-Tx ability is mandatory without capability for FDD-FDD inter-band CA. UL CA_n5-n8 is actually FDD-FDD inter-band CA, while it cannot perform simultaneous Rx-Tx, more specifically only “n5DL-n8UL”, while n5UL-n8DL has no problem. Thus, if network follows this principle, it may cause a problem. Another aspect is that if in the future, so-called a dedicated filter solution to take isolation between n5DL and n8UL is taken, network can differentiate those UEs, given that UE supporting UL CA_n5A-n8A with dedicated filter solution does not need to indicate that new UE capability. Regarding the latter one, even if we don’t introduce a new UE capability now, it may be solved if we introduce a new capability saying that UL CA_n5A-n8A can perform simultaneous Rx-Tx, but this makes the situation very complicated. Because till the dedicated filter solution comes to the market, network considers UL CA_n5A-n8 is actually, not able to perform simultaneous Rx-Tx (n5DL-n8UL only) despite the fact that CA_n5-n8 is FDD-FDD inter-band CA. Then, when dedicated filter solution comes, we introduce a new UE capability that CA_n5-n8 can do FDD-FDD inter-band CA despite the principle of FDD-FDD inter-band CA shall be capable of that. There may be an option that this twisted situation can be fixed if we introduce new bands when dedicated filter solution comes, though proponents of this CA may not like to take this approach, since this requires specifying all the band combinations including the new bands which correspond to all the band combinations including CA_n5-n8 that the operator has.

When it comes to the opinion that we shouldn’t introduce a new UE capability under the condition that UE behaviour is not crystal clear and if it is defined, this should take normal 3GPP process including RAN1/2 discussion. In a sense, this is also a reasonable opinion, given that as standardization, what that UE should additionally meet for a new feature with a new UE capability must be understood at least, though as said in Observation 2, the boundary of “unclear” may be different depending on companies.

Proposal 2: If no introduction of a new UE capability is the last piece to get the agreement to introduce UL CA_n5A-n8A in Rel-19, further discuss whether the new UE capability is introduced or not.

# Conclusion

In this contribution we further provided our views on how to move on for UL CA_n5A-n8A as follows.

Observation 1: From UE RF perspective, one of the root causes to make us reach impasse would be that the idea captured in the WF [R4-2507910] accompanies RF Front End (RFFE) switching, e.g., if the UE wants to monitor/measure some key DL information scheduled or configured by NW via n5 DL even if the CA configuration stays at dual UL CA.

Observation 2: Another issue associated with Observation 1 is that some think that if a new UE capability is defined, then, UE behaviours should be clearly defined, while some other think that standardization does not always specify all the details or corner cases.

Proposal 1: Discuss the below NOTE to be added to UL CA_n5A-n8A as an alternative as a starting point which takes into account the proposal in R4-2504116.

The UL configuration is supported for UEs that support [supportOfnon-ConcurrentInterbandULDL-r19] capability; During RRC configuration of UL CA_n5A-n8A with DL CA_n5A-n8A, UL transmission via n8 is not scheduled to the UEs in slots where DL reception via n5 is scheduled or configured to the UEs, or DL reception via n5 is not scheduled to the UEs in slots where UL transmission via n8 is scheduled to the UEs. The UEs are allowed to use switching period of 35 or 140 us depending on the capability, right before the start of n5 DL when switching from n8 UL or right before the start of the first n8 UL when switching from n5DL.

Observation 4: According to offline discussion, some believe that 3GPP shouldn’t introduce a new UE capability if UE behaviors are not defined probably in the same level of granularity introduced into other switching features. If this is the last piece to introduce UL CA_n5A-n8A, it would worth discussing whether the new UE capability is introduced or not.

Proposal 2: If no introduction of a new UE capability is the last piece to get the agreement to introduce UL CA_n5A-n8A in Rel-19, further discuss whether the new UE capability is introduced or not.

# References

[1] R4-2507910, WF on simultaneous Rx/Tx requirements, RAN4#115, May, 2025

[3] R4-2504116, CA_n5A-n8A capabilities, RAN4#114bis, April, 2025.