3GPP TSG RAN4 Meeting #117	R4-2520257

Dallas, USA, 17th November 2025- 21st November 2025

Agenda item:		5.5.2

Source:		Union Inter. Chemins de Fer, Nokia, Skyworks Solutions Inc., Qualcomm

Title:		TP to TR 38.795: FRMCS HPUE TDD band n101

Release:		19

Work Item:	HPUE_NR_FR1_bands_R19-Core

Document for:		Approval

# Introduction

In RAN4#116bis the WF R4-2514579 has been approved that, instead of defining FRMCS specific PC2 and PC1.5 UE, proposes to define PC1 Uplink CA configuration CA_n100-n101 and PC1 Uplink DC configuration DC_n100-n101 in order to support the railway FRMCS needs.

This contribution provides a corresponding text proposal for TR 38.795.

# Text Proposal

<Start of change>

7	Power class 2, 1.5 and 1 for FWVM within FR1 TDD

bands

7.1	NR band n101

7.1.1 UE maximum output power.

For NR band n101, PC1 has been defined in TS 38.101-1 for FRMCS cab radios. For FRMCS, railways need the ability to use uplink n100 / n101 inter-band CA/DC with the highest possible UE transmit power to achieve maximum coverage. As PC1 was not defined as a possible power class for NR multi carrier operation, it was considered to define HPUE inter-band CA/DC support based on FRMCS specific PC2 and PC1.5 classes that comply with the European regulatory required 37 dBc ACLR, and implemented based on FRMCS PC1 devices. However, as such FRMCS specific PC2 and PC1.5 devices would deviate from the underlying assumptions of the already defined NR  PC2 and PC1.5 devices,  it was agreed not to define those.

Instead, to support the FRMCS need for HPUE inter-band CA/DC, it was agreed to add in TS 38.101-1 new text to clarify that Uplink CA configuration CA_n100-n101 and Uplink DC configuration DC_n100-n101 are allowed only for FRMCS cab-radios, i.e. user equipment installed on trains with external antenna on top of the train roof at approximately 4m over ground, with UE having PC1 capability per band.

<End of change>

# References

[1]	WF R4-2514579

[2]	TR 38.795 V1.1.0 (2025-10)

[3]	RP-252606 NR_bands_xFeature_R19-Core

[4]	TS 38.101-1