3GPP TSG-RAN WG4 Meeting # 117	R4-2520279

Dallas, Texas, USA, 17th – 21st November 2025

Agenda Item:	7.1.3

Source: 	Nokia

Title: 	Simulation results for HPUE coexistence study

Document for:	Information

# 1.	Introduction

The new WI on UE RF enhancements for NR FR1, Phase 5 was approved at TSG RAN#109 [1]. One of the objectives of this WI is related to high power UE (HPUE) for NR single carrier operation:

High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class

The topic on coexistence study for ACLR was discussed at TSG RAN4#116bis [2] and the WF was agreed [3] where the simulation assumptions for preliminary simulation results were captured. This contribution provides preliminary simulation results at 7GHz (band n104) using the agreed assumptions in the WF.

# 2.	Simulation Results

The following inter-site distances were included in the WF [3]:

| Inter-site distance | Depending on frequency (ref. TR 37.829 for 2.6GHz, 3.5GHz and [TR 38.921, 600m] for other frequencies) |
| --- | --- |

Note that the following inter-site distances were included in Table 4.2.1.1-1 of TR 38.921 [4]:

| Inter-site distance | 0.45 km (urban) (Note 1) 0.9 km (suburban) | Based on cell range: 0.3 km (urban) 0.6 km (suburban) |
| --- | --- | --- |

Therefore, preliminary simulation results are provided below for 0.45, 0.6, and 0.9 km inter-site distances.

### 2.1		0.45 km inter-site distance

The CDFs of the UE transmit power and UL SINR of the victim UE, as well as the victim system UL throughput loss Vs ACLR offset, for 0.45 km inter-site distance are shown in Figure 2.1-1 below.

(a) UE transmit power								(b) UL SINR of victim UE

| ACLR offset 30+X [dB] | 0 | 1 |
| --- | --- | --- |
| Average throughput loss (23 dBm interfering UE) | 0.37% |  |
| 5%-tile throughput loss (23 dBm interfering UE) | 2.20% |  |
| Average throughput loss (32 dBm interfering UE) | 0.39% | 0.34% |
| 5%-tile throughput loss (32 dBm interfering UE) | 2.34% | 1.28% |

(c) Throughput loss

Figure 2.1-1: For 0.45 km inter-site distance

It can be seen from the CDFs of the UE transmit power in Figure 2.1-1 that the CDFs of the 23 dBm UE and the 32 dBm UE are identical until the UE reach their maximum output power. This is expected as the CLx-ile is adjusted according to the UE maximum output power. Comparing the CDFs of the UE transmit power, it can be seen that a higher percentage (~12%) of 23 dBm UE transmitted with their maximum output power than that (~6%) of 32 dBm UE aiming to achieve the target 15 dB UL SINR.

Moreover, it can be seen from the victim system UL throughput loss Vs ACLR offset in Figure 2.1-1 that 31 dB ACLR for 32 dBm UE (currently specified for power classes 1.5 and 2 UE) will ensure that the victim system performance degradation due to 32 dBm interfering UE is not larger than that due to 23 dBm interfering UE.

### 2.2		0.6 km inter-site distance

The CDFs of the UE transmit power and UL SINR of the victim UE, as well as the victim system UL throughput loss Vs ACLR offset, for 0.6 km inter-site distance are shown in Figure 2.2-1 below.

(a) UE transmit power								(b) UL SINR of victim UE

| ACLR offset 30+X [dB] | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Average throughput loss (23 dBm interfering UE) | 0.53% |  |  |  |
| 5%-tile throughput loss (23 dBm interfering UE) | 2.16% |  |  |  |
| Average throughput loss (32 dBm interfering UE) | 0.60% | 0.53% | 0.46% | 0.40% |
| 5%-tile throughput loss (32 dBm interfering UE) | 2.35% | 2.35% | 2.35% | 2.16% |

(c) Throughput loss

Figure 2.2-1: For 0.6 km inter-site distance

It can again be seen from the results in Figure 2.2-1 that the CDFs of the 23 dBm UE and the 32 dBm UE are identical until the UE reach their maximum output power, and a higher percentage (~20%) of 23 dBm UE transmitted with their maximum output power than that (~10%) of 32 dBm UE.

On the other hand, 31 dB ACLR for 32 dBm UE will ensure that average throughput loss and 5%-ile throughput loss due to 32 dBm interfering UE are less than 5% but will not ensure that the victim system performance degradation due to 32 dBm interfering UE is not larger than that due to 23 dBm interfering UE.

### 2.3		0.9 km inter-site distance

The CDFs of the UE transmit power and UL SINR of the victim UE, as well as the victim system UL throughput loss Vs ACLR offset, for 0.9 km inter-site distance are shown in Figure 2.3-1 below.

(a) UE transmit power								(b) UL SINR of victim UE

| ACLR offset 30+X [dB] | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Average throughput loss (23 dBm interfering UE) | 0.53% |  |  |  |  |
| 5%-tile throughput loss (23 dBm interfering UE) | NA |  |  |  |  |
| Average throughput loss (32 dBm interfering UE) | 0.82% | 0.72% | 0.63% | 0.56% | 0.49% |
| 5%-tile throughput loss (32 dBm interfering UE) | NA | NA | NA | NA | NA |

(c) Throughput loss

Figure 2.3-1: For 0.9 km inter-site distance

It can be seen from the results in Figure 2.3-1 that 5%-tile (cell edge) throughput is not achievable with 0.9 km inter-site distance even for 32 dBm UE. However, only the urban macro (UMa) channel model was included in TR 38.921 (which is a Rel-18 TR), while the suburban macro (SMa) channel model was introduced in TR 38.901 [5] as Rel-19 channel model enhancements [6]. The simulation results in Figure 2.3-2 below were obtained using the SMa channel model in TR 38.901.

.

(a) UE transmit power								(b) UL SINR of victim UE

| ACLR offset 30+X [dB] | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (23 dBm interfering UE) | 0.51% |  |  |  |  |  |  |
| 5%-tile throughput loss (23 dBm interfering UE) | 3.17% |  |  |  |  |  |  |
| Average throughput loss (32 dBm interfering UE) | 0.72% | 0.63% | 0.55% | 0.49% | 0.43% | 0.37% | 0.33% |
| 5%-tile throughput loss (32 dBm interfering UE) | 5.11% | 4.52% | 4.52% | 4.19% | 3.32% | 3.32% | 3.15% |

(c) Throughput loss

Figure 2.3-2: For 0.9 km inter-site distance

It can be seen from the results in Figure 2.3-2 that 5%-tile (cell edge) throughput is achievable even with 0.9 km inter-site distance using the SMa channel model. Similar observations as with 0.6 km inter-site distance can be made from the results in Figure 2.3-2, namely that the CDFs of the 23 dBm UE and the 32 dBm UE are identical until the UE reach their maximum output power, a higher percentage (~25%) of 23 dBm UE transmitted with their maximum output power than that (~25%) of the 32 dBm UE, and 31 dB ACLR for 32 dBm UE will ensure that average throughput loss and 5%-ile throughput loss due to 32 dBm interfering UE are less than 5% but will not ensure that the victim system performance degradation due to 32 dBm interfering UE is not larger than that due to 23 dBm interfering UE.

### 2.4		BS received signal power

The 99.99%-tile of the victim BS received signal power for the simulated cases are summarized in Table 2.4-1 below. It can be seen that the 99.99%-tile received signal power in all simulated cases, except with 32 dBm UE with 0.6 km inter-site distance using the UMs channel model, are lower than the current -43 dBm in-band blocking requirement specified in TS 38.104 [7] for wide-area BS. In the exception case, site engineering solutions (e.g., larger distance between victim BS and interfering HPUE, better RF filtering in the victim BS receiver chain) will be required to ensure satisfactory coexistence between the victim BS and interfering 32 dBm UE.

Table 2.4-1: 99.99%-tile victim BS received signal power

| UE maximum output power (dBm) | 0.45 km inter-site distance | 0.6 km inter-site distance | 0.9 km inter-site distance (SMa) |
| --- | --- | --- | --- |
| 23 | -49.3747 | -44.7359 | -47.0788 |
| 32 | -47.5586 | -42.2195 | -44.6808 |

# 3.	Conclusion

This contribution has provided preliminary simulation results at 7GHz (band n104) using the agreed assumptions in the WF.

# References

[1]	RP-252942, “New WID: UE RF enhancements for NR FR1, Phase 5”, Moderator (vivo).

[2]	R4-2514536, “Topic summary for [116bis][329] NR_UE_RF_Ph5_Part2”, Moderator (Samsung).

[3]	R4-2515085, “Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 - HPUE coexistence”, Huawei.

[4]	3GPP TR 38.921 v18.0.0: “NR; Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz”.

[5]	3GPP TR 38.901 v19.1.0: “Study on channel model for frequencies from 0.5 to 100 GHz”.

[6]	R1-2504964, “Introduction of Rel-19 7-24 GHz channel model enhancements”, Intel Corporation, ZTE Corporation.

[7]	3GPP TS 38.104 v19.2.0: “NR; Base Station (BS) radio transmission and reception”.