3GPP TSG-RAN WG4 Meeting # 117	R4-2520280

Dallas, Texas, USA, 17th – 21st November 2025

Agenda Item:	7.1.3

Source: 	Nokia

Title: 	Proposals on simulation assumptions for HPUE coexistence study

Document for:	Approval

# 1.	Introduction

The new WI on UE RF enhancements for NR FR1, Phase 5 was approved at TSG RAN#109 [1]. One of the objectives of this WI is related to high power UE (HPUE) for NR single carrier operation:

High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class

The topic on coexistence study for ACLR was discussed at TSG RAN4#116bis [2] and the WF was agreed [3] where the simulation assumptions for preliminary simulation results were captured. This contribution provides proposals on refinements of the simulation assumptions for HPUE coexistence study.

# 2.	Discussion

The following inter-site distances were included in the WF [3]:

| Inter-site distance | Depending on frequency (ref. TR 37.829 for 2.6GHz, 3.5GHz and [TR 38.921, 600m] for other frequencies) |
| --- | --- |

Note that the following inter-site distances were included in Table 4.2.1.1-1 of TR 38.921 [4]:

| Inter-site distance | 0.45 km (urban) (Note 1) 0.9 km (suburban) | Based on cell range: 0.3 km (urban) 0.6 km (suburban) |
| --- | --- | --- |

The preliminary simulation results provided in [5] show that 5%-tile (cell edge) throughput is not achievable with 0.9 km inter-site distance even for 32 dBm UE. However, only the urban macro (UMa) channel model was included in TR 38.921 (which is a Rel-18 TR), while the suburban macro (SMa) channel model was introduced in TR 38.901 [6] as Rel-19 channel model enhancements [7]. Therefore, it is proposed to use the SMa channel model in TR 38.901 for HPUE coexistence study with 0.9 km inter-site distance at 7GHz. The simulation parameters are provided in Table 2-1 below.

Table 2-1: Network layout model for suburban macro at 7GHz

| Parameters |  | Value |
| --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance |  | 0.9 km |
| Multi operators layout |  | Uncoordinated |
| BS antenna height |  | 25 m |
| UE location | Outdoor/indoor | Outdoor and indoor |
|  | Indoor UE ratio | 2% in commercial building, 18% in residential building (Table 7.4.2-1 in TR 38.901) |
|  | Low/high penetration loss ratio | 50% low loss and low loss A, 50% high loss (Table 7.4.3-2 in TR 38.901) |
|  | LOS/NLOS | LOS and NLOS |
|  | UE antenna height | 1.5 m for outdoor, 1.5 or 4.5 m for residential buildings, 1.5/4.5/7.5/10.5/13.5 m for commercial buildings (Table 7.2-5 in TR 38.921) |
| UE distribution (horizontal) |  | Uniform |
| Minimum BS - UE distance (2D) |  | 35 m For Uncoordinated deployment, minimum distance applies also to UEs and BS across different deployments. |
| Channel model |  | SMa (Table 7.4.1-1 in TR 38.901) |

# 3.	Conclusion

This contribution has provided proposals on refinements of the simulation assumptions for HPUE coexistence study. It is proposed to use the SMa channel model, as provided in Table 2-1 above. for HPUE coexistence study with 0.9 km inter-site distance at 7GHz.

# References

[1]	RP-252942, “New WID: UE RF enhancements for NR FR1, Phase 5”, Moderator (vivo).

[2]	R4-2514536, “Topic summary for [116bis][329] NR_UE_RF_Ph5_Part2”, Moderator (Samsung).

[3]	R4-2515085, “Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 - HPUE coexistence”, Huawei.

[4]	3GPP TR 38.921 v18.0.0: “NR; Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz”.

[5]	R4-2520279, “Simulation results for HPUE coexistence study”, Nokia.

[6]	3GPP TR 38.901 v19.1.0: “Study on channel model for frequencies from 0.5 to 100 GHz”.

[7]	R1-2504964, “Introduction of Rel-19 7-24 GHz channel model enhancements”, Intel Corporation, ZTE Corporation.