3GPP TSG-RAN WG4 Meeting # 117	R4-2520281

Dallas, Texas, US, 17th – 21st November 2025

Agenda Item:	8.4.3

Source: 	Nokia

Title: 	Coexistence studies for 6G Radio

Document for:	Approval

# 1.	Introduction

The revised SI on 6G Radio was approved at TSG RAN#108 [1]. One of the objectives of this SI is intra-3GPP coexistence studies:

- General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements

The topic on coexistence studies was discussed at TSG RAN4#116bis [2] and the WF was agreed [3] where it is captured:

Way Forward: List of candidate frequencies: 700 MHz, 2 GHz, 4 GHz, 7 GHz, 15 GHz and 30 GHz. For each exemplary frequency, companies proposing to redo coexistence studies should: - Identify the key assumptions differences (comparing to the assumptions used when RAN4 did the coex study e.g. the work done in TR 38.921 for 6.425-7.125GHz - Identify the potential impacts on the conclusions of previous coex studies (e.g. how much ACLR/ACS difference is expected). - Identify the next steps (update existing requirements? New requirements/new type? Regulation impacts?).

As shown above, companies proposing to redo coexistence studies should identify the key assumptions difference compared to the assumptions used for previous coexistence studies and the potential impacts on the conclusions. This contribution provides proposals on the assumptions to be used for coexistence studies for 6G Radio.

# 2.	Discussion

Coexistence studies are performed in RAN4 to decide the related coexistence requirements (ACLR, ACS, transmitter spurious emission for coexistence, receiver in-band blocking) for BS and UE in the targeted frequency bands. They are typically performed by system level simulation using the agreed sets of assumptions.

During the NR SI, coexistence studies were performed reusing largely the simulation assumptions (TR 38.803 [4]) from LTE (TR 36.942 [5]) and AAS (TR 37.840 [6] and TR 37.842 [7]). RAN4 should examine whether these simulation assumptions would continue to reflect real-life deployment for 6G Radio or they should be revised when performing coexistence studies for 6G Radio.

BS vertical coverage range is one of the antenna array parameters that was included in the LS replies [8-10] to ITU-R on IMT parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz and recorded in TR 38.922 [11]:

Table 6.1.2.3.2-3: Antenna array parameters Parameter; Indoor; Urban macro; Dense urban. Element gain (dBi) (Note 2); 5; 6.4; 6.4. Horizontal/vertical 3 dB beam width of single element (degree); 90º for H 90º for V; 90º for H 65º for V; 90º for H 65º for V. Horizontal/vertical front-to-back ratio (dB); 30 for both H/V; 30 for both H/V; 30 for both H/V. Antenna polarization; Linear ±45º; Linear ±45º; Linear ±45º. Antenna array configuration (Row × Column) (Note 4); 4 x 4; 16 x 24; 16 x 24. Horizontal/Vertical radiating sub-array spacing; 0.5 of wavelength for H, 0.5 of wavelength for V; 0.5 of wavelength for H, 2.8 of wavelength for V (Note 5); 0.5 of wavelength for H, 2.8 of wavelength for V (Note 5). Number of element rows in sub-array; N/A; 4; 4. Vertical element separation in sub-array (dv,sub); 0.5 of wavelength of V; 0.7 of wavelength of V; 0.7 of wavelength of V. Pre-set sub-array down-tilt (degrees); N/A; 3; 3. Array Ohmic loss (dB) (Note 2); 2; 2; 2. Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3); [2/8]; 17.14; 7.1. Base station horizontal coverage range (degrees); +/-90; +/-60; +/-60. Base station vertical coverage range (degrees) (Note 1); 0-180; 90-100; 90-100. Mechanical down-tilt (degrees); 90; 6; 6. Note 1: The vertical coverage range is given for the elevation angle θ, defined between 0° and 180°. Note 2: The element gain includes the loss and is per polarization. Note 3: The conducted power per sub-array assumes 16 × 24 sub-arrays and 2 polarizations for urban macro and dense urban cases; the conducted power per element assumes 4 x 4 elements and 2 polarizations for indoor case Note 4: 16 × 24 means there are 16 vertical and 24 horizontal radiating sub-arrays for urban macro and dense urban cases; 4 x 4 means there are 4 vertical and 4 horizontal radiating elements for indoor case. Note 5: For the case of 4 elements per sub array, dv will be 2.8 wavelengths..

BS vertical coverage range is one of the important design parameters in real-life deployment of AAS BS. However, it has not been used in any system level simulation in RAN4 up to now, where UE specific beamforming (TR 37.842) has been used to point the beam directly to each UE being served by the BS (without limitation on the elevation angle). Simulation results using the BS vertical coverage range to limit the elevation angle of the beam are provided below to show the potential impacts on the conclusions of coexistence studies.

2.1	Downlink Results

The urban macro simulation results of the downlink SINR of the victim UE at 15 GHz carrier frequency are provided in Figure 1 below using the agreed simulation assumptions in TR 38.922, without and with BS vertical coverage range. Here the downlink ACIR is obtained using 45 dB BS ACLR and 33 dB UE ACS (currently defined for BS and UE in FR1). It can be seen from the figure that in both cases enough downlink SINR (up to 70 dB and 65 dB without and with BS vertical coverage range, respectively) can be provided using 46 dBm BS conducted output power together with AAS BS beamforming. Moreover, the downlink SINR with BS vertical coverage range is lower than that without BS vertical coverage range, as the beam may not be directly pointed to each served UE.

(a) Without BS vertical coverage range			(b) With BS vertical coverage range

Figure 1: Urban Macro DL SINR of victim UE

The urban macro simulation results of the downlink throughput losses of the victim UE with different ACIR offsets (compared to the ACIR using 45 dB BS ACLR and 33 dB UE ACS) without and with BS vertical coverage range are provided, respectively, in Tables 1 and 2 below. Comparing the results between Tables 1 and 2, both average and 5%-tile throughput losses with BS vertical coverage range are higher than those without BS vertical coverage range, and the 5%-tile throughput loss with BS vertical coverage range are notably higher as the beam is not directly pointed to those UE at the cell edge.

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (%) | 0.17 | 0.28 | 0.45 | 0.70 | 1.06 | 1.55 |
| 5%-tile throughput loss (%) | 8.8485e-04 | 0.18 | 0.35 | 1.00 | 1.38 | 3.53 |

Table 1: Urban Macro DL throughput loss of victim UE without BS vertical coverage range

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (%) | 0.32 | 0.49 | 0.75 | 1.13 | 1.64 | 2.34 |
| 5%-tile throughput loss (%) | 3.93 | 3.98 | 4.09 | 11.57 | 11.57 | 11.57 |

Table 2: Urban Macro DL throughput loss of victim UE with BS vertical coverage range

2.2	Uplink Results

The urban macro simulation results of the victim and interfering UE transmit power at 15 GHz carrier frequency are provided in Figure 2 below using the agreed simulation assumptions in TR 38.922, without and with BS vertical coverage range. Here the uplink ACIR is obtained using 30 dB UE ACLR and 45 dB BS ACS (currently defined for BS and UE in FR1). It can be seen from the figure that around 20% and 32% of the UE without and with BS vertical coverage range, respectively, are transmitting at maximum power.

(a) Without BS vertical coverage range			(b) With BS vertical coverage range

Figure 2: Urban Macro UE transmit power

The urban macro simulation results of the victim UE UL SINR are provided in Figure 3 below. Here the uplink ACIR is obtained using 30 dB UE ACLR and 45 dB BS ACS (currently defined for BS and UE in FR1). It can be seen from the figure that the target UL SINR of 15 dB is achieved by around 80% and 68% of the UE without and with BS vertical coverage range, respectively.

(a) Without BS vertical coverage range			(b) With BS vertical coverage range

Figure 3: Urban Macro UE UL SINR

The urban macro simulation results of the victim BS received blocking signal power at the TAB connector (i.e. including the antenna array gain) are provided in Figure 4 below. The 99.99%-tile received blocking signal power levels are less than -48 dBm and -39 dBm without and with BS vertical coverage range, respectively.

(a) Without BS vertical coverage range			(b) With BS vertical coverage range

Figure 4: Urban Macro received blocking signal power

The urban macro simulation results of the uplink throughput losses of the victim UE with different ACLR offsets (compared to the 30 dB UE ACLR) without and with BS vertical coverage range are provided, respectively, in Tables 3 and 4 below. Comparing the results between Tables 3 and 4, the average throughput loss with BS vertical coverage range is higher than those without BS vertical coverage range, and 5%-tile (cell edge) throughput is not achievable with BS vertical coverage range as the beam is not directly pointed to those UE at the cell edge.

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (a) | 7.6453e-02 | 0.12 | 0.20 | 0.30 | 0.46 | 0.68 |
| 5%-tile throughput loss (a) | 1.14 | 2.21 | 3.52 | 3.55 | 3.61 | 3.73 |

Table 3: Urban Macro UL throughput loss of victim UE without BS vertical coverage range

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (a) | 0.19 | 0.26 | 0.37 | 0.52 | 0.44 | 0.61 |
| 5%-tile throughput loss (a) | NA | NA | NA | NA | NA | NA |

Table 4: Urban Macro UL throughput loss of victim UE with BS vertical coverage range

# 3.	Conclusion

This contribution has provided simulation results using the BS vertical coverage range to limit the elevation angle of the beam to show the potential impacts on the conclusions of coexistence studies. It is proposed that:

Proposal: BS vertical coverage range should be used as one of the assumptions in system level simulation for coexistence studies for 6G Radio.

# References

[1]	RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone.

[2]	R4-2514510, “Topic Summary for [116bis][103] 6G BS RF and co-existence”, Moderator (Ericsson).

[3]	R4-2514638, “WF on 6G BS RF and co-existence”, Ericsson.

[4]	3GPP TR 38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

[5]	3GPP TR 25.942 v18.0.0: “Radio Frequency (RF) system scenarios”.

[6]	3GPP TR 37.840 v12.1.0, “Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station”.

[7]	3GPP TS 37.842 v13.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Universal Terrestrial Radio Access (UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”.

[8]	R4-2410576: “LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, Ericsson.

[9]	R4-2414449: “LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, Ericsson.

[10]	R4-2420385: “LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, Ericsson.

[11]	3GPP TR 38.922 v19.2.0, “NR; Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”.