3GPP TSG RAN4 Meeting #117		R4-2520285
Dallas, US, 17th – 21st November, 2025

Agenda item:			5.11.3

Source:	Nokia

Title:	On 4940-4990 MHz BS requirements

Document for:	Approval

# Introduction

During RAN4#116bis, the WF on 4.9GHz band was agreed in [1]. This document addresses BS specification impact on power limits due to introduction of new 4.9GHz band.

# Discussion

During the last RAN4 meeting, BS maximum output power requirement impact was discussed without conclusion. Below is highlighted relevant text from § 90.1215 which should be taken into account when new band is introduced in BS specifications. It should be also noted power limits listed in Table 1 below allow to define requirements for Local Area and Medium Range BS only.

Furthermore, the formula used in (f)(2) is used e.g. in § 101.143 and applies to fixed microwave services. Therefore, it should be not considered for further 3GPP band work in RAN4.

§ 90.1215 Power limits.

Except as provided in paragraph (f) of this section, the transmitting power of stations operating in the 4940-4990 MHz band must not exceed the maximum limits in this section.

(a)

(1) For base, mobile, and temporary fixed operations, the maximum conducted output power must not exceed:

| Table 1 to Paragraph (a)(1) |  |  |
| --- | --- | --- |
| Channel bandwidth (MHz) | Low power maximum conducted output power (dBm) | High power maximum conducted output power (dBm) |
| 1 | 7 | 20 |
| 5 | 14 | 27 |
| 10 | 17 | 30 |
| 15 | 18.8 | 31.8 |
| 20 | 20 | 33 |
| 30 | 21.8 | 34.8 |
| 40 | 23 | 36 |
| 50 | 24 | 37 |

(2) High power devices are also limited to a peak power spectral density of 21 dBm per one MHz. High power devices using channel bandwidths other than those listed above are permitted; however, they are limited to peak power spectral density of 21 dBm/MHz. If transmitting antennas of directional gain greater than 9 dBi are used, both the maximum conducted output power and the peak power spectral density should be reduced by the amount in decibels that the directional gain of the antenna exceeds 9 dBi. However, high power point-to-point and point-to-multipoint operations (both fixed and temporary-fixed rapid deployment) may employ transmitting antennas with directional gain up to 26 dBi without any corresponding reduction in the maximum conducted output power or spectral density. Corresponding reduction in the maximum conducted output power and peak power spectral density should be the amount in decibels that the directional gain of the antenna exceeds 26 dBi.

(b) Low power devices are also limited to a peak power spectral density of 8 dBm per one MHz. Low power devices using channel bandwidths other than those listed above are permitted; however, they are limited to a peak power spectral density of 8 dBm/MHz. If transmitting antennas of directional gain greater than 9 dBi are used, both the maximum conducted output power and the peak power spectral density should be reduced by the amount in decibels that the directional gain of the antenna exceeds 9 dBi.

(c) The maximum conducted output power is measured as a conducted emission over any interval of continuous transmission using instrumentation calibrated in terms of an RMS-equivalent voltage. If the device cannot be connected directly, alternative techniques acceptable to the Commission may be used. The measurement results shall be properly adjusted for any instrument limitations, such as detector response times, limited resolution bandwidth capability when compared to the emission bandwidth, sensitivity, etc., so as to obtain a true maximum conducted output power measurement conforming to the definitions in this paragraph for the emission in question.

(d) The peak power spectral density is measured as conducted emission by direct connection of a calibrated test instrument to the equipment under test. If the device cannot be connected directly, alternative techniques acceptable to the Commission may be used. Measurements are made over a bandwidth of one MHz or the 26 dB emission bandwidth of the device, whichever is less. A resolution bandwidth less than the measurement bandwidth can be used, provided that the measured power is integrated to show total power over the measurement bandwidth. If the resolution bandwidth is approximately equal to the measurement bandwidth, and much less than the emission bandwidth of the equipment under test, the measured results shall be corrected to account for any difference between the resolution bandwidth of the test instrument and its actual noise bandwidth.

(e) The ratio of the peak excursion of the modulation envelope (measured using a peak hold function) to the maximum conducted output power shall not exceed 13 dB across any 1 MHz bandwidth or the emission bandwidth whichever is less.

(f) The transmitting power of permanent fixed point-to-point and point-to-multipoint stations operating in the 4940-4990 MHz band must not exceed the maximum limits in this paragraph (f). Moreover, applicants should request no more power than necessary for a particular use.

(1) The maximum equivalent isotropically radiated power (EIRP), as referenced to an isotropic radiator, must not exceed 55 dBW (85 dBm).

(2) For path lengths shorter than 17 kilometers, the EIRP shall not exceed the value derived from the following equation: New EIRP limit = 55 dBW—40*log(17/B) dBW, where B = the actual path length in kilometers.

# Conclusion

This document addresses BS specification impact on power limits due to introduction of new 4.9GHz band with the following proposals:

Proposal 1: 3GPP 4.9GHz band BS requirements should take into account power limits defined in Table 1 above, requirements to be defined for Local Area and Medium Range BS only.

Proposal 2: 3GPP 4.9GHz band BS requirements should not consider limits defined in (f) above.

# Reference

[1] R4-2515103; Way Forward for [116bis][322] NR_TDD_4.9GHz_band; Fujitsu