3GPP TSG RAN4 Meeting #117		R4-2520288
Dallas, US, 17th – 21st November, 2025

Source:	Nokia

Title:	6GR Considerations on BS RF

Agenda item:		8.4.1

Document for:	Approval

# 1	Introduction

During RAN4#116bis, WF on 6G BS RF and coexistence was agreed in [1]. This document addresses number of issues addressed by this WF.

# 2	Discussion

2.	1		Requirements to be re-assessed

During last RAN4 meeting, it was agreed 5G BS RF requirements should be considered as a starting point when defining the 6G BS RF requirements. In the table below, analysis are provided how 5G BS RF requirements should be considered for 6G, according to agreement in [1]:

| 5G BS RF requirements | No impact expected, might be reused as is for 6G | To be re-evaluated based on system parameters decision for 6G (SU, channel BW, …), following TR 38.817-02 principles/formula | Would need to be studied in the 6G SI scope |
| --- | --- | --- | --- |
| Conducted requirements |  |  |  |
| Base station output power | X |  |  |
| Output power dynamics |  |  |  |
| RE power control dynamic range |  | X (depending on SU and channel BW) |  |
| Total power dynamic range |  | X (depending on SU and channel BW) |  |
| Transmit ON/OFF | X (for existing bands considering legacy deployment) |  | X (for new 6G bands) |
| Transmitted signal quality |  |  |  |
| Frequency error | X |  |  |
| Modulation quality |  |  | X (studies are needed for EVM requirement and measurement methodologies) |
| Time alignment error |  | X (depending on MRTD discussions) |  |
| Unwanted emissions |  |  |  |
| Occupied bandwidth | X (regulatory requirement) |  |  |
| ACLR | X (for existing bands considering legacy deployment) |  | X (for new 6G bands depending on results of coexistence studies) |
| Operating Band Unwanted Emissions |  | X (depending on SU and channel BW) |  |
| Transmitter spurious emission |  |  |  |
| General requirement | X (regulatory requirement) |  |  |
| Protection of BS receiver of own or different BS | X |  |  |
| Additional spurious (coex) | X (for existing bands considering legacy deployment) |  | X (for new 6G bands depending on results of coexistence studies) |
| Co-location | X (for existing bands considering legacy deployment) |  | X (for new 6G bands) |
| Transmitter intermodulation |  |  | X (possible enhancement on test efforts) |
| Receiver sensitivity level |  | X (depending on SU and channel BW) |  |
| Dynamic range |  | X (depending on SU and channel BW) |  |
| In-band selectivity and blocking |  |  |  |
| ACS | X (for existing bands considering legacy deployment) |  | X (for new 6G bands depending on results of coexistence studies) |
| In-band blocking | X (for existing bands considering legacy deployment) |  | X (for new 6G bands depending on results of coexistence studies) |
| Out of band blocking | X (for existing bands considering legacy deployment) |  | X (for new 6G bands) |
| Receiver spurious emissions | X |  |  |
| Receiver intermodulation |  | X (pending what is the internal guard-band to make sure IM product falls to relevant wanted signal RB) |  |
| In channel selectivity |  | X (depending on SU and channel BW) |  |
| Radiated requirements |  |  |  |
| Radiated transmit power | X |  |  |
| OTA base station output power | X |  |  |
| OTA output power dynamics |  |  |  |
| OTA RE power control dynamic range |  | X (depending on SU and channel BW) |  |
| OTA Total power dynamic range |  | X (depending on SU and channel BW) |  |
| OTA transmit ON/OFF |  |  | X (for new 6G bands, for existing bands depending on the Rel-20 co-location requirements enhancement) |
| OTA transmitted signal quality |  |  |  |
| OTA Frequency error | X |  |  |
| OTA Modulation quality |  |  | X (studies are needed for EVM requirement and measurement methodologies) |
| OTA Time alignment error |  | X (depending on MRTD discussions) |  |
| OTA Unwanted emissions |  |  |  |
| OTA Occupied bandwidth | X (regulatory requirement) |  |  |
| OTA ACLR | X (for existing bands considering legacy deployment) |  | X (for new 6G bands depending on results of coexistence studies) |
| OTA Operating Band Unwanted Emissions |  | X (depending on SU and channel BW) |  |
| OTA Transmitter spurious emission |  |  |  |
| OTA General requirement | X (regulatory requirement) |  |  |
| OTA Protection of BS receiver of own or different BS | X |  |  |
| OTA Additional spurious (coex) | X (for existing bands considering legacy deployment) |  | X (for new 6G bands depending on results of coexistence studies) |
| OTA Co-location |  |  | X (for new 6G bands, for existing bands depending on the Rel-20 co-location requirements enhancement) |
| OTA transmitter intermodulation |  |  | X (possible enhancement on test efforts) |
| OTA spatial emission |  | X (mandatory for n104, consideration if required for other new bands) |  |
| OTA sensitivity |  | X (depending on SU and channel BW) |  |
| OTA receiver sensitivity level |  | X (depending on SU and channel BW) |  |
| OTA dynamic range |  | X (depending on SU and channel BW) |  |
| OTA in-band selectivity and blocking |  |  |  |
| OTA ACS | X (for existing bands considering legacy deployment) |  | X (for new 6G bands depending on results of coexistence studies) |
| OTA In-band blocking | X (for existing bands considering legacy deployment) |  | X (for new 6G bands depending on results of coexistence studies) |
| OTA out of band blocking | X (for existing bands considering legacy deployment) |  | X (for new 6G bands) |
| OTA receiver spurious emissions | X |  |  |
| OTA receiver intermodulation |  | X (pending what is the internal guard-band to make sure IM product falls to relevant wanted signal RB) |  |
| OTA in channel selectivity |  | X (depending on SU and channel BW) |  |

It is proposed to take into account above analysis for further work related to 6GR BS requirements.

2.2	BS type 1-H enhancement

The BS type 1-H enhancement should be studied requirement by requirement and if benefits seen to e.g. ease the testing burden, necessary enhancements can be considered.

It is proposed to further study 1-H enhancement and consider it when benefits are justified.

2.	3		cmWave receiver requirements

Number of BS Rx requirements for 7-15 GHz should be derived on the basis of co-existence studies and are further discussed in document [2]. We expect both conducted and radiated Rx requirements should be defined for this frequency range.

Number of BS Rx requirements for 7-15 GHz should be derived on the basis of co-existence studies. Both conducted and radiated requirements should be defined for this frequency range.

2.	4		MSR

During the last RAN4 meeting, it was agreed to consider only new capability set(s) with 4G, 5G and 6G for MSR as a starting point. As there are some frequency bands defined only for NR (but not LTE), it is proposed to define two new capability sets as shown below:

| Capability Set supported by the BS | CS20 | CS21 |
| --- | --- | --- |
| Supported RATs | NR3, 6GR | E-UTRA2, NR3, 6GR |
| Supported configurations | SR NR3 (SC, MC, CA) SR 6GR (SC, MC, CA) MR NR3 + 6GR | SR E-UTRA2 (SC, MC, CA) SR NR3 (SC, MC, CA) SR 6GR (SC, MC, CA) MR E-UTRA2 + 6GR MR NR3 + 6GR MR E-UTRA2 + NR3 MR E-UTRA2 + NR3 + 6GR |
| Applicable BC | BC1, BC2 or BC3 | BC1, BC2 or BC3 |
| NOTE 1: MC denotes multi-carrier in single RAT; SC denotes single carrier; MR denotes multi-RAT; SR denotes single-RAT. NOTE 2: Includes optional (declared by the manufacturer) support of NB-IoT in-band and/or NB-IoT guard band operation within E-UTRA carrier(s) NOTE 3: Includes optional (declared by the manufacturer) support of NB-IoT operation in NR in-band within NR carrier(s). |  |  |

It is proposed to take into account proposed new capability sets above for further work on MSR requirements.

2.	5		SBFD

RAN4 did a significant work on studying the feasibility of SBFD and on defining requirements for SBFD-capable BS in 5G during the study item phase in Rel-18, the normative work in Rel-19 and in the ongoing work item in Rel-20. Therefore, our view is that the work should not start from scratch in 6GR, instead it should be heavily based on the conclusions from the study item and on the requirements defined in Rel-19, taking also in consideration the requirements to be defined for SBFD-SBFD coexistence in Rel-20.

The requirements defined by RAN4 for the 5G SBFD-capable BS should be used as baseline for the 6GR study whenever possible.

# 4	Conclusion

In this contribution we present our view on BS RF aspects for 6GR. The following proposals have been made:

Proposal 1: It is proposed to take into account above analysis for further work related to 6GR BS requirements.

Proposal 2: It is proposed to further study 1-H enhancement and consider it when benefits are justified.

Proposal 3: Number of BS Rx requirements for 7-15 GHz should be derived on the basis of co-existence studies. Both conducted and radiated requirements should be defined for this frequency range.

Proposal 4: It is proposed to take into account proposed new capability sets above for further work on MSR requirements.

# References

[1] R4-2514638; WF on 6G BS RF and co-existence; Ericsson

[2] R4-2520281; Coexistence studies for 6G Radio; Nokia