# 3GPP TSG-RAN WG4 Meeting #117	R4-2520295

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	7.6.4.1

Source: 	Sharp

Title: 	UE RF requirements to support HD-FDD for Ku/Ka bands with FR1 numerologies

Document for:	Discussion

Introduction

For a new Rel-20 work item “Enhanced requirements for NR NTN and IoT NTN Phase 2” in RP-252962, the following way forward was approved at RAN4#116-Bis in R4-2515082:

- Sub-topic 1-1: Issues for HD-FDD requirements

- Issue 1-1-1: Antenna type assumption for HD-FDD VSAT

- WF

  - The working assumption for Phase 1 is that the HD-FDD VSAT antenna type is a single physical aperture with Tx/Rx switching capability.

  - Other implementations are not precluded.

- Issue 1-1-2: RF requirements for fixed VSAT for HD-FDD

- WF

  - For FR1-NTN Ku band n248, n247, the FDD requirements for Type 2 VSAT and Type 3 VSAT are based on the Type 5 and Type 6 requirements regarding maximum output power, OTA reference sensitivity level, and maximum input level. The same principle should be applied to HD-FDD.

  - Further check the regulation related requirements related to NS_203N~NS_208N for the fixed VSAT operating in n248 and n247.

- Issue 1-1-3: Transmit ON/OFF time mask for HD-FDD

- WF

  - Consider reusing existing Transmit ON/OFF time mask requirement as starting point.

- Issue 1-1-4: Treatment on HD-FDD specifications.

- WF

  - Start with FDD requirements as the baseline and further check if any RF requirements need to be modified for HD-FDD.

  - Update the SAN RF requirements if necessary.

- Issue 1-1-5: EIRP requirements for HD-FDD

- WF

  - Consider the existing FDD VSAT EIRP requirements for HD-FDD as the starting point

  - FFS on whether to modify.

- Issue 1-1-6: EIS requirements for HD-FDD

- WF

  - Consider the existing FDD VSAT EIS requirements for HD-FDD as the starting point

  - FFS on whether to modify.

- Sub-topic 1-2: Issues for VSAT with small antennas (e.g., 20x20cm)

- Issue 1-2-1: New VSAT type

- WF

  - Study potential new VSAT type(s) for small antenna with the following candidates:

    - VSAT that support LEO only

    - VSAT that support both GSO and LEO

- Issue 1-2-2: Assumptions for VSAT with small antenna

- WF

  - Conduct implementation and feasibility studies for the small antenna considering, but not limited to, the following aspects:

    - Antenna size, number of antenna elements, antenna gain, Tx power per element and link budget evaluation for supporting LEO only or supporting both GSO and LEO.

- Issue 1-2-3: ACLR and ACS for the new VSAT with small antenna

- WF

  - Consider the existing Ku-band ACLR and ACS as the starting point

  - FFS on whether to modify.

    - Encourage companies to provide justifications for whether to re-use the existing specifications.

- Issue 1-2-4: Specifications impact for HD-FDD VSAT with small antenna

- WF

  - FFS in the next meeting.

This contribution provides Sharp’s views on the issues above.

Discussions

Issues for HD-FDD requirements

Transmit ON/OFF time mask for HD-FDD

For HD-FDD operation, a switching device will be introduced to use a single aperture for both Tx and Rx. According to the current specification, the transmit ON/OFF time mask is defined as follows (R4-2513711). The question here is whether this time mask is sufficient to support switching operations of a HD-FDD VSAT terminal

| Sub-clause | Sub-clause title | Conducted/Radiated | FR1/FR2 | OFF level | Transient time |
| --- | --- | --- | --- | --- | --- |
| 6.3.2/3 | Transmit ON/OFF time mask | Conducted | FR1 | -50dBm | 10us 1 |
| 9.3.2/3 | Transmit ON/OFF time mask | Radiated | FR1 | -36dBm | 10us 2 |
| 9.3.2/3 | Transmit ON/OFF time mask | Radiated | FR2 | -36dBm | 5us |

It is noted that RAN1 defines Rx-Tx and Tx-Rx transition time as follows (R4-2513711), which is longer than RAN4 ON/OFF time mask requirements.

| Transition time | FR1 | FR2 | Unit |
| --- | --- | --- | --- |
| From Tx to Rx | 13.02 | 7.01 | us |
| From TX to TX | 13.02 | 7.01 | us |

In our view, 10us for FR1-NTN and 5us for FR2-NTN are sufficient to support switching operations of a HD-FDD VSAT terminal

Proposal 1: Reuse existing Transmit ON/OFF time mask requirement for HD-FDD

EIRP requirements for HD-FDD

In our view, the purpose of the introduction of HD-FDD is to commonize the Rx and Tx aperture required for FD-FDD, which means the assumption of FD-FDD is dual apertures. Hence, the gain obtained by elimination of duplexer (i.e. insertion loss) cannot be expected for Ku-band, as dual aperture architecture is not mandated for Rel-19 FD-FDD. Meanwhile, additional insertion loss by a switching device may be considered. In our view, additional insertion loss of 1 dB (which is 0.2~1 dB according to the online discussion at RAN4#116-bis) allows more flexibility for the VSAT implementation, but keeping the current EIRP requirement for FD-FDD is also acceptable. Therefore, the following proposal can be made.

Proposal 2: For min peak EIRP requirement on HD-FDD without small antenna, discuss the following options:

- Option 1: Reuse the existing min peak EIRP requirements for FD-FDD

- Option 2: Relax existing min peak EIRP requirements with 1 dB considering the additional insertion loss by switching device

Proposal 3: No change for EIRP max for HD-FDD without small antenna

EIS requirements for HD-FDD

Similar discussion with EIRP is applicable to EIS, and hence the following proposal can be made.

Proposal 4: Proposal 2: For EIS requirement on HD-FDD without small antenna, discuss the following options:

- Option 1: Reuse the existing EIS requirements

- Option 2: Relax existing EIS requirements with 1 dB considering the additional insertion loss by switching device

Proposal 5: No change for maximum input level for HD-FDD without small antenna

Issues for VSAT with small antennas

New VSAT type

It was concluded in RAN4#116bis that RAN4 will further discuss potential new VSAT type(s) for small antenna with the following candidates, (1) VSAT that support LEO only, or (2) VSAT that support both GSO and LEO. In our understanding, the 20 x 20cm requirement comes from the automotive industry, which enable the installation of VSAT on the rooftop of a car. Therefore, it is important for RAN4 to address this requirement.

Proposal 6: At least new VSAT type, i.e. type 7, is specified for LEO assuming 20x20 cm antenna size to address the requirement from automotive industry.

Assumptions for VSAT with small antenna

For 20x20 cm antenna, our initial assessment is provided by the table below (reused from the table provided in R4-2514126). The analysis for VSAT type 7 is performed based on VSAT type 6 to reuse its hardware design as much as possible, and the potential loss from VSAT type 6 is considered.

| antenna elements | antenna size assumption | antenna gain (with assumption of 3.5 dBi element gain) | max EIRP (with assumption of 10W Tx power) | Min peak EIRP in TS 38.101-5 |
| --- | --- | --- | --- | --- |
| 45x45 VSAT type 6 | 60cm equivalent antenna aperture | 36.6 dBi | 76.6 dBm | 60 dBm (band n509, n508, n248 and n 247) |
| 15x15 VSAT type 7 | ~20x20 cm | 27 dBi | 67 dBm | 50 dBm |

Comparing with 45x45 elements, antenna gain of 9.54 dBi is lost due to a reduced number of antenna elements. Further improvement for max EIRP and min peak EIRP can be considered if more element gain (more than 3.5 dBi) and/or Tx power (more than 10W) is used. However, it is not our preference to increase the per element gain or Tx power because the commonization of hardware design between VSAT type 6 and 7 would be lost. With this analysis, the min peak EIRP of 50 dBm would be appropriate for 20x20 cm antenna. The max EIPR would be 67 dBm based on the same logic above, but further discussion is necessary considering the coexistence aspects discussed in section 2.2.3.

For the EIS requirement, comparing with the G/T of VSAT type 6, the array gain is simply lost (i.e. -9.54 dBi) and further improvement (e.g. by better noise figure) is not easy. Therefore, our preference is to relax EISREFSENS_50MHz with 9.54 dBm compared with that for VSAT type 6.

Proposal 7: For the new VSAT type 7 for HD-FDD with 20x20 cm antenna, the following UE RF requirements are considered

- Max EIRP: 67 dBm or less, depending on the results on coexistence analysis

- Min peak EIRP: 50 dBm

- EISREFSENS_50MHz: <= -112 dBm (i.e. 9.54 dB relaxation compared with VSAT type 6)

ACLR and ACS for the new VSAT with small antenna

As discussed in RAN1#116bis, it is preferrable to avoid the coexistence study because it consumes many TUs. The issue here is that small antenna will cause additional interference due to the different antenna pattern as compared to Rel-19 VSAT type-6. More concretely, the major concern is wider main lobe.

Our analysis for EIRP pattern considering the antenna pattern is shown in Figure 1. In this figure, EIRP patterns for 45x45 elements (orange line) and 15x15 elements (blue line) are shown, where 40 dBm total Tx power and 3.5 dBi element gain are assumed. As shown in this figure, EIRP of 15x15-element antenna between -5.5 to -2 and 2 to 5.5 degree becomes higher than that for 45x45-element antenna. Thus, simulation-based study is needed to analyze the actual negative impact of increased EIRP for the angle range. Meanwhile, the impact of the wider main lobe may be relieved if the EIRP itself is reduced for VSAT type 7 (i.e. lower element gain and total Tx power), which requires further discussion in RAN4.  For example, the green line in the figure shows the EIRP pattern where the peak EIRP is reduced to 63.5 dBm, which is the same level as the 1st side robe of the 45x45-element antenna. With this change, the EIRP pattern for small antenna is mostly lower than that for 45x45-element antenna, and additional simulation is not necessary for small antenna and the existing ACLR and ACS requirement for VSAT type 6 can be reused.

Figure 1. ERIP pattern

Observation 1: The necessity of coexistence study for small antenna depends on the number of elements, total Tx power and element antenna gain. Additional simulation campaign may be avoided if reduced max EIRP (FFS the value) is targeted for small antenna, and the ACLR and ACS defined for VSAT type 6 can be reused

Conclusion

Proposal 1: Reuse existing Transmit ON/OFF time mask requirement for HD-FDD

Proposal 2: For min peak EIRP requirement on HD-FDD without small antenna, discuss the following options:

- Option 1: Reuse the existing min peak EIRP requirements for FD-FDD

- Option 2: Relax existing min peak EIRP requirements with 1 dB considering the additional insertion loss by switching device

Proposal 3: No change for EIRP max for HD-FDD without small antenna

Proposal 4: Proposal 2: For EIS requirement on HD-FDD without small antenna, discuss the following options:

- Option 1: Reuse the existing EIS requirements

- Option 2: Relax existing EIS requirements with 1 dB considering the additional insertion loss by switching device

Proposal 5: No change for maximum input level for HD-FDD without small antenna

Proposal 6: At least new VSAT type, i.e. type 7, is specified for LEO assuming 20x20 cm antenna size to address the requirement from automotive industry.

Proposal 7: For the new VSAT type 7 for HD-FDD with 20x20 cm antenna, the following UE RF requirements are considered

- Max EIRP: 67 dBm or less, depending on the results on coexistence analysis

- Min peak EIRP: 50 dBm

- EISREFSENS_50MHz: <= -112 dBm (i.e. 9.54 dB relaxation compared with VSAT type 6)

Observation 1: The necessity of coexistence study for small antenna depends on the number of elements, total Tx power and element antenna gain. Additional simulation campaign may be avoided if reduced max EIRP (FFS the value) is targeted for small antenna, and the ACLR and ACS defined for VSAT type 6 can be reused