3GPP TSG-RAN WG4 Meeting #117	R4-2520305

Dallas, USA, Nov. 17-21, 2025

Source: 	KDDI Corporation

Title: 	Discussion on 6G general UE RF requirement

Agenda Item:	8.3

Document for:	Discussion

- 1. Introduction

In RAN4#116-bis, RAN4 began the discussion on 6G general UE RF requirement and agreed the WF on single carrier output power requirements as follows[1].

Topic #3: Single carrier output power requirements - Agreements - Study 6G output power requirements to enable more flexibility to different UE implementations to reflect the RF front-end capability for single band - Encourage inputs on how to allow UE to deliver more output power and how power class framework includes e.g. Tx diversity and UL-MIMO - Study improvements on configured maximum output power requirements and associated parameters - Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements - Study the default power class for FDD and TDD bands for 6G - Study how to handle SAR/MPE for 6GR

In this contribution, we focus on single carrier output power requirements and propose some issues.

- 2. Discussion

  - Default power class

Default power class for 6G FDD band

In the last meeting, RAN4 discussed the issue on default power class for 6G as follows[2].

Issue 3-1-3: Default power class Multiple companies made comments to increase to default power class to 26 dBm, especially for TDD. For FDD there were both suggestions to use 26 dBm as well as do further study. It was also recognized that some regions have regulatory restrictions on output power. Proposals - Proposal 1: Study the default power class for FDD and TDD bands for 6G - Consider [26] dBm for TDD bands - Consider [23, 26 dBm] for FDD bands - Take HPUE progress in 5G NR into account - Recommended WF - TBA

Japanese regulation on HPUE operation for FDD bands

In Japan, currently HPUE operation for any FDD bands is not allowed, and also this domestic regulatory situation will be same in 2030 era due to co-existence requirements with other domestic services. RAN4 should take into account that there will be countries not being allowed to operate HPUE for any FDD bands.

Proposal 1: Take into account that there will be countries not being allowed to operate HPUE for any FDD bands.

Proposal 2: Default power class for FDD band is 23dBm.

- 3. Conclusion

Proposal 1: Take into account that there will be countries not being allowed to operate HPUE for any FDD bands.

Proposal 2: Default power class for FDD band is 23dBm.

- 4. Reference

[1] R4-2514646, WF for [116bis][102] 6G general RF and UE RF, Qualcomm, RAN4#116bis