3GPP TSG-RAN WG4 Meeting #117	R4-2520311

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	7.9.3

Source:	CAICT

Title:	Discussion on 	Inter-vendor training collaboration and interoperability for two-sided AI/ML models

Document for:	Discussion

1	Introduction

In last meeting, moderator provided a comprehensive list of topics for discussion. In this contribution, we provide our views on selective topics.

# Recommendations for RAN4 #117 The following are not agreements, but are recommendations for contributions and discussions at RAN4#117: - Proposals for assumptions for the dataset including how to cover scalability, channel models, number of samples - Proposals for how to evaluate the dataset including potential metrics to compare different dataset contributions - Proposals on model hyperparameters, considering in particular complexity, considering also quantization size - Proposals and reasoning for scalability solutions - Proposals and reasoning for training parameters - Proposals for performance metrics for evaluating the different potential model structures - Proposals for how to capture the model and the dataset in 3GPP (format, how to store etc.)

2	Discussion

Topic 1: assumptions for the dataset including how to cover scalability, channel models, number of samples

In the previous RAN4 meeting, it was agreed that unless significant issues are identified, RAN4 would focus only on discussing scalability issues and options that had already been agreed upon in RAN1. In RAN1, the scalability of CSI compression (Case 0) was primarily evaluated across the following dimensions: the number of Tx ports, payload sizes, and bandwidth settings—including variations in both the number of sub-bands and sub-band sizes. Based on our understanding, since the dataset includes only the target CSI (i.e., the input to the encoder), the payload size does not affect the fundamental assumptions of the dataset.

The dataset construction can be structured along three key dimensions: the number of Tx ports, the configuration of {number of sub-bands, sub-band size}, and the channel models. RAN4 could specify a minimal set of candidate values for each dimension to cover typical test configurations. The global dataset would then be built by considering all combinations of these dimensions. For instance, if M candidate values are defined for Tx ports, N for {number of sub-bands, sub-band size}, and P for channel models, the total number of dataset combinations would be M × N × P.

On size of each dataset, the method used to construct mixed dataset in the study item could be taken for reference, at least 10K samples per contributing company to ensure balanced representation and the minimum total size should exceed a threshold, e.g., 100K samples.

Proposal 1: As starting point, the dataset construction can be structured along three key dimensions: the number of Tx ports, the configuration of {number of sub-bands, sub-band size}, and the channel models. RAN4 could specify a minimal set of candidate values for each dimension to cover typical test configurations. The global dataset would then be built by considering all combinations of these dimensions.

Topic 2: how to evaluate the dataset including potential metrics to compare different dataset contributions

Based on the experience of R19 evaluation, the discrepancy of dataset distribution would highly affect the performance of the AI/ML models. Thus, RAN4 could use the R19 CNN-based CSI encoder/decoder or Transformer-based CSI encoder/decoder defined in RAN1 as a benchmark to test performance on each company’s dataset. If the performance is consistent across datasets, they can be deemed as comparable.

Proposal 2: Considering to use the R19 CNN-based CSI encoder/decoder or Transformer-based CSI encoder/decoder defined in RAN1 as a benchmark to test performance on each company’s dataset.

Topic 3: how to capture the model and the dataset in 3GPP (format, how to store etc.)

Regarding the model, the discussion can be divided into model structure and model weight. The model structure can be documented in the specification using text descriptions and diagrams/ flowcharts, along with essential hyperparameters. The model weights can be stored on the 3GPP FTP server and referenced in the specification via URLs. The weight files should be stored in mainstream formats that are widely supported and can be easily loaded by common machine learning tools.

Regarding the dataset, we suggest that the specification should include a descriptive paragraph outlining its key features. The actual dataset files can be hosted in a designated directory on the 3GPP FTP server and referenced via URLs in the specification.

Proposal 3: The model structure can be documented in the specification using text descriptions and diagrams/ flowcharts, along with essential hyperparameters.

Proposal 4: The model weights can be stored on the 3GPP FTP server and referenced in the specification via URLs. The weight files should be stored in mainstream formats that are widely supported and can be easily loaded by common machine learning tools.

Proposal 5: The specification should include a descriptive paragraph outlining key features of the dataset. The actual dataset files can be hosted in a designated directory on the 3GPP FTP server and referenced via URLs in the specification.

Conclusion

In this contribution, inter-vendor training collaboration and interoperability for two-sided AI/ML models are discussed with following proposals.

Proposal 1: As starting point, the dataset construction can be structured along three key dimensions: the number of Tx ports, the configuration of {number of sub-bands, sub-band size}, and the channel models. RAN4 could specify a minimal set of candidate values for each dimension to cover typical test configurations. The global dataset would then be built by considering all combinations of these dimensions.

Proposal 2: Considering to use the R19 CNN-based CSI encoder/decoder or Transformer-based CSI encoder/decoder defined in RAN1 as a benchmark to test performance on each company’s dataset.

Proposal 3: The model structure can be documented in the specification using text descriptions and diagrams/ flowcharts, along with essential hyperparameters.

Proposal 4: The model weights can be stored on the 3GPP FTP server and referenced in the specification via URLs. The weight files should be stored in mainstream formats that are widely supported and can be easily loaded by common machine learning tools.

Proposal 5: The specification should include a descriptive paragraph outlining key features of the dataset. The actual dataset files can be hosted in a designated directory on the 3GPP FTP server and referenced via URLs in the specification.