3GPP TSG-RAN WG4 Meeting #117											R4-2520316

Dallas, US, Nov. 17 – 21, 2025

Agenda item:	5.11.2

Source: 	Fujitsu

Title: 	Discussion on system parameters for US 4.9GHz new band

Document for:	Approval

# 1. Introduction

At the RAN#108 meeting, a spectrum-related work item [1] was approved to specify a new NR TDD 4.9GHz band for US operation. In this paper, we would like to provide our views on system parameters for US 4.9GHz new band.

# 2. Discussion

Firstly, the band numbering for this new band is discussed. The latest TS 36.101 and 38.101-1 states band 113 has already been used as operating band for LTE based 5G terrestrial broadcast. Considering it, the next number, band n114, should be used for the new NR TDD 4.9GHz band.

Proposal 1: RAN4 to use n114 as the band number for new NR TDD 4.9GHz band.

Regarding the other system parameters, the contributions [2-6] were brought at the last meeting for new NR TDD 4.9GHz band. The candidate options were summarized in the moderator summary [7]. Regarding the Operating band table below, companies’ proposals were the same, and it can be used.

Proposal 2: RAN4 to use the Table 1 below for new NR TDD 4.9GHz band.

Table 1 NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| nXXX | 4940 MHz – 4990 MHz | 4940 MHz – 4990 MHz | TDD |

For the channel bandwidth table, the difference among companies is only whether the note for PC1 is added or not. Regarding the HPUE for new NR TDD 4.9GHz band, the conclusion in the last RAN plenary meeting is that HPUE is not included in this WI. Considering it, we think that the note for PC1 should not be added to the channel bandwidth table so far. Our proposal is to use the following channel bandwidth table.

Proposal 3: RAN4 to use the Table 2 below for new NR TDD 4.9GHz band.

Table 2  Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| nXXX | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |

Regarding the channel raster, two patterns regarding the range of NREF as follows were proposed at the last RAN4 meeting [7].

- Option 1:

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nXXX | 15 | 729334 – <1> – 732666 | 729334 – <1> – 732666 |
|  | 30 | 729334 – <2> – 732666 | 729334 – <2> – 732666 |

- Option 2:

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nXXX | 15 | 729333 – <1> – 732667 | 729333 – <1> – 732667 |
|  | 30 | 729333 – <2> – 732667 | 729333 – <2> – 732667 |

The RF reference frequencies of Option 1 and Option 2 can be calculated as follows based on TS 38.101-1 Clause 5.4.2.1.

- Option 1: 4940.01 – 4989.99 (MHz)

- Option 2: 4939.995 – 4990.005 (MHz)

Considering the above results, Option 1 should be used because its range is within 4940 – 4990 MHz.

Proposal 4: RAN4 to use the Table 3 below for new NR TDD 4.9GHz band.

Table 3  Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nXXX | 15 | 729334 – <1> – 732666 | 729334 – <1> – 732666 |
|  | 30 | 729334 – <2> – 732666 | 729334 – <2> – 732666 |

Regarding the synchronization raster, the situation is almost the same with that for channel raster. Two patterns regarding the range of GSCN as follows were proposed at the last RAN4 meeting [7].

- Option 1:

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nXXX | 30 kHz | Case C | 8850 – <1> – 8877 |

- Option 2:

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nXXX | 30 kHz | Case C | 8848 – <1> – 8880 |

In Jan. 2018 meeting, the following guideline for synchronization raster points calculation was agreed [8].

- Include GSCN that correspond to SS block that completely fit within the channel bandwidth accounting for guard bands

- Guard bands are calculated based on the minimum channel BW, the SCS for the SS block and the corresponding NRB (spectrum utilization), assuming the RF reference frequency at the center of a block

When we calculated the range of GSCN considering this guideline and the current specs for synchronization raster, the values are the same of Option 1. We propose Option 1 should be used.

Proposal 5: RAN4 to use the Table 4 below for new NR TDD 4.9GHz band.

Table 4  Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nXXX | 30 kHz | Case C | 8850 – <1> – 8877 |

# 3. Conclusions

In this contribution, we provided our view on system parameters for US 4.9GHz new band as follows.

Proposal 1: RAN4 to use n114 as the band number for new NR TDD 4.9GHz band.

Proposal 2: RAN4 to use the Table 1 below for new NR TDD 4.9GHz band.

Table 1  NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| nXXX | 4940 MHz – 4990 MHz | 4940 MHz – 4990 MHz | TDD |

Proposal 3: RAN4 to use the Table 2 below for new NR TDD 4.9GHz band.

Table 2  Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| nXXX | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |

Proposal 4: RAN4 to use the Table 3 below for new NR TDD 4.9GHz band.

Table 3  Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nXXX | 15 | 729334 – <1> – 732666 | 729334 – <1> – 732666 |
|  | 30 | 729334 – <2> – 732666 | 729334 – <2> – 732666 |

Proposal 5: RAN4 to use the Table 4 below for new NR TDD 4.9GHz band.

Table 4  Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nXXX | 30 kHz | Case C | 8850 – <1> – 8877 |

# 4. Reference

- RP-251887, New WID on Introduction of NR TDD 4.9GHz Band for US Operation, Fujitsu

- R4-2513181, System parameters for 4.9GHz new band, CATT

- R4-2513738, Discussion on system parameters for US 4.9GHz new band, Fujitsu Limited

- R4-2513835, New 4.9GHz US band - System parameters, Ericsson

- R4-2514046, Discussion on System parameters for US 4.9 GHz band, Nokia

- R4-2514280, Introduction of US 4.9GHz band: system parameters, Huawei, HiSilicon

- R4-2514445, Topic summary for [116bis][322] NR_TDD_4.9GHz_band, Moderator (Fujitsu)

- R4-1801316, WF on Sync Raster Calculations, Ericsson