# 3GPP TSG-RAN WG4 Meeting #117                                                                  R4-2520318

Dallas, USA, Nov. 17-21, 2025

Source:                Tejas Networks

Title:                     SBFD BS to SBFD BS adjacent channel coexistence

Agenda Item:       7.2.4

Document for:     Discussion

# Introduction

At the RAN4#116bis [1] meeting, Phase 2 of the NR base station RF requirements evolution was initiated. This contribution presents an initial discussion of the SBFD BS to SBFD BS adjacent channel coexistence.

SBFD BS to SBFD BS adjacent channel coexistence Agreements: - For next meeting further consider following options, with the intention to down-select: Option 1: UD (Operator 1) and DU (Operator 2) Option 2: DUD (Operator 1 and 2) Option 3: UD and DUD Provide more description on each specific case. - As baseline use simulation parameters defined in TR 38.858. For 6 GHz, new parameters for BS antenna should be established for SBFD.

# Discussion

## SBFD BS to SBFD BS adjacent channel coexistence

Release 20 introduces enhanced support for SBFD-only deployments in FR1 wide-area bands including n104, n79, and n41. These enhancements are designed to enable independent operation from legacy TDD systems. SBFD architecture separates DL and UL sub-bands, improving UL coverage and reducing latency. However, this separation introduces coexistence challenges, particularly due to high-power downlink transmissions occurring adjacent to sensitive uplink receivers. SBFD-to-SBFD configurations demonstrate superior coexistence performance compared to mixed TDD-SBFD scenarios, provided that ACIR remains within acceptable limits. Strengthened base station transmitter and receiver requirements are critical, especially regarding unwanted emissions and in-band blocking. Strategic placement of downlink sub-bands contributes to interference mitigation. Three representative sub-band configurations are proposed to evaluate feasibility. These leverage SBFD’s split-band structure to create implicit guard bands, reducing the need for additional filtering. The evaluation focuses on interference paths, filtering requirements, and ACIR targets. Each configuration reflects realistic deployment scenarios and emphasizes downlink-to-uplink leakage, the primary coexistence concern.

Configuration 1: UD (Operator 1) and DU (Operator 2): Operator 1 allocates uplink followed by downlink, while Operator 2 uses the reverse order. This results in the aggressor’s downlink being directly adjacent to the victim's downlink. The victim’s downlink sub-band serves as a natural guard, typically providing around 50 MHz of separation. This configuration simplifies transmitter filtering requirements and represents a baseline case for coexistence analysis. Slot allocation is asymmetric, with Operator 1 favouring uplink-heavy traffic and Operator 2 prioritizing downlink-heavy scheduling.

Configuration 2: DUD (Operator 1 and Operator 2): Both operators implement a three-sub-band structure: downlink at the lower edge, uplink in the center, and downlink at the upper edge. This creates dual interference paths, with outer downlink emissions potentially affecting the neighbouring uplink, and vice versa. The uplink sub-band offers significant guard separation, imposing stricter requirements on transmitter emissions and receiver blocking. This configuration is suitable for wide allocations such as 200 MHz and reflects advanced deployment strategies, though it increases filter complexity.

Configuration 3: UD (Operator 1) and DUD (Operator 2): This hybrid scenario combines asymmetric operator strategies. Operator 1 uses a two-sub-band UD structure, while Operator 2 adopts the three-sub-band DUD arrangement. The aggressor’s downlink may align with either the victim’s outer downlink or uplink, resulting in variable guard distances. Multiple interference paths emerge, with the most significant being downlink-to-uplink leakage. This configuration captures deployment diversity and is well-suited for sensitivity analysis under asymmetric assumptions. Hence, three SBFD sub-band configurations are considered for coexistence analysis. Option 1 (UD/DU) uses two sub-bands per operator, where one operator allocates uplink followed by downlink and the other does the reverse. This creates a simple edge alignment with the victim’s downlink acting as a buffer, making it the baseline and reference case for analysis. Option 2 (DUD/DUD) applies a symmetric three-sub-band structure for both operators, placing uplink in the center and downlink on both edges. This introduces dual interference paths and relies on the center uplink as a guard, serving as a test case for symmetric setups. Option 3 (UD/DUD) combines two and three sub-band structures, resulting in variable guard distances and multiple interference paths. This configuration can be used for sensitivity checks to understand coexistence under operator mismatches.

Observation 1: The coexistence issue comes from downlink signals interfering with uplink reception when they are placed next to each other.

Observation 2: SBFD’s split-band design helps reduce interference by creating natural guard bands, especially when both operators follow similar patterns.

Observation 3: DUD configurations introduce tighter guard spacing, requiring more stringent transmitter emission control and receiver blocking performance.

Proposal 1: Begin with the UD/DU configuration as the baseline, then introduce DUD/DUD to assess performance when both operators adopt identical SBFD structures.

Proposal 2: Consider the UD/DUD configuration to study how coexistence behaves when operators use different SBFD setups.

Proposal 3: Define updated base station transmitter and receiver requirements, including emission masks and explicit in-band blocking thresholds.

2.2	Grid Shift requirement

In the TDD-SBFD study, a 100% GS was assumed for simulation analysis. However, given the relaxed ACIR requirement for SBFD coexistence, a 10% GS can be considered as a practical starting point for further evaluations.

Proposal 4: Adopt a 10% GS as the baseline assumption for SBFD coexistence studies.

# Conclusion

Observation 1: The coexistence issue comes from downlink signals interfering with uplink reception when they are placed next to each other.

Observation 2: SBFD’s split-band design helps reduce interference by creating natural guard bands, especially when both operators follow similar patterns.

Observation 3: DUD configurations introduce tighter guard spacing, requiring more stringent transmitter emission control and receiver blocking performance.

Proposal 1: Begin with the UD/DU configuration as the baseline, then introduce DUD/DUD to assess performance when both operators adopt identical SBFD structures.

Proposal 2: Consider the UD/DUD configuration to study how coexistence behaves when operators use different SBFD setups.

Proposal 3: Define updated base station transmitter and receiver requirements, including emission masks and explicit in-band blocking thresholds.

Proposal 4: Adopt a 10% GS as the baseline assumption for SBFD coexistence studies.

# Reference

- R4-2515091, Way-Forward for NR_BS_RF_Ph2, RAN4#116bis, Ericsson