3GPP TSG-RAN WG4 Meeting #117                                           R4-2520320

Dallas, USA, Nov 17 – 21, 2025

Source: 	Huawei, HiSilicon

Title: 	On system parameters for 6G —— Waveform

Agenda Item:	8.2.1

Document for:	Approval

Introduction

6G system parameters have been discussed extensively in last RAN4 meeting [1], and WF was agreed in [2].

This contribution provides our further consideration on system parameters for 6G on waveform, including PA model.

Discussion

Waveform

Evaluation cases

RAN4 agreement on waveform is excerpted as below.

RAN4 Agreement - The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1 - To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects - Tx assumption including PA model - Related RF requirements which should be taken into consideration - Both UL and DL are considered - Take the candidate waveforms identified/discussed in RAN1 as the baseline - Identify the evaluation metric, e.g. net gain - Implementation constraints - Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs - Evaluation cases could be selected step-by-step, based on RAN1 progress - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement

As confirmed by RAN4, the primary objective of the RAN4 study is to evaluate candidate waveforms and potential low PAPR techniques. To our knowledge, while CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink, two additional candidate waveforms are currently under discussion in RAN1: namely, DFT-s-OFDM for downlink and DFT-s-OFDM for uplink with number of layers > 1. No conclusions have yet been reached in RAN1 regarding these two DFT-s-OFDM enhancements. However, RAN1 has at least agreed to study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1.

In NR, the lack of UL MIMO support for DFT-s-OFDM was not a technical limitation. Therefore, if it is concluded that UL MIMO can be supported for DFT-s-OFDM, the remaining task would be how to specify the corresponding requirements in RAN4 during the WI phase. We do not see a need for RAN4 to conduct further evaluation on this matter in parallel with RAN1, unless explicitly requested by RAN1.

Proposal 1: No parallel RAN4 study is foreseen for evaluation on DFT-s-OFDM for UL with number of layers > 1. Any necessary requirements for this waveform would be handled during the WI phase, if needed.

Regarding the potential support of DFT-s-OFDM for downlink, the evaluation would primarily involve considerations from the BS side, specifically the possibility of increased output power. Historically, there has been no established BS PA model for such evaluation, as BS maximum output power is typically subject to manufacturer declaration. Therefore, we think it is impractical for RAN4 to define a PA model for this purpose. While DFT-s-OFDM's single-carrier property indeed offers lower PAPR than CP-OFDM, potentially enhancing maximum output power, as evidenced in UL transmissions, the determination of applicable DL usage scenarios falls within RAN1's study scope.

Observation 1: DFT-s-OFDM with lower PAPR compared to CP-OFDM would deliver higher output power, which has been justified for UL from UE side.

Observation 2: Maximum output power for BS side is up to manufacturer’s declaration and no PA model has been discussed and adopted before for BS evaluation.

Proposal 2: DFT-s-OFDM for DL should be a pure RAN1 evaluation, which should not rely on a RAN4 PA model for DL. No need for RAN4 to have discussion on DL PA model.

For low PAPR techniques, we see several schemes are under discussion in RAN1 now. However, it is not desirable that RAN4 to perform evaluations for all these schemes. More study should be done in RAN1 with a converged scope before sending potential low PAPR techniques to RAN4. Upon the request from RAN1 in LS R1-2508069,

For waveform evaluations, e.g. evaluations of PAPR reduction/MPR, PA models are needed. RAN1 respectfully asks RAN4 to provide appropriate PA model(s) and other relevant RF parameters for both downlink and uplink evaluations, at least for around 7GHz. Any timely feedback is appreciated, before which RAN1 continues waveform evaluations with PA model assumptions that might be based on companies’ inputs.

RAN4's primary focus should be on the PA model and other evaluation assumptions, including pertinent requirements. These requirements are currently under discussion in the parallel UE RF topic and will remain subject to further modification as the study advances.

Proposal 3: RAN4 focus on the PA model and other evaluation assumptions, including applicable requirements, in the absence of inputs from RAN1 pertinent to waveform evaluation.

Another issue that requires emphasis is the timeline for the interim milestone. According to the interim checkpoint set by RAN, the waveform along with several other parameters is expected to be finalized in the second quarter next year. Therefore, consensus on issues related to the waveform, particularly concerning low PAPR techniques to be further evaluated by RAN4, must be reached no later than the April meeting. Otherwise, there will be no sufficient time for RAN4 to conduct a meaningful evaluation.

Proposal 4: Notify RAN1 that it is imperative to stable the minimum set of waveforms for RAN4's further evaluation no later than April meeting.

Evaluation assumptions

As indicated in the RAN1 LS, further waveform evaluation depends on inputs from RAN4 regarding necessary assumptions. In line with the discussion in Section 2.1.1, the assumptions provided by RAN4 should focus on the uplink, particularly for the evaluation of low-PAPR waveforms. The following agreements are cited from the WF for UE RF:

UE RF Agreement - PA modelling is discussed in system parameter thread - In initial 6GR evaluations, use 5G NR unwanted emission requirements as starting point until requirement framework for 6GR is clear - Spurious emissions requirements, SEM and ACLR will be defined for 6GR - FFS for - OOB boundary - potential accommodations for different frequency sub-ranges - Encourage inputs on regulatory survey being conducted in spectrum agenda - Study the UE Tx impairment requirements/assumptions (e.g. carrier leakage, image rejection, CIM3 and CIM5) - In RAN4#117, prioritize discussion for UE Tx impairments and unwanted emission framework

Since transmitter impairments (e.g., carrier leakage, I/Q image, CIM3) and unwanted emissions will be covered under the UE RF agenda, corresponding discussions need not be repeated under system parameters. However, to support RAN1 evaluation, certain necessary assumptions must be provided by RAN4.

The PA model will be further discussed in the next session. In addition, the key requirements to be considered in waveform evaluation primarily include SEM and ACLR. Although Spurious Emissions (SE), In-Band Emissions (IBE), and EVM are also related to MPR study, SEM and ACLR are sufficient to assess the MPR impact using the "Net Gain" metric especially for edge/outer RB allocations.

In the absence of finalized UE RF conclusions and channel bandwidth definitions, the evaluation can be performed based on existing 5G requirements for a 100 MHz channel bandwidth around 7 GHz, assuming a Power Class 2 PA. The suggested assumptions (excluding the PA model) are summarized in the table below:

Table 1: Waveform evaluation assumptions

| Parameter/Requirements |  | Assumptions/Value | Note |
| --- | --- | --- | --- |
| PA model |  | TBD | Memory effect should be considered |
| Band under evaluation |  | around 7GHz | n104 could be assumed |
| Channel Bandwidth (CBW) |  | 100MHz |  |
| Power class |  | PC2 (26dBm) |  |
| Complied requirements | SEM | TS 38.101-1 §6.5.2.2 | Subject to further adjustment pending on progress of UE RF, co-existence study |
|  | ACLR | TS 38.101-1 §6.5.2.4 |  |
|  | EVM | TS 38.101-1 §6.4.2.1 | Considered for high modulation order/inner RB allocation, pending on RAN1 discussion |
|  | IBE | TS 38.101-1 §6.4.2.3 |  |
| Tx impairments | Carrier Leakage | -28dBc | Subject to further adjustment pending on progress of UE RF study |
|  | IQ image | -28dBc |  |
|  | CIM3 | -60dB |  |
| PA calibration conditions | CBW | [20MHz full RB allocation] | Other options are not precluded, pending on the further study in RAN4 |
|  | SCS | 15kHz |  |
|  | Waveform | DFT-s-OFDM |  |
|  | Modulation | QPSK |  |
|  | Power class | PC2 |  |
|  | Power backoff to meet ACLR | 1dB |  |

The above assumptions should be sent to RAN1 for consistent evaluation.

Proposal 5: Existing 5G requirements on 100MHz CBW around 7GHz with a power class 2 PA could be considered as starting point for initial waveform evaluation. Assumptions could be adjusted upon the progress of 6G study across different topics in RAN4.

Evaluation metric

According to discussion by RAN1 in last meeting, the “Net Gain” evaluation criterion was already agreed.

RAN1 Agreement - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding

Generally speaking, the agreed metric is similar to those proposed by several companies during the last RAN4 meeting. It is worth noting that the "net gain" criterion was already adopted in Rel-18 when RAN4 evaluated low-PAPR schemes under the coverage enhancement topic. Therefore, we believe the same metric agreed upon by RAN1 could also be adopted by RAN4, should further evaluation be required.

Proposal 6: Adopt the same the “Net Gain” evaluation criterion as agreed by RAN1 for further low PAPR evaluation when necessary.

PA model

The PA model discussions started last meeting with the following WF:

Agreement (R4-2514641) - Consider PA modelling at least for the following aspects for RAN4 discussion - Memory effects for UE supporting larger CBW - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3) - Calibration conditions - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc. - PA models for different device types, handheld UE is prioritized - Applicable requirements - 5G-A requirements as starting point - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled

As was agreed in the last meeting the PA models in RAN1 waveform evaluations and RAN4 requirements can be decoupled, therefore we need to consider the PA models for RAN1 and RAN4 use cases, separately. In other words, the study of the PA modeling should be continued even after the LS reply to RAN1, if the model is not suitable for RAN4 requirements.

Observation 3: The study of the PA modeling should be continued even after the LS reply to RAN1, if the model is not suitable for RAN4 requirements.

Different frequency ranges support different CBWs. For example, the FDD bands (especially below 2 GHz) supported a max CBW of 50 MHz in 5G and TDD bands could reach 100 MHz of CBW. For 6G SI phase the 100 MHz maximum CBW is being discussed in RAN1 for FDD bands, therefore one can simplify those studies and use 5G PA models for them. Conversely, industry discussions are increasingly focused on expanding the maximum CBW for legacy FR1 bands to 200 MHz. Furthermore, for spectrum allocations around 7 GHz, even broader CBW configurations—specifically 200 MHz, with exploratory evaluations for potential extension to 400 MHz—are now under active consideration. The PAs, under such high CBWs, behave differently and their output are subject to the dynamic out of band emission (due to memory effect) and in-band PSD imbalance. Therefore, new models should be developed which include all or some of these factors.

Observation 4: Memory effect and PSD imbalance are present when PAs are derived with large CBWs (such as 200 MHz).

Proposal 7: Use 5G PA models for the FDD bands below 2 GHz, as they won’t include very large CBWs.

Proposal 8: Consider new PA models for the bands higher than 2 GHz and around 7 GHz where 200 MHz CBW are considered.

As different power classes are considered for 6G SI, it would be beneficial to have different PA models for different PCs. Moreover, for 6G technologies, if a new PC is introduced (especially with higher output powers) then their corresponding PA models should be studied, to ensure proper RAN4 requirements. In case the new models are not introduced and PA models are pushed to operate more than their initial power class, the behavior of the model would not be close to reality and the evaluation results will be either over or under estimated.

Proposals 9: Different PA models to be consider for different PCs and introduce new models when a new PC is introduced in 6G.

PAs used in real applications can use different power management techniques such as Envelope Tracking (ET), Average Power Tracking (APT) which could increase the out of band emissions while increasing the PA efficiency. On the other hand, such power management techniques are very product specific, therefore it is better to not include them in the PA modelling.

Proposal 10: Consider fixed biased PA as baseline for the models. Other advanced power management techniques shall remain within implementation.

In 5G the PA was calibrated with the following configuration as:

- 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

The 1dB MPR was taken into account to ensure a more linear PA. Moreover, DFT-s-OFDM was a legacy of 4G continued in 5G, therefore it was used in calibration procedure. Finally, 20 MHz (100RB) was chosen for the CBW of the calibration procedure. For 6G SI, and probably the WI phase, one can keep the 1 dB back-off as larger CBWs (such as 200 MHz) are to be considered. The DFT-s-OFDM waveform can also be reused, unless there are strong arguments from proponents of CP-OFDM. On the other hand, as in large CBWs PAs behave differently, a calibration configuration with 20 MHz will not include many of those effects. Therefore, the calibration BW should be increased at least for TDD bands where 200 MHz CBW (or maybe 400MHz) are to be used.

Proposal 11: consider the larger CBW for calibration configuration for 6G SI for TDD bands.

A: 1dB MPR: DFT-s-OFDM QPSK 100MHz, 270RB or

B: 1dB MPR: DFT-s-OFDM QPSK 200MHz, [540RB].

As was mentioned previously the memory effect should be considered, especially for large CBWs.in Figure 2.1.4-1 a single PA was measured with 20 MHz, 100 MHz and 200 MHz CBWs using the same input power.

Couple of issues can be observed, first, the ACLR increases significantly in 200 MHz comparing to the other two cases which is a clear indication of large out of band emissions due to large CBW (dynamic memory effect). Moreover, it can be observed that for the ∆ACLR=ACLRl-ACLRhincreases for high Channel BWs {20 MHz (1.7dB), 100 MHz (3.2dB), 200 MHz (3.6dB)}. Additionally, the in-band PSD imbalance is clearly visible in the case of 200 MHz comparing to the other cases.

Figure 2.1.4-1 ACLR evaluations for 20 MHz, 100 MHz and 200 MHz CBW

In [4] the memory effects in PA models were shown and polynomial model coefficients for the case of a 100 MHz CBW were proposed. Figure 2.1.4-2 shows the impact of memory effect on SEM, where memory-based model (which includes dynamic out-of-band emissions) hits the SEM limits.

Figure 2.1.4-2 SEM is impacted by the memory effect.

Proposal 12: Memory effect has to be included in the models as it has impact on ACLR and SEM.

Using the model explained in [4], and reproduced below, a PC2 PA was modelled for 100MHz CBW.

youtputn= m=0Mk=1Kam,2k-1xinputn-mxinputn-m2(k-1)

The PA was calibrated to pass ACLR (31dBm) and SEM requirements of 5G and normalized mean squared error (NMSE) metric was used to evaluate the PA models. Figure 2.1.4-3 depicts the measured PA output signal and the models for different nonlinearity order and memory depth.

Figure 2.1.4-3 PC2 PA output power measurement and models for K= [5, 7, 9] and M= [0,…,6], Pa_in power= -1.2dBm, PA_out power at antenna port= 25.7dBm( 4dB IL )

Several interesting issues can be seen observed from the plots. Firstly, the ACLR is impacted by the memory effect as was explained earlier. The PSD is symmetrical on memoryless models (M=0, dark blue PSDs) while PA output measured data and the memory-based models show clearly higher PSD levels on higher ACLR1 region than the lower ACLR1 region. secondly, the performance of the models for different nonlinearity orders (K=5,7,9) are quite similar however one can observe for higher Ks the models are closer to the PA measured PSD on the frequency region shown in the black box, however such an improvement come at the cost of using more coefficients in the calculations. Thirdly, with the inclusion of the first memory sample the NMSE is improved by at least 5.4dB comparing to the memory-less case. Finally, one can observe that a memory sample of 2 or 3 could be considered a good trade-off as the NMSE values become almost unchanged after it.

Remark1: The coefficients of the above models can be found in the attached Excel file.

Observation 5: Memory-less models ignore the asymmetrical ACLR behavior of the PA, which is crucial for RAN1/4 evaluations.

Proposal 13: Send the PC2 PA model with K=7 and M=3 to RAN1 for their evaluations on large channel bandwidths.

youtputn= m=0Mk=1Kam,2k-1xinputn-mxinputn-m2(k-1)

Where xinput, youtput, am,2k-1, M and K are the PA input signal, PA output signal, model coefficients, memory depth, and the polynomial order, respectively.

| Memory based PC2 PA Polynomial model |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| a0,1 | 1.92670821e+01+ 4.28654318e+00j | a1,1 | 7.99369914e+00-1.19662446e+01j | a2,1 | -8.06256490e+00 +1.01223949e+01j | a3,1 | 2.64587932e+00-3.12638149e+00j |
| a0,3 | 1.22620920e+02+ 1.04139154e+02j | a1,3 | -6.61969533e+01+ 2.03859399e+01j | a2,3 | 1.52084895e+00-1.18868244e+01j | a3,3 | 8.00596896e+00+ 5.36545885e+00j |
| a0,5 | -5.18300555e+03-3.12894497e+03j | a1,5 | 3.03165098e+03-7.20789286e+02j | a2,5 | -3.57361279e+02+ 3.25891527e+02j | a3,5 | -1.84889686e+02-2.17736957e+02j |
| a0,7 | 7.87331187e+04+ 3.95229160e+04j | a1,7 | -3.53677746e+04+ 8.23798720e+03j | a2,7 | 1.07342168e+02-3.27771737e+03j | a3,7 | 4.85963637e+03+ 3.05471379e+03j |
| a0,9 | -6.29759394e+05-2.59187374e+05j | a1,9 | 1.91536254e+05-4.97792719e+04j | a2,9 | 3.69009699e+04+ 1.29979229e+04j | a3,9 | -5.06559645e+04-1.96186125e+04j |
| a0,11 | 2.48830981e+06+ 8.68652330e+05j | a1,11 | -4.98282321e+05+ 1.61154342e+05j | a2,11 | -2.38453329e+05-1.45556247e+04j | a3,11 | 2.24363753e+05+ 5.97038737e+04j |
| a0,13 | -3.78283688e+06-1.17070303e+06j | a1,13 | 5.00074073e+05-2.16479788e+05j | a2,13 | 4.42501999e+05-1.43905335e+04j | a3,13 | -3.59990032e+05-6.93553051e+04j |
| Note: the front-end IL of 4dB is already considered in the model |  |  |  |  |  |  |  |

Conclusion

This contribution provides our consideration on system parameters for 6G.

Evaluation cases

Proposal 1: No parallel RAN4 study is foreseen for evaluation on DFT-s-OFDM for UL with number of layers > 1. Any necessary requirements for this waveform would be handled during the WI phase, if needed.

Observation 1: DFT-s-OFDM with lower PAPR compared to CP-OFDM would deliver higher output power, which has been justified for UL from UE side.

Observation 2: Maximum output power for BS side is up to manufacturer’s declaration and no PA model has been discussed and adopted before for BS evaluation.

Proposal 2: DFT-s-OFDM for DL should be a pure RAN1 evaluation, which should not rely on a RAN4 PA model for DL. No need for RAN4 to have discussion on DL PA model.

Proposal 3: RAN4 focus on the PA model and other evaluation assumptions, including applicable requirements, in the absence of inputs from RAN1 pertinent to waveform evaluation.

Proposal 4: Notify RAN1 that it is imperative to stable the minimum set of waveforms for RAN4's further evaluation no later than April meeting.

Evaluation assumptions

Proposal 5: Existing 5G requirements on 100MHz CBW around 7GHz with a power class 2 PA could be considered as starting point for initial waveform evaluation. Assumptions could be adjusted upon the progress of 6G study across different topics in RAN4.

Table 1: Waveform evaluation assumptions

| Parameter/Requirements |  | Assumptions/Value | Note |
| --- | --- | --- | --- |
| PA model |  | TBD | Memory effect should be considered |
| Band under evaluation |  | around 7GHz | n104 could be assumed |
| Channel Bandwidth (CBW) |  | 100MHz |  |
| Power class |  | PC2 (26dBm) |  |
| Complied requirements | SEM | TS 38.101-1 §6.5.2.2 | Subject to further adjustment pending on progress of UE RF, co-existence study |
|  | ACLR | TS 38.101-1 §6.5.2.4 |  |
|  | EVM | TS 38.101-1 §6.4.2.1 | Considered for high modulation order/inner RB allocation, pending on RAN1 discussion |
|  | IBE | TS 38.101-1 §6.4.2.3 |  |
| Tx impairments | Carrier Leakage | -28dBc | Subject to further adjustment pending on progress of UE RF study |
|  | IQ image | -28dBc |  |
|  | CIM3 | -60dB |  |
| PA calibration conditions | CBW | [20MHz full RB allocation] | Other options are not precluded, pending on the further study in RAN4 |
|  | SCS | 15kHz |  |
|  | Waveform | DFT-s-OFDM |  |
|  | Modulation | QPSK |  |
|  | Power class | PC2 |  |
|  | Power backoff to meet ACLR | 1dB |  |

Evaluation metric

Proposal 6: Adopt the same the “Net Gain” evaluation criterion as agreed by RAN1 for further low PAPR evaluation when necessary.

PA model

Observation 3: Memory effect and PSD imbalance are present when PAs are derived with large CBWs (such as 200 MHz).

Observation 4: Memory effect and PSD imbalance are present when PAs are derived with large CBWs (such as 200 MHz).

Proposal 7: Use 5G PA models for the FDD bands below 2 GHz, as they won’t include very large CBWs.

Proposal 8: Consider new PA models for the bands higher than 2 GHz and around 7 GHz where 200 MHz CBW are considered.

Proposals 9: Different PA models to be consider for different PCs and introduce new models when a new PC is introduced in 6G.

Proposal 10: Consider fixed biased PA as baseline for the models. Other advanced power management techniques shall remain within implementation.

Proposal 11: consider the larger CBW for calibration configuration for 6G SI for TDD bands.

A: 1dB MPR: DFT-s-OFDM QPSK 100MHz, 270RB or

B: 1dB MPR: DFT-s-OFDM QPSK 200MHz, [540RB].

Proposal 12: Memory effect has to be included in the models as it has impact on ACLR and SEM.

Observation 5: Memory-less models ignore the asymmetrical ACLR behavior of the PA, which is crucial for RAN1/4 evaluations.

Proposal 13: Send the PC2 PA model with K=7 and M=3 to RAN1 for their evaluations on large channel bandwidths.

youtputn= m=0Mk=1Kam,2k-1xinputn-mxinputn-m2(k-1)

Where xinput, youtput, am,2k-1, M and K are the PA input signal, PA output signal, model coefficients, memory depth, and the polynomial order, respectively.

| Memory based PC2 PA Polynomial model |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| a0,1 | 1.92670821e+01+ 4.28654318e+00j | a1,1 | 7.99369914e+00-1.19662446e+01j | a2,1 | -8.06256490e+00 +1.01223949e+01j | a3,1 | 2.64587932e+00-3.12638149e+00j |
| a0,3 | 1.22620920e+02+ 1.04139154e+02j | a1,3 | -6.61969533e+01+ 2.03859399e+01j | a2,3 | 1.52084895e+00-1.18868244e+01j | a3,3 | 8.00596896e+00+ 5.36545885e+00j |
| a0,5 | -5.18300555e+03-3.12894497e+03j | a1,5 | 3.03165098e+03-7.20789286e+02j | a2,5 | -3.57361279e+02+ 3.25891527e+02j | a3,5 | -1.84889686e+02-2.17736957e+02j |
| a0,7 | 7.87331187e+04+ 3.95229160e+04j | a1,7 | -3.53677746e+04+ 8.23798720e+03j | a2,7 | 1.07342168e+02-3.27771737e+03j | a3,7 | 4.85963637e+03+ 3.05471379e+03j |
| a0,9 | -6.29759394e+05-2.59187374e+05j | a1,9 | 1.91536254e+05-4.97792719e+04j | a2,9 | 3.69009699e+04+ 1.29979229e+04j | a3,9 | -5.06559645e+04-1.96186125e+04j |
| a0,11 | 2.48830981e+06+ 8.68652330e+05j | a1,11 | -4.98282321e+05+ 1.61154342e+05j | a2,11 | -2.38453329e+05-1.45556247e+04j | a3,11 | 2.24363753e+05+ 5.97038737e+04j |
| a0,13 | -3.78283688e+06-1.17070303e+06j | a1,13 | 5.00074073e+05-2.16479788e+05j | a2,13 | 4.42501999e+05-1.43905335e+04j | a3,13 | -3.59990032e+05-6.93553051e+04j |
| Note: the front-end IL of 4dB is already considered in the model. |  |  |  |  |  |  |  |

- References

[1] R4-2514508, Feature lead summary for [116bis][101] 6G system parameter

[2] R4-2514641, WF on 6GR system parameter

[3] R4-2514642, WF for 6G general RF and UE RF

[4]	M. Abdi Abyaneh and Y. Liu, “Evaluation of PA Memory Effects from 5G-NR to 6G Standards,” presented at the IEEE Conference on Standards for Communications and Networking (CSCN) 2025, Bologna, Italy, Sep. 2025.