3GPP TSG-RAN WG4 Meeting #117                                           R4-2520327

Dallas, TX, United States, Dec 17 – 21, 2025

Source: 	Huawei, HiSilicon

Title: 	On co-existence study for 6G

Agenda Item:	8.4.3

Document for:	Approval

Introduction

Research on co-existence study for 6G was mentioned in the issue 2-1-1 of WF on 6G BS RF and co-existence[1]. Candidate frequency bands, key assumptions, potential impacts, etc., need to be discussed under this topic.

This document presents initial considerations of 6G simulation scenarios and assumptions for the co-existence study.

Discussion

- 2.1 Simulation scenario

Table 1 summarizes the considered scenarios. The ~7 GHz band is regarded as a core frequency band for 6G and is therefore assigned the highest priority. In accordance with TR 38.921, only urban macro and indoor hotspot scenarios are considered, as the dense urban scenario exhibits the least stringent ACIR requirements among the three simulated scenarios outlined in TR 38.803. Given that the 700 MHz band is primarily intended for wide-area coverage, the rural macro scenario is included, while the indoor hotspot scenario is deemed inapplicable and thus excluded.

Table 1 Summary of considered co-existence scenarios

| No. | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Priority |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 6GR, 100,[200/400]MHz | Same as aggressor | DL to DL | 7 GHz | Urban macro, Indoor hotspot | First |
| 2 | 6GR, 100,[200/400]MHz | Same as aggressor | UL to UL | 7 GHz | Urban macro, Indoor hotspot |  |
| 3 | 6GR, 100MHz | Same as aggressor | DL to DL | 4 GHz | Urban macro, Indoor hotspot | Second |
| 4 | 6GR, 100MHz | Same as aggressor | UL to UL | 4 GHz | Urban macro, Indoor hotspot |  |
| 5 | 6GR, 20MHz | Same as aggressor | DL to DL | 2 GHz | Urban macro, Indoor hotspot |  |
| 6 | 6GR, 20MHz | Same as aggressor | UL to UL | 2 GHz | Urban macro, Indoor hotspot |  |
| 7 | 6GR, 20MHz | Same as aggressor | DL to DL | 700 MHz | Urban macro, Rural macro |  |
| 8 | 6GR, 20MHz | Same as aggressor | UL to UL | 700 MHz | Urban macro, Rural macro |  |
| 9 | 6GR, 200MHz | Same as aggressor | DL to DL | 30 GHz | Urban macro, Indoor hotspot | Third |
| 10 | 6GR, 200MHz | Same as aggressor | UL to UL | 30 GHz | Urban macro, Indoor hotspot |  |
| 11 | 6GR, 200MHz | Same as aggressor | DL to DL | 15GHz | Urban macro, Indoor hotspot |  |
| 12 | 6GR, 200MHz | Same as aggressor | UL to UL | 15GHz | Urban macro, Indoor hotspot |  |

Proposal 1: Co-existence studies should be prioritized as specified in Table 1: the ~7 GHz band has the highest priority, while the 4 GHz, 2 GHz, and 700 MHz bands are designated as the second priority.

Figure 1 illustrates the interference scenario where operator 1 is the aggressor and operator 2 is the victim.

Figure 1 Interference Diagram

Specifically, DL to DL and UL to UL interference scenarios should be considered for the 6GR-to-6GR co-existence study.

- 2.2 Simulation assumption

For 7GHz, the previous co-existence study was done in TR 38.921 and we can take this as a starting point. For 4GHz, some assumptions are provided in TR 38.922, but the co-existence study is not conducted. As 4GHz is used for some key scenarios for 6G, such as UAV and ISAC, the co-existence study should also be done in 6G to obtain the proper RAN4 requirement.

Details on urban macro network layout model are listed in Table 2. The cell radius in TR 38.921 is only 0.3km for urban macro and it is very smaller than that of other bands below 6GHz. The main reason is the antenna configuration in TR 38.921 is16x8 without sub-array, while the antenna configuration in TR 38.922 for 7GHz is (8x3) x16 with sub-array. In order to expand the communication rang of 7GHz, the antenna size should be further increased at least match that of TR 38.922. For indoor ratio, the 20% is defined in urban macro. This value does not match the real deployment scenario, because the most of UEs are indoor. The indoor UE ratio for RAN 1[1] is 80% and the in ITU 5D[3] is 70%, therefore, an indoor UE ratio of 80% is proposed to align with the RAN 1 assumption.

Table 2: Network layout model for urban macro

| Parameters |  | Value | Note |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 500m for 4/7GHz 750m for 700MHz/2GHz[7] | It depends on the frequency. |
| Multi operator’s layout |  | Uncoordinated |  |
| BS antenna height |  | 25 m for 2/4/7GHz[6][7] 30m for 700MHz[7] | It depends on the frequency. |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80% |  |
|  | Low/high penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

Details on indoor network layout model are listed in Table 3.

Table 3:Network layout model for Indoor hotspot

| Parameters |  | Values | Note |
| --- | --- | --- | --- |
| Network layout |  | 50 m x 120 m, 12 BSs | TR 38.921 |
| Inter-site distance |  | 20m |  |
| Multi operator’s layout |  | Coordinated operation |  |
| BS antenna height |  | 3 m | ceiling |
| UE location | Outdoor/indoor | Indoor |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | 1 m |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 0 m |  |
| Channel model |  | Indoor Office |  |
| Shadowing correlation |  | NA |  |

Table 4 lists the summary of propagation model.

Table 4: Propagation model

| Propagation loss | Value |
| --- | --- |
| Path loss | TR 38.901 |
| LOS probability | TR 38.901 |
| Penetration loss | TR 38.901 O2I building penetration loss |

Table 5 summarizes the BS antenna characteristics. For the 7 GHz and 4 GHz bands, the antenna configurations defined in TR 38.922 can serve as references. For the 700 MHz band, it is reasonable to adopt parameters of an AAS antenna rather than a non-AAS type, considering the evolution of 6G base stations. Additionally, the requirements for ACLR and ACS should be re-evaluated.

Table 5: BS antenna characteristics

| Scenario | Frequency range | Transceiver configuration | Array configuration (Msubx1, MxN) | Note |
| --- | --- | --- | --- | --- |
| Urban macro | 700 MHz | 16 TX, 16 RX | 4x1, 2x4 |  |
|  | 2 GHz | 64 TX, 64 RX | 3x1, 4x8 | TR 38.922 |
|  | 4 GHz | Same as 2 GHz | Same as 2 GHz | Same as 2 GHz |
|  | 7 GHz | 256 TX, 256 RX | 3x1, 8x16 | TR 38.922 |
| Indoor hotspot | 2 GHz | [32 Tx,32 Rx] | [1X1,4X4] |  |
|  | 4 GHz | Same as 2 GHz | Same as 2 GHz |  |
|  | 7 GHz | [32 Tx,32 Rx] | [1X1,4X4] |  |

It is assumed that UE antenna patterns are omnidirectional with 0 dBi gain. However, the potential use of directional antennas, especially in the ~7 GHz band, merits further discussion.Table 6: UE antenna characteristic

| Parameter | Value |
| --- | --- |
| Antenna pattern | isotropic with 0dBi [Directional antenna] |

Other simulation parameters are provided in Table 7. While TR 38.921 and TR 38.922 study only power class PC3, it is noted that many companies are considering PC2 as the default power class for TDD bands for 6G. Therefore, we recommend that PC2 is considered in co-existence studies for the 7 GHz and 4 GHz bands, which were not taken in previous releases. For FDD bands, with the growing prevalence of HPUE, PC2 could also be considered in the 700 MHz co-existence study. If these simulation assumptions are updated, the co-existence study should be revisited to determine the corresponding ACLR and ACS requirements.

Table 7: Other simulation parameters

| Parameters | Value |
| --- | --- |
| Frequency reuse | 1x3x1 |
| The number of active UE (UL) | 3 |
| The number of active UE (DL) | 1 |
| ACLR and ACS modelling | TR 38.921 |
| Transmission power control model | TR 38.921 |
| Link Level Performance Model | TR 38.921 |
| Received power model | TR 38.921 |
| Traffic model | Full buffer |
| BS max TX power in dBm | 46 for macro urban 24 for indoor hotspot (Note 1) |
| UE max TX power in dBm | 23 and 26 (Note 2) |
| UE min TX power in dBm | -33 or -40 |
| BS Noise figure in dB | 6 (7 GHz) 5 (700MHz/2GHz/4GHz) |
| UE Noise figure in dB | 12 (7 GHz) 9 (700MHz/2GHz/4GHz) |
| Handover margin | 3 dB |
| Note 1: BS max TX power is defined two polarizations. For 7GHz, the provided max TX power is assumed across 100 MHz bandwidth. For other bandwidths (e.g. 200 MHz), the same power spectral density as assumed across the 100 MHz bandwidth should be maintained, (e.g. the max TX power for a 200 MHz channel will be 3dB higher than a 100 MHz channel). Note 2: The CLx-ile of 26dBm should be increased by 3 dB. Note 3: It depends on the bandwidth. The min TX power is -40 dBm for 20MHz and -33 dBm for 100MHz. |  |

Proposal 2: Simulation assumptions, including network layout models, propagation models, BS/UE antenna characteristics, and other necessary parameters, shall be determined alongside co-existence scenarios.

Proposal 3: The following specific updates are proposed for simulation assumptions:

- For 7 GHz, update parameters such as ISD, antenna configuration, indoor UE ratio, and UE power class beyond the TR 38.921 baseline.

- For 700 MHz, use AAS antenna parameters instead of non-AAS base station assumptions.

- Consider Power Class 2 in co-existence studies, at least for TDD bands.

Conclusion

Focused on 6G, this document covers some initial considerations regarding simulation scenarios and assumptions for the co-existence study.

Proposal 1: Co-existence studies should be prioritized as specified in Table 1, i.e. the ~7 GHz band has the highest priority, while the 4 GHz, 2 GHz, and 700 MHz bands are designated as the second priority.

Table 1 Summary of considered co-existence scenario

| No. | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Priority |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 6GR, 100,[200/400]MHz | Same as aggressor | DL to DL | 7 GHz | Urban macro, Indoor hotspot | First |
| 2 | 6GR, 100,[200/400]MHz | Same as aggressor | UL to UL | 7 GHz | Urban macro, Indoor hotspot |  |
| 3 | 6GR, 100MHz | Same as aggressor | DL to DL | 4 GHz | Urban macro, Indoor hotspot | Second |
| 4 | 6GR, 100MHz | Same as aggressor | UL to UL | 4 GHz | Urban macro, Indoor hotspot |  |
| 5 | 6GR, 20MHz | Same as aggressor | DL to DL | 2 GHz | Urban macro, Indoor hotspot |  |
| 6 | 6GR, 20MHz | Same as aggressor | UL to UL | 2 GHz | Urban macro, Indoor hotspot |  |
| 7 | 6GR, 20MHz | Same as aggressor | DL to DL | 700 MHz | Urban macro, Rural macro |  |
| 8 | 6GR, 20MHz | Same as aggressor | UL to UL | 700 MHz | Urban macro, Rural macro |  |
| 9 | 6GR, 200MHz | Same as aggressor | DL to DL | 30 GHz | Urban macro, Indoor hotspot | Third |
| 10 | 6GR, 200MHz | Same as aggressor | UL to UL | 30 GHz | Urban macro, Indoor hotspot |  |
| 11 | 6GR, 200MHz | Same as aggressor | DL to DL | 15GHz | Urban macro, Indoor hotspot |  |
| 12 | 6GR, 200MHz | Same as aggressor | UL to UL | 15GHz | Urban macro, Indoor hotspot |  |

Proposal 2: Simulation assumptions, including network layout models, propagation models, BS/UE antenna characteristics, and other necessary parameters, shall be determined alongside co-existence scenarios.

Proposal 3: The following specific updates are proposed for simulation assumptions:

- For 7 GHz, update parameters such as ISD, antenna configuration, indoor UE ratio, and UE power class compared to the TR 38.921 baseline.

- For 700 MHz, use AAS antenna parameters instead of non-AAS base station assumptions.

- Consider Power Class 2 in co-existence studies, at least for TDD bands.

- References

- R4-2514638, WF on 6G BS RF and co-existence, Ericsson

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

- ITU WP 5D/792. Working document on characteristics of terrestrial component of IMT sharing and compatibility studies in preparation for WRC-27(Annex 4.32 to Working Party 5D Chair’s Report)

- R4-2513944, Draft LS to ECC PT on parameters for AAS BS operating in bands below 1 GHz, Ericsson

- TR 38.921, Parameters for IMT studies on 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz.

- TR 38.922, Study on International Mobile Telecommunications (IMT) parameters for 4 400-4 800 MHz, 7 125-8 400 MHz and 14 800-15 350 MHz.

- ITU M.2292. Characteristics of terrestrial IMT-Advanced systems for frequency sharing interference analyses.