3GPP TSG-RAN WG4 Meeting #117                                           R4-2520328

Dallas, USA, Nov 17 – 21, 2025

Source: 	Huawei, HiSilicon

Title: 	Further consideration on spectrum for 6G

Agenda Item:	8.5

Document for:	Approval

Introduction

This contribution provides our further consideration on spectrum for 6G based on the WF [2].

Discussion

New spectrum/bands and regulations

Potential new spectrum for 6G

The next WRC (World Radiocommunication Conferences) will be held in Oct 2027 meaning that WRC-27 comes after the completion of the SI (June, 2027). Hence, RAN4 somehow needs to select potential and promising spectrum for 6G day 1 use. It is noted that the existing legacy bands defined in 5G may not require band specific study in the SI phase, while those can be handled in a more generic way together with new concepts such as UL/DL decoupling etc.

As is very well known, 4.4 – 4.8 GHz, 7.125 – 8.4 GHz and 14.8 – 15.35 GHz are potential new IMT bands. In fact, RAN4 had already conducted some study to derive basic RF parameters for those spectra in response to LS(s) from ITU-R in R19 SI of FS_NR_IMT_4400_7125_14800MHz.

The latter two spectrum portions are completely new so that co-existence simulation was performed in the R19 SI, while the first one (4.4 - 4.8GHz) is confined within n79 (4400 MHz – 5000 MHz) so that the corresponding RF parameters for n79 were referred to and no co-existence study was conducted in the R19 SI.

It is also noted that the co-existence study was conducted only for UE power class 3, while it is expected that in reality, even higher power is requested to be used to overcome larger pathloss compared to lower frequency ranges.

Observation 1-1: 4.4 – 4.8 GHz, 7.125 – 8.4 GHz and 14.8 – 15.35 GHz were studied in Ｒ19 SI of FS_NR_IMT_4400_7125_14800MHz, where a spectrum like 4.4-4.8GHz didn’t do co-existence study and the co-existence study for all the spectra didn’t consider practical aspects such as higher power classes.

In addition, in R20 RAN led SI (FS_6G_RAN_Scen_Req) and in R20 RAN1 led SI (FS_6G_Radio), some parameters essential for co-existence study such as the number of Tx/Rx, channel bandwidth etc., are under discussion. Therefore, it would be better to do co-existence study based on the new parameters for 6G.

Observation 1-2: Some parameters impacting on co-existence results such as Channel bandwidth, the number of Tx/Rx antennas etc., are under discussion in R20 RAN led SI (FS_6G_RAN_Scen_Req) and in R20 RAN1 led SI (FS_6G_Radio).

With consideration of the observation 1 and 2, it may be better for RAN4 to at least revisit the co-existence study for those frequency ranges.

Observation 1-3: From observation 1 and 2, it would worth revisiting and re-evaluating the SI with new parameters which under discussion in TSG RAN/RAN1.

Last but not least, n104 frequency range (6425 MHz – 7125 MHz) is a part of 4.4 – 15.35GHz, and the band is not yet deployed at full scale. From the frequency perspective, it may be possible to refer to new parameters to impact on co-existence study for 4.4 – 4.8 GHz or 7.125 – 8.4 GHz.

Observation 1-4: Frequency range of 6425 MHz – 7125 MHz has not been yet fully unleashed and it may be possible to obtain even more suitable requirements like ACLR with consideration of new parameters like channel bandwidth, the number of Tx/Rx antenna etc., under R20 RAN level study item and RAN1 for WG level SI.

Proposal 1-1: Consider at least 4.4 – 4.8 GHz, 7.125 – 8.4 GHz, 14.8 – 15.35 GHz and 6.425 – 7.125 MHz for further study with co-existence evaluation.

Frequency ranges

The frequency ranges defined in 5G for Terrestrial Networks (TN) can be found in the table below:

Table 1: Definition of TN frequency ranges

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

It is worth noting that the upper boundary of FR1 was previously extended from 6000 MHz to 7125 MHz during the RAN#82 meeting.

In the last RAN plenary meeting (#109), spectrum is one of the topics discussed for RAN 6G study item. One controversial issue regarding frequency range is:

How to categorize the frequency range? Should the spectrum resource from 7.125GHz to 8.4GHz be categorized into the lower group or the higher group?

- Option 1: extend FR1 to 8.4 GHz and define a separate mid-high band (8.4–24.25 GHz) with its own single numerology.

- Option 2: extend FR1 to 8.4 GHz and merge 8.4–24.25 GHz into FR2, creating a unified FR2 family.

- Option 3: define separate FR for 7.125 – 24.25 GHz.

No consensus was reached by the end of the session. Given that this matter is still under active discussion in both RAN1 and the RAN, it is more appropriate that RAN4 proceed with the discussion with close coordination with these groups.

From RAN4 perspective, however, the 7-24 GHz frequency range was already been studied during the Rel-16 study item. RAN4 could build on the findings from that work to further investigate possible limitations at both UE and BS side, or boundaries that would support conducted requirements in FR1. Such analysis could provide a valuable technical reference to help RAN1 and RAN plenary determine the most appropriate frequency range categorization.

Proposal 1-2: RAN4 to conduct studies pertaining to the TN frequency range from a UE and BS implementation perspective, particularly regarding the applicability of conducted requirements.

Furthermore, it shall be noted that the frequency ranges defined in 5G for Non-Terrestrial Networks (NTN) differ from the one presented in Table 1 for TN. Before the Ku bands introduction, the following NTN frequency ranges were defined:

Table 2: Definition of NTN frequency ranges, including Ka bands (Rel-18)

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 7125 |
| FR2-NTN (NOTE 2) | 17300 – 30000 |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

For FR1, despite terminology itself (e.g., FR1 vs. FR1-NTN), the allocated frequency limits are the same for both FR1 and FR1-NTN.

In case of FR2, despite terminology itself (e.g., FR2 vs. FR2-NTN), the allocated frequency limits are different. This was due to the need to account for Ka bands.

With the introduction of Ku bands in Rel-19, the NTN frequency range were further modified, as follows:

Table 3: Definition of NTN frequency ranges, including Ka and Ku bands (Rel-19)

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 14500 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

Compared to Table 2, the FR1-NTN upper frequency range was extended from 7125 MHz up to 14500 MHz, and the FR2-NTN lower frequency range was extended from 17300 MHz down to 10700 MHz. With such arrangement, now FR1-NTN does overlap with the FR2-NTN in 10700-14500 MHz range.

Observation 1-5: Definitions of FR1 and FR2 frequency ranges for TN and for NTN is already mis-aligned.

Observation 1-6: NTN specification (of FR1-NTN and FR2-NTN) does already cover 7 – 24 GHz range.

With the above considerations, it is observed that introduction of any new Frequency Range (FR) definition (for 7.125 – 24.25 GHz, of for 8.4 – 24.25 GHz) would only complicate more the overall picture of Frequency Ranges for 6G.

It shall be also considered, that introduction of any new terminology for Frequency Ranges, e.g., “FR3”, shall not fragment any of the existing TN, nor NTN bands. This is especially important for the newly introduced Rel-19 Ku bands, as captured in Table 4 below:

Table 4: Ku satellite operating bands in FR1-NTN and FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n248 / n509 | 14000 MHz - 14500 MHz | 10700 MHz – 12750 MHz | FDD |
| n247 / n508 | 13750 MHz - 14000 MHz | 10700 MHz – 12750 MHz | FDD |

Observation 1-7: Range of 10700 – 14500 MHz is already specified to FR2-NTN.

Therefore, it is suggested to investigate Frequency Ranges framework for 6G without consideration of any new (e.g., FR3) terminology introduced as it would only further complicate the overall picture.

Proposal 1-3: Refrain from introducing FR3 to cover any frequency range within 7 – 24 GHz.

It can be discussed whether it would be reasonable to consider FR1-NTN and FR2-NTN ranges are starting point for 6G conducted requirements, and 6G OTA requirements, respectively.

It shall be noticed, that with 6G, there was intention to further integrate TN and NTN designs towards future network integrations. Keeping this in mind, one may ask whether definitions of Frequency ranges shall be also aligned among TN and NTN. Based on the above discussion and observations, it is clear that the current situation is far from aligned (with FR1 and FR2 separated with 7-24 GHz range, while FR1-NTN and FR2-NTN overlapping already and covering 7-24 GHz range). As TN and NTN frequency ranges definitions were derived based on the WRC radio regulations, with distinct spectrum allocation for terrestrial and satellite operations, it is not envisioned that those baseline regulations would be aligned. With this, it is suggested to keep separate definitions of Frequency Ranges for TN, and for NTN, in the same manner as already defined in 5G NR. Definition of TN Frequency Range(s) would be subject to updates to account for WRC-27 decisions, and proposals 1-1 an 1-2 above.

Proposal 1-4: In 6G, keep separate discussion of frequency ranges of TN and NTN.

Framework on impact of updated regulations to the defined bands

In-band spectrum allocation with access restrictions

The n77 band deployments in the US and Canada were discussed extensively in previous releases. The figure below illustrates the background scenario (figure cited from R4-2206568, LS to RAN2).

NR Band n77 was initially defined in 3GPP Rel-15 as a TDD band spanning 3300-4200 MHz. In the United States, the FCC auctioned the 3700–3980 MHz range in 2020 and later a new spectrum segment was introduced into 3450-3550 MHz in 2021. A key issue was that n77 UEs certified only for the original spectrum segment, i.e., 3700-3980 MHz, would not be permitted to camp on cells operating in the 3450-3550 MHz segment, while n77 UEs certified for both of the original and new segments are allowed to camp on both cells (segments) Since FCC type approval is mandatory for the U.S. market, a new signaling mechanism became necessary in order to meet the above conditions.

To address this, 3GPP introduced in Rel-16 a UE capability (extendedBand-n77) and a new Network Signaling (NS) value, i.e., 55. The capability bit explicitly indicates support for the 3450-3550 MHz range, while the NS value is used to bar non-compliant UEs from accessing cells in this segment—preventing repeated unsuccessful attachment attempts. This NS mechanism diverges from its conventional use for additional spurious emissions, instead, serving for access control. A similar approach was later adopted for Canada’s 3450-3650 MHz spectrum. It is also noted that the discussion for intra band CA was even more controversial, though the details are omitted here.

These band-specific issues highlighted a broader challenge: the need to handle increasing regional spectrum fragmentation and sub-band allocations in a scalable manner. Consequently, a RAN-level study item was established in Rel-18, documented in TR 38.893, to analyze root causes and evaluate generalized solutions.

The root causes identified include:

1.	A regional regulator makes an initial frequency allocation of portions/subsets of the 3GPP band, i.e., not including the entire band

2.	The region requires UE certification for all UEs operating within the borders of the region.

3.	The regional certification only applies to the frequency allocation at the time of certification.

4.	The UE must confine its operation to the frequency band for which it was certified.

5.	The regional regulator subsequently allocates other portions/subsets of the band [and/or regulations change within the existing sub-bands].

6.	Newer UEs are certified for both the original allocation and subsequent allocations.

Several approaches were discussed for addressing regional sub-band requirements, including:

- Solution A: Introducing a new band for the sub-band

- Solution B: Assigning a new band number

- Solution C: Reusing the existing NR band number with new signaling

Due to time constraints, no generalized mechanism was finalized in 5G. The TR instead emphasizes ensuring consistent UE behavior both within and outside region-specific sub-bands.

As we move toward 6G, it is essential to prioritize a unified signaling framework and certification-aware operation to enable dynamic adaptation to regional sub-band allocations, thereby reducing spectrum fragmentation and enhancing agility in cross-regional connectivity.

Proposal 1-5: Study in 6G to develop a unified and extensible signaling mechanism that provides flexible support for regional sub-band allocations within a single band, thereby avoiding fragmented solutions.

Updated additional spectrum requirements to the defined bands

Regulatory changes typically evolve slowly, though exceptions do occur, which can significantly impact RAN4 requirements for the already defined frequency bands.

One of the notable examples stems from ITU WRC-19, which specified an unwanted emission limit for IMT mobile stations operating in 24.25–27.5 GHz to protect the Earth Exploration Satellite Service (EESS) (passive) in the adjacent 23.6–24 GHz spectrum. The limit is set at -29 dBW/MHz until September 1, 2027, and will be tightened to -35 dBW/MHz thereafter. It should be noted that devices brought into use before this date remain subject to the influence of the earlier -29 dBW/200 MHz limit.

However, regional regulations in markets such as China and the EU have already implemented the more stringent spurious emission requirement of -35 dBW/200 MHz (equivalent to -5 dBm/200 MHz) ahead of the ITU-R timeline:

- China requires compliance with -5 dBm/200 MHz for commercial deployments.

- EU Decision 2020/590 mandates -35 dBW/200 MHz (-5 dBm/200 MHz) effective January 1, 2024.

To align with these regulatory developments and to support commercial deployment, RAN4 introduced corresponding requirements. Key conclusions from Rel-16 regarding EESS protection include:

- Introduction of a new Network Signaling (NS) value for bands n258 and n257, along with a modified Maximum Power Reduction (MPR) method and an extended scope of modified MPR.

- Obsoleting the -8 dBm/200 MHz requirement under NS_201.

- Agreement to apply -5 dBm/200 MHz after September 2027, though specification updates remain pending due to lack of consensus.

- No general agreement on mandatory support for all NS values per band, though NS_203 was made mandatory for EESS protection.

In Rel-19, as these requirements came into force, RAN4 further updated FR2 requirements, including:

- Revised NS_202 definition to protect legacy devices and enforce stringent emission limit.

- Introduction of new NS_205 for bands n257 and n258.

- Addition of new A-MPR requirements for NS_205.

- Updates to Annex H to ensure backward compatibility for NS_202 and support for NS_205 in bands n257 and n258.

One of the key issues addressed during the study was whether NS support should be mandatory or not. Although not generally agreed, mandatory NS was adopted for EESS protection due to regulatory necessity.

Observation 1-8: Regulation (e.g., additional spectrum emissions), changes could have impact on RAN4 requirements in terms of such as mandatory/optional, respective applicable release(s) for the applicable operating bands

Another example discussed recently is that Japan's regulations on spurious emissions related to PHS have been updated, which incurred a lot of discussion on the A-MPR requirements as well as the issue of modifiedMPR such as inconsistencies across bands and/or specifications (between 38.101-1 and -2) in terms of modal verbs such as shall/may, applicable releases, inclusion of spec version (no spec version is seen in 38.101-2 while it is seen in 38.101-1).

More specifically, following inconsistencies are found in 5G-A discussion.

Table 5: Three rationales used in the definitions of modifiedMPR-behaviour

| Rationale | Instances |
| --- | --- |
| #1: “may be set to 1” for earlier releases, “shall be set to 1” for current and future releases | Band n30, n71 n41 bit 0, bit 1 |
| #2: “may be set to 1” for earlier releases, not needed for current and future releases | Band n96, n102 n41 bit 2 |
| #3: “may be set to 1” for any releases | Band n34, n39, n40, n78, n79 n41 bit 3 |

It can be observed that:

Observation 1-9:	There are mixed usages of “shall” and “may” in the definitions of the modified MPR behaviour. Different bands and even different bits for the same band follow different rationales, leading to confusion.

Table 6: Definitions of the bits in the field modifiedMPR-Behavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| … |  |  |  |
| n41 | 0 (leftmost bit) | EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
|  | 1 | EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | EN-DC contiguous and non-contiguous intraband MPR and A-MPR as defined in 38.101-3 v16.4.0. If this bit is not set the UE uses Rel-15 MPR or A-MPR for EN-DC contiguous and non-contiguous intraband MPR and A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |
|  | 3 | PC 1.5 MPR as defined in Table 6.2D.2-3 | This bit may be set to 1 by a UE of any release supporting power class 1.5. This bit is intended to be set by larger form factor FWA devices. If the bit is not set for a Rel-17 and later UE, PC 1.5 MPR as defined in Table 6.2D.2-2 applies. If the bit is not set for a Rel-16 and earlier UE, MPR in Table 6.2.2-4 of 38.101-1 v16.5.0 applies. |

It can be observed that:

Observation 1-10:	For band n41, both bit 0 and bit 1 “shall be set to 1”. It’s unclear how to interpret bit 2 if it’s also set to 1. It’s questionable whether bit 2 can fulfil its intended purpose under the existing definitions.

In RAN4, still all those above are under discussion based on the way forward of R4-2511776. It is also noted that there is an issue not discussed in 5G that the current modifiedMPR scheme requires CRs for earlier releases down to the release, where an associated band is introduced. This increases more workload (the number of CRs).

Observation 1-11: modifiedMPR scheme requires CRs for earlier releases down to the release, where an associated band or feature is introduced.

The resolution(s) to be taken in 5G would not be ideal due to avoiding NBC. Thus, all these issues found in 5G should be solved with a clean-slate.

Proposal 1-6: Issues written in Observations 6-8 should be addressed in 6G with a clean slate.

Band combination definition and simplification

In last meeting, band combination definition and simplification were barely discussed despite a number of contributions were mentioned about those aspects. In the WF [2], just a boundary about what should be discussed in 6G Spectrum thread was clarified. We would like to share our thinking on how to simplify band combination in following part.

Besides the discussion on band combination definition, some approaches to simplify/streamline requirements for band combinations were also discussed and captured in the WF, such as delta Rib and Tib. We would also provide our consideration how to simplify these BC related requirements as a part of the framework on band combination simplification.

Overview of band combination status in NR

Defining band combinations for operator holding spectrum normally involves many parties, including co-existence analysis, specification definition and maintenance, complex calibration, testing and verification. To support those band combinations, it costs 90%+ overall UE capability signalling overhead that also increases the overall system overhead of the air-interface signalling. But currently less than 10% band combinations are deployed in the real network.

Observation 2-1: Although there are quite a lot of band combinations have been specified and considerable UE capability signalling overheads are needed to support those band combinations, the current deployment in the real network is not that much.

Carrier aggregation is introduced for enhancing UL/DL throughput to aim to fully utilize operator’s available spectrum with limited channel bandwidth for each individual component carrier, especially the introduced max channel bandwidth was 20 MHz in LTE. A significant amount of study for impact on maximum output power (MOP) and REFSENS has been conducted per band combination basis in both LTE and NR with definition of ∆RIB, ∆TIB and MSD, which are caused by additional RF components such as diplexer/multiplexer for isolation and RF nonlinear behaviours such as IMD, harmonics etc. on top of the additional RF components respectively.

Observation 2-2: With the increasing number of supported band combinations, UE Tx/Rx performance degradation becomes severe due to insertion loss from multiplexer, switch and other additional RF components.

Over recent releases, the feasibility of specific band combinations have been provided. For instance, in Rel-18, the study outcomes of Study on enhancement for 700/800/900MHz band combinations for NR (FS_NR_700800900_combo_enh) are captured in TR 38.872. The common implementation challenges among different assumed RF architectures are how to achieve acceptable filter isolation between two or three bands with close frequency ranges and how to design antenna considering the excessive bandwidth ratio. Consequently, the study revealed the fact that, with all the foreseeable difficulties, supporting a band combination with two or three bands under Sub-1GHz can lead to considerable Tx/Rx performance degradation as well as the huge impact on antenna efficiency which cannot be reflected in conducted RF requirements.

In Rel-19, RAN4 tried another route to better utilize the spectrum in low bands considering the practicalities of handset RF front end architectures challenges in the industry, where Low band CA via switching (NR_LBCA_Sw)[4] was proposed to make full use of low band spectrum with accommodation of RF front end challenges. With network configuration of specific switching pattern with 40ms periodicity so that both low bands can be used by switching:

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

More importantly, the WI concluded that if a UE supports a LB-LB CA for a band combination, but the UE doesn’t support conventional CA for the same band combination, ΔTIB,c and ΔRIB,c are zero. It says that this approach gives some flexibility to utilize SDL band, while by avoiding concurrent operation between bands, the impact on MOP and REFSENS can be avoided as well.

Observation 2-3: Rel-19 Low-Low band CA via switching is an alternative to balance the performance degradation from normal CA and the spectrum aggregation restriction from single carrier operation.

Analysis on support band combination

Higher throughput can be achieved by combing more carriers or bands in terms of spectrum size used simultaneously. This can be especially beneficial with low and/or middle frequency bands with small channel bandwidth allocated. However, with more and more middle and high spectrum introduction, more issues are observed in the UE implementation and testing to support more band combinations and other parameters/features such as wider channel bandwidth and/or higher MIMO layers to increase throughput. In practice, many aspects have become intricately intertwined simultaneously, but here we share our analysis on the observed issues in one-on-one basis.

UE Antenna gain

UE needs to simultaneously support different RATs, such as cellular network, shorter-distance communications, satellite, NFC and UWB, and different frequency ranges, such as FR1, FR2-1 and around 7GHz. But UE antennas can’t simultaneously achieve optimal gain for all supported bands as the size of antenna is fixed which is restricted by UE’s physical size: G=(ka)2+2ka, a is antenna size. So, it is necessary to consider the antenna design when RAN4 define the band combination to achieve the possible largest antenna gain.

MOP/REFSENS degradation caused by IL from multiplexer

Adjacent band combination brings complex front-end design that involves complex and flexible control logic, multiplexer should be used to ensure the concurrent transmission and reception that brings high design cost and increases the insertion loss of front-end. The maximum output power and sensitivity will be degraded due to the insertion loss of front-end, about 0.7~1dB. Even more than 1dB insertion loss can be caused compared to single band in some cases if a UE supports multiple band combinations.

REFSENS degradation caused by non-linearity

If a UE supports band combinations with more bands, the possibility for the UE to come across REFSENS degradation (MSD) increases due to IMD, cross band isolation, harmonics and harmonic mixing. Nowadays, higher power classes have been introduced into bands so that the impact of the above non-linearly behaviour becomes even larger. To reduce the sensitivity degradation caused by harmonic interference, IMD and/or high power, UE has to add filters to provide suppression, increase antenna isolation, and/or adopt digital domain elimination of interference which is unstable and scenario dependent, which addressing all the cases would cause a serious issue to achieve gain when viewed as a whole. All these seriously increase UE implementation complexity and cost.

Loss of SRS antenna switching gain

The SRS antenna switching for one band will interrupt the transmission and/or reception of another band, for example n41 switch DPDT1 will interrupt the transmission and reception of B3 as shown in in Figure 1 below. In addition, complex RF front design may cause more ILs so that output power per SRS antenna port may have lager imbalance.

Figure 1: SRS antenna switching

Complex circuit design and board-level routing result in the inability to fully reap the benefits of SRS under CA.

The number of transceiver paths v.s. support CA for band with larger channel bandwidth and MIMO layers

With the introduction of new 6GR spectrum with larger channel bandwidth and higher MIMO layers, such as 4.4 – 4.8 GHz, 7.125 – 8.4 GHz and 14.8 – 15.35 GHz as mentioned above, most operators will have low, medium and high frequency spectrum resources. Operators will have more choices to select suitable band combinations to meet the real service request with more cost-effective approach. From UE perspective, it is highly beneficial to make the best use of the total available number Rx chain and PLL supported by UE, i.e. allocate the Rx chain to band with higher channel bandwidth and MIMO layer as much as possible to achieve higher throughput. Such as the inter-band 4CC band combination among sub1GHz, 1.5~2.7GHz, 3.3~7GHz and 5~8GHz, the total max throughput 5.72Gbps for DL (111Mbps+400Mbps+1.74Gps+3.47Gbps) and max UL 1.12Gbps can be achieved that can well meet most of possible service scenarios as shown in Table 7 below:

Table 7: Analysis on max throughput for single CC for different frequency range

| Frequency Range | Channel Bandwidth per CC | MIMO layer# | Max throughput for single CC | Target Scenario |
| --- | --- | --- | --- | --- |
| sub1GHz | 10MHz | DL：2 UL：1 | DL：111Mbps UL：60Mbps | Larger coverage, basic communication service |
| 1.5~2.7GHz | 20MHz | DL：4 UL：2 | DL：400Mbps UL：200Mbps | General traffic coverage |
| 3.3~5GHz | 100MHz | DL：4 UL：2 | DL：1.74Gbps UL：285Mbps | Heavy-traffic service estimated TDD pattern is 7D1S2U with S=6:4:4 |
| 5~8GHz | 200MHz | DL：4 UL：2 | DL：3.47Gbps UL：571Mbps | Hotspot with TDD pattern 7D1S2U with S=6:4:4 |

Observation 2-4: Total max throughput of 5.72Gbps for DL and 1.12Gbps for UL can be achieved with 4CC inter-band combinations among different frequency ranges of sub1GHz, 1.5~2.7GHz, 3.3~7GHz and 5~8GHz.

Some operators may not have higher spectrum frequency with larger channel bandwidth and MIMO layers in some region, it can be analysed case by case to address those operator’s request on low band combinations.

Observation 2-5: Following aspects which could lead to performance degradation when supporting normal CA:

- UE Antenna gain limitation for support more bands with fixed UE physical size

- Maximum output power and REFSENS degradation caused by front-end IL from multiplexer

- REFSENS degradation caused by non-linearity and IMD

- Loss of SRS antenna switching gain caused by interruption and power imbalance across ports

- The balance between the available number of transceiver paths and support CA for band with larger channel bandwidth and MIMO layers.

To efficiently utilize operator’s available spectrums of low, mid and higher frequency ranges, maximize the spectrum efficiency to achieve higher performance gain and/or throughput by taking into account the industry development, implementation feasibility and complexity, and power consumption that is directly related to user experience, new mechanism need to be investigated in 6GR. Band group concept as mentioned by many companies is a feasible and meaningful way forward that can be used to greatly simplify the band combination definition and some RF requirements with per BC granularity. As shown in Figure 2 below, all possible available IMT frequency ranges can be divided into 7 band groups as per the UE’s antenna layout and/or RF front-end implementation, but more or less band groups than 7 can be further discussed. Any band combinations between/among different band groups can be supported, but only one band can be selected within one band group by switching. As analyzed in Table 7, the band combinations that consist of bands from different band group can improve the UL coverage by maximizing the antenna gain, reduced IL and avoided SRS switching interruption, at the same time higher throughput can be achieved by fully utilizing the wider channel bandwidth and higher MIMO layers from mid/high frequency range by optimizing UE transceiver path allocation. The proposal is mainly target for new band combinations that may be introduced in 6GR with consideration of new available spectrum resources and the migration of the band combinations defined in NR but not commercialized yet.

Figure 2: Exemplary band group definition

Observation 2-6: With the exemplary band grouping as shown in Figure 2, UE implementation could have more appropriate RF resource allocation for the bands fall in different band groups so that both coverage and throughput performance can be improved especially for mid and high frequency bands since the UE is more likely to support e.g. wider channel bandwidth and higher MIMO layer number.

Some vendors or operators may have some concerns, here we summarize them and share our current thinking:

- The commercialized band combinations in the real network: Those should be kept and migrated to 6GR directly if related bands are re-farmed to 6GR

- Operators have strong demands on some band combinations within one group due to available spectrum resource limitation: Those requests should be considered and treated separately as exceptional cases

- The specification definition: The general principle can be captured in the specification. If all the existing co-existence requirements and RF requirements defined in the specification can meet the support of certain band combinations from both UE implementation and network deployment, it is not necessary to explicitly list them in the specification that can avoid huge band combination tables definition and even additional application tool development as did in NR; Otherwise, if different RF requirements need to be defined due to some regulatory request, specific band combination definition need to be considered. Anyway, those special cases are finite and the overall workload can be manageable.

In a word, even with those exceptions listed above, the heavy workload for band combinations introduction and some RF requirements analysis and definition can still be greatly reduced with consideration of band group concept for band combination introduction in 6GR.

Proposal 2-1: Study band group concept aiming to minimize the performance degradation from normal carrier aggregation so that both coverage and throughput performance can be improved at least for the bands within the same group. Following aspect can be taken into consideration as starting point.

- Operators’ spectrum holding in low, mid and high frequency range

  - Exemplary band grouping

    - Band group#1 – Sub 1GHz, Band group #2 – 1.4~2.2GHz, Band group#3 – 2.3~2.7GHz, Band group#4 – 3.3~5GHz, Band group#5 – 5~7GHz, Band group#6 – 7~15GHz, Band group#7 – 24~71GHz

    - For inter-band group, simultaneous UL and/or DL scheduling in the component bands is allowed just like normal CA operation. For intra-band group, only one band can be scheduled for UL/DL transmission at a time

  - If specific operator request cannot be covered by band group concept e.g. all fall into the same band group, it can be considered as exceptional case

- UE behavior and RF requirements impact

  - If carrier aggregation is configured for bands within a band group, study how to support switching UL/DL transmissions between each two of the bands and how to improve the UL/DL performance by e.g. delta Tib/Rib, MSD etc.

  - Otherwise, study based on normal CA operation UE behavior and RF requirements.

Study to support BCS 4/5 for band combinations

The Bandwidth Combination Set (BCS) was a feature with considerable historical legacy attached to band combinations, dating back to Release 10. It was originally introduced to facilitate chipset development, particularly for devices that might not support all defined channel bandwidths within a band combination.

Unlike LTE, NR supports a wider range of channel bandwidths, with additional CBWs expected to be introduced in future releases. RAN4 experience revealed that the growing variety of BCS led to significant complexity and operational challenges. This issue was compounded by the fact that each BCS was typically defined per spectrum usage scenario at the request of individual operators, often failing to accommodate the full range of deployment needs across different operators.

To address these complications, BCS4 and BCS5 were subsequently introduced. The concept of BCS4/BCS5 are excepted from TR 38.862 as below:

BCS4/BCS5 are allowed to be applied to new band combinations in RAN4 which indicate UE to support up to all of the channel bandwidths for the band in the band combination. BCS4 is release independent to Rel-15 with no new signalling while BCS5 is functionally equivalent to BCS4 except that the new signalling [supportedMinBandwidthDL /supportedMinBandwidthUL] introduced in Rel-17 such as the limitation to the supporting channel bandwidth in each band within the band combination would apply, and BCS5 with the signalling [supportedMinBandwidthDL /supportedMinBandwidthUL] is allowed for early implementation from Rel-15. For a legacy gNB that was not upgraded to understand BCS4 or BCS5 with the new signalling, it would enable to ignore BCS4 or BCS5 with the new signalling.

Ideally, adopting BCS4/BCS5 for all band combinations would significantly reduce the workload in RAN4 related to band combination standardization, while simultaneously accommodating diverse operator requirements. However, whether BCS4/BCS5 can serve as the baseline or default option for all combinations requires further study. Due to implementation constraints, certain band combinations may still require exceptional treatment. Identifying such cases and determining how to address them individually and reflect these exceptions in the specifications should be further discussed.

Proposal 2-2: Study in 6GR whether BCS4/BCS5 could be considered as default option for requested band combinations while exceptions are allowed.

Study generic delta Tib/Rib

Within 3GPP, a vast number of band combinations have been defined to support Carrier Aggregation (CA). In practice, supporting these combinations often requires multiplexers to allow multiple bands to share common antennas, which introduces additional insertion loss. This is particularly challenging for certain complex combinations, such as those involving low-low band pairs, where multiplexer design must also satisfy strict isolation requirements between transmit and receive bands—often resulting in even higher insertion losses.

To account for these implementation-specific losses, RAN4 introduced the parameter of delta Tib and delta Rib, representing additional insertion loss on the transmit and receive paths, respectively. These values are typically defined on a case-by-case basis with a granularity of 0.1dB. However, in the standards development process, it is common practice to refer to similar existing band combinations when specifying these values, leading to inaccuracies and inconsistencies across many combinations.

A more efficient and consistent approach would be to define fixed delta values based on a combinations of frequency ranges, such as low-low, low-mid, or low-high combinations, rather than specifying losses for each individual band combination. Some constructive proposals have already been proposed previously in 5G to simplify delta Tib/Rib values. It is evident that RAN4 has expended considerable effort dealing with these granular band-combination-related issues. Therefore, generic values should be studied and standardized from the very beginning of 6G.

Proposal 2-3: Develop a unified and simplified framework for defining delta Tib/Rib values to streamline CA implementation and reduce standardization complexity.

Conclusion

This contribution provides our consideration on spectrum for 6G.

New spectrum/bands and regulations

- Potential new spectrum for 6G

Observation 1-1: 4.4 – 4.8 GHz, 7.125 – 8.4 GHz and 14.8 – 15.35 GHz were studied in Ｒ19 SI of FS_NR_IMT_4400_7125_14800MHz, where a spectrum like 4.4-4.8GHz didn’t do co-existence study and the co-existence study for all the spectra didn’t consider practical aspects such as higher power classes.

In addition, in R20 RAN led SI (FS_6G_RAN_Scen_Req) and in R20 RAN1 led SI (FS_6G_Radio), some parameters essential for co-existence study such as the number of Tx/Rx, channel bandwidth etc., are under discussion. Therefore, it would be better to do co-existence study based on the new parameters for 6G.

Observation 1-2: Some parameters impacting on co-existence results such as Channel bandwidth, the number of Tx/Rx antennas etc., are under discussion in R20 RAN led SI (FS_6G_RAN_Scen_Req) and in R20 RAN1 led SI (FS_6G_Radio).

With consideration of the observation 1 and 2, it may be better for RAN4 to at least revisit the co-existence study for those frequency ranges.

Observation 1-3: From observation 1 and 2, it would worth revisiting and re-evaluating the SI with new parameters which under discussion in TSG RAN/RAN1.

Observation 1-4: Frequency range of 6425 MHz – 7125 MHz has not been yet fully unleashed and it may be possible to obtain even more suitable requirements like ACLR with consideration of new parameters like channel bandwidth, the number of Tx/Rx antenna etc., under R20 RAN level study item and RAN1 for WG level SI.

Proposal 1-1: Consider at least 4.4 – 4.8 GHz, 7.125 – 8.4 GHz, 14.8 – 15.35 GHz and 6.425 – 7.125 MHz for further study with co-existence evaluation.

- Frequency ranges

Proposal 1-2: RAN4 to conduct studies pertaining to the TN frequency range from a UE and BS implementation perspective, particularly regarding the applicability of conducted requirements.

Observation 1-5: Definitions of FR1 and FR2 frequency ranges for TN and for NTN is already mis-aligned.

Observation 1-6: NTN specification (of FR1-NTN and FR2-NTN) does already cover 7 – 24 GHz range.

Observation 1-7: Range of 10700 – 14500 MHz is already specified to FR2-NTN.

Proposal 1-3: Refrain from introducing FR3 to cover any frequency range within 7 – 24 GHz.

Proposal 1-4: In 6G, keep separate discussion of frequency ranges of TN and NTN.

- In-band spectrum allocation with access restrictions

Proposal 1-5: Study in 6G to develop a unified and extensible signaling mechanism that provides flexible support for regional sub-band allocations within a single band, thereby avoiding fragmented solutions.

- Updated additional spectrum requirements to the defined bands

Observation 1-8: Regulation (e.g., additional spectrum emissions), changes could have impact on RAN4 requirements in terms of such as mandatory/optional, respective applicable release(s) for the applicable operating bands

Observation 1-9:	There are mixed usages of “shall” and “may” in the definitions of the modified MPR behaviour. Different bands and even different bits for the same band follow different rationales, leading to confusion.

Observation 1-10:	For band n41, both bit 0 and bit 1 “shall be set to 1”. It’s unclear how to interpret bit 2 if it’s also set to 1. It’s questionable whether bit 2 can fulfil its intended purpose under the existing definitions.

Observation 1-11: modifiedMPR scheme requires CRs for earlier releases down to the release, where an associated band or feature is introduced.

Proposal 1-6: Issues written in Observations 6-8 should be addressed in 6G with a clean slate.

Band combination definition and simplification

Observation 2-1: Although there are quite a lot of band combinations have been specified and considerable UE capability signalling overheads are needed to support those band combinations, the current deployment in the real network is not that much.

Observation 2-2: With the increasing number of supported band combinations, UE Tx/Rx performance degradation becomes severe due to insertion loss from multiplexer, switch and other additional RF components.

Observation 2-3: Rel-19 Low-Low band CA via switching is an alternative to balance the performance degradation from normal CA and the spectrum aggregation restriction from single carrier operation.

Observation 2-4: Total max throughput of 5.72Gbps for DL and 1.12Gbps for UL can be achieved with 4CC inter-band combinations among different frequency ranges of sub1GHz, 1.5~2.7GHz, 3.3~7GHz and 5~8GHz.

Observation 2-5: Following aspects which could lead to performance degradation when supporting normal CA:

- UE Antenna gain limitation for support more bands with fixed UE physical size

- Maximum output power and REFSENS degradation caused by front-end IL from multiplexer

- REFSENS degradation caused by non-linearity and IMD

- Loss of SRS antenna switching gain caused by interruption and power imbalance across ports

- The balance between the available number of transceiver paths and support CA for band with larger channel bandwidth and MIMO layers.

Observation 2-6: With the exemplary band grouping as shown in Figure 2, UE implementation could have more appropriate RF resource allocation for the bands fall in different band groups so that both coverage and throughput performance can be improved especially for mid and high frequency bands since the UE is more likely to support e.g. wider channel bandwidth and higher MIMO layer number.

Proposal 2-1: Study band group concept aiming to minimize the performance degradation from normal carrier aggregation so that both coverage and throughput performance can be improved at least for the bands within the same group. Following aspect can be taken into consideration as starting point.

- Operators’ spectrum holding in low, mid and high frequency range

  - Exemplary band grouping

    - Band group#1 – Sub 1GHz, Band group #2 – 1.4~2.2GHz, Band group#3 – 2.3~2.7GHz, Band group#4 – 3.3~5GHz, Band group#5 – 5~7GHz, Band group#6 – 7~15GHz, Band group#7 – 24~71GHz

    - For inter-band group, simultaneous UL and/or DL scheduling in the component bands is allowed just like normal CA operation. For intra-band group, only one band can be scheduled for UL/DL transmission at a time

  - If specific operator request cannot be covered by band group concept e.g. all fall into the same band group, it can be considered as exceptional case

- UE behavior and RF requirements impact

  - If carrier aggregation is configured for bands within a band group, study how to support switching UL/DL transmissions between each two of the bands and how to improve the UL/DL performance by e.g. delta Tib/Rib, MSD etc.

  - Otherwise, study based on normal CA operation UE behavior and RF requirements.

Proposal 2-2: Study whether BCS4/BCS5 could be considered as default option for requested band combinations while exceptions are allowed.

Proposal 2-3: Develop a unified and simplified framework for defining delta Tib/Rib values to streamline CA implementation and reduce standardization complexity.

- References

[1] RP-251881, New SID: Study on 6G Radio

[2] R4-2514626, WF on 6G Spectrum

[3] RP-252882 TR 38.914 v0.2.0

[4] RP-251315 Revised WID on Low NR band carrier aggregation via switching, RAN#108, Telus