3GPP TSG-RAN WG4 Meeting #117	R4-2520330

# Dallas, US, November 17th-21st, 2025

Title: 	Consideration on demodulation study for 6G

Source: 	Huawei, HiSilicon

Agenda item:	8.7

Document for:	Discussion

Background

A WF[1] on 6G demod was agreed in last meeting, where many open issues are left for discussion. In this contribution we provide our views on these open issues.

Discussion

General discussion

Many 6G demod topics were raised in last meeting, based on our understanding, some topics are highly relevant to 6G physical channel design, which are currently being discussed in RAN1. RAN4 should postpone these topics until RAN1’s study is stable. We propose RAN4 to only focus on topics that are less dependent on RAN1 design. In table 2-1 we analysed the feasibility of studying these 6G demod topics based on impact of RAN1.

Table 2-1: Summary of RAN4’s proposed demod topics

| RAN4 6G demod topics | Details | Analyse |
| --- | --- | --- |
| General aspects | Waveform and modulation study | Deprioritized, highly dependent on RAN1. |
|  | SCS | Deprioritized, highly dependent on RAN1. |
|  | Demodulation specification principles | Less dependent on RAN1, high priority for study. |
|  | Broadcast and feedback-less channels/signals testing | Need feedback from TE vendor. |
|  | ISAC study | No demodulation impact. |
|  | Conducted and radiated testing | Deprioritize FR3 test methodology |
| Channel models | Channel type, Specialized propagation channels, Frequency related aspects of channel model, Uplink aspects of channel model, AI/ML aspects of channel model, Channel model alignment, PMI bias | Less dependent on RAN1, high priority for study. |
| Receiver assumptions | Receiver assumption for UE, Receiver assumption for BS | Less dependent on RAN1, high priority for study. |
| TxEVM and SNR | Introducing more practical Tx EVM values and define new SNR range. | Deprioritized, highly dependent on RAN1. TxEVM and SNR range are related to new modulation schemes which are currently being discussing in RAN1. |
| Interference modelling aspects | Study more practical interference model. | Deprioritized, highly dependent on RAN1. Interference model need system simulation evaluation, which depends on 6G basic physical structure design such as frequency, SCS, modulation and waveform. |
| Performance testing and requirement | Demodulation and CSI reporting test methodologies | Less dependent on RAN1, high priority for study. |
| New TE functionalities | OLLA, SRS based precoding, time/frequency phase offset pre-compensation, other new TE functionalities | Less dependent on RAN1, high priority for study. |
| UE classification and applicability | UE classification, Applicability rules, Device types | Deprioritized, highly dependent on RAN1 |
| Uplink demod | Digital Pre-Distortion and Post-Distortion Techniques | Deprioritized, discussed in RF firstly. |

Based on the above summary, we propose the following:

Proposal 1:  RAN4 shall treat general following topics as 1st priority

Demodulation specification principles

Channel model

Receiver assumption

New TE functionalities

Demodulation and CSI reporting test methodologies

Proposal 2:  RAN4 shall postpone following topics until RAN1 study is stable.

Waveform and modulation study

SCS

Broadcast and feedback-less channels/signals testing

Introducing more practical Tx EVM values and define new SNR range.

Study more practical interference model.

UE classification, Applicability rules, Device types

Channel model

Channel type

One issue is channel type, the options are:

Channel type • Proposals o Option 1: Clarify use of TDL and SCM models for 6G  Option 1A: Use rCDL baseline for MIMO.  Option 1B: Use CDL/rCDL as baseline for 6G.  Option 1C: Maintain TDL and CDL.  Option 1D: Continue TDL for simplicity.  Option 1E: Include xTDL.  Option 1F: Define default CDL propagation models.  Option 1G: Adoption of SCM should be justified for each test purpose.  Option 1H: Study practical MIMO correlation matrices for TDL.  Option 1I: Use rCDL baseline for MIMO and a limited number of TDL requirements for MIMO features with single layer transmission.  Option 1J: select one channel model (either TDL or CDL) for one specific feature. • Option 1Ja: The criteria of selection should be clarified and applied for all features.

CDL channel exhibits better spatial properties than legacy TDL channel + correlation, which means CDL channel is more suitable for test of the feature that is related to spatial properties, such as high layer cases and MIMO related feature as studied in NR [X]. For other cases with low MIMO layer cases, e.g. PDCCH/PBCH/PDSCH with smaller than 4 MIMO layer cases, legacy TDL channel would be sufficient. Furthmore, CDL channel is not configurable, deterministic spatial properties may make it challengeable to satisfy all test purposes. For example, one advanced receiver exhibiting small gain under CDL-C channel could have sufficient gain under TDL channel by configuring different correlation parameters. xTDL channel is configurable and flexible, the number of clusters and power ratio between the clusters could be adjusted to satisfy the specific test purpose. Hence xTDL channel shall also be introduced as alternative options to ensure the configurability and scalability in case CDL channel doesn’t work.

Proposal 3: RAN4 to introduce TDL, xTDL and CDL channel in 6G with following application cases:

CDL channel is selected only for limited tests of feature related to spatial properties such as high MIMO layer.

TDL channel is selected for cases with low MIMO layers, e.g. PDCCH/PBCH/ PDSCH with smaller than 4 MIMO layer cases.

xTDL channel is selected in case feature cannot be reasonably verified under CDL channel.

Correlation:

Current MIMO correlation matrixes are generated based on XPL or ULA assumption for BS and UE side. However, UE antennas are always non-uniformly distributed. We proposed RAN4 to study more practical UE correlation matrix. From the experience of 6 layers feasibility study, measurement-based correlation matrix could be a starting point. One approach is to measure the UE correlation matrix from channel coefficient of the CDL channel where practical antenna assumptions have been introduced based on v19.0.0 of 38.901. The other approach is to measure the UE correlation matrix in the real field.

Proposal 4:  RAN4 to consider measurement-based UE correlation matrices.

Approach 1: Measure the UE correlation matrix from channel coefficient of the CDL channel where practical antenna assumptions have been introduced based on v19.0.0 of 38.901.

Approach 2:  Companies to provide correlation matrix measured in the real field.

Current BS antenna correlation matrices are derived based on fixed distance between first antenna and last antenna in the array regardless number of antennas. Such assumption leads to large condition number for MIMO correlation matrix when antenna array is very large. Hence, it’s necessary to consider new α1, α2 parameters to make the condition number of MIMO correlation matrix in the reasonable range. One alternative way is to assume fixed distance between two adjacent antennas, as the number of antennas increase.

Proposal 5:  RAN4 to reuse current BS correlation model and consider new α1, α2 parameters for BS correlation matrices for large antenna array. E.g. Assume fixed distance between two adjacent antennas.

Frequency related aspects of channel model

One issue is frequency related aspects of channel model:

- Proposals - Option 1: Study new frequency range. - Option1A: Study new frequency ranges of 6G - Derive CDL for 7-15 GHz. - Option1B: Study FR2. - Option 1C: Evaluate necessity and study spatial channel model for other frequency ranges in 6GR. - Option 2: Study if channel model is agnostic to different carrier frequencies or separate models are needed. - Option 3: Discuss the number of antennas required for different frequency ranges.

In Rel-20, it has been agreed to use single CDL channel model for band agnostic performance requirements. However, angular spread is function of frequency, considering 6G frequency spans more than 5G and the general intention is to define more practical demodulation requirements, it’s necessary to study and compare CDL channel spatial properties for different typical 6G band before making the decision whether unified or separate channel models are needed.

Proposal 6:  RAN4 to further study and compare channel spatial properties for different typical 6G bands before making the decision whether unified or separate channel models are needed.

Uplink aspects of channel model

One issue is whether to study UL CDL for BS variants:

- Proposals - Option 1: Study UL CDL for BS variants. - Option 1a: Study UL CDL for BS variants if key issues in DL CDL study are settled.

Our understanding is that UL CDL is simple conversion of DL CDL, RAN4 could focus on DL CDL study and simplify apply the outcome to UL CDL.

Proposal 7:  RAN4 to focus on DL CDL study, while UL CDL study could be simplified by leveraging the outcome of DL CDL channel study.

Channel model alignment

One issue is channel model alignment, the proposal is:

- Proposals - Option 1: Study how to ensure the alignment of CDL implementation. Channel properties such as PSD, time correlation coefficient and frequency correlation coefficient may need to be aligned apart from SNR points alignment.

In Rel-19 SCM study, the number of cases for simulation results alignment for CDL channel is quite limited, only aligning the target SNR point may not be sufficient. To better verify the CDL implementation alignment across companies, channel properties such as power spectrum distribution, time correlation coefficient and frequency correlation coefficient should also be considered as metric for alignment judgement.

Proposal 8:  Include channel properties such as power spectrum distribution, time correlation coefficient and frequency correlation coefficient as metric for alignment judgement.

PMI bias

One issue is PMI bias:

- Proposals - Option 1: Consideration of CDL modeling in 6GR for PMI report evaluation should follow the identification of robust countermeasures to prevent overfitting to deterministic channel behavior.

This issue is being discussed in WI performance evolution phase 6, which is expected to be discussed in 6G SI only if there is no effective countermeasure found in WI performance evolution phase 6.

Proposal 9: RAN4 to start discussing PMI bias issue in 6G when there is no effective countermeasure found in WI performance evolution phase 6.

UE antenna modelling for CDL

UE antenna placemen and radiation power pattern were newly defined in latest version of 38.901, following summarizes the new part:

- Two UE antenna models are defined in latest version of 38.901: Handheld UE and FWA.

- Follow text summarizes the antenna models for handheld UE:

- Antenna placement candidate locations and some typical cases for orientation are specified

- Polarized antenna modelling is specified

- Radiation power pattern of a single antenna element is specified.

For antenna models for FWA, antenna placement candidate locations are specified. Figure 2-2 illustrates the antenna placement candidate locations for handheld UE and FWA:

Figure 2-2: Antenna placement candidate locations for handheld UE and FWA

Based on our understanding, such new UE antenna assumptions could better reflect the real UE implementation, which should be as a reference for 6G CDL model.

Proposal 10:  6G CDL model should be created referring to newly UE antenna assumption defined in v19.0.0, 38.901.

Specialized propagation channels

One issue is whether to study specialized propagation channels:

- Proposals - Option 1: Evaluate candidate channel model for DL and UL considering new use cases including AI, ISAC, NTN, HST. - Option 2: Initiate a similar study for NTN scenarios, focusing on the CDL-D variant to reflect the LOS-dominant nature of NTN links and enable more accurate performance evaluations for NTN systems under practical deployment scenarios.

We think it’s value to do this study, but we prefer to focus on basic CDL channel study at the 1 day of 6G and postpone the study until basic 6G CDL channel is stable.

Proposal 11:  RAN4 to focus on basic CDL channel study firstly and postpone the study until basic 6G CDL channel is stable.

Receiver assumption

One issue is receiver assumption for UE, the candidate options are:

Receiver assumption for UE - Proposals - Option 1: MMSE-IRC as a baseline receiver. - Option 2: MMSE-IRC and R-ML as baseline receivers. - CMCC: with the prerequisite that the receiver is transparent to the network and does not require any PHY layer modification and additional assistance information. - Option 3: Cover advanced receivers (R-ML, soft-IC) - CT: Study the required information for advanced Rec for MU-MIMO. - Option 4: Study baseline and simplified structures. - Option 5: Study widely linear MMSE-IRC.

MMSE-IRC is used as baseline receiver for both demodulation and CQI calculation in NR and LTE. In NR, R-ML receiver is selected as optional feature for few PDSCH demodulation cases with up to 4 layers, but not used for CSI reporting and adaption link PDSCH requirements. Considering R-ML receiver is widely implemented in most UE vendors and for the purpose of defining more practical requirements, we see the need to study the feasibility of defining R-ML receiver as baseline. Mentioning that there are various reduced complexity methods for Maximum likelihood receiver, it is questionable whether the companies’ results can be well aligned, especially for higher layers (8 layers) and close-loop tests.  We propose RAN4 to study the feasibility of defining R-ML receiver for both demodulation and CSI reporting as baseline under both open loop and link adaption test conditions. The study should focus on possibility of alignment.

Observation 1: In NR, R-ML receiver is selected as optional feature for few PDSCH demodulation cases with up to 4 layers, but not used for CSI reporting and adaption link PDSCH requirements.

Proposal 12:  For UE receiver assumption, study the feasibility of defining R-ML receiver for both PDSCH and CSI reporting including both open-loop PDSCH test cases and link adaption PDSCH test cases, where R-ML receiver are assumed for both demodulation and CSI calculation. The study should focus on possibility of alignment.

Mentioning that RAN1 is discussing geometric shaping and probabilistic shaping which highly impact R-ML implementation. It’s proposed to study R-ML receiver based on 6G modulation schemes once RAN1’s study become stable.

Proposal 13:  RAN4 to study R-ML receiver based on 6G modulation schemes including geometric shaping and probabilistic shaping once RAN1’s study become stable.

One issue is receiver assumption for BS, the candidate options are:

Receiver assumption for BS - Proposals - Option 1: MMSE-IRC as a baseline receiver. - Option 2: Study feasibility of considering higher than 8Rx scenarios.

MMSE-IRC is widely used in BS side, option 1 is confirmed. For option 2, the limitation of up to 8Rx requirements in NR is unacceptable complexity of matrix inversion operation on Ruu matrix in the simulator when number of Rx is higher than 8.

Proposal 14: Confirm MMSE-IRC as BS baseline receiver.

Observation 2: The limitation of up to 8Rx requirements in NR is unacceptable complexity of matrix inversion operation on Ruu matrix in the simulator when number of Rx is higher than 8.

Based on observation 2, we think it’s challengeable to consider higher than 8Rx scenarios unless the simulator has made significant improvements.

Proposal 15: Not to consider higher than 8Rx BS requirements in 6G.

Test framework

Test scope for demodulation testing and CSI reporting testing

One issue is test scope for demodulation testing and CSI reporting testing:

Demodulation testing One issue is demodulation testing, the options are listed as following: - Proposals - Option 1: For 6G demodulation study, use FRC style, MCS value, fixed rank, fixed channel bandwidth, fixed subframe configuration as a starting point. - Option 2: Study extending scope of demodulation tests with link adaptation. - Option 3: Discuss the additional margins and measurements uncertainty for requirements definition of 6GR. - Option 4: Discuss the SNR derivation procedure for 6GR, the span of ideal results span is <= [X] dB. - Proposals - Option 1: Streamline CQI reporting testing into 1-step approach and setting requirements in terms of throughput/SNR and BLER limits. - Option 2: Study the necessity of CQI reporting requirements in addition to combined demodulation and link adaptation testing. - Option 3: Simplify the PMI reporting testing process and setting requirements directly in terms of throughput/SNR instead of measuring γ. - Option 4: Study the necessity of PMI reporting requirements in addition to combined demodulation and link adaptation testing. - Option 5: Study RI reporting requirements test metrics and test methodologies. - Option 5A: Investigate alternative metric that measures consistency and accuracy of Rank Indicator (RI).

All RAN1 evaluation are based on link adaption, which complies with the conditions in the field. Considering in the market the KPI is overall performance rather than the performance of individual module, it would be more reasonable to replace individual CQI, PMI and RI tests by link adaption cases. Generally, we propose to category 6G demod tests into open-loop PDSCH cases which is used to verify demodulation performance and link adaption which is used to verify CSI reporting performance.

Proposal 16: Replace individual CQI, PMI and RI tests by link adaption cases.

Proposal 17: Category 6G demod tests into open-loop PDSCH cases which is used to verify demodulation performance and link adaption which is used to verify CSI reporting performance.

New TE functionalities

One issue is new TE functionalities, the options are captured as follows:

OLLA with link adaptation - Feature Lead notes - Option 1 discussion has been captured in Chairman and Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on OLLA in the next meeting as several companies see value to study OLLA as part of 6G demod SI. - Proposals - Option 1: Study to include OLLA in ATP requirements. - Option 1A: Use proposed OLLA model from R4-2300703 as a starting point. - Option 1B: Study the feasibility to include OLLA in ATP requirements. SRS based precoding - Feature Lead notes - Option 1 discussion has been captured in Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on SRS based precoding in the next meeting as several companies see value to study SRS based precoding as part of 6G demod SI. - Proposals - Option 1: Study whether and how to define baseline SRS based precoding procedure in TE to enable aligned simulation assumptions. Time/frequency/phase offset precompensation - Feature Lead notes - Discussion of Option 1 has been captured in Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on time/frequency/phase offset precompensation in the next meeting as several companies see value to study time/frequency/phase offset precompensation as part of 6G demod SI. - Proposals - Option 1: Study inclusion of higher layer aspects in demodulation requirements via dynamic TE decisions using known algorithms, e.g., applying timing offset reports (CJT). Other new TE functionalities - Feature Lead notes - Discussion of following options has been captured in Ad-hoc minutes. - Proposals - Option 1: Study inclusion of higher layer aspects in demodulation requirements via increased and dynamic application of DUT feedback in the TE. - Option 2: Study inclusion of higher layer aspects in demodulation requirements via dynamic TE decisions using known algorithms, e.g., SU/MU scheduling, dynamic resource allocation/slots. - Option 3: Explore utilizing test equipment algorithms to evaluate features that rely on network-side processing, enabling realistic UE performance assessment without limiting network implementation flexibility. - Option 4: Identify features dependent on network-side processing and aim to define corresponding performance requirements, where feasible, using test equipment-based evaluation methods

Regarding OLLA, the challenge is standardization of OLLA algorithm, which fully depends on the BS implementation and it is questionable if the OLLA algorithm for testing could represent most BS implementation. Hence the OLLA study needs to consider practicality and complexity of TE implementation is also to be considered. For option 1A, it would be premature to consider any specific OLLA algorithm as baseline now since we need more time for more contribution’s input especially from BS vendors.

Proposal 18: Regarding OLLA, encourage BS vendors to provide proposed OLLA algorithms with practicality and complexity of TE implementation to be considered rather than referring any specific OLLA algorithm as baseline.

Regarding SRS based precoding, SRS based channel estimation and precoding make demodulation test more uncertain. To avoid the uncertainty as much as possible, TE should skip channel estimation and use ideal channel coefficient to calculate precoder matrix. Also, specific precoding algorithm needs to be standardized, e,g, SVD based precoding and it’s also proposed to consider both SU-MIMO and MU-MIMO scenario.

Proposal 19: Regarding SRS based precoding, TE should skip channel estimation and use ideal channel coefficient to calculate precoder matrix. Also, specific precoding algorithm with SU-MIMO and MU-MIMO needs to be standardized, e,g, SVD based precoding.

Conclusion

In this contribution we provide our views on 6G demodulation topics. The proposals are:

General:

Proposal 1:  RAN4 shall treat general following topics as 1st priority

Demodulation specification principles

Channel model

Receiver assumption

New TE functionalities

Demodulation and CSI reporting test methodologies

Proposal 2:  RAN4 shall postpone following topics until RAN1 study is stable.

Waveform and modulation study

SCS

Broadcast and feedback-less channels/signals testing

Introducing more practical Tx EVM values and define new SNR range.

Study more practical interference model.

UE classification, Applicability rules, Device types

Channel model:

Proposal 3: RAN4 to introduce TDL, xTDL and CDL channel in 6G with following application cases:

CDL channel is selected only for limited tests of feature related to spatial properties such as high MIMO layer.

TDL channel is selected for cases with low MIMO layers, e.g. PDCCH/PBCH/ PDSCH with smaller than 4 MIMO layer cases.

xTDL channel is selected in case feature cannot be reasonably verified under CDL channel.

Proposal 4:  RAN4 to consider measurement-based UE correlation matrices.

Approach 1: Measure the UE correlation matrix from channel coefficient of the CDL channel where practical antenna assumptions have been introduced based on v19.0.0 of 38.901.

Approach 2:  Companies to provide correlation matrix measured in the real field.

Proposal 5:  RAN4 to reuse current BS correlation model and consider new α1, α2 parameters for BS correlation matrices for large antenna array. E.g. Assume fixed distance between two adjacent antennas.

Proposal 6:  RAN4 to further study and compare channel spatial properties for different typical 6G bands before making the decision whether unified or separate channel models are needed.

Proposal 7:  RAN4 to focus on DL CDL study, while UL CDL study could be simplified by leveraging the outcome of DL CDL channel study.

Proposal 8:  Include channel properties such as power spectrum distribution, time correlation coefficient and frequency correlation coefficient as metric for alignment judgement.

Proposal 9:  RAN4 to focus on basic CDL channel study firstly and postpone the study until basic 6G CDL channel is stable.

Proposal 10:  RAN4 to start discussing PMI bias issue in 6G when there is no effective countermeasure found in WI performance evolution phase 6.

Proposal 11:  6G CDL model should be created referring to newly UE antenna assumption defined in v19.0.0, 38.901.

Receiver assumption:

Observation 1: In NR, R-ML receiver is selected as optional feature for few PDSCH demodulation cases with up to 4 layers, but not used for CSI reporting and adaption link PDSCH requirements.

Proposal 12:  For UE receiver assumption, study the feasibility of defining R-ML receiver for both PDSCH and CSI reporting including both open-loop PDSCH test cases and link adaption PDSCH test cases, where R-ML receiver are assumed for both demodulation and CSI calculation. The study should focus on possibility of alignment.

Proposal 13:  RAN4 to study R-ML receiver based on 6G modulation schemes including geometric shaping and probabilistic shaping once RAN1’s study become stable.

Proposal 14: Confirm MMSE-IRC as a baseline BS receiver.

Observation 2: The limitation of up to 8Rx requirements in NR is unacceptable complexity of matrix inversion operation on Ruu matrix in the simulator when number of Rx is higher than 8.

Proposal 15: Not to consider higher than 8Rx BS requirements in 6G.

Test framework

Proposal 16: Replace individual CQI, PMI and RI tests by link adaption cases.

Proposal 17: Category 6G demod tests into open-loop PDSCH cases which is used to verify demodulation performance and link adaption which is used to verify CSI reporting performance.

New TE functionalities

Proposal 18: Regarding OLLA, encourage BS vendors to provide proposed OLLA algorithms with practicality and complexity of TE implementation to be considered rather than referring any specific OLLA algorithm as baseline.

Proposal 19: Regarding SRS based precoding, TE should skip channel estimation and use ideal channel coefficient to calculate precoder matrix. Also, specific precoding algorithm with SU-MIMO and MU-MIMO needs to be standardized, e,g, SVD based precoding.

Reference

[1] R4-2514588 WF on [116bis][106] 6G Demod