3GPP TSG-RAN WG4 Meeting #117												            R4-2520334

Dallas, USA, 17 – 21 November, 2025

Agenda item:			8.11

Source:	Huawei, HiSilicon

Title:	Further consideration on testability and OTA study for 6G

Document for:	Approval

Introduction

RAN4 #116bis agrees broad scope for testability and OTA [1]. This contribution addresses some of the issues in more details.

Discussion

For issue 1-2-1 in OTA WF [1], the guiding principle should be to use existing TRP and TRS measurement setups to accommodate some of tests currently performed by conducted measurements. For example, on transmit side, EVM, out of band emission and spurious emission can be readily included in TRP test setups. Given that it is possible to perform some tests in OTA setups, it does not mean that it is necessary to do those tests under OTA. Because antennas do not introduce much nonlinearity in the context of EVM, out of band emission or spurious emissions, which are dominated by filter performance or power amplifier nonlinearity. Namely, analyses on a case by case basis are needed before deciding what conducted tests are to be performed under OTA setups.

On receive side, TRS already measures receive sensitivity. Tests associated with blocking could, in theory, be included in TRS setup by combining the blocking signals with those transmitted by the link antenna. However, test labs have their own arrangement for link antennas. Some have them underneath the turn table, while others place the link antenna above the turn table. There is also an issue with blocking power calibration to ensure that the same power level reaches the device antenna regardless of test lab setups. This is more difficult to achieve because the device received power under OTA tests is a convolution of antenna patterns between the device receive antenna and the link antenna. This implies that all test labs need to use the same type of link antenna and placed it in the same spatial location relative the device under test.

In addition, with blocking signal power and link antenna position fixed, difference devices may not experience identical blocking power level due to the facts antenna locations vary from device to device. This would increase the associated measurement uncertainty.

Observation 1: it is more difficult to introduce variations to existing TRP and TRS test setups to accommodate tests such as blocking for FR1 frequency range.

Proposal 1: if justified, it is possible to introduce some OTA tests using current TRP and TRS test setups, e.g. EVM, out of band or spurious emission tests for FR1 frequency range.

Transmit antenna switching has been an issue for TRP tests in both LTE and NR. Current test methodologies disable transmit antenna switching and measure the switching antennas separately. As antenna switching becomes more and more common in devices, it is high time to develop a suitable test method for it.

Proposal 2: study and develop a test method to measure TRP with transmit antenna switching on.

Issue 1-2-1: Whether it is valuable to improve some conducted requirements to be verified via OTA approach

Agreements:

- RAN4 can further study the testability limitations of some conducted test cases, consider the following aspects as starting point, e.g.,

  - FR1 conducted test cases already consider antenna-performance impacts but not verified via radiated approach in a case-by-case manner, e.g. MSD

  - FR1 test case simplification with both conducted and radiated considered, e.g., spurious emission

  - Discuss which conducted conformance testing could either yield testability issues, e.g., lack of physical Tx/Rx ports, and/or highly dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern.

For issue 2-1-1 in OTA WF [1], conducted tests should be used for new “FR1” frequency range. Depending on the outcome of discussion under spectrum, the new “FR1” frequency range would likely be below 15 GHz. In such frequency range, antennas are unlikely to be packaged together with chipsets as done in NR FR2 frequency range under Antenna In Package (AIP) scheme because AIP is typically used above 20 GHz. Namely a test port is still possible behind the antenna and would not cause significant impact on RF performance such as impedance mismatch, etc. for frequencies below 15 GHz.

Proposal 3: use conducted tests for cases where antennas are accessible, for example, below 15 GHz.

For the devices where antennas are not accessible, e.g. under AIP, or creation of an antenna port would significantly degrade the existing RF performance, e.g. due to impedance mismatch, OTA tests should be adopted. Most FR2 OTA test methods in Release 19 can be re-used or modified for 6G OTA tests.

Proposal 4: For the devices where antennas are not accessible or creation of an antenna port would significantly degrade the existing RF performance, for example, above 15GHz, OTA tests should be used.

Issue 2-1-1: Testability for New Frequencies between FR1 and FR2

Agreements:

RAN4 study the testability including conducted testing and radiated testing for the new frequencies. The following aspects can be considered:

- The frequency range definition will be discussed under the spectrum agenda.

- For conducted test, use existing test method as a starting point

- For OTA test

  - Further study potential test methodologies including the full-package, i.e., supported frequency range, test setup, configuration, positioning, procedure, validation/calibration, quiet-zone/test zone, MU, testing time reduction

For issue 4-1-1 in OTA WF [1], discussion on UE types has been deferred to RAN#110 in December. However, NR UE types include FWA, handheld devices, RedCap, vehicle UE, high speed train roof mounted UE, etc. From OTA perspective, different device types simply mean various device sizes. OTA test systems can therefore be designed to accommodate the largest device size if the desire is to use one test setup to cover all device types. It is difficult to perform in-situ tests for vehicle UE, high speed train roof mounted UE. Perhaps they can be ruled out in the initial phase of 6G OTA tests.

Proposal 5: depending on RAN plenary decision on 6G device types, a single OTA test system is feasible if it can be designed to accommodate the maximum size of all device types.

Issue 4-1-1: OTA testability applicability for different UE types in 6GR day-1

Agreements:

- RAN4 consider the study of OTA test system to better accommodate different UE types, new feature/functionalities, new performance metrics and test cases.

- The UE types and form factor considered in testability can be further discussed in this SI.

- RAN4 study the feasibility of developing a single system to cover above aspects

For issue 5-1-1, WF on 6G AI [7] identified a few use cases. However, all the use cases can be tested under conducted measurement. For example, demodulation tests or DPD/DPoD performance on power amplifiers. Namely, current scope of 6G AI use cases does not seem to require OTA tests. Obviously, any of the 6G AI use cases can be tested under OTA. However, OTA tests may not offer additional performance insight or include major performance impacting factors which are not covered by conducted tests.

Observation 2: 6G AI use cases do not seem to require OTA tests.

Issue 5-1-1: RAN4 study on enhancement of AI/ML testability for 6GR

Agreements:

- The corresponding test method including both OTA and conducted, to verify the AI/ML features (FFS details) should be studied in RAN4 in 6G SI.

  - The methodologies identified in 5G will be considered as the starting point for both conducted and OTA

For issue 6-1-1, same device types, e.g., handheld UE, used for both TN and NTN under similar frequency ranges can be tested under a single test system such as anechoic chambers, albeit care should be taken on large round trip time in NTN scenarios. However, attachment such as VSAT to NTN devices tends to be larger than handheld devices and may need a different test system other than anechoic chambers. Test systems such as IFF suitable for testing VSAT is also applicable handheld devices. However, IFF and anechoic chambers may have different cost structures. As a result, a single test system can accommodate majority of test cases for both TN and NTN, while exception such as VSAT should also be allowed.

Observation 3: it is possible to design a test system to accommodate OTA tests for both TN and NTN either in the form of anechoic chambers or IFF.

Issue 6-1-1: Harmonized OTA test methodologies for TN and NTN

Agreements:

- Targeting a harmonized test system/test method for TN and NTN devices for FR1.

  - The test complexity and cost should be considered.

For issue 8-1-1, 3D probe setups in anechoic chambers for MIMO OTA tests could produce more precise channel emulation than 2D probe setups. Typical 3D probe configurations are multiple rings of probes [2, 3] or a sector of probes [4]. However, the benefit of more precise channel emulation in 3D probes may be outweighed by its implementation cost. Besides, the main aim of 6G SI is to come up with new test methodologies rather than channel emulation improvement.

Proposal 6: do not consider 3D probe setups for MIMO OTA tests.

Browsing mode is one of the popular positions for users to hold their devices when reading. MIMO OTA tests up to release 19 have used free space mode. In addition, the positioning guideline for browsing already exists for TRP and TRS tests and can be reused for MIMO OTA tests. The remaining task would be to define a measurement campaign under dynamic channels. The same performance metric for free space mode, if agreed, can also be used for browsing mode. In short, inclusion of browsing mode performance for MIMO OTA would not consume a lot of meeting time.

Proposal 7: include browsing mode in MIMO OTA measurement campaign under dynamic channels.

There are numerous articles published in communication conferences and journals on potential 6G radio access network architecture. A common theme among them is cell free architecture [5, 6], where a device would be connected to more than one base stations or multiple TRP (Transmit Receive Point). In addition, NR also supports multi-TRP functionality under current NR network architecture. Together with dynamic channels, the inclusion of multi-TRP in MIMO OTA would cover a key aspect of MIMO OTA performance in 6G.

Proposal 8: include multi-TRP in MIMO OTA test methodology development under dynamic channels.

Issue 8-1-1: RAN4 consider MIMO OTA for 6GR day-1

Agreements：

RAN4 will study the dynamic MIMO OTA (at least dynamic channel model and link adaptation) for 6G. static MIMO is not precluded. The following can be considered as starting point:

- 2D or 3D channel model

- FFS phantom involved testing

- FFS multi-TRP

- FFS focus on FR1 as first priority

  - FR2 and new frequency range may also be considered

Conclusion

This contribution provides the following proposals and observations.

Observation 1: it is more difficult to introduce variations to existing TRP and TRS test setups to accommodate tests such as blocking for FR1 frequency range.

Observation 2: 6G AI use cases do not seem to require OTA tests.

Observation 3: it is possible to design a test system to accommodate OTA tests for both TN and NTN either in the form of anechoic chambers or IFF.

Proposal 1: If justified, it is possible to introduce some OTA tests using current TRP and TRS test setups, e.g., EVM, out of band or spurious emission tests for FR1 frequency range.

Proposal 2: Study and develop a test method to measure TRP with transmit antenna switching on.

Proposal 3: Use conducted tests for cases where antennas are accessible, for example, below 15 GHz.

Proposal 4: For the devices where antennas are not accessible or creation of an antenna port would significantly degrade the existing RF performance, for example, above 15GHz, OTA tests should be used

Proposal 5: Depending on RAN plenary decision on 6G device types, a single OTA test system is feasible if it can be designed to accommodate the maximum size of all device types.

Proposal 6: Do not consider 3D probe setups for MIMO OTA tests

Proposal 7: Include browsing mode in MIMO OTA measurement campaign under dynamic channels.

Proposal 8: Include multi-TRP in MIMO OTA test methodology development under dynamic channels.

References

- R4-2514647 WF for [116bis][110] 6G testability and OTA

- ”Robustness Analyses of 3D Probe Configuration for Different Channel Models in MIMO OTA Setups”, X. Huang, et al, 2021 IEEE International Workshop on Electromagnetics: Applications and Student Innovation Competition

- ” Time-Domain Nonstationary Channel Emulation in Multiprobe Anechoic Chamber Setups for Over-the-Air Testing”, Z. Jiang, et al, IEEE ANTENNAS AND WIRELESS PROPAGATION LETTERS, VOL. 20, NO. 12, DECEMBER 2021, pp.2511-2515

- ”On Radiated Performance Evaluation of Massive MIMO Devices in Multiprobe Anechoic Chamber OTA Setups”, P/ Kyosti, et al, IEEE TRANSACTIONS ON ANTENNAS AND PROPAGATION, VOL. 66, NO. 10, OCTOBER 2018, pp.5485-5497

- ”Interdependent Cell-Free and Cellular Networks: Thinking the Role of Cell-Free Architecture for 6G”, Y. Zhang, et al, IEEE Network, Vol.38, Issue 5, 2024, pp.247-254

- ” A Self-Driven Virtual Elastic Infrastructure for Cell-Free Based 6G Networks”, J. Vardakas, et al, IEEE Wireless Communications, April 2025, pp.188-195

- R4-2514637 WF on 6G AI