3GPP TSG-RAN WG4 Meeting # 117                                       R4-2520345

Agenda Item: 8.8.2

Source: MediaTek Inc.

Title: Views on 6G AI/ML

Document for: Discussion

# Introduction

In RAN4#116bis, many 6G AI/ML use cases were proposed and discussed [1]. As guided, in this paper, we would like to provide our views on some of the 6G AI/ML use cases.

# Discussion

## 2.1 AI-nonlinearity compensation

According to the proposals and discussions last meeting, now there are 4 sub-use cases for AI-nonlinearity compensation on table. Two are for UL (UE: AI DPD & NW: AI-DPoD) and another two are for DL (NW: AI DPD & UE: AI-DPoD).

AI-nonlinearity compensation for DL

Due to limited transmission power, cost and size at UE, UL is usually the bottleneck. But BS is more powerful than UE. As far as we know, there is no obvious nonlinearity issue for DL. Usually, BS transmits with fixed power. The deployment will take BS realization and coverage into consideration. The use of high order demodulation mainly depends on SINR. It is not clear to us about the motivation of AI-nonlinearity compensation for DL. We suggest deprioritizing the study on AI-nonlinearity compensation for DL before the motivation is well justified.

Proposal 1: Deprioritize the study on AI-nonlinearity compensation for DL before the motivation is well justified.

AI-nonlinearity compensation for UL

Usually, DPD is adopted at transmitter to compensate PA non-linearity. On the other hand, at NW side, BS can use DPoD to recover the signal, even if there is some nonlinear distortion at Tx side. So both AI DPoD at NW and AI DPD at UE are proposed last meeting.

- AI DPoD at NW

For AI DPoD at NW, there is a parallel discussion on non-AI DPoD. In the early stage, RAN4 need to discuss non-linearity model(s) of transmission signals which is common for both non-AI DPoD and AI DPoD. As agreed last meeting, non-linearity model(s) will be discussed in other 6G agenda (e.g., system parameter agenda and UE RF agenda). After that, it seems there is divergence on having a joint session or separate session of AI and non-AI DPoD at NW. We are kind of open. But we believe that the performance will be compared with non-AI DPD in the end no matter AI and non-AI DPoD at NW are discussed jointly or separately. After the study, RAN4 shall conclude whether to differentiate AI DPoD and non-AI DPoD when define related requirements and signalling.

Proposal 2: Open to have a joint session or separate session of AI and non-AI DPoD at NW.

Proposal 3: AI DPoD at NW shall be compared with non-AI DPoD in the end.

- AI DPD at UE

RAN4 only defines minimum requirement, UE can always do better, no matter adopting AI solution or non-AI solution. Without specification, UE can still use AI based DPD. 6G Day-1 should focus on the features that require specification. On another aspect, similar as AI DPoD at NW, the performance shall be compared with non-AI DPD as well. But there is no non-AI DPD benchmark now. Considering the above, we propose not to study AI based DPD at UE.

Proposal 4: Not to study AI based DPD at UE.

## 2.2 AI/ML use case for RRM

Last meeting, plenty of AI/ML use cases for RRM were proposed. As guided, we would like to provide our views on the proposed use cases.

### 2.2.1 L1 beam measurement related AI/ML use cases

As summarized in RAN1 moderator summary [2], RAN1 is discussing the following BM related AI/ML use cases.

It can be observed that most of the proposed use cases are already covered by R19 AI/ML BM or overlapped with the use cases discussing in RAN1. Specifically,

- Time domain (intra-cell): The use case seems the same as R19 AI/ML BM Case 2.

- Time and spatial domain (intra-cell): The use case seems the same as R19 AI/ML BM Case 2 when set A and set B are different.

- Freq. and spatial domain (inter-cell): Same as Sub-case B in RAN1 discussion.

- Spatial domain (inter-cell): Covered by Sub-case A in RAN1 discussion.

- Time and spatial domain (inter-cell): Covered by Sub-case A in RAN1 discussion.

We suggest avoiding duplicated discussion in different WGs. In addition, the above use cases are mainly for FR2 in our understanding. In R19 AI/ML BM, testability is still in evaluation. We prefer to solve the testability issue at first.

Based on the above analysis, we propose not to discuss these L1 beam measurement related AI/ML use cases in RAN4.

Proposal 5: Not to discuss the listed L1 beam measurement related AI/ML use cases in RAN4:

- Time domain (intra-cell): Same as R19 AI/ML BM Case 2.

- Time and spatial domain (intra-cell): Same as R19 AI/ML BM Case 2 when set A and set B are different.

- Freq. and spatial domain (inter-cell): Same as Sub-case B in RAN1 discussion.

- Spatial domain (inter-cell): Covered by Sub-case A in RAN1 discussion.

- Time and spatial domain (inter-cell): Covered by Sub-case A in RAN1 discussion.

### 2.2.2 L3 beam measurement related AI/ML use cases

In our view, the use cases can be categorized to three types:

- Extending the use cases in R20 AI mobility from cell-level to beam-level.

From our perspective, there is no fundamental difference between cell-level and beam-level prediction. Furthermore, there is an ongoing discussion in RAN2 on how to harmonize L1 and L3 measurement procedure. Therefore, it is not necessary to have a study phase for those use cases.

- Hybrid use cases of R20 AI mobility, e.g., time domain + frequency domain.

For this kind of use cases, it is better to start the study after there is sufficient progress in R20 AI mobility.

- Once discussed in RAN2 but not included in R20 AI mobility.

We are not sure whether these use cases will be discussed in RAN2 again. Therefore, in RAN4 it is fine to keep them on the WF, but we need to coordinate with RAN2 to avoid duplicating works.

For the proposed use cases, they can be categorized to:

- Time domain (intra-cell): Type#1

- Time and spatial domain (intra-cell): Type#2,3

- Freq. and spatial domain (inter-cell): Type#2,3

- Time and freq. domain (inter-cell): Type#2

- Freq. domain prediction for non-collocated cells (inter-cell): Type#3

- Freq. domain prediction for N carriers single cell: Type#1

Based on the above analysis, we propose not to deprioritize these L3 beam measurement related AI/ML use cases in RAN4.

Observation 1: For use cases that just extends the use cases in R20 AI mobility from cell-level to beam-level, there is no fundamental difference between cell-level and beam-level prediction. Furthermore, there is an ongoing discussion in RAN2 on how to harmonize L1 and L3 measurement procedure.

Proposal 6: For use cases that just extends the use cases in R20 AI mobility from cell-level to beam-level, it is not necessary to have a study phase.

Proposal 7: For hybrid use cases of R20 AI mobility, it is better to start the study after closing the work in R20 AI mobility.

Proposal 8: Deprioritize the listed L3 beam measurement related AI/ML use cases in the study phase in RAN4:

- Time domain (intra-cell)

- Time and spatial domain (intra-cell)

- Freq. and spatial domain (inter-cell)

- Time and freq. domain (inter-cell)

- Freq. domain prediction for N carriers single cell

### 2.2.3 L3 cell measurement related use cases

For the proposed use cases, time domain (intra-cell) prediction and freq. domain (inter-cell) prediction are already included in R20 AI mobility. For the use case “Freq. domain prediction for non-collocated cells (inter-cell)”, we agree it is an important use case that should be studied. For frequency domain prediction, R20 AI mobility focus on co-located scenarios. However, after some study, we found that if only co-located scenarios are considered, the frequency domain RSRP predictions becomes very straightforward. As the example shown in below figure, the RSRPs of the same site from 2 different frequency have a very obvious correlation. In this case, the prediction seems to be too easy and it may cause some problem when RAN4 wants to design a test to differentiate AI and non-AI UEs.

RSRP of co-located cell pairs

UE trajectory

Therefore, we believe that it is important to study non-co-located scenario for the use case of frequency domain RSRP predictions. But the use case was once discussed in RAN2 but not included in R20 AI mobility. We are not sure whether these use cases will again be discussed in RAN2. Therefore, in RAN4 it is fine to keep them on the WF, but we need to coordinate with RAN2 to avoid duplicating works.

Proposal 9: For the use case once discussed in RAN2 but not included in R20 AI mobility, RAN4 keeps them tentatively in the WF, but need to later coordinate with RAN2 to avoid duplicating work.

Proposal 10: For L3 cell measurement with freq. domain (inter-cell) prediction, it is important to also consider non-collocated scenarios. But due to the uncertainty of RAN2 study, this category of use case can be postponed now.

Proposal 11: No need to study L3 cell measurement with time domain (intra-cell) prediction and freq. domain (inter-cell, co-located) prediction which are already included in R20 AI mobility.

### 2.2.4 Multi-Domain L3 level Prediction for Measurement-Gap Reduction

If we are correct, this use case is also a hybrid use case of R20 AI mobility, i.e., time domain + frequency domain + spatial domain.

We prefer to start the study after there is sufficient progress in R20 AI mobility.

Proposal 12: Start the study on the use case “Multi-Domain L3 level Prediction for Measurement-Gap” after closing the work in R20 AI mobility.

### 2.2.5 Dynamic Adaptation of Measurement Procedure

This use case is proposed by us. We would like to explain the motivation of this use case.

It is observed  that UE may not detect any cell on some configured inter-frequency in the field. The scenario may be as shown in Fig.1. f2 is deployed for hot spots. f1 is deployed for coverage. There are some areas that f2 is not covered. But NW has no information of UE location. NW configures inter-frequency measurement on f2 regardless UE location. According to current requirements, even UE is not in the coverage area of f2, UE still needs to measure f2 periodically. The overall measurement delay is extended unnecessarily.

With the help of AI, UE could be able to identify whether it is in the coverage of f2 by simply measuring f1. With such a knowledge, UE can choose not to measure f2 or measure f2 less frequently. Instead, UE can measure other inter-frequencies more frequently or save some power by skipping some measurements. Such an AI-assisted measurement scheduling at UE side can help UE to smartly spend its measurement efforts on more important frequency layers, i.e., which frequency layers to measure more, which to measure less. In our view, it would in the end enhance the mobility performance.

Fig.1

Observation 2: It is beneficial to allow Dynamic Adaptation of Measurement Procedure at UE side.

As guided, we also provide some of our initial thinking on this use case:

| Dynamic Adaptation of Measurement Procedure |  | MediaTek |
| --- | --- | --- |
| AI model input | Input in training | RSRP of serving cell and neighbor cells |
|  | Input in inference | RSRP of serving cells and neighbor cells |
| AI model output | Label in training (if applicable) | Suppose inter-f measurement on f2 is configured. The label is whether any cell on f2 is [detectable] |
|  | Output in inference | Whether any cell on f2 is [detectable] |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | Through legacy inter-f measurement |
| model location for inference |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | No collaboration/interaction |
| Evaluation methodology |  | System level simulation |
| Evaluation assumption |  | Serving cell is on f1. To-be-predicted frequency is on f2. Set up a scenario that f1 has a full coverage but f2 does not. |
| Evaluation KPI |  | number of preditions that is righttotal number of preditions |
| Evaluation benchmark |  | Measurement following legacy requirements |
| preliminary evaluation results |  |  |
| High level potential specification impact |  | New measurement delay requirements to allow dynamic adaptation of measurement procedure. |
| Feasibility issues, including complexity, and other aspects related to implementation |  | The model complexity is expected to be not high as it is just a classification model. |

Proposal 13: For use case “Dynamic Adaptation of Measurement Procedure”, the detail information is proposed as:

| Dynamic Adaptation of Measurement Procedure |  | MediaTek |
| --- | --- | --- |
| AI model input | Input in training | RSRP of serving cell and neighbor cells |
|  | Input in inference | RSRP of serving cells and neighbor cells |
| AI model output | Label in training (if applicable) | Suppose inter-f measurement on f2 is configured. The label is whether any cell on f2 is [detectable] |
|  | Output in inference | Whether any cell on f2 is [detectable] |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | Through legacy inter-f measurement |
| model location for inference |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | No collaboration/interaction |
| Evaluation methodology |  | System level simulation |
| Evaluation assumption |  | Serving cell is on f1. To-be-predicted frequency is on f2. Set up a scenario that f1 has a full coverage but f2 does not. |
| Evaluation KPI |  | number of preditions that is righttotal number of preditions |
| Evaluation benchmark |  | Measurement following legacy requirements |
| preliminary evaluation results |  |  |
| High level potential specification impact |  | New measurement delay requirements to allow dynamic adaptation of measurement procedure. |
| Feasibility issues, including complexity, and other aspects related to implementation |  | The model complexity is expected to be not high as it is just a classification model. |

### 2.2.6 Spatial domain RX beam sweeping reduction (L1/L3)

In our understanding, the use case is targeting for FR2. The main idea is to measure with only a subset of UE Rx beams and predict “RSRP of the other unused Rx beams” or “RSRP of the best Rx beam & the corresponding Rx beam ID”. If to support this feature, RAN4 may define new measurement delay requirements and predicted RSRP accuracy requirements. We think testability of this feature should be studied at first. For AI/ML BM, there are multiple AoA and single AoA test method on table. At least, we think single AoA can be workable for AI/ML BM. But for AI based Rx beam sweeping reduction, we don’t think it is possible to use single AoA test system. We are also not sure whether the multiple AoA test system in discussion in AI/ML BM is workable here, e.g., if the signals are always transmitted from the same 4 AoAs, how do we know UE passes the test case via the help of AI or by just simply conducting measurements only with Rx beams pointing to the 4 AoAs.

Proposal 14: Before starting the work on Spatial domain RX beam sweeping reduction (L1/L3), testability of this feature should be studied at first.

### 2.2.7 AI/ML based RLM/BFD/CBD, LTM by measurement prediction

AI/ML based RLM has once discussed in RAN2 but not included in R20 AI mobility. AI/ML based BFD/CBD are similar as AI/ML based RLM. As proposed in proposal 9: For the use case once discussed in RAN2 but not included in R20 AI mobility, RAN4 keeps them tentatively in the WF, but need to later coordinate with RAN2 to avoid duplicating work.

We don’t think there is any limitation on what procedure to apply the measurement prediction and event prediction results in R20 AI mobility. If we understand correct, for AI/ML based LTM, the intention is to apply the measurement prediction and event prediction to LTM. If so, then there is no need to independent study AI/ML based LTM.

Proposal 15: The use case “AI/ML based RLM/BFD/CBD” can be postponed now due to the uncertainty of RAN2 study.

Proposal 16: For the use case “AI/ML based LTM”, if the intention is just to apply the measurement prediction and event prediction results to LTM, then there is no need for independent study.

### 2.2.8 AI/ML based SCell related enhancement by measurement prediction

If we understand correct, for AI/ML based SCell related enhancement, the intention is to apply the measurement prediction to SCell. In our view, this scenario is very similar to frequency-domain prediction with either co-located or non-co-located sites. Therefore, it can be merged into L3 cell (or beam) measurement related use cases.

Proposal 17: For the use case “AI/ML based SCell related enhancement by measurement prediction”, it can be merged into L3 cell (or beam) measurement related use cases.

### 2.2.9 AI-based prediction for enhanced s-measure mechanism

If we understand correct, the intention is also to adapt measurement behavior dynamically. We see some similarity as use case#5a (Dynamic Adaptation of Measurement Procedure). Maybe case 5a and 11 can be merged.

Proposal 18: Further check whether the use case “AI-based prediction for enhanced s-measure mechanism” can be merged with “Dynamic Adaptation of Measurement Procedure”.

### 2.2.10 AI/ML based AGC through temporal/frequency domain prediction

If we are correct, the intention is to reduce measurement delay. Measurement prediction for temporal/frequency domain in discussion can reduce the measurement but not only the time for AGC. . We are not sure about the detail of the use case. More detail information is needed.

Proposal 19: For use case “AI/ML based AGC through temporal/frequency domain prediction”, more information is needed to check the feasibility and difference from other use cases.

### 2.2.11 AI/ML based DL sync through temporal/frequency domain prediction

If we are correct, the intention is to predict coarse timing of the cell and to skip PSS/SSS detection. This is highly depending on whether and how much network is already synchronized. If NW are well synchronized, UE may not need to detect PSS/SSS to get coarse timing. But if NW are not well synchronized, we doubt whether the use case can work as timing at NW may drift to make the predicted timing inaccurate. On another aspect, we prefer the NW can be well synchronized instead of seeking for fancy solutions at UE side.

Proposal 20: Prefer to have a well synchronized NW instead of studying the use case “AI/ML based AGC through temporal/frequency domain prediction”.

### 2.2.12 AI abstraction of the wireless environment to facilitate data scheduling and network power saving

It is not clear to us what spec impact there will be and whether it can be done just by NW implementation. More information seems needed to check the necessity for further study.

Proposal 21: For use case “AI abstraction of the wireless environment to facilitate data scheduling and network power saving”, more information is needed to check the necessity for further study.

# Summary

In this paper, we provide some views on 6G RAN4 AI/ML use cases. We have the following proposals and observations:

Proposal 1: Deprioritize the study on AI-nonlinearity compensation for DL before the motivation is well justified.

Proposal 2: Open to have a joint session or separate session of AI and non-AI DPoD at NW.

Proposal 3: AI DPoD at NW shall be compared with non-AI DPoD in the end.

Proposal 4: Not to study AI based DPD at UE.

Proposal 5: Not to discuss the listed L1 beam measurement related AI/ML use cases in RAN4:

- Time domain (intra-cell): Same as R19 AI/ML BM Case 2.

- Time and spatial domain (intra-cell): Same as R19 AI/ML BM Case 2 when set A and set B are different.

- Freq. and spatial domain (inter-cell): Same as Sub-case B in RAN1 discussion.

- Spatial domain (inter-cell): Covered by Sub-case A in RAN1 discussion.

- Time and spatial domain (inter-cell): Covered by Sub-case A in RAN1 discussion.

Observation 1: For use cases that just extends the use cases in R20 AI mobility from cell-level to beam-level, there is no fundamental difference between cell-level and beam-level prediction. Furthermore, there is an ongoing discussion in RAN2 on how to harmonize L1 and L3 measurement procedure.

Proposal 6: For use cases that just extends the use cases in R20 AI mobility from cell-level to beam-level, it is not necessary to have a study phase.

Proposal 7: For hybrid use cases of R20 AI mobility, it is better to start the study after closing the work in R20 AI mobility.

Proposal 8: Deprioritize the listed L3 beam measurement related AI/ML use cases in the study phase in RAN4:

- Time domain (intra-cell)

- Time and spatial domain (intra-cell)

- Freq. and spatial domain (inter-cell)

- Time and freq. domain (inter-cell)

- Freq. domain prediction for N carriers single cell

Proposal 9: For the use case once discussed in RAN2 but not included in R20 AI mobility, RAN4 keeps them tentatively in the WF, but need to later coordinate with RAN2 to avoid duplicating work.

Proposal 10: For L3 cell measurement with freq. domain (inter-cell) prediction, it is important to also consider non-collocated scenarios. But due to the uncertainty of RAN2 study, this category of use case can be postponed now.

Proposal 11: No need to study L3 cell measurement with time domain (intra-cell) prediction and freq. domain (inter-cell, co-located) prediction which are already included in R20 AI mobility.

Proposal 12: Start the study on the use case “Multi-Domain L3 level Prediction for Measurement-Gap” after closing the work in R20 AI mobility.

Observation 2: It is beneficial to allow Dynamic Adaptation of Measurement Procedure at UE side.

Proposal 13: For use case “Dynamic Adaptation of Measurement Procedure”, the detail information is proposed as:

| Dynamic Adaptation of Measurement Procedure |  | MediaTek |
| --- | --- | --- |
| AI model input | Input in training | RSRP of serving cell and neighbor cells |
|  | Input in inference | RSRP of serving cells and neighbor cells |
| AI model output | Label in training (if applicable) | Suppose inter-f measurement on f2 is configured. The label is whether any cell on f2 is [detectable]. |
|  | Output in inference | Whether any cell on f2 is [detectable]. |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | Through legacy inter-f measurement |
| model location for inference |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | No collaboration/interaction |
| Evaluation methodology |  | System level simulation |
| Evaluation assumption |  | Serving cell is on f1. To-be-predicted frequency is on f2. Set up a scenario that f1 has a full coverage but f2 does not. |
| Evaluation KPI |  | number of preditions that is righttotal number of preditions |
| Evaluation benchmark |  | Measurement following legacy requirements |
| preliminary evaluation results |  |  |
| High level potential specification impact |  | New measurement delay requirements to allow dynamic adaptation of measurement procedure. |
| Feasibility issues, including complexity, and other aspects related to implementation |  | The model complexity is expected to be not high as it is just a classification model. |

Proposal 14: Before starting the work on Spatial domain RX beam sweeping reduction (L1/L3), testability of this feature should be studied at first.

Proposal 15: The use case “AI/ML based RLM/BFD/CBD” can be postponed now due to the uncertainty of RAN2 study.

Proposal 16: For the use case “AI/ML based LTM”, if the intention is just to apply the measurement prediction and event prediction results to LTM, then there is no need for independent study.

Proposal 17: For the use case “AI/ML based SCell related enhancement by measurement prediction”, it can be merged into L3 cell (or beam) measurement related use cases.

Proposal 18: Further check whether the use case “AI-based prediction for enhanced s-measure mechanism” can be merged with “Dynamic Adaptation of Measurement Procedure”.

Proposal 19: For use case “AI/ML based AGC through temporal/frequency domain prediction”, more information is needed to check the feasibility and difference from other use cases.

Proposal 20: Prefer to have a well synchronized NW instead of studying the use case “AI/ML based AGC through temporal/frequency domain prediction”.

Proposal 21: For use case “AI abstraction of the wireless environment to facilitate data scheduling and network power saving”, more information is needed to check the necessity for further study.

# 5 Reference

- [1] R4-2514636, Topic Summary for [116bis][107] 6G AI, Samsung (FL)

- [2] R1-2508118, Moderator summary #5 on AI/ML for 6GR, RAN1#122bis, Samsung (Moderator)