3GPP TSG-RAN WG4 Meeting # 117                                                            R4-2520346

Dallas, USA, 17th – 21st Nov., 2025,

Agenda item:	6.11.4

Source:	MediaTek Inc.

Title:	Discussion on RRM core requirement and testing framework for beam management

Document for:	Discussion

# Introduction

In this contribution, we provide our views on the remaining issues of AI/ML BM use case.

# Discussion

## 2.1 Prediction report delay

Last meeting, RAN4 reached the following agreement on prediction report delay:

Issue 2-1: Measurement period for inference – case 2 Agreement: Measurement period for inference – case 2 K(number of samples)*[M]*N*P K as defined by RAN1

The only remaining issue is whether “M” is needed for BM case 2. According to RAN1 agreement as shown below, “K” is the least number of consecutive transmission occasions of RS resources in Set B before the CSI report. Literally speaking, it seems M is not necessary. But other factors, e.g., N and P, which also contribute to the number of transmission occasions, are not considered by RAN1. In our understanding, RAN1 didn’t consider the factors that will affect measurement delay. So we prefer to keep M in the prediction report delay.

Agreement: Support the following on the reporting condition of CSI reporting for inference For BM-Case2, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, UE transmit a CSI report for inference only if receiving at least K latest consecutive transmission occasion for each of the RS resources in Set B no later than the corresponding CSI reference resource, where K is indicated by a new UE capability.

Proposal 1: For AI/ML BM case 2, keep M in the prediction report delay.

      - 2.2 Simulation results

### 2.2.1 General analysis

In previous meeting, the simulation settings were further discussed to evaluate the impact of measurement error in R4-2505152. In our simulations, Option 2 in R4-2505152 is adopted to get the distribution of BB error under different SNRs, i.e., TDL-C channel and single Tx beam. For a single SNR, Tx power is set as the normalized power 1 and noise is generated following Gaussian distribution. After simulations, the average and variance of BB error under a fixed SNR can be obtained. Following above procedures, we get the mean and variance of BB measurement error under SNR= [-24, 10] dB with step 3dB at first (as in Fig. 1).

Fig.1

During inference, the way to add measurement error to the inputs of model inference is shown in Fig. 2:

- In SLS simulation, SINR of each Tx-Rx beam pair is calculated dependently.

  - In the calculation of SINR, noise is generated as Gaussian distribution and UE receiver noise figure is set to 10dB which is as guided in TR38.843 Table 6.3.1-1. Useful signal and interferences from other cells are calculated considering Tx power, path-loss, channel gain and beamforming gains.

- After calculating SINR of each beam pair, we look up the table to get the corresponding variance.

  - If the SINR is not an integer or out of the range [-24:3:9] dB, we use interpolation method to obtain the mean and variance.

- A random BB error is generated following Gaussian distribution with the mean and variance corresponding to the SINR value.

Fig. 2 Method of adding measurement error

### 2.2.2 Simulation results using our dataset

Note: There is no further update compared the simulation results submitted last meeting.

We focus on the scenario with 100% outdoor UEs to check the performance of different metrics. The simulation results are summarized as in Table 1 and 2. To check the predicted RSRP accuracy, regression model is adopted in the simulations. We also increased the size of training dataset to larger than 100,000 to check the impact of the size of training dataset.

Table 1. Simulation results for Scenario 1: spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement

| error consideration |  |  | case 1 (6300) | case 3 (6300) |  |  | case 1 (105000) | case 3 (105000) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| KPI | K | X | N.A. | BB error only | RF error only | BB+RF error | N.A. | BB+RF error |
| KPI 1 | 1 | 0 | 68 | 65 | 52 | 51 | 73 | 53 |
|  | 2 | 0 | 84 | 83 | 73 | 72 | 87 | 75 |
|  | 3 | 0 | 91 | 90 | 84 | 82 | 92 | 84 |
|  | 4 | 0 | 94 | 93 | 90 | 88 | 95 | 89 |
|  | 5 | 0 | 95 | 95 | 92 | 92 | 96 | 92 |
|  | 1 | 1 | 75 | 73 | 59 | 58 | 82 | 61 |
|  | 1 | 2 | 81 | 80 | 67 | 65 | 87 | 69 |
|  | 1 | 3 | 86 | 85 | 74 | 72 | 90 | 75 |
| KPI 2 | 1 | N.A. | 4.9 | 5.7 | 7.1 | 7.5 | 4 | 6.3 |
|  | 3 |  | 5.4 | 6 | 7.3 | 7.8 | 4.4 | 7 |
|  | 5 |  | 5 | 6.2 | 7.2 | 8 | 4.6 | 7 |

Table 2. Simulation results for Scenario 2: Spatial domain prediction with 32 CSI-RS beams in Set A and 8 SSB beams in Set B

| error consideration |  |  | case 1 (6300) | case 3 (6300) |  |  | case 1 (105000) | case 3 (105000) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| KPI | K | X | N.A. | BB+RF error | RF error only | BB+RF error | N.A. | BB+RF error |
| KPI 1 | 1 | 0 | 82 | 79 | 57 | 55 | 85 | 57 |
|  | 2 | 0 | 94 | 92 | 79 | 76 | 95 | 78 |
|  | 3 | 0 | 96 | 96 | 87 | 86 | 98 | 87 |
|  | 4 | 0 | 98 | 98 | 92 | 92 | 99 | 92 |
|  | 5 | 0 | 98 | 99 | 95 | 94 | 99 | 94 |
|  | 1 | 1 | 90 | 88 | 65 | 62 | 94 | 66 |
|  | 1 | 2 | 97 | 93 | 72 | 70 | 97 | 73 |
|  | 1 | 3 | 98 | 96 | 79 | 76 | 98 | 80 |
| KPI 2 | 1 | N.A. | 2.5 | 2.7 | 5.3 | 5.6 | 1.9 | 4.9 |
|  | 3 |  | 3.5 | 4.1 | 7.2 | 7.6 | 2.4 | 6.8 |
|  | 5 |  | 4.4 | 5.2 | 9 | 8.7 | 3.1 | 7.8 |

The prediction accuracy is improved a bit by increasing the training dataset size from 6300 to 105,000.

Observation 1: The prediction accuracy is improved a bit by increasing the training dataset size.

      - 2.3 Metrics

### 2.3.1 Metric: absolute/relative RSRP requirements

Issue 2-9: Metrics/KPIs for beam ID prediction Agreement: - If RSRP prediction is reported, absolute/relative RSRP requirements applies to - reported beams where SNR side condition is met - The exact RSRP reporting including both absolute and relative will be defined in RAN1 - The related test can only apply to the top-K, where the value of K is FFS, beams. - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K - The feasibility of the test will be further discussed and decided.

RAN4#116 reached the above agreement on absolute/relative RSRP requirements for predicted RSRP. One of the remaining issue is how to set SNR side condition. The legacy L1 measurement requirements are defined at SNR= -3dB and the requirements are applicable under the side condition that SNR≥-3dB. In the LLS simulation, we can set SNR to -3dB and get the accuracy requirements under SNR= -3dB. But for AI BM, we are confused on how to set SNR side condition as SLS simulations are performed. We checked SNR distribution of the strongest beam in set A and found that there is no data with SNR of the strongest beam in set A equal to -3dB in our generated dataset as shown in Fig.3. We once tried to decrease the RSRP artificially to make SNR of the strongest beam in set A equal to -3dB. But we received some offline comments that this may lead to a different scenario and is not a good way. Considering the above fact, we think the RSRP accuracy requirements for the strongest beam can be defined by filtering out the data with SNR of the strongest beam in set A no less than -3dB.

Fig. 3

Proposal 2: RSRP accuracy requirements of Top-1 of predicted beams can be defined by filtering out the data with SNR of the Top-1 beam in set A no less than -3dB.

For none Top-1 beams, it can be observed that the absolute accuracy of each predicted beam varies as shown in Table 1 (KPI 2). We checked the CDF of the RSRP/SINR difference between the (x+1)th strongest beam and the strongest beam (E.g., 1th gap: strongest beam - 2nd strongest beam, 3th gap: strongest beam - 4th strongest beam) in Fig. 4. As shown by figure on the right in Fig. 4, there is a 60% probability that SINR of the 2nd strongest beam is lower than the strongest beam, i.e., SINR difference >0dB. Such a SINR difference will lead to the difference in predicted absolute RSRP accuracy of top-1 beam and other beams.

Observation 2: For different beams, e.g., predicted 1st strongest and 5th strongest beam, the predicted absolute RSRP accuracy varies. It may be the SINR difference that results in the difference between the predicted absolute RSRP accuracy of top-1 beam and other beams.

Fig. 4

Due to limited time, we don’t have time to check the predicted absolute RSRP accuracy of none top-1 beams if excluding those datasets with low SNR. We can further check it to see whether it is workable to define a single absolute accuracy requirement which is applicable to all/some of the predicted beams with some side conditions, e.g., SNR≥-3dB.

Proposal 3: Further discuss whether it is workable to define a single absolute accuracy requirement which is applicable to all/some of the predicted beams with some side conditions.

      - 2.4 Analysis on Multiple AoA Test setup for AI/ML BM

RAN4#116 and RAN4#116bis reached the following progress on test system for AI/ML BM. Although we still think single AoA test system is more workable, we can focus on the evaluation of multi-AoA test system this meeting as guided.

Issue 2-6: Test system setup Agreement: Consider multi AoA based RRM testing setup as baseline(enhanced IFF system with 4 probes used for FR2 RRM conformance testing) Single AoA tests can also be performed in the multi AoA based RRM test setup Further analyze what spatial channel models can be emulated with this test setup - Companies are invited to bring analysis on what simplified spatial channel (multiple clusters coming from multiple directions) model can be emulated in this test setup Single AoA tests are not precluded Issue 2-7: Test system channel model Agreement: Companies to bring analysis into what simplifications to CDL channels are needed to be able to emulate the channels in the enhanced IFF chamber. Take CDL-C Umi as one example Other channel models can be further discussed Issue 2-9: Simplified Channel models Agreement: RAN4 will further discuss the simplified Uma and UMi channel model by taking the following aspects into consideration - Commercial test system limitation, including the number of probes and the separation - Model simplification should not result in inference performance degradation beyond the margin when the training is done based on the original models. - The margin can be discussed and decided further. - Other aspects are not precluded.

We believe that companies are supposed to simulate using the final agreed simplified CDL channel if confirmed workable. To carry out the simulations, RAN4 needs to discuss how to simulate the simplified CDL. We think at least the following aspects should be discussed.

How to simulate the channel “from BS to clusters” and the channel “from clusters to UE”

In the simulation, it is the channel from BS to UE simulated. As shown in Fig.5, the channel between BS and UE includes two parts in CDL channel model: BS  Clusers and Clusters  UE.  As enhanced IFF system with 4 probes is used as baseline and there are far more than 4 clusters in the CDL channel model, it is obvious that how to simply the part “from clusters to UE” should be discussed. In R4-2511597, several methods have been proposed on this. But it seems that the contribution focused on how to simplify the channel “from clusters to UE” and didn’t touch the part “from BS to clusters”. To get the whole channel, how to simulate the part from BS to clusters should also be discussed.

Fig.5

Observation 3: CDL channel includes two parts “from BS to clusters” and the channel “from clusters to UE”.

Proposal 4: Discuss how to simulate the channel “from BS to clusters” in the simplified CDL-channel as well.

How to emulate the channel for different locations?

In general, the channel that the chamber simulates is link level channel, which is just one realization, i.e., for UE at a certain location. For AI/ML BM test, it is necessary to check the performance at different UE locations in a cell. In our understanding, there may be two methods. One is through UE rotation. But according to the information provided by TE vendors [2-5], it may be difficult and costly to support frequent and flexible UE rotation, i.e., rotation 2o, 5o. Now, the time needed to rotate UE is about 1s, which is not acceptable during one iterative test.

Another method may be through adjusting TE transmission power at each probe, which is similar as single AoA test system. Using this method, RAN4 needs to conclude on the methodology of simplifying the CDL-channel. In the simulation, the procedures will be as the below:

- Cast the UE in the cell randomly.

- Generate the channel between BS and UE according to 38.901, including path-loss, shadowing, and small-scale fading.

- Simplify small scale fading between BS and UE based on the to-be-agreed simplification method.

- Calculate received RSRP of each beam.

Proposal 5: Discuss how to emulate the channel for different locations in multiple AoA test systems.

Observation 4: It is not workable to emulate different UE locations through rotating UE during the test as the time needed to rotate UE is about 1s.

Proposal 6: Evaluate whether it is workable to emulate different UE locations through adjusting TE transmission power at each probe in multiple AoA test systems.

Proposal 7: The procedures to emulate the channel for different UE locations using simplified CDL channel in the simulation will be:

- Cast the UE in the cell randomly.

- Generate the channel between BS and UE according to 38.901, including path-loss, shadowing, and small-scale fading.

- Simplify small scale fading between BS and UE based on the to-be-agreed simplification method.

- Calculate received RSRP of each beam.

How to get training dataset?

Another issue is how to get the training dataset. In computer simulations, a pre-defined UE radiation pattern can be used to generate the training dataset and inference dataset. But in the test, we are not clear about how to get the training dataset. We think there may be two ways. One is collecting the training data in the chamber for each test UE before training. This method is time-consuming and not preferred. Another is to generate the training dataset assuming a certain UE radiation pattern. We evaluated the performance of models trained with a UE antenna radiation pattern (case 2) and inference on UEs with varied antenna radiation patterns (case 1, 3, 4) as shown in Fig. 6. It can be observed that AI/ML BM Tx beam prediction model generalize well for different antenna radiation patterns. Therefore, we think it is enough to use a specified UE radiation pattern during the test. We guess, even simplified CDL channel is adopted, we may have similar observation.

Fig. 6

Proposal 8: Discuss how to get training dataset if to use multiple AoA test systems:

- Alt 1: collect the training data in the chamber for each test UE before training

- Alt 2: generate the training dataset assuming a certain UE radiation pattern through simulation

Dynamic range

In early releases, dynamic range of IFF and DFF test systems had been analysed. Since several years have passed, there may be some new progress. But we are not experts, so here we just copy the previous results in TR38.810 here for information.

Table 3

| Test methods | Wanted signal (dBm/SCS) at reference point: upper bound | SNR at baseband (dB): upper bound | SNR at baseband (dB): upper bound – 30dB |
| --- | --- | --- | --- |
| IFF, peak fine beam | -83.5 | 25.1 | -4.9 |
| IFF, peak rough beam | -83.5 | 18.1 | -11.9 |
| IFF, non-peak fine beam | -83 | 13 | -17 |
| IFF, non-peak rough beam | -83 | 6 | -24 |
| DFF, peak fine beam | -83.8 | 24.8 | -5.2 |
| DFF, peak rough beam | -83.8 | 17.8 | -12.2 |
| DFF, non-peak fine beam | -83.3 | 12.7 | -17.3 |
| DFF, non-peak rough beam | -83.3 | 5.7 | -24.3 |

If to use multiple AoA test system, the Rx beam will be tuned by UE and it is not possible to always use the peak fine beam. So for multiple AoA test system, we only need to check the highlighted rows in Table 3. It can be observed that the highest SNR is only 13dB, which is very low.

Observation 5: Multiple AoA test systems cannot use peak fine beam direction only.

Considering the upper bound of SNR with rough beam are much lower than that with fine beam as shown in Table 1, fine beam should be used in AI/ML BM test to get a larger dynamic power range. Next we will only focus testing using fine beam.

Observation 6: The upper bound of SNR with multiple AoA test system is 13dB.

Proposal 9: Take the upper bound of SNR with multiple AoA test system into consideration during simulation.

# Conclusion

In this contribution, we provide our views for AI/ML BM use case. Observations and proposals are summarized as follows.

Proposal 1: For AI/ML BM case 2, keep M in the prediction report delay.

Observation 1: The prediction accuracy is improved a bit by increasing the training dataset size.

Proposal 2: RSRP accuracy requirements of Top-1 of predicted beams can be defined by filtering out the data with SNR of the Top-1 beam in set A no less than -3dB.

Observation 2: For different beams, e.g., predicted 1st strongest and 5th strongest beam, the predicted absolute RSRP accuracy varies. It may be the SINR difference that results in the difference between the predicted absolute RSRP accuracy of top-1 beam and other beams.

Proposal 3: Further discuss whether it is workable to define a single absolute accuracy requirement which is applicable to all/some of the predicted beams with some side conditions.

Observation 3: CDL channel includes two parts “from BS to clusters” and the channel “from clusters to UE”.

Proposal 4: Discuss how to simulate the channel “from BS to clusters” in the simplified CDL-channel as well.

Proposal 5: Discuss how to emulate the channel for different locations in multiple AoA test systems.

Observation 4: It is not workable to emulate different UE locations through rotating UE during the test as the time needed to rotate UE is about 1s.

Proposal 6: Evaluate whether it is workable to emulate different UE locations through adjusting TE transmission power at each probe in multiple AoA test systems.

Proposal 7: The procedures to emulate the channel for different UE locations using simplified CDL channel in the simulation will be:

- Cast the UE in the cell randomly.

- Generate the channel between BS and UE according to 38.901, including path-loss, shadowing, and small-scale fading.

- Simplify small scale fading between BS and UE based on the to-be-agreed simplification method.

- Calculate received RSRP of each beam.

Proposal 8: Discuss how to get training dataset if to use multiple AoA test systems:

- Alt 1: collect the training data in the chamber for each test UE before training

- Alt 2: generate the training dataset assuming a certain UE radiation pattern through simulation

Observation 5: Multiple AoA test systems cannot use peak fine beam direction only.

Observation 6: The upper bound of SNR with multiple AoA test system is 13dB.

Proposal 9: Take the upper bound of SNR with multiple AoA test system into consideration during simulation.

# Reference

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2411177, Testability and interoperability issues for beam management, Korea Testing Laboratory, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411254, OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases, Keysight Technologies, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411279, Study on CDL channel model for AI/ML beam management in FR2, Anritsu Corporation, 3GPP TSG-RAN4 # 112, Aug. 2024.

- R4-2411587, Discussion on test setup for AI/ML based beam management, Rohde & Schwarz, 3GPP TSG-RAN4 # 112, Aug. 2024.