3GPP TSG-RAN WG4 Meeting # 117   	                                                           R4-2520347

Dallas, USA, 17th – 21st Nov., 2025,

Agenda item:	7.12.2

Source:	MediaTek Inc.

Title:	Discussion on general aspects for AI mobility

Document for:	Discussion

# Introduction

RAN#109 plenary has agreed the updated version of the WID with the following RAN4 objectives [1]:

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

In this contribution, we would like to discuss some general issues.

# Discussion

## 2.1 Prioritize measurement event prediction for temporal domain prediction

According to the WID, there are two cases, RRM measurement prediction and measurement event prediction. If indirect prediction is used for measurement event prediction, RRM measurement prediction is the basis. Due to this, both RAN2 and RAN4 took RRM measurement prediction as high priority in SI phase. But in the field, event triggered L3 measurement report is widely used, while periodic L3 measurement report is seldomly used.

Observation 1: In real field, event triggered L3 reporting is way more commonly used than periodic reporting.

For temporal domain prediction, there are two cases, Case A and Case B. As in temporal domain prediction Case A, NW will not skip any SSB transmission. As AI/ML model is power consuming, for the sake of power saving, we hope UE not to be mandatory to predict the future all the time, which is also not necessary. When to do inference should be totally up to UE. We think it is more reasonable that UE does the prediction only when an event may occur, i.e., only predict for measurement events.

Proposal 1: For the sake of power saving, UE should not be mandatory to predict the future all the time.

Proposal 2: For temporal prediction Case A, it is up to UE when to do the prediction.

For temporal domain prediction Case B, according to TR38.744, there are mainly two skipping patterns as shown below.

From TR38.744 Figure 5.2.1.1-2: Skipping pattern example 1 of intra-frequency temporal domain case B Figure 5.2.1.1-3: Skipping pattern example 2 of intra-frequency temporal domain case B

Example 1 would have big impact on legacy UEs (not supporting Case B) as SSB is not transmitted periodically. Example 2 may have impact on legacy UEs too, if SSB is not transmitted periodically. Last meeting, RAN2 reached the consensus that “For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping.” Considering that there will be legacy non-AI UEs in the field, NW has to transmit SSB periodically to avoid the impact on legacy non-AI UEs. So RAN4 only needs to consider the case that NW transmit SSB periodically for Case B. Then legacy non-AI UE will perform measurement based on the configured SMTC and other MOs. As the interpretation of “skipping pattern” is not the timing of UE's SSB measurement/skipping, when and how to do the measurement and prediction is totally up to UE. It is also not necessary for UE to predict the skipped occasions especially when RSRP of serving cell is high enough and neighboring cell is low. That is why it is workable to extend the measurement delay for UEs not at cell edge in R16 power saving. For both Case A and Case B, NW is supposed to transmit SSB periodically and when/how to do the prediction is totally up to UE. To us, it seems there is no difference between the two cases. For mobility, we think the most important thing about measurement is to report the event timely and accurately. For both case A and case B, we think event prediction should be the focus.

Proposal 3: For temporal prediction Case B, only consider the case that NW transmit SSB periodically.

Proposal 4: For temporal prediction Case B, when and how to do the measurement and prediction is totally up to UE.

Proposal 5: For temporal domain prediction, measurement event prediction should be the focus and RAN4 only need to define test cases for measurement event prediction.

## 2.2 Simulation Assumptions

Based on the experience of AI/ML BM, it is essential to check whether the agreed performance metric is reasonable through simulation. And it is time consuming to align the simulation results. It is better to agree on the simulation assumptions as early as possible. Last meeting, there were some discussions on the simulation assumptions and reached some agreements. In this meeting, we believe there will be further discussions on the simulation assumptions. To fully utilize RAN2’s evaluation work, we propose to use the simulation assumptions in Table 5.1-1, Table 5.3.1-1 and Table 5.3.1-2 in TR38.744 as a basis.

Proposal 6: Use the simulation assumptions in Table 5.1-1, Table 5.3.1-1 and Table 5.3.1-2 in TR38.744 as a basis.

Last meeting, RAN4 discussed the scenarios to discuss RRM requirements for. We think one of the purposes is to decide whether to do the simulations for all the scenarios. For FR2 related scenarios, Rx beam sweeping delay is not considered in both AI/ML BM simulations and RAN2 simulations. For AI/ML BM, RAN4 only discussed the simulations for spatial domain. For temporal domain, there were seldom simulation related discussion. For BM, there is a scenario “predict RSRP of CSI-RS in Set A based on the measurement result of CSI-RS in Set B”. For CSI-RS with repetition off, Rx beam sweeping is not expected. But for AI mobility, Rx beam sweeping is necessary for SSB measurement in FR2.

Proposal 7: In the simulation, take Rx beam sweeping delay into consideration in FR2.

For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), in RAN2 simulations, only co-located cases are considered. Even the cells of two frequency layers are co-located. Cells from two frequency layers may share the same BS configurations (e.g., antenna pattern, downtilt angle) or may not. They may have different horizontal coverage ranges as shown in Fig.1. At least in our RAN2 simulations, the BS configurations (e.g., antenna pattern, downtilt angle) and the horizontal coverage of the two frequency layers are also the same. It is not clear to us whether the case that the co-located cells have different horizontal coverage is in scope or not. But usually, antenna pattern of 4GHz and 2GHz are different. We think at least in the simulation, the case that cells from different frequency layers with different antenna pattern should be considered.

Fig. 1

If the horizontal coverage ranges of the two frequency layers are the same, low band is usually deployed for coverage, and higher band is deployed for capacity. HO between the two frequencies is usually for load balance. If current serving cell is on 4GHz, then the collocated 2GHz cell is surely of good quality. In this case, we think prediction on 2GHz is redundant. If current serving cell is on 2GHz, only when RSRP of serving cell is lower than a threshold, there is a need to predict RSRP of the cell on 4GHz in case the RSRP of the target cell is too low.

Proposal 8: For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), discuss whether co-located cells on different frequencies are with varied BS configurations (e.g., antenna pattern, downtilt angle) or not.

Observation 2: For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), if co-located cells from the low and high bands share the same horizontal coverage range, there is no need to predict RSRP of cells in low band through RSRP of cells in high band.

Proposal 9: For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), if co-located cells from the low and high bands share the same horizontal coverage range and UE is supposed to predict RSRP of high band through RSRP of cells in low band, UE only needs to do the prediction when RSRP of the cell in low band is lower than a threshold.

## 2.3 Ground truth

In SI phase, RAN4 reached the following progress on how to determine the ground truth. In the next, we would like to discuss the next steps to do.

RAN4#114 Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: Take Option 1-1-FR2 as baseline, and further discuss any additional aspects need to be considered. - Option 1-1-FR2: The ground truth of L3 RPRP is the reported L3 RSRP measurement under sufficient high SNR in FR2. Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list RAN4#114bis Issue 2-1-5: Ground truth definition for RSRP Accuracy for FR1 Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

Even during simulation, we will compare the predicted result with “ground truth”.

Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

The key issue of Alt 1 is that the ground truth can be calculated based on a specified L1 filtering and L3 filtering while it is up to UE implementation on how to do L1 filtering and L3 filtering. There will be some gap between the calculated ground truth at TE and the measured result at UE. To check whether alt1 is workable, simulations are needed to check the gap between UE’s implementation and the calculated ground truth.

Although we think using sliding window is more reasonable, RAN4 requirements were defined assuming non-sliding window. So both in the simulation or in the test, the ground truth of Alt1 shall be calculated using non-sliding window.

- We take Fig. 2 as an example to illustrate our thought.

  - The ground truth of f1 at occasion#10 can be calculated as (1-α)*Fn-1+α*Mn, where

    - Mn: the average transmission power of occasion #6, 7, 8, 9, 10 (L1 filtering with 5 samples).

    - Fn-1: the updated filtered RSRP at occasion#5 (occasion#10 – 200ms) (assume non-sliding window following the specification of L3 filtering).

    - α is derived according to 38.331 cl. 5.5.3.2.

  - The ground truth of f1 at occasion#9 can be calculated as (1-α)*Fn-1+α*Mn, where

    - Mn: the average transmission power of occasion #5, 6, 7, 8, 9 (L1 filtering with 5 samples).

    - Fn-1: the updated filtered RSRP at occasion#4 (occasion#9 – 200ms) (assume non-sliding window following the specification of L3 filtering).

    - α is derived according to 38.331 cl. 5.5.3.2.

In this way, even assuming non-sliding window, it is possible to get the ground truth which is calculated based on the transmitted power at TE.

Fig. 2

Proposal 10: If to get the ground truth based on the transmitted or reception power in FR1, the ground truth for occasion X can be calculated as (1-α)*Fn-1+α*Mn, where

- Mn: the average transmission power of occasion #X-160ms, X-120ms, X-80ms, X-40ms, X (L1 filtering with 5 samples).

- Fn-1: the updated filtered RSRP at occasion #X– 200ms) (assume non-sliding window following the specification of L3 filtering).

- α is derived according to 38.331 cl. 5.5.3.2.

To get the margin to be added on top of the calculated ground truth based on the transmitted or reception power, we checked the difference among the measurement results at difference occasions in 200ms. The measurement results for each occasion are calculated as Proposal 10. We believe that the worst case shall be using non-sliding window but with different offsets. The distribution of max {result at occasion #X – result at occasion #Y} is shown as in Fig. 3, e.g., X and Y are from Occasion {6,7,8,9,10}. The RSRP difference increases with UE speed. With 90km/h, the RSRP difference is about 4dB at 95% tile point. With 30km/h, the RSRP difference is about 3dB at 95% tile point.

Fig. 3

Proposal 11:  If to derive the ground truth through transmitted power at TE, additional margin should be considered due to different L1 filtering and L3 filtering may be adopted by UE vendors. The value of the margin can be different for varied UE speed, which is proposed as 3dB for 30km/h and 4dB for 90km/h.

Another alternative is to use the reported results as ground truth. There was proposal that the ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Yms before and after the time instance X. If non-sliding window is adopted, the margin Yms seems necessary. If to use the reported results as ground truth, the test will contain two steps. In 1st step for AI prediction, TE will not transmit SSB on some occasions/frequency layers and UE will predict the measurement results at those occasions/frequency layers. UE uses the output of L3 filter as the input to AI model for the AI prediction model. Note that L3 filter results is obtained based on a certain L1 filtering behavior, e.g., under non-sliding window assumption, there could be many different window offsets as illustrated in Fig 4.

Fig 4

In 2nd step for ground truth reporting, TE will transmit SSB on all the occasions/frequency layers. UE will anyway adopt a certain window offset of the L1-filtering to generate measurement report. Even it is possible to make the channel realization in the two steps the same, it is not guaranteed that the offsets  in 1st and 2nd rounds are the same. Such a difference in offsets will inevitably lead to additional margin in the final accuracy requirement.

In 1st step, UE may get the measurement results with L1 filtering #1 (every 200ms) as the example shown in Fig 4. In the second step, UE may get the measurement results based on L1 filtering #4. In the first step, assuming UE reports the predicted result for occasion#10 after getting the measurement result at occasion#5. In the next step, due to L1 filtering #4, UE will report the measurement results obtained at occasion#3, #8 and #13. Clearly, it is more reasonable to compare the predicted result for occasion#10 with the measurement results at occasion#8 or #13. Because of the uncertainty of UE’s L1 filtering window offset, the final test requirement needs to consider the worst case among all possible offsets, of which the value is upper bounded by the defined measurement delay.

Based on the above analysis, if non-sliding window is adopted, there will be some RSRP difference introduced by the Yms and additional margin should be added. Considering the worst case, we think the additional margin added should be the same as getting the ground truth based on the transmitted or reception power.

Proposal 12: If to use reported RSRP as ground truth, the ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. Y is the defined measurement delay considering all kinds of UE implementation.

Proposal 13: When comparing the predicted results with reported RSRP, due to there may be Yms gap, additional RSRP difference margin should be added. The value of the margin shall be the same as getting the ground truth based on the transmitted or reception power, if non-sliding window is adopted.

However, if sliding window is adopted, UE can get the measurement results every SMTC. If UE also reports the measurement results for every SMTC occasion, the predicted result for an occasion can be compared with the measurement result at that occasion. Then there is no need to add additional margin.

Proposal 14: When comparing the predicted results with reported RSRP, if UE also reports the measurement results for every SMTC occasion, then there is no need to add additional margin.

If the final decision to use reported RSRP as ground truth, RAN4 need to discuss how to determine the additional margin, i.e., based on the worst case or based on companies input.

Proposal 15: If to use reported RSRP as ground truth, RAN4 need to discuss how to determine the additional margin, e.g., based on the worst case or based on companies input.

## 2.4 How to handle cells not detectable?

It is observed that there were some proposals to study the impacts on cell identification for L3 cell-level measurement prediction by AI/ML or whether the predicted cell is known or unknown. We agree that this is an important issue for further discussion.

For scenario 2 and 4, since they are intra-cell prediction, UE is supposed to detect and measure the cell before prediction. So cell identification is not an issue in the test or in the field. But in the simulations, the data with low SNR (i.e., the cell is not detectable) shall be excluded in the dataset at first.

Proposal 16: When do the simulations, the data containing cells with too low SNR (e.g., <-6dB) should be excluded from the dataset.

For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), since measurement on target cell is not always conducted, RAN4 should discuss the impact on cell identification. As discussed in Section 2.2, we are not sure whether RAN4 consider the case that the horizontal coverage ranges of the two frequency layers are different. If to consider the case, it is hard to know which cell is detectable on the to-be-predicted frequency layer without measurement.

Though cell detection, UE can get Cell ID and rough timing. Regarding Cell ID, if Cell#X’ have similar coverage as Cell#X, it is possible to inform UE cell ID of Cell#X’ and UE can know which cell’s RSRP that UE is predicting. But if Cell#X’ have different coverage from Cell#X, we are not sure whether it is workable to inform UE cell ID of the neighbor cell(s) by serving cell. If cell ID of the to-be-predict cell is unknown, we don’t know how NW understands UE’s prediction results.

Proposal 17: For scenario 3 (inter-frequency measurement prediction), discuss how UE gets cell ID of the to-be-predict cell(s) for reporting the prediction results to network.

## 2.5 Impact of Measurement error

### 2.5.1 Measurement error model

In this part, we would like to discuss how to add measurement error for AI mobility in FR1. Although it is L3 measurement considered in AI mobility, UE is supposed to measure each sample and derive L3 measurement result after L1 filtering and L3 filtering. BB error is introduced due to random noise and interference. For each measurement occasion, noise is different, so BB error shall be added independently per measurement occasion (i.e., at point A in Fig.6). After L1 filtering, BB error will be further flattened. In other words, when derive BB error distribution, one shot measurement is assumed.

The way to add BB error is shown in Fig. 5:

- In SLS simulation, SINR of each Tx beam is calculated dependently.

  - In the calculation of SINR, noise is generated as Gaussian distribution and UE receiver noise figure is set to 9dB which is widely used in FR1. Useful signal and interferences from other cells are calculated considering Tx power, path-loss, channel gain and beamforming gains.

- After calculating SINR of each beam, we look up the table to get the corresponding variance.

  - If the SINR is not an integer or out of the range [-24:3:9] dB, we use interpolation method to obtain the mean and variance.

- A random BB error is generated following Gaussian distribution with the mean and variance corresponding to the SINR value.

Fig. 5

Proposal 18: For AI mobility, in FR1, BB error should be added as:

- Added independently per Tx beam per cell per measurement occasion according to the corresponding SINR.

- The distribution of BB error under certain SINR is obtained through LLS assuming one shot measurement.

RF error is the ripples introduced by non-perfect RF components. If all the RF components remain unchanged, RF error will remain the same. If RF error is added independently before L1 filtering, then RF error will be averaged after L1 and L3 filtering, and the impact of RF error will be underestimated. Another way is to add RF error after L1 filtering, which is the same as defining measurement accuracy requirements. We had some initial check on this way and found the impact of RF error is much larger. We understand that the impact of RF error will be overestimated if adding after L1 filtering. But it is difficult to model RF error as it is in the reality. We are not sure which way is better. We are also thinking about whether it is reasonable to add RF error with larger variance but before L1 filtering. We raise this issue for further discussion.

Fig. 6

Observation 3: The impact of RF error will be underestimated if adding before L1 filtering. The impact of RF error will be overestimated if adding after L1 filtering.

Proposal 19: For AI mobility, in FR1, discuss how to add RF error.

## 2.6 Generalization

RAN2 has done some evaluation on generalization performance, i.e.,

- across different UE speeds

- across different cell configurations

  - Umi: ISD = 200m, BS antenna height = 10m, BS Tx power = 40dBm

  - Uma: ISD = 500m, BS antenna height = 25m, BS Tx power = 44dBm

- across different frequency prediction directions

  - 2GHz  4GHz

  - 4GHz  2GHz

However, we don’t think it is sufficient as the generalization performance over some measurement/evaluation related parameters has not been evaluated yet. E.g., different L3 filtering configurations, event thresholds, SMTC period, MGRP. We prefer the model can be agnostic to these configurations. Therefore, we suggest RAN4 continuing working on the generalization issue w.r.t. different parameters in the work item phase.

Observation 4: For generalization performance evaluation, RAN2 didn’t touch measurement/evaluation related parameters, e.g., different L3 filtering configurations, event thresholds, SMTC period, MGRP.

Proposal 20: RAN4 shall keep evaluating the generalization performance over measurement/evaluation related parameters, e.g., different L3 filtering configurations, event thresholds, SMTC period, MGRP and etc.

We have simulated the model performance under different L3 filtering configurations, i.e., k= 0, 2,4,6,8. In the simulation, the settings are as below:

- Scenario: temporal prediction

- OW: 4 x 40ms, PW: 40ms, 80ms, 120ms, 160ms

- L1 filtering: Sliding window

- Input to AI model: RSRP of two cells (the first two strongest) at 4 occasions in OW (2x4)

- Output from AI model: RSRP of two cells at 4 predicted occasions (2x4)

- Measurement error is added at both training and inference phase. The measured RSRP are used as ground truth, i.e., there is measurement error in ground truth.

- Inference performed when enter condition of A3 event (threshold 2dB, hysteresis=0) is met.

Table. 2 shows the performance of absolute RSRP difference and relative RSRP difference at 90% tile point with different k (RSRP difference at all predicted instances during the PWs are counted together). It can be observed that the predicted RSRP is more accurate with a larger k (historical samples share a greater contribution/weight). When L3 filtering is not used, i.e., k=0, the accuracy performance is the worst. As the performance difference between k=0 and other values is large, it seems not good to define a unified requirements for all L3 filtering configurations. RAN4 needs to discuss how to define the accuracy requirements for different L3 filtering configures.

Table 2

| k | Absolute RSRP difference [dB] | Relative RSRP difference [dB] |
| --- | --- | --- |
| 0 | 3.42 | 4.25 |
| 2 | 1.12 | 1.34 |
| 4 | 0.76 | 0.93 |
| 6 | 0.49 | 0.62 |
| 8 | 0.35 | 0.45 |

Observation 5: The performance with different L3 filtering configurations diverge.

Proposal 21: Further discuss how to define RSRP accuracy requirements for different L3 filtering configurations.

# Conclusion

In this contribution, we provide our views for AI mobility. Proposals and observations are summarized as follows.

Observation 1: In real field, event triggered L3 reporting is way more commonly used than periodic reporting.

Proposal 1: For the sake of power saving, UE should not be mandatory to predict the future all the time.

Proposal 2: For temporal prediction Case A, it is up to UE when to do the prediction.

Proposal 3: For temporal prediction Case B, only consider the case that NW transmit SSB periodically.

Proposal 4: For temporal prediction Case B, when and how to do the measurement and prediction is totally up to UE.

Proposal 5: For temporal domain prediction, measurement event prediction should be the focus and RAN4 only need to define test cases for measurement event prediction.

Proposal 6: Use the simulation assumptions in Table 5.1-1, Table 5.3.1-1 and Table 5.3.1-2 in TR38.744 as a basis.

Proposal 7: In the simulation, take Rx beam sweeping delay into consideration in FR2.

Proposal 8: For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), discuss whether co-located cells on different frequencies are with varied BS configurations (e.g., antenna pattern, downtilt angle) or not.

Observation 2: For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), if co-located cells from the low and high bands share the same horizontal coverage range, there is no need to predict RSRP of cells in low band through RSRP of cells in high band.

Proposal 9: For Scenario 3, FR1 to FR1 inter-frequency (frequency domain), if co-located cells from the low and high bands share the same horizontal coverage range and UE is supposed to predict RSRP of high band through RSRP of cells in low band, UE only needs to do the prediction when RSRP of the cell in low band is lower than a threshold.

Proposal 10: If to get the ground truth based on the transmitted or reception power in FR1, the ground truth for occasion X can be calculated as (1-α)*Fn-1+α*Mn, where

- Mn: the average transmission power of occasion #X-160ms, X-120ms, X-80ms, X-40ms, X (L1 filtering with 5 samples).

- Fn-1: the updated filtered RSRP at occasion #X– 200ms) (assume non-sliding window following the specification of L3 filtering).

- α is derived according to 38.331 cl. 5.5.3.2.

Proposal 11:  If to derive the ground truth through transmitted power at TE, additional margin should be considered due to different L1 filtering and L3 filtering may be adopted by UE vendors. The value of the margin can be different for varied UE speed, which is proposed as 3dB for 30km/h and 4dB for 90km/h.

Proposal 12: If to use reported RSRP as ground truth, the ground truth of L3-RSRP predicted for the time instance X is the L3-RSRP measurement result reported within Y ms before and after the time instance X. Y is the defined measurement delay considering all kinds of UE implementation.

Proposal 13: When comparing the predicted results with reported RSRP, due to there may be Yms gap, additional RSRP difference margin should be added. The value of the margin shall be the same as getting the ground truth based on the transmitted or reception power, if non-sliding window is adopted.

Proposal 14: When comparing the predicted results with reported RSRP, if UE also reports the measurement results for every SMTC occasion, then there is no need to add additional margin.

Proposal 15: If to use reported RSRP as ground truth, RAN4 need to discuss how to determine the additional margin, e.g., based on the worst case or based on companies input.

Proposal 16: When do the simulations, the data containing cells with too low SNR (e.g., <-6dB) should be excluded from the dataset.

Proposal 17: For scenario 3 (inter-frequency measurement prediction), discuss how UE gets cell ID of the to-be-predict cell(s) for reporting the prediction results to network.

Proposal 18: For AI mobility, in FR1, BB error should be added as:

- Added independently per Tx beam per cell per measurement occasion according to the corresponding SINR.

- The distribution of BB error under certain SINR is obtained through LLS assuming one shot measurement.

Observation 3: The impact of RF error will be underestimated if adding before L1 filtering. The impact of RF error will be overestimated if adding after L1 filtering.

Proposal 19: For AI mobility, in FR1, further discuss how to add RF error.

Observation 4: For generalization performance evaluation, RAN2 didn’t touch measurement/evaluation related parameters, e.g., different L3 filtering configurations, event thresholds, SMTC period, MGRP.

Proposal 20: RAN4 shall keep evaluating the generalization performance over measurement/evaluation related parameters, e.g., different L3 filtering configurations, event thresholds, SMTC period, MGRP and etc.

Observation 5: The performance with different L3 filtering configurations diverge.

Proposal 21: Further discuss how to define RSRP accuracy requirements for different L3 filtering configurations.

# Reference

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.

# Appendix