3GPP TSG-RAN WG4 Meeting #117                                                               R4-2520374

Dallas, USA, 17th – 21st November, 2025

Source:	Samsung

Title:	Discussion on RAN4 operation efficiency for 6GR

Agenda item:			8.12.1

Document for:	Approval

# 1. Introduction

In RAN#108, RAN plenary approved a new RAN working group level SID on 6G Radio [RP-251881], which has been further revised into [1, RP-252912] in RAN#109. Specifically, the topic of RAN4 operation efficiency in the RAN4 6G topic is planned with the following scopes [2]:

| 6G Topics | Scope | Scope in Q4'25 and Q1'26 |
| --- | --- | --- |
| RAN4 operation efficiency | CR, spec improvement and modernization in general, new tools for band/band combinations, other aspects | CR, spec improvement and modernization in general, new tools for band/band combinations, other aspects |

In this contribution, we would like to provide our viewpoints on RAN4 operation efficiency in the 6G study.

# 2. Discussion on general aspects

## 2.1 General specification improvement

Based on last meeting agreement, because of the existence of 6GSM SI, for modernization and new tools, the relevant discussion is agreed to be postponed until March 2026, and companies are encouraged to be actively involved in SID discussion:

On the conclusions of 6GSM SID Conclusions: - Postponed until March 2026 - Companies encouraged to be actively involved into SID discussion..

By looking into the identified specification related problem, we can see 6GSM is inclusive and we have not yet identified any non-technical issues discussed in RAN4 are missed in 6GSM discussion.

Proposal 1: All non-technical non-RAN4-specific specification issues should be discussed in 6GSM, and there is no need to continue the discussion on non-technical non-RAN4-specific specification issues (even after March 2026).

## 2.2 RAN4-specific specification issues

How to capture the standardized AI/ML model (e.g., AI/ML model for conformance testing, AI/ML model used for reference purpose, etc.) and standardized dataset (e.g., dataset to derive reference model, etc.) in the 3GPP specification is very likely to be the problem only for RAN4. Other working group could rely on RAN4 to define reference model for operation, and RAN4 may also the needs to define testing model. For dataset, it is likely that RAN4 may need to use the specified dataset for training reference model purpose, while other WGs may only specify the data format for transmission and no needs to specify certain dataset.

Observation 1: How to capture the standardized AI/ML model (e.g., AI/ML model for conformance testing, AI/ML model used for reference purpose, etc.) and standardized dataset (e.g., dataset to derive reference model, etc.) in the 3GPP specification can be RAN4-specific issue.

In Rel-19 discussion on RAN1-led AI/ML work item, RAN4 has practice both dataset and model sharing for the practice of two-sided model interoperability study. The following are agreements achieved for logistical issues:

Agreement For AI/ML model formats: - Use at least ONNX format for sharing. - Companies can also share PyTorch model - The models in different formats from one company shall be the same - Use version v1.16 or later - ONNX version and opset version number to be included in the file Agreement: For dataset format - Use NumPy for dataset sharing - Do not use pickled data(compression mechanism) - Use npy – single array in each file Agreement: Dataset/model input file format: - N (samples) X 2 (IQ) X nSB (number of subbands) X nPorts (number of CSI-RS ports) X nLayers (number of layers) - In other words, dataset should content following info: - 1st dimension: Number of samples - 2nd dimension: Real and imaginary - 3rd dimension: Number of sub-bands - 4th dimension: Number of antenna ports - 5th dimension: Number of layers – present only if >1 layer - Note: Each element of the dataset will be a float32 real number The dataset includes eigenvectors calculated based on average covariance channel matrix for the given MIMO layer for each sub-band based on pre-processing assumptions in WF R4-2414447 Single file for fixed scalar quantization(only dataset), TBD whether additional file is needed for variable/trainable quantization Dataset files can be split into multiple files to enable easier upload. - Use the 2 digits for split files starting from 00, increment for each additional file Split files and then archive each file Agreement: For file naming scheme - Folders for AI/ML data sharing and current WI/use case to be created under “RAN4 folder” - Subfolder created for each meeting File naming scheme( ML model file and dataset file) - a unique identifier for the company (4 characters – list to be maintained by RAN4 secretary) - meeting number - differentiate model and dataset by identifier (2 characters ml and ds) - additional identifier with 2 characters (TBD) - ec for encoder - dc for decoder - ei for encoder input - do for decoder output - lt for latent (decoder input or encoder output) - others to be added as needed - dataset can be split in multiple files – 2 digits - files to be compressed to zips and uploaded Agreement: Files to be shared, to be decided based on option under study - Dataset containing encoder input per subband - Encoder and/or decoder model

Based on Rel-19 experience, at least the following aspects needs to be studied for how to capture the standardized AI/ML model and standardized dataset in the 3GPP specification.

Proposal 2: For specification modernization for capturing the standardized AI/ML model and the standardized dataset, the following issues can be studied in RAN4:

- Unified AI/ML model format for sharing

- Dataset format and data description

- AI/ML model and dataset naming rules

- Methods of sharing (including the current FTP-based method) by considering download/upload efficiency, the convenience for model/dataset update, and the availability of the location link for easy citation

## 2.3 RAN4-specific working procedure optimization

Based on Rel-15 experience, the specification editors played the most important role to control the specification quality. RAN4 specification contains very broad areas from RF to baseband, which further require editor-led procedure is important for 6G specification quality control.

Proposal 3: For specification quality improvement, the following procedures led by specification editors can be considered:

- Before CR to be agreed, it is encouraged to use the running CR approach to allow more time for CR editing.

- CR drafted by companies can only be approved after the review of specification editor.

- For each specification, the online drafting session can be organized by the specification editor in the gap period between two RAN4 meetings.

# 3. RAN4 6G Specification Enhancement

## 3.1 RRM

Streamlined RRM requirements

In 5GNR, RAN4 RRM specified so many RRM requirements in specs. It was complained that the specs are convoluted. Moreover, many of the RRM requirements haven’t never actually been utilized in real-world deployments. In 6GR discussion, there is no further progress from other groups. However, RAN4 can align with high-level principles for 6G Day-1 RRM spec.

Firstly, it is not “Must” to define RRM requirements for all features and procedures in Day-1. We propose to use two criteria:

a) Must to have actual impacts and guidance on implementation design. As mentioned above, many of the RRM requirements haven’t never actually been utilized in real-world deployments. Take an example, several MGs have never been utilized in practice.

b) Must to be tested and testable in conformance testing: If RRM requirements cannot be tested with testability issue, there is really no need to waste time discussing corner cases and cases in paper work.

Secondly, even if the procedure name is the same, the procedures and components can be different in 6GR compared to 5GNR. Take an example, in many RRM requirements, RRM requirement are discussed by assumption of timeline of UE behavior including:

-        Whether RF retuning is needed. In the very beginning of Rel-15, the assumption of RF retuning time is long.

-        Whether AGC is needed.

-        Whether the bandwidth change is needed

-        Whether BB processing is needed

-        Etc.

With the development of UE in 6GR, theses factors may be update by new assumption.

Proposal 4: For 6G Day-1 RRM requirements, we propose to align with high-level principles for:

- RAN4 to define necessary RRM requirements for key features and procedures. It is not mandatory to define RRM requirements for all features and procedures. To consider by two criteria:

  - Must to have actual impacts and guidance on implementation design. As mentioned above, many of the RRM requirements haven’t never actually been utilized in real-world deployments. Take an example, several MGs have never been utilized in practice.

  - Must to be tested and testable in conformance testing: If RRM requirements cannot be tested with testability issue, there is really no need to waste time discussing corner cases and cases in paper work.

- Even the names of procedures are the same as in 5GNR, it doesn’t mean RAN4 will reuse the exactly same RRM requirements in 5GNR. Take an example, RRM with timeline procedures can be changed in 6GR.

- RAN4 to discuss and achieve the common assumption of each component for different UE capabilities, including assumption of RF and BB processing, like: RF retuning time, AGC time, time for change bandwidth, time for BB processing, T/F tracking, number of searchers, etc. It can be shared and utilized in different RRM requirements to avoid different and excursive assumption for timeline RRM requirements.

RRM Spec structure and drafting rules

In 6GR, we think the overall spec structure can be inherited such as: RRC_IDLE/INACTIVE/ CONNECTED state mobility, Timing, Signaling, Measurement. etc. However, there are a few pain points to identify. Take an example, in current 5GNR spec, many sub-clauses for CCA are naming with x.A. Many sub-clauses for RedCap are naming with x.B. Many sub-clauses for NTN are naming with x.C. When the new features are introducing one by one, the number is excursive and lot of repeated wording in the similar clauses. RAN4 need a high-level principle to decide when a new feature is introduced, whether it can be specified in new set of RRM requirements. For example, if RedCap used in new sub-clauses set, whether if FWA, high speed train UE, normal UEs, ATG UEs, UAV UEs; NTN UEs, have similar parallel clauses in spec. It led to more and more parallel and repeated wording for similar procedures.

Proposal 4: For 6GR RRM spec structure and drafting rules, the overall spec structure in 5GNR can be inherited such as: RRC_IDLE/INACTIVE/ CONNECTED state mobility, Timing, Signaling, Measurement. etc.

- RAN4 to discuss and decide the high-level principle to decide whether a new feature is introduced, new sub-clauses can be allowed or not. We prefer to category the clauses from procedures and different assumptions rather than UE types.

For the drafting rule, we can use the discussions on Rel-19 RRM Specification quality improvement as start point, the following rules are recommended to enhance the readability and consistency of the RRM spec:

Proposal 5: RAN4 can use the following aspects as start point:

- Reuse the Big CR procedure and RAN4 Chair and MCC’s rules of Big CR: no [], TBD, FFS clean up in the Big CR and specs.

- Reuse the rules of “Forward section” to ensure consistent usage of frequently used terms, notation, abbreviations, CA configuration vocabulary, etc.

- For new features, determine the common rule of whether to add a new sub-clause. If new sub-clauses are introduced:

  - It is recommended to clearly declare the numbering corresponding to a feature in an appendix or designated location.

  - For situations where similar text needs to be repeated across multiple sections (or specifications), the general text should first be agreed upon as a reference and then used across different sections/CRs/specifications to improve consistency.

## 3.2 Demod

For FRCs in 6G Demodulation specification, prefer to use a formula-based or pseudo-code-based definition for FRCs instead of table-based approach listing every parameter combination. Especially for FRCs in multiple BS specifications (38.104, 38.141-1, 38.141-2), suggest at least to align the numbering among them to avoid confusing references.

Proposal 6: Use a formula-based or pseudo-code-based definition for FRCs instead of table-based approach listing every parameter combination.

# 4. Conclusion

In this contribution, we have provided our viewpoints on RAN4 operation efficiency in the 6G study:

# 4. Reference

[1] RP-252912 (revision from RP-251881), “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN#109.

[2] R4-2511652, “RAN4 6G Plan and Meeting Arrangement”, RAN4 chair, RAN4#116.

[2] R4-2514648, “Way Forward for [116bis][111] 6G operation efficiency”, CATT, RAN4#116bis.