3GPP TSG-RAN WG4 Meeting #117                                                               R4-2520376

Dallas, USA, Nov. 17-21, 2025

Source:	Samsung

Title:	Discussion on Inter-vendor training collaboration and interoperability

Agenda item:			7.9.3

Document for:	Discussion

# 1. Introduction

Rel-20 new WID was approved in RAN#108 and was further updated in RAN#109 [1]. Among objectives of this work item, the following are RAN4 relevant work scope on inter-vendor training collaboration for two-sided AI/ML models, and RAN4 requirements of interoperability and RRM:

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

After the discussion in the first RAN4 meeting in Rel-20, following agreements are achieved on the steps to complete the WI objectives [2], in which some preliminary consensus on model backbone and model developing steps is given.

Agreement Which SI option to use for the WI Based on the current WID RP-251870, RAN4 only defines the requirements for the encoder. RAN4 will discuss option 3 only this WI. Test decoder and reference encoder backbone The test decoder and the reference encoder in Option 3 will have the same backbone. - It is FFS if a single test decoder will be specified. - The test decoder and the reference encoder can have different structures, e.g. # of layers Dataset for model evaluation RAN4 will discuss how to introduce the new mixed dataset for model structure evaluation purpose at the beginning of this WI, by taking into consideration of the scalability and/or various configurations. - Details are FFS including the channel model(s). Scalability Unless the significant issues are identified, RAN4 is going to only discuss the scalability issues and options, which have been agreed in RAN1. Model backbone Transformer is assumed as the baseline Only Quantized parameters are specified Steps to develop the test decoder and reference encoder: - Step 1: Create dataset - Step 1-1: Align on configurations for mixed dataset construction, taking factors such as scalability over Tx port number and subband size and payload size into account - Step 1-2 Decide how align and on which metrics can be used for evaluating dataset alignment, if any - Step 1-3 Companies create datasets - Step 1-4 Identify which company datasets to include in the mixed dataset(s) - Step 1-5 Create new dataset(s) - Step 2: Align on the model backbone, model hyperparameters and the scalability solution. - Step 2-1: Proposed model structures (model backbone and model hyperparameters) are brought by companies, as well as proposed scalability solution. - This can be in parallel with step 1. - Transformer is agreed as baseline for backbone - Same backbone for test decoder and reference encoder - Step 2-2: Preliminary selection of structures, taking into account complexity - [Step 2-2a Align on training parameters] - Step 2-3: Agree on metrics and steps for evaluation - Step 2-4: Check on performance alignment -> see simulation results from contributing companies. - Step 2-5: Share models (encoder or decoder or both)/datasets (training/testing/inference), if needed. - Step 2-6: Select one or more model structures considering complexity and robustness and decide how many models. - Step 2-7: Select one or more scalability solutions. This step can be done in parallel with Steps 2-2 to 2-4. - Step 3: Specify the model for the following work in WID. Where and how to specify may need further discussion. Note: At this stage, models using the agreed structures, dataset, scalability have already been created by most companies - Step 3-1 Decide how to select from the trained model(s) from each company - Step 3-2: Freeze model(s) - Step 3-3: Agree on how and where to capture the models, the dataset and relevant information

The WF also recommends the following discussion topics to provide guidance for RAN#117.

- Proposals for assumptions for the dataset including how to cover scalability, channel models, number of samples - Proposals for how to evaluate the dataset including potential metrics to compare different dataset contributions - Proposals on model hyperparameters, considering in particular complexity, considering also quantization size - Proposals and reasoning for scalability solutions - Proposals and reasoning for training parameters - Proposals for performance metrics for evaluating the different potential model structures - Proposals for how to capture the model and the dataset in 3GPP (format, how to store etc.)

In this contribution, we aim to provide our views on these topics.

# 2. Discussion

## 2.1 Consideration on dataset assumption

Channel model

In Rel-19, Uma channel model in SLS was used in RAN4 to generate dataset and evaluate the AI/ML models. In Rel-19, RAN4 work was concentrated on the alignment of models through convergence of intermediate SGCS for Option3 and Option 4. However, in Rel-20, RAN4 relevant work consists of specifying reference encoder and test decoder with scalability capability, and defining the performance requirement of the encoders. In the RAN4 test, LLS is utilized for demod testing, with TDL and CDL channel models available. The specified models need to be embedded into LLS to evaluate the final performance. Naturally, the dataset for training models should be generated by LLS. Otherwise, the models trained on SLS channel data require extra verification on the LLS channel data.

Observation 1: The specified reference encoder and test decoder will be embedded into LLS to define the requirement of the encoder.

Proposal 1: RAN4 uses LLS to generate dataset for model training.

Proposal 2: RAN4 should study which of the following channel models is used in LLS for generating dataset and for AI/ML CSI enhancement evaluation,

- Atl. 1, TDL channel model;

- Atl. 2, CDL channel model.

TDL channel model has been calibrated for demod testing in RAN4. In Rel-19, RAN4 led one study item to calibrate CDL channel model for demod testing. At the end of the study item, further study on performance alignment with CDL channel model for PMI testing was concluded. CDL channel model has not be calibrated among companies. It can not be used for RAN4 to perform AI/ML CSI enhancement evaluation.

Observation 2: RAN4 has not calibrated CDL channel for PMI testing.

Generalization for various scenarios in the practical environment is the primary issue of AI/ML model deployment. TDL channel model is established per propagation route between one Tx port and one Rx port. Different propagation routes are independently initialized by different random seeds. There are no spatial features such as propagation angles included in TDL channel model. In contrast, spatial channel models (e.g., Uma in SLS and CDL in LLS) better match the practical wireless propagation characteristics.  However, in [3], vivo showed that large SGCS degradation occurs when models trained on Uma channel data infers on CDL-C channel data. Based on our experience, the reason may be that Uma channel model generates channel samples with various propagation angles (AOD, ZOD) based on the random dropping positions, while CDL channel model generates channel samples with the fixed mean angles of AOD and ZOD (Annex A.2 in TR 38.901) according to the specified clusters. The fixed mean angles of departure correspond to almost changeless eigenvectors. The lack of angle diversity leads to the SGCS degradation. To address this issue, a range of different mean angles of AOD and ZOD can be applied to initialize different CDL channel models.

Observation 3: TDL channel model is established per propagation route between one Tx port and one Rx port. There are no spatial features such as propagation angles included in it.

Observation 4: Compared to TDL channel model, CDL channel model more closely approximates real-world wireless propagation channels.

Proposal 3: If RAN4 agrees on TDL channel model, RAN4 should consider methods to add spatial features / restrictions such as propagation angles for approximating real-world wireless propagation channels. .

Observation 5: Initializing different CDL channel modes with a range of mean angles of AOD and ZOD could provide data diversity for generating the dataset.

Proposal 4: If RAN4 agrees on CDL channel model, RAN4 should consider the data distribution diversity from the specified CDL channel models and consider methods to enrich the data diversity for generating the dataset.

Dataset content

In Rel-19, the mixed dataset contains precoding matrixes associated with a single layer. Similarly, new mixed dataset for Rel-20 is proposed to contain precoding matrixed associated with multiple layers. RAN4 is to discuss the maximum layer number. According to the maximum layer number, the right eigenvectors of channel response are extracted in sequence in the descending sorting order of eigen values. To facilitate alignment and attain good intermediate performance, unified column normalization and phase normalization should be adopted to normalize the eigenvectors.

Observation 6: In Rel-19, RAN4 mixed dataset contains precoding matrixes associated with a single layer.

Proposal 5: The mixed dataset contains precoding matrixes associated with a maximum layer number.

Proposal 6: RAN4 is to discuss the maximum layer number. Maximum Rank 2 shall be studied.

Proposal 7: RAN4 determines a unified method to achieve the precoding matrix associated with the maximum layer number. Following is recommended.

- Select the right eigenvectors corresponding to the top-maximum- layer-number eigen values

- Perform column normalization by normalizing the L2 norm of each eigenvector in to 1

- Perform phase normalization by subtracting the angle of the first element from the angles of all the elements in each eigenvector

Metric to compare datasets from companies, Principles to select company datasets

Define a dissimilarity metric to quantify the change of eigenvectors across the sub-bands per layer. 1-SGCS(vi,vbasis) can be treated as one dissimilarity metric. vi-vbasis could plays the same role, where vi is one eigenvector on sub-band i, vbasis is the eigenvector on the basis sub-band and ∙ means L2 norm. The eigenvector of the middle sub-band constitutes the basis eigenvector. The dissimilarity metric is used to measure the spread of dissimilarity compared to the basis along the sub-bands per layer.  The averaged spread of dissimilarity over the samples and the distribution of spread of dissimilarity are reported per layer as the subsequent selection metric.

Proposal 8: A spread of dissimilarity between basis eigenvector and eigenvectors along the sub-bands is proposed as the metric to compare datasets. Details are as following:

- Define a dissimilarity metric to quantify the change of eigenvectors across sub-bands per layer. 1-SGCS(vi,vbasis) and vi-vbasis are the candidate metrics.

- The eigenvector on the middle sub-band constitutes the basis eigenvector.

- The spread of dissimilarity between basis eigenvector and eigenvectors along the sub-bands is measured. Averaged spread of dissimilarity over samples and the distribution of spread of dissimilarity are reported per layer as the selection metric for the mixed dataset.

The principles to select company datasets should be 1) identifying valid change trend of eigenvectors along the sub-bands that conforms to the frequency selectivity, 2) covering sufficient feature distributions for model generalization. So, methods should be discussed to select company datasets according to the reported metrics.

Observation 7: To select company datasets, the following characteristics represented by the proposed metrics can be considered:

- valid change trend of eigenvectors along the sub-bands that conforms to frequency selectivity

- a range of lower bound and upper bound on averaged spread of dissimilarity.

Channel configurations for evaluate the scalability solutions

To evaluate the scalability solutions on Tx port number, sub-band size and payload size, typical values on the three scalable aspects should be discussed and determined before generating the training dataset.

Proposal 9: Following typical values are proposed to cover the scalability:

- Tx port number {32, 16, 8}

- Sub-band size {13, 11, 9}

- Payload size {eType II PC=6, PC=5} as an example

In Rel-19, only precoding matrixes associated with a single layer is considered. If precoding matrixes associated with multiple layers are supported, it needs further discussion whether rank-common layer-common model or rank-specific model is adopted. Different types of models correspond to different payload size.

Observation 8: If multiple layers are supported in Rel-20, there are two types of models to handle the generalization for layer number, rank-common layer-common model and rank-specific model.

Observation 9: If multiple layers are supported in Rel-20, in the case of rank-common layer-common model, the payload size is in proportion to layer number; in the case of rank-specific model, the payload size is directly the output size of quantizer.

Observation 10: If multiple layers are supported in Rel-20, for rank-common layer-common model and rank-specific model, the scalability solution on payload size may be different.

Proposal 10: If multiple layers are supported in Rel-20, it needs discussion whether rank-common layer-common model or rank-specific model is adopted. Because the payload size and the scalability solution on payload size are associated with the model types.

## 2.2 Consideration on encoder and decoder structure

Since Transformer was agreed as the backbone of reference encoder and test decoder in the last meeting, a unified model structure with configurable model hyperparameters should be determine for the evaluation of performance-complexity trade-off. Different model hyperparameters produce models with different complexities. Taking the study outcome from RAN1, one unified configurable model structure is proposed.

Proposal 11: RAN4 is to determine a unified model structure with Transformer backbone and configurable model hyperparameters for evaluation of performance-complexity trade-off. Following model structure is proposed.

Table 1. unified model structure with Transformer backbone and configurable hyperparameters

| Encoder – FC1 (Embedding layer) | Output dimension dmodelEC per token |
| --- | --- |
| Encoder – Positional encoding | Dimension of positional encoding for each token position: dmodelEC |
| Encoder – Transformer encoder blocks | Number of transformer encoder blocks NEC Dimension of transformer encoder block dmodelEC Number of self-attention heads NheadEC Dimension of attention head dheadEC=dmodelEC/NheadEC Dimension of latent space inside feedforward module dFFNEC Activation function: relu |
| Encoder – FC2(Output linear layer) | Reshape the matrix to a vector Output dimension doutEC |
| Quantization and Dequantization | Scalar quantization: Number of bits per latent dimension: Nbit Total payload size for Scalar quantization = Nbit ×doutEC |
| Decoder – FC1 (Embedding layer) | Reshape the vector to a matrix Output dimension dmodelDC per token |
| Decoder – Positional encoding | Dimension of positional encoding for each token position: dmodelDC |
| Decoder – Transformer encoder blocks | Number of transformer encoder blocks NDC Dimension of transformer encoder block dmodelDC Number of self-attention heads NheadDC Dimension of attention head dheadEC=dmodelDC/NheadDC Dimension of latent space inside feedforward module dFFNDC Activation function: relu |
| Decoder – FC2(Output linear layer) | Output dimension doutDC=NTx×L×2, L is the layer number per inferring |

Models with different hyperparameters have different complexities, usually represented as computation complexity (e.g., FLOPs, MACs) and storage complexity (e.g., parameter number). FLOPs and MACs are both metrics for computation complexity and they are confusable. FLOPs (Floating Point Operations) means the total number of floating point operations. MACs (Multiply ACcumulate operations) means the computing steps that involves multiplying two numbers and adding the product to a running total in an accumulator. One MACs approximately equals two FLOPs. But some statistical tools output misleading FLOPs results. To accurately align model complexity and conduct performance-complexity trade-off, precise computation complexity must be reported by companies. As quantized parameters are agreed, the storage size of parameters are also recommended to be reported, because different quantized precision, INT8 and BF16 as examples, produce different storage sizes.

Observation 11: FLOPs and MACs are both metrics for computation complexity and they are confusable. Some statistical tools output misleading FLOPs results.

Observation 12: One MACs approximately equals two FLOPs. FLOPs (Floating Point Operations) means the total number of floating point operations. MACs (Multiply ACcumulate operations) means the computing steps that involves multiplying two numbers and adding the product to a running total in an accumulator.

Observation 13: Different quantized precision, INT8 and BF16 as examples, produce different storage sizes.

Proposal 12: To report the accurate complexity, FLOPs, MACs, parameter numbers and storage sizes are proposed to be reported together.

In Rel-19, RAN4 verified the feasibility of compatibility between a common CNN decoder and several encoders with different backbones and with different complexities. In common, the smaller the complexity of the encoder is, the worse the SGCS performance is. To evaluate the performance-complexity trade-off, several encoders with different complexities should be trained by the same N(encoders)-to-1(decoder) method developed in Rel-19. And the several encoders with different complexities are suggested to be aligned. Finally, based on the trade-off between throughput gains and complexities, at least one encoder is specified as the reference encoder.

Observation 14: In Rel-19, RAN4 verified the feasibility of compatibility between a common CNN decoder and several encoders with different backbones and with different complexities.

Observation 15: It was observed that the smaller the complexity of the encoder is, the worse the SGCS performance is.

Proposal 13: Only one decoder with reasonable and acceptable complexity is specified in Rel-20.

Proposal 14: To evaluate the performance-complexity trade-off, several encoders with different complexities should be trained with the same one decoder by the same N(encoders)-to-1(decoder) method developed in Rel-19.

Proposal 15: Based on the trade-off between throughput gains and model complexities, at least one encoder can be used as the reference encoder to derive the performance requirement.

## 2.3 Consideration on scalability methods

Based on the above unified model structure, to avoid duplicate work, RAN4 should downselect the scalability solutions from RAN1 study outcome. We proposed the following solutions. It should be noted that in theory, scalability on the sub-band size can be naturally handle by the transformer blocks, as the varying sequence length in NLP. There is no need of padding at the input. We will verify this.

Observation 16: When taking sub-band as the token dimension, theoretically, scalability on the sub-band size can be naturally handle by the transformer blocks, as the varying sequence length in NLP.

Proposal 16: We proposed to downselect the following methods from RAN1 study outcome to reduce duplicate work.

Table 2. methods for model scalability.

| choice of token dimension and feature dimension | Use sub-band as the token dimension and Tx port as a feature dimension |
| --- | --- |
| scalability on the Tx ports | a common embedding layer with padding |
| scalability on the sub-band size | - The number of tokens varies with the sub-band size and no additional scalability method is needed. - Padding at the input |
| scalability on payload size | truncation/masking of the output linear layer output |

To evaluate the scalability solutions, two cases are design to evaluate the performance and identify the performance losses from scalability solutions. The performance gaps between Case 1 and Case 2 are the losses from the scalability methods.

- Case 1 (scalable structure with joint training on scalable scenarios): use the scalable model structure and scalability methods, train the model with single parameter set across different {Tx port number, payload size, sub-band size} data.

- Case 2 (dedicated model structure trained on dedicated data): train one dedicated model with the input size and output size exactly matches the dedicated {Tx port number, payload size, sub-band size} data.

Proposal 17: RAN4 use Case 1 (scalable model structure with joint training on scalable scenarios) and Case2 (dedicated model structure trained on dedicated data) to evaluate the performance of scalability solutions and identify the losses brought by scalability solutions.

## 2.4 Consideration on test metric for the performance requirements

The overall work in Rel-20 could be divided into two parts. The first one is to align and specify the test decoder and reference encoder. The second one is to evaluate the throughput gains using the specified models. For the first one, intermediate metric is set to evaluate the absolute accuracy of reconstructed precoding matrixes. Same as in Rel-19, SGCS is proposed. For the second one, final metric is set to evaluate the absolute throughput and/or throughput gains over the legacy non-AI CSI feedback method if the gain is testable.

Proposal 18: For results aligning for two-sided model training and inference evaluation, absolute SGCS is proposed as the performance metric.

Proposal 19: For verify the throughput gains and specify the performance requirements of the encoder, RAN4 can take the throughput related metrics as performance metric, e.g., absolute throughput or throughput gain over the non-AI CSI feedback or others depending on testability.

When comparing with non-AI CSI feedback, eType II codebook should be treated as the benchmark. And which parameter combination should be used needs further discussion. The overhead of non-AI method should be comparable with the one of AI/ML models. For the best, the two overheads of the two methods are equivalent.

Proposal 20: When RAN4 discusses the achievable performance gain of AI/ML-based CSI feedback enhancement, the eType II-based codebook can be taken as the baseline. The overheads of eType II and AI/ML models should be comparable or equivalent.

## 2.5 Consideration on capture the model and the dataset in 3GPP

To define the requirement of one encoder, RAN4 needs to specify the test decoder, reference encoder and the dataset. Model structures of the test decoder and reference encoder can be detailed in specifications. But model parameters are usually three-dimension or four-dimension tensors, which are difficult to specified in specifications. Instead, model parameters stored in open-source file format, like ONNX, are finely structure and convenient to obtain. The parameter file can be stored in 3GPP server and obtained by specified links. In the same way, the dataset can be stored and obtained in the open-source file format.

Proposal 21: Model structures are specified in specifications. Model parameters and dataset are stored in open-source file format, like ONNX, in 3GPP server with specified links.

It was agreed in the last meeting that only quantized parameters are specified. It should be noted that the quantized parameters, regardless of in INT8, BF16 or FP16 format, will bring in degradation of intermediate performance, compared to float32 format parameters. If quantized parameters are specified for consideration of practical deployment, the degradation must be considered in advance. In general, there are two types of model quantization methods to obtain quantized parameters, Quantization-Aware Training (QAT) and Post-Training Quantization (PTQ). Different companies may use different methods with different detailed configurations.

Observation 17: Quantized parameters, regardless of in INT8, BF16 or FP16 format, will bring in degradation of intermediate performance, compared to float32 format parameters.

Observation 18: There are two types of model quantization methods to obtain quantized parameters, Quantization-Aware Training (QAT) and Post-Training Quantization (PTQ)

Proposal 22: RAN4 should perform model alignment using the models with quantized parameters.

Proposal 23: RAN4 should study which quantization method is adopted and the detailed quantization configurations,

- Quantization-Aware Training (QAT)

- Post-Training Quantization (PTQ)

Proposal 24: RAN4 should also achieve the alignment of quantization method and detailed quantization configuration of the quantization method, regardless of QAT and PTQ.

# 3. Conclusion

In this contribution, we provided our viewpoints on the recommended aspects of CSI feedback enhancement, accordingly the following observations and proposals are obtained:

Observation 1: The specified reference encoder and test decoder will be embedded into LLS to define the requirement of the encoder.

Proposal 1: RAN4 uses LLS to generate dataset for model training.

Proposal 2: RAN4 should study which of the following channel models is used in LLS for generating dataset and for AI/ML CSI enhancement evaluation,

- Atl. 1, TDL channel model;

- Atl. 2, CDL channel model.

Observation 2: RAN4 has not calibrated CDL channel for PMI testing.

Observation 3: TDL channel model is established per propagation route between one Tx port and one Rx port. There are no spatial features such as propagation angles included in it.

Observation 4: Compared to TDL channel model, CDL channel model more closely approximates real-world wireless propagation channels.

Proposal 3: If RAN4 agrees on TDL channel model, RAN4 should consider methods to add spatial features / restrictions such as propagation angles for approximating real-world wireless propagation channels. .

Observation 5: Initializing different CDL channel modes with a range of mean angles of AOD and ZOD could provide data diversity for generating the dataset.

Proposal 4: If RAN4 agrees on CDL channel model, RAN4 should consider the data distribution diversity from the specified CDL channel models and consider methods to enrich the data diversity for generating the dataset.

Observation 6: In Rel-19, RAN4 mixed dataset contains precoding matrixes associated with a single layer.

Proposal 5: The mixed dataset contains precoding matrixes associated with a maximum layer number.

Proposal 6: RAN4 is to discuss the maximum layer number. Maximum Rank 2 shall be studied.

Proposal 7: RAN4 determines a unified method to achieve the precoding matrix associated with the maximum layer number. Following is recommended.

- Select the right eigenvectors corresponding to the top-maximum- layer-number eigen values

- Perform column normalization by normalizing the L2 norm of each eigenvector in to 1

- Perform phase normalization by subtracting the angle of the first element from the angles of all the elements in each eigenvector

Proposal 8: A spread of dissimilarity between basis eigenvector and eigenvectors along the sub-bands is proposed as the metric to compare datasets. Details are as following:

- Define a dissimilarity metric to quantify the change of eigenvectors across sub-bands per layer. 1-SGCS(vi,vbasis) and vi-vbasis are the candidate metrics.

- The eigenvector on the middle sub-band constitutes the basis eigenvector.

- The spread of dissimilarity between basis eigenvector and eigenvectors along the sub-bands is measured. Averaged spread of dissimilarity over samples and the distribution of spread of dissimilarity are reported per layer as the selection metric for the mixed dataset.

Observation 7: To select company datasets, the following characteristics represented by the proposed metrics can be considered:

- valid change trend of eigenvectors along the sub-bands that conforms to frequency selectivity

- a range of lower bound and upper bound on averaged spread of dissimilarity.

Proposal 9: Following typical values are proposed to cover the scalability:

- Tx port number {32, 16, 8}

- Sub-band size {13, 11, 9}

- Payload size {eType II PC=6, PC=5} as an example

Observation 8: If multiple layers are supported in Rel-20, there are two types of models to handle the generalization for layer number, rank-common layer-common model and rank-specific model.

Observation 9: If multiple layers are supported in Rel-20, in the case of rank-common layer-common model, the payload size is in proportion to layer number; in the case of rank-specific model, the payload size is directly the output size of quantizer.

Observation 10: If multiple layers are supported in Rel-20, for rank-common layer-common model and rank-specific model, the scalability solution on payload size may be different.

Proposal 10: If multiple layers are supported in Rel-20, it needs discussion whether rank-common layer-common model or rank-specific model is adopted. Because the payload size and the scalability solution on payload size are associated with the model types.

Proposal 11: RAN4 is to determine a unified model structure with Transformer backbone and configurable model hyperparameters for evaluation of performance-complexity trade-off. Following model structure is proposed.

Table 1. unified model structure with Transformer backbone and configurable hyperparameters

| Encoder – FC1 (Embedding layer) | Output dimension dmodelEC per token |
| --- | --- |
| Encoder – Positional encoding | Dimension of positional encoding for each token position: dmodelEC |
| Encoder – Transformer encoder blocks | Number of transformer encoder blocks NEC Dimension of transformer encoder block dmodelEC Number of self-attention heads NheadEC Dimension of attention head dheadEC=dmodelEC/NheadEC Dimension of latent space inside feedforward module dFFNEC Activation function: relu |
| Encoder – FC2(Output linear layer) | Reshape the matrix to a vector Output dimension doutEC |
| Quantization and Dequantization | Scalar quantization: Number of bits per latent dimension: Nbit Total payload size for Scalar quantization = Nbit ×doutEC |
| Decoder – FC1 (Embedding layer) | Reshape the vector to a matrix Output dimension dmodelDC per token |
| Decoder – Positional encoding | Dimension of positional encoding for each token position: dmodelDC |
| Decoder – Transformer encoder blocks | Number of transformer encoder blocks NDC Dimension of transformer encoder block dmodelDC Number of self-attention heads NheadDC Dimension of attention head dheadEC=dmodelDC/NheadDC Dimension of latent space inside feedforward module dFFNDC Activation function: relu |
| Decoder – FC2(Output linear layer) | Output dimension doutDC=NTx×L×2, L is the layer number per inferring |

Observation 11: FLOPs and MACs are both metrics for computation complexity and they are confusable. Some statistical tools output misleading FLOPs results.

Observation 12: One MACs approximately equals two FLOPs. FLOPs (Floating Point Operations) means the total number of floating point operations. MACs (Multiply ACcumulate operations) means the computing steps that involves multiplying two numbers and adding the product to a running total in an accumulator.

Observation 13: Different quantized precision, INT8 and BF16 as examples, produce different storage sizes.

Proposal 12: To report the accurate complexity, FLOPs, MACs, parameter numbers and storage sizes are proposed to be reported together.

Observation 14: In Rel-19, RAN4 verified the feasibility of compatibility between a common CNN decoder and several encoders with different backbones and with different complexities.

Observation 15: It was observed that the smaller the complexity of the encoder is, the worse the SGCS performance is.

Proposal 13: Only one decoder with reasonable and acceptable complexity is specified in Rel-20.

Proposal 14: To evaluate the performance-complexity trade-off, several encoders with different complexities should be trained with the same one decoder by the same N(encoders)-to-1(decoder) method developed in Rel-19.

Proposal 15: Based on the trade-off between throughput gains and model complexities, at least one encoder can be used as the reference encoder to derive the performance requirement.

Observation 16: When taking sub-band as the token dimension, theoretically, scalability on the sub-band size can be naturally handle by the transformer blocks, as the varying sequence length in NLP.

Proposal 16: We proposed to downselect the following methods from RAN1 study outcome to reduce duplicate work.

Table 2. methods for model scalability.

| choice of token dimension and feature dimension | Use sub-band as the token dimension and Tx port as a feature dimension |
| --- | --- |
| scalability on the Tx ports | a common embedding layer with padding |
| scalability on the sub-band size | - The number of tokens varies with the sub-band size and no additional scalability method is needed. - Padding at the input |
| scalability on payload size | truncation/masking of the output linear layer output |

Proposal 17: RAN4 use Case 1 (scalable model structure with joint training on scalable scenarios) and Case2 (dedicated model structure trained on dedicated data) to evaluate the performance of scalability solutions and identify the losses brought by scalability solutions.

Proposal 18: For results aligning for two-sided model training and inference evaluation, absolute SGCS is proposed as the performance metric.

Proposal 19: For verify the throughput gains and specify the performance requirements of the encoder, RAN4 can take the throughput related metrics as performance metric, e.g., absolute throughput or throughput gain over the non-AI CSI feedback or others depending on testability.

Proposal 20: When RAN4 discusses the achievable performance gain of AI/ML-based CSI feedback enhancement, the eType II-based codebook can be taken as the baseline. The overheads of eType II and AI/ML models should be comparable or equivalent.

Proposal 21: Model structures are specified in specifications. Model parameters and dataset are stored in open-source file format, like ONNX, in 3GPP server with specified links.

Observation 17: Quantized parameters, regardless of in INT8, BF16 or FP16 format, will bring in degradation of intermediate performance, compared to float32 format parameters.

Observation 18: There are two types of model quantization methods to obtain quantized parameters, Quantization-Aware Training (QAT) and Post-Training Quantization (PTQ)

Proposal 22: RAN4 should perform model alignment using the models with quantized parameters.

Proposal 23: RAN4 should study which quantization method is adopted and the detailed quantization configurations,

- Quantization-Aware Training (QAT)

- Post-Training Quantization (PTQ)

Proposal 24: RAN4 should also achieve the alignment of quantization method and detailed quantization configuration of the quantization method, regardless of QAT and PTQ.

# 4. Reference

[1] RP-252445 (revised from RP-251870), “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm, RAN#109.

[2] R4-2514614, “WF on R20 NR_AIML_air_Ph2”, Ericsson, RAN4#116bis.

[3] R4-2513417, “Discussion on inter-vendor training collaboration and interoperability for two-sided AI/ML models”, vivo, RAN4#116bis.