3GPP TSG-RAN WG4 Meeting #117	R4-2520378

# Dallas, TX, USA, 17th – 21st, November, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | - | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | dCR to TS 38.141-2 on SBFD TX conformance tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | dCR to TS 38.141-2 to add TX radiated conformance test for SBFD-capable BS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add TX radiated conformance test clauses for SBFD-capable BS, including - General clause for radiated TX conformance testing - Radiated TX power for sBFD - OTA BS output power for SBFD - Ouput power dynamics for SBFD - Transmit ON/OFF power for SBFD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The relevant clauses of TX radiated conformance tests for SBFD-capable BS are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

## 9.2 Radiated transmitter characteristics for SBFD-capable BS

### 9.2.1	General

Radiated transmitter characteristics requirements apply on the SBFD-capable BS type 1-H, BS type 1-O, or BS type 2-O including all its functional components active and for SBFD operation.

### 9.2.2	Radiated transmit power for SBFD

#### 9.2.2.1 	General

The BS radiated transmit power for SBFD operation is specified over SBFD symbol, in which the EIRP for SBFD symbol may be different from the EIRP for transmitter ON period as specified in clause 6.2.1

#### 9.2.2.2	Minimum requirement for BS type 1-H, BS type 1-O, and BS type 2-O

The same minimum requirement as clause 6.2.2 for BS type 1-H, BS type-1-O, and BS type-2-O is applicable to BS radiated transmit power requirement for SBFD operation.

#### 9.2.2.3 	Test purpose

The same test purpose as clause 6.2.3 for BS type 1-H, BS type-1-O, and BS type-2-O is applicable to BS radiated transmit power requirement for SBFD operation.

#### 9.2.2.4 	Method of test

The same method of test as clause 6.2.4 is applicable to BS radiated transmit power requirement for SBFD operation.

#### 9.2.2.5 	Test requirement

The same test requirement as clause 6.2.5 for BS type 1-H, BS type-1-O, and BS type-2-O is applicable to BS radiated transmit power requirement for SBFD operation.

### 9.2.3	OTA base station output power for SBFD

#### 9.2.3.1		General

The OTA base station output power for SBFD operation is specified over SBFD symbol, in which the TRP for SBFD symbol may be different from the TRP for transmitter ON period as specified in clause 6.3.1.

#### 9.2.3.2		Minimum requirement for BS type 1-O and BS type 2-O

The same requirement as clause 6.3.2 for BS type 1-O and BS type-2-O is applicable to OTA base station output power requirement for SBFD operation.

#### 9.2.3.3 	Test purpose

The same test purpose as clause 6.3.3 is applicable to OTA base station output power requirement for SBFD operation.

#### 9.2.3.4 	Method of test

The same method of test as clause 6.3.4 is applicable to OTA base station output power requirement for SBFD operation.

#### 9.2.3.5 	Test requirement

The same test requirement as clause 6.3.5 for BS type 1-O and BS type 2-O is applicable to OTA base station output power requirement for SBFD operation.

### 9.2.4	OTA output power dynamics for SBFD

#### 9.2.4.1	General

The requirements in clause 9.2.4 apply during SBFD symbol. Transmit signal quality (as specified in clause 9.2.6) shall be maintained for the output power dynamics requirements of this clause.

The OTA output power requirements are directional requirements and apply to the beam peak directions over the OTA peak directions set.

#### 9.2.4.2		OTA RE power control dynamic range

##### 9.2.4.2.1	General

The RE power control dynamic range for SBFD operation is the difference between the power of an RE and the average RE power for a BS at maximum output power (Pmax,c,AC or Pmax,c,TABC) during SBFD symbol for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

##### 9.2.4.2.2	Minimum requirement for BS type 1-O

The OTA RE power control dynamic range for SBFD operation is specified the same as the conducted RE power control dynamic range requirement for BS type 1-H in TS 38.104 [2], clause 6.3.3.2.

##### 9.2.4.2.3	Method of test

The same method of test as clause 6.4.3.4 is applicable to OTA RE power control dynamic range requirement for SBFD operation.

#### 9.2.4.3	OTA total power dynamic range

##### 9.2.4.3.1	General

The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

NOTE 1:	The upper limit of the OTA total power dynamic range for SBFD is the BS maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all DL RBs in transmission bandwidth configuration for SBFD at maximum output power. The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbol carries PDSCH and not contain RS or SSB.

##### 9.2.4.3.2	Minimum requirement for BS type 1-O and 2-O

The OTA total power dynamic range minimum requirement for each NR carrier shall be larger than or equal to the level calculated as 10log10(NRB,SBFD,DL) if one SBFD DL subband is configured, i.e., DU or UD pattern, and as 10log10(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured, i.e., DUD pattern.

##### 9.2.4.3.3	Test purpose

The same test purpose as clause 6.4.3.3 is applicable to OTA RE power control dynamic range requirement for SBFD operation.

##### 9.2.4.3.4	Method of test

The same method of test as clause 6.4.3.4 for BS type 1-O and BS type 2-O is applicable to OTA RE power control dynamic range requirement for SBFD operation.

##### 9.2.4.3.5	Test requirement

The same test requirement as clause 6.4.3.5 for BS type 1-O and BS type 2-O is applicable to OTA RE power control dynamic range requirement for SBFD operation.

### 9.2.5	Transmit ON/OFF power for SBFD

#### 9.2.5.1	Transmitter OFF power for SBFD

##### 9.2.5.1.1		General

The general description for OTA transmitter OFF power as clause 6.5 shall be applied.

##### 9.2.5.1.2		Minimum requirement for BS type 1-O and 2-O

For the OTA transmitter OFF power requirements for non-SBFD symbols, the same requirement as specified in clause 6.5 for BS type 1-O and BS type 2-O shall be applied. Requirement is not applicable for SBFD symbols.

Requirements related with multi-band and non-contiguous spectrum are not applied to SBFD.

##### 9.2.5.1.3		Method of test and test requirements

No specific test or test requirements are defined for transmitter OFF power for SBFD symbols.

==============End of change==============