3GPP TSG-RAN WG4 Meeting #117	R4-2520401

Dallas, USA, Nov. 17-21, 2025

Source: 	vivo

Title: 	On 6G spectrum sharing

Agenda Item:	8.9

Document for:	Discussion

Introduction

The RAN WG study on 6G radio has been approved by RAN#108 as SID in [1].The relevant objectives of RAN4 are copied below for reference.

- Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

In the SID, one of the objectives is to study the migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR. The spectrum sharing was initially discussed at RAN4 116bis meeting [2]. In this contribution, further analysis and consideration on open issues for 6G spectrum sharing are provided.

Discussion

- The discussion on the spectrum sharing at RAN4 116bis meeting is diverse, which is as expected as the first open discussion. In the end some agreements for “general aspects” and “Aspects to facilitate MRSS” were achieved [2] and the agreements are copied below for reference.

      - Issue 1-1-1: Migration scheme

- Agreements

- RAN4 focus on MRSS between 6GR and NR until further RANP guidance

- RAN4 focus on inter-RAT mobility between 6GR and NR

      - Issue 1-1-2: Scenarios for MRSS between 6GR and NR

- Agreements

- For operating frequency range or band for MRSS between 6GR and NR

- Start RAN4 6G-5G MRSS study in FR1

- Whether to consider 6G-5G MRSS in FR2-1 is further discussed

      - Issue 1-2-0: General consideration

Agreement

- To facilitate MRSS between 6GR and NR, take following aspects as starting point for RAN4 study

- System parameters

- RF requirements

- RRM requirements

- Interference handling

- Taking following table as starting point to discuss which system parameters are impacted by MRSS.

| System parameters | Whether impacted by MRSS |
| --- | --- |
| Numerology | Maybe impacted |
| Channel raster | Maybe impacted |
| Sync raster | Maybe impacted |
| Waveform | FFS |
| Modulation | FFS |
| Channel bandwidth | FFS |

RRM requirements

From RRM perspective, it is agreed to that RRM requirements can be taken as starting point for RAN4 study. One particular point mentioned during MRSS discussion is about inter-RAT mobility. It is agreed that RAN4 to focus on inter-RAT mobility between 6GR and NR. Based on the same logic above, RRM requirements for inter-RAT mobility is not a MRSS specific issue, which should be discussed under RRM agenda with sufficient RAN1/2 progress.

- From RRM requirements perspective, inter-RAT mobility is not a MRSS specific issue, which should be discussed under RRM agenda with sufficient RAN1/2 progress.

Whether to reuse legacy NR signals/channels for 6GR

Regarding whether to reuse legacy NR signals/channels for 6GR, by tracking the progress in RAN1/RAN2 discussion of 6G study, there is also no concrete design/principle for MRSS between 6G and NR. Taking NR-LTE DSS as reference, SSB is not impacted by LTE CRS by MBSFN to avoid performance degradation on very fundamental procedures, e.g., initial access, synchronization. Considering MRSS between 6GR and NR, the always on signals are reduced compared with LTE, and for 6GR, the overhead of always on signals are expected to be further reduced based on promising mechanism, e.g., on-demand signals, SSB periodicity extension. Thus, colliding between 6G sync signals and NR always on signals are expected to be less severe than NR-LTE MRSS. Based on analysis above, from RRM requirements perspective, RAN4 assumes 6GR sync signals are not impacted by MRSS, and NR signals/channels (e.g., SSB) are not reused for 6GR in MRSS.

- From RRM requirements perspective, RAN4 assumes 6GR sync signals are not impacted by MRSS, and NR signals/channels (e.g., SSB) are not reused for 6GR in MRSS.

Conclusion

In this contribution we further discuss potential issues regarding 6G spectrum sharing and have the following proposals:

- From RRM requirements perspective, inter-RAT mobility is not a MRSS specific issue, which should be discussed under RRM agenda with sufficient RAN1/2 progress.

- From RRM requirements perspective, RAN4 assumes 6GR sync signals are not impacted by MRSS, and NR signals/channels (e.g., SSB) are not reused for 6GR in MRSS.

- References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

[2] R4-2514646, WF for [116bis][108] 6G spectrum sharing, CMCC, RAN4 116bis