3GPP TSG-RAN WG4 Meeting #117	R4-2520409

Dallas, US, 17th – 21st November, 2025

Source: 	Huawei, HiSilicon

Title: 	RF requirements for A-IoT BS

Agenda Item:	4.20.1.2

Document for:	Approval

Introduction

The core part of WI on solutions for Ambient IoT in NR has been completed at RAN#109 and TS38.194 has been released [1]. Last meeting, a WF was agreed [2]. This contribution we discuss maintenance issues on Ambient-IoT BS.

Discussion

Foffset for A-IoT operation

It was agreed apply Foffset for TX side, and FFS for RX side last meeting.

Copied from [2] WF: - For Tx side of A-IoT operation, A-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset as defined in Table 3 below where CBW is defined in 5.3.1 and 5.3.2 of TS38.194. Table 3 Foffset for A-IoT operation Carrier; Foffset. Ambient IoT; 100 kHz+CBW/2. - FFS on the Rx side.

For NB-IoT standalone operation, NB-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset  as defined in Table 5.6-3A in TR36.104.

Table 5.6-3A (in TR36.104): Foffset for NB-IoT standalone operation

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Standalone NB-IoT | 200 kHz |

To ensure TX and RX consistency, it is recommended that the Foffset also apply for Rx side.

Proposal 1: For A-IoT operation, A-IoT requirements for receiver shall apply with a frequency offset Foffset  as defined in Table 3 below where CBW is defined in 5.3.1 and 5.3.2 of TS38.194.

Table 3 Foffset for A-IoT operation

| Carrier | Foffset |
| --- | --- |
| Ambient IoT | 100 kHz+CBW/2 |

ACS

Last meeting, it was an open issue on interfering signal.

Copied from [2] Issue 2-4: interfering signal WF: - FFS whether to update the interfering signal for ACS, in-band blocking, and narrowband intermodulation from 3 MHz E-UTRA/NR to 5 MHz NR.The motivation is that 5 MHz NR is more widely deployed than 3 MHz NR, making these requirements more representative of real-world conditions.

In the endorsed draft CR R4-2515143[3] for TS38.194, the ACS requirements are as follows:

Copied from draft CR R4-2515143 [3]

- Table 7.3.1.2-1: Base station ACS requirement

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB (Note) | -53 | ±100 | 3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal |
| 3520 | PREFSENS + 6dB (Note) | -53 | ±100 | 3 MHz DFT-s-OFDM NR signal |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1 |  |  |  |  |

- Table 7.3.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (kHz) | Type of interfering signal |
| --- | --- | --- |
| 200 | ±100 | 3 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB, closest to wanted signal |
| 3520 | ±1500 | 3MHz DFT-s-OFDM NR signal |

Given that 3 MHz NR is not widely deployed, it is proposed to use a 5 MHz NR signal as the interfering source to better align with real-world network scenarios.

Proposal 2: Update the interfering signal for ACS, in-band blocking, and narrowband intermodulation from 3 MHz E-UTRA/NR to 5 MHz NR.

For 200kHz wanted signal, when use a 5MHz NR signal with 15kHz SCS and 1RB closest to wanted signal as interfering signal, the NR guardband on one side is 250kHz, and half of an RB occupies 90kHz, so the interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge need to adjusted to 250kHz +90kHz=340kHz.

Proposal 3: For 200kHz D2R CBW, the ACS interfering signal center frequency offset to the lower/upper Base Station RF Bandwidth need to correct from +/-100kHz to +/-340kHz.

Proposal 4: For 3.52M D2R CBW, correct the ACS interfering signal center frequency offset to the lower/upper Base Station RF Bandwidth from +/-100kHz to +/-2500kHz.

- Table 7.3.1.2-1: Base station ACS requirement

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB (Note) | -53 | ±340 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal |
| 3520 | PREFSENS + 6dB (Note) | -53 | ±2500 | 5 MHz DFT-s-OFDM NR signal |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1 |  |  |  |  |

- Table 7.3.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (kHz) | Type of interfering signal |
| --- | --- | --- |
| 200 | ±340 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal |
| 3520 | ±2500 | 5 MHz DFT-s-OFDM NR signal |

In-band blocking

In current TS38.194, the in-band blocking requirements are as follows:

Copied from draft CR R4-2515143 [3]

Table 7.3.2.2-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6 dB | -38 | ±7.5 | 3MHz DFT-s-OFDM NR signal 15 kHz SCS, 15 RBs |
| 3520 | PREFSENS + 6 dB | -38 | ±4.5 | 3 MHz DFT-s-OFDM NR signal 15 kHz SCS, 15 RBs |
| NOTE 1: PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1 |  |  |  |  |

Proposal 5: For 200kHz D2R CBW, the type of general blocking interfering signal change from 3M NR signal to 5M NR signal.

Correspondingly, the interfering signal center frequency offset to the lower/upper Base Station RF Bandwidth need to adjust to 1.5 times interfering signal bandwith.

Proposal 6: For 3.52M D2R CBW, the type of general blocking interfering signal change from 3M NR signal to 5M NR signal. Change the interfering signal center frequency offset to the lower/upper Base Station RF Bandwidth from +/-4.5MHz to +/-7.5MHz.

Table 7.3.2.2-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6 dB | -38 | ±7.5 | 5MHz DFT-s-OFDM NR signal 15 kHz SCS, 15 RBs |
| 3520 | PREFSENS + 6 dB | -38 | ±7.5 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |
| NOTE 1: PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1 |  |  |  |  |

Narrowband intermodulation

In current TS38.194, the narrowband intermodulation requirements are as follows:

Copied from TS38.194 [3]

Table 7.6.2-1: Narrowband intermodulation performance requirement for A-IoT Medium Range BS

| Channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edg [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB (Note 1) | -53 | ±340 | CW |
|  |  | -53 | ±880 | 5 MHz E-UTRA signal, 1 RB (Note 2) |
| 3520 | PREFSENS + 6dB (Note 1) | -53 | ±270 | CW |
|  |  | -53 | ±780 | 3 MHz E-UTRA signal, 1 RB (Note 2) |
| NOTE 1: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1. NOTE 2: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |  |  |

Since A-IoT is belong NR, better to use NR signal rather than E-UTRA signal.

Proposal 7: For narrowband intermodulation , change the type of interfering signal from LTE signal to 5M NR signal.

Table 7.6.2-1: Narrowband intermodulation performance requirement for A-IoT Medium Range BS

| Channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edg [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB (Note 1) | -53 | ±340 | CW |
|  |  | -53 | ±880 | 5 MHz NR signal, 1 RB (Note 2) |
| 3520 | PREFSENS + 6dB (Note 1) | -53 | ±270 | CW |
|  |  | -53 | ±780 | 5MHz NR signal, 1 RB (Note 2) |
| NOTE 1: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1. NOTE 2: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |  |  |

Conclusion

This contribution we share our considerations regarding the forthcoming band combination specifications, and have the following proposals:

Proposal 1: For A-IoT operation, A-IoT requirements for receiver shall apply with a frequency offset Foffset  as defined in Table 3 below where CBW is defined in 5.3.1 and 5.3.2 of TS38.194.

Table 3 Foffset for A-IoT operation

| Carrier | Foffset |
| --- | --- |
| Ambient IoT | 100 kHz+CBW/2 |

Proposal 2: Update the interfering signal for ACS, in-band blocking, and narrowband intermodulation from 3 MHz E-UTRA/NR to 5 MHz NR.

Proposal 3: For 200kHz D2R CBW, the ACS interfering signal center frequency offset to the lower/upper Base Station RF Bandwidth need to correct from +/-100kHz to +/-340kHz.

Proposal 4: For 3.52M D2R CBW, correct the ACS interfering signal center frequency offset to the lower/upper Base Station RF Bandwidth from +/-100kHz to +/-2500kHz.

- Table 7.3.1.2-1: Base station ACS requirement

| A-IoT channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering signal centre frequency offset to the lower/upper Base Station RF Bandwidth edge [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB (Note) | -53 | ±100 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal |
| 3520 | PREFSENS + 6dB (Note) | -53 | ±2500 | 5 MHz DFT-s-OFDM NR signal |
| Note: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1 |  |  |  |  |

- Table 7.3.1.2-2: Base Station ACS interferer frequency offset values

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (kHz) | Type of interfering signal |
| --- | --- | --- |
| 200 | ±100 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 1 RB，closest to wanted signal |
| 3520 | ±2500 | 5 MHz DFT-s-OFDM NR signal |

Proposal 5: For 200kHz D2R CBW, the type of general blocking interfering signal change from 3M NR signal to 5M NR signal.

Proposal 6: For 3.52M D2R CBW, the type of general blocking interfering signal change from 3M NR signal to 5M NR signal. Change the interfering signal center frequency offset to the lower/upper Base Station RF Bandwidth from +/-4.5MHz to +/-7.5MHz.

Table 7.3.2.2-1: Base station general blocking requirement

| BS channel bandwidth of the lowest/highest carrier received (kHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6 dB | -38 | ±7.5 | 5MHz DFT-s-OFDM NR signal 15 kHz SCS, 15 RBs |
| 3520 | PREFSENS + 6 dB | -38 | ±7.5 | 5 MHz DFT-s-OFDM NR signal 15 kHz SCS, 25 RBs |
| NOTE 1: PREFSENS depends also on the BS channel bandwidth as specified in tables 7.2.2-1 |  |  |  |  |

Proposal 7: For narrowband intermodulation, change the type of interfering signal from LTE signal to 5M NR signal.

Table 7.6.2-1: Narrowband intermodulation performance requirement for A-IoT Medium Range BS

| Channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edg [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB (Note 1) | -53 | ±340 | CW |
|  |  | -53 | ±880 | 5 MHz NR signal, 1 RB (Note 2) |
| 3520 | PREFSENS + 6dB (Note 1) | -53 | ±270 | CW |
|  |  | -53 | ±780 | 5MHz NR signal, 1 RB (Note 2) |
| NOTE 1: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1. NOTE 2: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |  |  |

- References

[1] RP-252745, TS 38.194 v1.0.0 NR; Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception, Huawei, RAN#109, Sept., 2025

[2] R4-2515147, WF on [116bis][326] A-IoT_Maintenance

[3] R4-2515143 CR for TS 38194 on A-IoT BS RF requirements.docx

[4] RP-250796 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR