3GPP TSG-RAN WG4 Meeting # 117	R4- 2520426

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	7.9.3

Source: 	National Taiwan University

Title: 	On Dataset Alignment and Reference Model Selection Criterion for Inter-Vendor Training Collaboration and Interoperability for Two-Sided AI/ML Models

Document for:	Approval

Introduction

We present our view on partially specified decoder and reference decoder derivation procedure in this contribution.

Discussion

Dataset Alignment

In the previous meeting, alignment metrics including power spectral entropy (PSE), frequency coherence (FC) and spectral coherence (SC) are discussed. However, these metrics all captures mainly the channel characteristics, and when we ignore the channel estimation error, the channel generated from the same statistical model should have exactly the same PSE, FC and SC. Therefore, unless the channel estimation schemes introduce too much distortion to these statistics, these metrics don’t contribute too much additional information. In addition, in CSI compression, we encode the precoders, whose statistics is related by not directly reflected in these channel related statistics. Therefore, we propose to perform data alignment directly on the precoder dataset.

As we explained in our previous contributions [1], the CSI compression is equivalent to the optimal vector quantization problem. Therefore, when we want to align the dataset, we should consider how the dataset distributed in the vector space spanned by the precoders to be encoded and transmitted to the base station. The best method to capture the characteristics of precoder distribution on the vector space is principle component analysis. Therefore, we have the following proposal:

Proposal 1: Design dataset alignment procedures based on principle component analysis of the encoder input dataset.

We suggest the following procedures:

Proposal 2: We propose the following procedures of dataset alignment based on principle component analysis:

Step 1: Companies perform principle component analysis on their own encoder input dataset, and submit the top k principle components (or the principle components with singular value larger than an agreed threshold).

Step 2: Use the k’(< k) top principle components from each company, apply Gran-Schmit process to derive a set orthonormal basis forming a space (called it the initial space). For the rest of submitted principle components from all the companies, remove the principle components when the residual error of the vectors projecting on the initial space is larger than an agreed threshold. When the residual error is smaller than the threshold, keep the principle component. After checking all the remaining principle components, forming a new set of orthonormal basis by applying Gran-Schmit process on the kept principle components, the new set of orthonormal basis spans a new space (called it the aligned space).

Step 3: For the aggregated mixed dataset, remove the encoder input when the residual error of projecting the encoder input on the aligned space is larger than an agreed threshold.

Discussion for Reference Decoder Selection Criterion and Inter-Operability

As pointed out in [1], the inter-vendor collaboration procedure targets the best system performance with reasonable complexity to limit implementation cost, as well as a flexible design space to allow vendors to develop their own models based on the reference model(s). In RAN4#116 meeting, the following criteria for decoder selection are agreed:

“Encoder / Decoder selection criteria: Achievable performance, complexity and robustness in different conditions need to be taken into account in selecting the model.”

In RAN4#116bis, RAN4 agreed this as the reference decoder derivation procedure: “Select one or more model structures considering complexity and robustness and decide how many models.”  Following on the TR and the agreement, based on the study done in [2], we propose further details of the selection criterion below:

Proposal 3: Among the three criterion, we propose to use the achievable performance and complexity as the prioritized criterion for choosing a small sets of encoder/decoder model structures with reasonable performance and feasible/implementable complexity. Among these small sets of model structures, RAN4 can decide the final model structures to be standardized as reference model by the robustness criterion, and the robustness criterion is checked against the selected small sets of model structures.

The evaluation of performance metrics and complexity is more straightforward. The robustness metric is for guaranteeing test repeatability and interoperability. We propose the following procedure to derive the robustness metric as part of selection criterion:

Proposal 4: For all the small selected sets of model structures, evaluate the robustness metrics based on test repeatability and interoperability by the following procedure

- Companies contribute encoders trained with the candidate decoders (one encoder per candidate decoder)

- Connect an encoder to the corresponding decoder, run the test based on the agree test dataset and record the SGCS

- Repeat step 2 for all the encoder and decoder pairs, record all the SGCS

- For each candidate decoder, capture the variation of the SGCSs when connecting to different encoders, and evaluate the variation together with performance metrics and complexity

Test Decoder Verification Procedure

In addition, for option 4, none of the options are complete enough for TE vendors to derive the test decoders that meet the test repeatability requirements. For example, in 4a, there are different ways to train the test decoders and the achieved loss functions from the specified dataset can vary a lot, and we still don’t have any test repeatability guarantee across different test decoders. Similarly, the joint training with reference encoder and yield different test decoders without guaranteed test repeatability. Therefore, additional requirements/specifications are needed for dataset based and reference encoder-based approach. We provide the suggestions for further specifications for each option in the following:

Dataset based approach 4a: we can specify a loss/similarity function of the test decoder output and the specified dataset for a given input as follows

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset, and f is the chosen loss/similarity function to evaluate the difference between the decoder output from the dataset corresponding to the latent message z, gref(z), and the test decoder output from z, gtest(z).

The drawback of the approach is that both the test decoder (and implicitly the DUT encoder) are heavily restricted by the dataset due to the specified constraint, and we don’t expect that the DUT encoder can work in practice unless in an environment that is very closely resembled with the specified dataset. In fact, the purpose of the test is, as agreed, “verify whether the minimum performance of AI/ML functionality/feature can be achieved”. But we don’t expect such dataset shared between the UE vendors and infra vendors is available for all the application scenarios, and therefore the performance achieved here is definitely not the minimum performance we can expect in the field. Alternatively, if we consider the approach proposed in the previous meeting (and included also in section 2.2), we can derive a more flexible requirement that can enable both TE vendors and UE vendors to develop decoders and encoders that can be trained by different datasets than the specified one. Such a flexibility can get us closer to the agreed test goal, since we allow the encoders and decoders to be trained by other dataset as long as the specified requirement in the form of latent message/decoder output ordering is fulfilled. To be more specified, the alternative requirement is:

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space.

Similarly, we can add the requirement for option 4b as

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input during test decoder verification procedures. Therefore, we also need to specified the generation procedures of X.

Alternatively, based on the proposal in section 2.2, we can specify the requirements of the test decoder as

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

In summary, we have the following proposal:

Proposal 5: Additional steps for option 4a and 4b feasibility study:

- Define the decoder verification criterion from one of the following options (for 4a and 4b, respectively):

(Option 4a: specify dataset)

- 4a-1: RAN4 derive the threshold δ via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset (from step 2), and f is the chosen loss/similarity function gref(z) is the test decoder output and gtest(z) is from the dataset.

- 4a-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where the notations are the same as 4a-1, and Ytest is the test decoder output space.

(Option 4b: specify reference encoder)

- 4b-1: RAN4 derive the threshold δ and the encoder input X generation procedure via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input in the test decoder verification procedures. Note that RAN4 needs to specified the generation procedures of X and specifying

- 4b-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

- RAN4 check if the decoders derived from the above procedure can satisfy test repeatability requirement, i.e., the loss function delta between two different decoders when connecting to the same encoder, is within a RAN4 agreed margin. If yes, we can conclude that option 4 (at least one sub-option) is feasible.

Conclusion

Proposal 1: Design dataset alignment procedures based on principle component analysis of the encoder input dataset.

Proposal 2: We propose the following procedures of dataset alignment based on principle component analysis:

Step 1: Companies perform principle component analysis on their own encoder input dataset, and submit the top k principle components (or the principle components with singular value larger than an agreed threshold).

Step 2: Use the k’(< k) top principle components from each company, apply Gran-Schmit process to derive a set orthonormal basis forming a space (called it the initial space). For the rest of submitted principle components from all the companies, remove the principle components when the residual error of the vectors projecting on the initial space is larger than an agreed threshold. When the residual error is smaller than the threshold, keep the principle component. After checking all the remaining principle components, forming a new set of orthonormal basis by applying Gran-Schmit process on the kept principle components, the new set of orthonormal basis spans a new space (called it the aligned space).

Step 3: For the aggregated mixed dataset, remove the encoder input when the residual error of projecting the encoder input on the aligned space is larger than an agreed threshold.

Proposal 3: Among the three agreed selection criteria, performance, complexity and robustness, we propose to use the achievable performance and complexity as the prioritized criteria for choosing a small sets of encoder/decoder model structures with reasonable performance and feasible/implementable complexity. Among these small sets of model structures, RAN4 can decide the final model structures to be standardized as reference model by the robustness criterion, and the robustness criterion is checked against the selected small sets of model structures.

Proposal 4: For all the small selected sets of model structures, evaluate the robustness metrics based on test repeatability and interoperability by the following procedure

- Companies contribute encoders trained with the candidate decoders (one encoder per candidate decoder)

- Connect an encoder to the corresponding decoder, run the test based on the agree test dataset and record the SGCS

- Repeat step 2 for all the encoder and decoder pairs, record all the SGCS

- For each candidate decoder, capture the variation of the SGCSs when connecting to different encoders, and evaluate the variation together with performance metrics and complexity

Proposal 5: Additional steps for option 4a and 4b feasibility study:

        - Define the decoder verification criterion from one of the following options (for 4a and 4b, respectively):

(Option 4a: specify dataset)

- 4a-1: RAN4 derive the threshold δ via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset (from step 2), and f is the chosen loss/similarity function gref(z) is the test decoder output and gtest(z) is from the dataset.

- 4a-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where the notations are the same as 4a-1, and Ytest is the test decoder output space.

(Option 4b: specify reference encoder)

- 4b-1: RAN4 derive the threshold δ and the encoder input X generation procedure via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input in the test decoder verification procedures. Note that RAN4 needs to specified the generation procedures of X and specifying

- 4b-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

        - RAN4 check if the decoders derived from the above procedure can satisfy test repeatability requirement, i.e., the loss function delta between two different decoders when connecting to the same encoder, is within a RAN4 agreed margin. If yes, we can conclude that option 4 (at least one sub-option) is feasible.

Reference

[1] R4-2412130

[2] C.-H. Huang, C.-K. Wen, G. Y.  Li, “AI/ML Life Cycle Management for Interoperable AI Native RAN”, submitted to IEEE Communication Standard Magazine, available at http://arxiv.org/abs/2507.18538