# 3GPP TSG-RAN WG4 Meeting #117                             R4-2520444

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	8.9

Source: 	CMCC

Title: 			 Discussion on spectrum sharing for 6GR study

Document for:	Discussion

- Introduction

In RAN #108 meeting, the SID on 6G radio was approved [1]. The objectives on spectrum sharing are duplicated as following.

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

In last RAN4 meeting, there was initial discussion on spectrum sharing for 6GR study, and a WF was approved [2]. This contribution provides further discussion on spectrum sharing.

- Discussion

In last RAN4 meeting, there was initial discussion on spectrum sharing for 6GR study, and following agreements are reached.

Sub-topic 1-0: Agenda handling - Agreements MRSS should be considered in the ultimate RAN4 decision, including RAN4’s input to interim milestone. Regarding some of the system parameters (e.g. SCS, sync raster, channel raster etc.), separated agenda will be designated for the scenarios of with (in spectrum sharing) and without (in system parameter) MRSS considered. - If feasible, RAN4 strives to define unified system parameters for the scenarios with and without MRSS. - Revisit this agenda arrangement after Feb. meeting in 2026 for potentially merging the discussion under the single agenda. Before RAN4 receives concrete design from RAN1/2, RAN4 discussion will be based on the agreed hypothesis. Sub-topic 1-1: General aspects Issue 1-1-1: Migration scheme - Agreements - RAN4 focus on MRSS between 6GR and NR until further RANP guidance - RAN4 focus on inter-RAT mobility between 6GR and NR Issue 1-1-2: Scenarios for MRSS between 6GR and NR - Agreements - For operating frequency range or band for MRSS between 6GR and NR - Start RAN4 6G-5G MRSS study in FR1 - Whether to consider 6G-5G MRSS in FR2-1 is further discussed Sub-topic 1-2: Aspects to facilitate MRSS Issue 1-2-0: General consideration Agreement - To facilitate MRSS between 6GR and NR, take following aspects as starting point for RAN4 study - System parameters - RF requirements - RRM requirements - Interference handling - Taking following table as starting point to discuss which system parameters are impacted by MRSS. System parameters; Whether impacted by MRSS. Numerology; Maybe impacted. Channel raster; Maybe impacted. Sync raster; Maybe impacted. Waveform; FFS. Modulation; FFS. Channel bandwidth; FFS.

MRSS provides a smooth and efficient evolution from NR to 6G by enabling NR and 6G systems to dynamically share the same spectral resources. For legacy frequency range, e.g. FR1 and FR2-1, the typical case for MRSS is FR1, considering the better coverage of low band and limited bandwidth. In last meeting, it was agreed to start RAN4 6G-5G MRSS study in FR1, and FFS for 6G-5G MRSS in FR2-1. For FR2-1, firstly, the coverage is limited. Secondly, large channel bandwidth, e.g. 100MHz, is assumed. Based on these consideration, we are not sure whether MRSS is needed for FR2-1. If there is no operator demand on MRSS between 6G and 5G in FR2-1, no need to consider 6G-5G MRSS in FR2-1.

Proposal 1: If there is no demand from operator on 6G-5G MRSS in FR2-1, no need to consider 6G-5G MRSS in FR2-1.

One issue is about spectrum sharing with NTN. Some companies propose to consider spectrum sharing scenarios including NR NTN + 6G NTN, NR TN + 6G NTN, NR NTN + 6G TN. Firstly, spectrum sharing between TN and NTN is not in the scope, which are also pending on WRC-27 decision. As for spectrum sharing between NR NTN and 6G NTN, we are not sure whether there is such demand from NTN operators. Considering the limited study time, it is better to firstly focus on the TN network for study on 6G-5G MRSS. When the study on 5G-6G MRSS for TN is stable, we can further check whether the solution can be reused for NR NTN + 6G NTN.

Proposal 2: it is proposed to postpone the discussion on spectrum sharing with NTN. It is proposed to firstly focus on the spectrum sharing between NR TN and 6G TN.

In last meeting, it was agreed to take following aspects as starting point for RAN4 study on MRSS between 6GR and NR.

- System parameters

- RF requirements

- RRM requirements

- Interference handling

As for which system parameters are impacted by MRSS, it was agreed to take following table as starting point.

| System parameters | Whether impacted by MRSS |
| --- | --- |
| Numerology | Maybe impacted |
| Channel raster | Maybe impacted |
| Sync raster | Maybe impacted |
| Waveform | FFS |
| Modulation | FFS |
| Channel bandwidth | FFS |

Different sub-carrier spacing for MRSS between 5G and 6G will increase the spectral leakage and introduce ICI. The scheduling coordination would be very complicated and the efficiency for spectrum sharing will be decreased. To facilitate MRSS between 5G NR and 6GR within legacy bands, it is expected that 6GR support the SCS configurations currently predominant in 5G NR deployments.

In last meeting, RAN1 has agreed that 15kHz SCS for FDD and 30kHz SCS for TDD are supported for sub 6GHz. 120kHz is supported for the frequency range between 24.25GHz - 52.6GHz. FFS other SCS options. It is proposed to follow RAN1 agreements.

Agreements in RAN#109 - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion. Agreements in RAN1#112bis - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

Proposal 3: the alignment between 5G and 6G numerology will facilitate MRSS. It is proposed to follow RAN1 agreements to take 15KHz SCS for FDD and 30KHz SCS for TDD for sub 6GHz.

To facilitate MRSS, channel raster also need to be considered. If the channel raster are not aligned for MRSS between 5G and 6G, it will complex the scheduling and reduce spectrum efficiency. In NR, 100KHz channel raster are considered for band below 3GHz except band n41 to align with LTE channel raster. For MRSS between NR and 6G, we are not sure whether 100KHz channel raster are still needed for low band, and can be further discussed.

Proposal 4: For MRSS between NR and 6GR, it is proposed to discuss whether 100KHz channel raster for low band are still needed.

7.5KHz uplink shift is considered for Dynamic Spectrum Sharing (DSS) between LTE and NR. Since LTE uplink and NR uplink adopt different waveform, 7.5KHz Uplink shift is used to guarantee the orthogonality between carriers and avoid inter-carrier inference. As discussed above, RAN1 has agreed that the waveforms defined in 5G NR are supported as the basis for 6GR, i.e. CP-OFDM and DFT-s-OFDM waveforms are supported for 6GR for uplink. No need to consider 7.5KHz Uplink shift for MRSS between NR and 6GR.

Proposal 5: No need to consider 7.5KHz uplink shift for MRSS between NR and 6GR.

The 6G waveform need to be orthogonal with 5G waveform to facilitate MRSS. Whether 5G and 6G transmissions can co-exist within shared spectrum is impacted by time-frequency orthogonality.

Agreements in RAN1 #122 meeting CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc. Agreements in RAN1 #122bis meeting Agreement - For uplink low-PAPR proposals, the link level performance evaluation criterion is Net Gain assuming same spectrum efficiency as the reference - Net Gain [dB] = Tx power gain relative to the reference – SNR degradation relative to the reference @10% BLER - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account. - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged. - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding Agreement - Study the evaluation method for evaluating DFT-s-OFDM for UL with number of layers > 1.

In last RAN1 meeting, it was agreed that the waveforms defined in 5G NR are supported as the basis for 6GR. FFS other waveforms. To enable simultaneous operation, eliminate guard band, simplify receiver complexity for MRSS, it is expected to consider the impact on MRSS for the discussion on new waveform.

Proposal 6: it is proposed to consider the MRSS impact when discuss the waveform for 6GR

- RAN1 agreed that the waveforms defined in 5G NR are supported as the basis for 6GR

- For other waveform for 6GR, impact on MRSS need to be considered

RAN1 is discussing smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective. Normally, the minimum CBW is defined in RAN4 taking operators’ spectrum allocation, SSB minimum bandwidth, etc, into account. For NR, the CBW multiples of 5MHz is the basic. However, irregular channel bandwidth is considered in later release. There are some candidate solutions, e.g. overlapping channels from network perspective, are considered in the study phase. In the work phase, RAN4 adopt the solution to define native irregular channel BWs, in detail, 7MHz is defined in Release 19 and 6MHz is defined in Release 20. For 6GR, One issue is how to support irregular CBW in 6GR. Defining native irregular channel BWs, e.g. 6MHz, 7MHz, may not cover all the possible irregular channel bandwidth from operators, and will introduce RAN4 workload. If the solution like overlapping channels from network perspective are considered, it may have impact on MRSS. It is proposed to discuss whether MRSS is considered for irregular channel bandwidth pending on the solution to support irregular channel bandwidth.

Proposal 7: it is proposed to consider the impact on MRSS when discussing irregular channel bandwidth. The solution to support irregular channel bandwidth may have impact on MRSS support.

Inter-RAT mobility between NR and 6G, including handover and cell re-selection, is an important and simple way to ensure UE’s service continuity when it moves between a 6G cell and a 5G cell. However, the corresponding interruption during inter-RAT handover cannot be avoided. For 6G, we may aim for a solution to minimize the interruption time. Normally, handover interruption comprises the time required to search the target cell, time for fine time tracking, time for UE UE processing. One way to reduce interruption delay is to discuss whether the components of interruption and the time for each components can be reduced. Also, for MRSS scenario, inter-RAT mobility can be performed on the same frequency carriers, some optimization can be considered.

Proposal 8: it is proposed to minimize handover interruption time for inter-RAT handover between NR and 6GR.

Another consideration is to minimize the data loss when performing inter-RAT measurement. In Rel-18, inter-RAT measurements without gaps, including inter-RAT NR measurement and inter-RAT LTE measurement, are introduced in RAN4 and helpful in reducing the impact on throughput due to measurement gap. It is proposed to support inter-RAT measurements without gaps from 6G day-1.

Proposal 9: it is proposed to support inter-RAT measurements without gaps, including inter-RAT NR measurement without gap and inter-RAT 6GR measurement without gap, from 6G day-1.

For MRSS between 4G and 5G, CRS-IM receiver was introduced to handle the CRS interference from LTE cells. For MRSS between 5G and 6G, one difference from MRSS between 4G and 5G is that there is no always on signals, e.g. SSB, CSI-RS are periodic. We are not sure how severe the interference is for this scenario, and whether to introduce advanced receiver to mitigate the interference. It is proposed to have further study on this.

Proposal 10: it is proposed to study the interference handling between 5G and 6G.

- Conclusion

This contribution provides discussion on spectrum sharing. The proposals are:

Proposal 1: If there is no demand from operator on 6G-5G MRSS in FR2-1, no need to consider 6G-5G MRSS in FR2-1.

Proposal 2: it is proposed to postpone the discussion on spectrum sharing with NTN. It is proposed to firstly focus on the spectrum sharing between NR TN and 6G TN.

Proposal 3: the alignment between 5G and 6G numerology will facilitate MRSS. It is proposed to follow RAN1 agreements to take 15KHz SCS for FDD and 30KHz SCS for TDD for sub 6GHz.

Proposal 4: For MRSS between NR and 6GR, it is proposed to discuss whether 100KHz channel raster for low band are still needed.

Proposal 5: No need to consider 7.5KHz uplink shift for MRSS between NR and 6GR.

Proposal 6: it is proposed to consider the MRSS impact when discuss the waveform for 6GR

- RAN1 agreed that the waveforms defined in 5G NR are supported as the basis for 6GR

- For other waveform for 6GR, impact on MRSS need to be considered

Proposal 7: it is proposed to consider the impact on MRSS when discussing irregular channel bandwidth. The solution to support irregular channel bandwidth may have impact on MRSS support.

Proposal 8: it is proposed to minimize handover interruption time for inter-RAT handover between NR and 6GR.

Proposal 9: it is proposed to support inter-RAT measurements without gaps, including inter-RAT NR measurement without gap and inter-RAT 6GR measurement without gap, from 6G day-1.

Proposal 10: it is proposed to study the interference handling between 5G and 6G.

- Reference

- RP-251881, New SID: Study on 6G Radio

- R4-2514646, WF for [116bis][108] 6G spectrum sharing, CMCC

- R4-2511652, RAN4 6G Plan and Meeting Arrangement

- RP-252870, pCR for 38.914 on requirements for architecture and migration for 6G

- RP-252960, pCR for 38.914 on Requirements of new and existing services for 6G