# 3GPP TSG-RAN WG4 Meeting #117                              R4-2520455

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.11.4

Source: 	CMCC

Title: 			Discussion on RRM core requirement and testing framework for beam management

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on RRM requirements for beam management.

- Discussion

Issue 2-1: Measurement period for inference – case 2 Agreement: Measurement period for inference – case 2 K(number of samples)*[M]*N*P K as defined by RAN1 Issue 2-2: Reporting delay Agreement for reporting delay: Reuse d’ as to be agreed by RAN1 d’ to be added to the measurement period to reflect the extra time needed by the UE to perform inference Depending on the RAN1 specifications, d’ shall not be double counted in the overall delay for periodic, aperiodic or semi-persistent reporting Final specifications to be agreed after RAN1 includes d’ in the specs Issue 2-3: Reporting delay requirements for monitoring Agreement: Do not define any explicit requirements for reporting delay of RS-PAI performance monitoring metric.

For case 2 ( temporal downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams), it was agreed that the measurement period is related with K(number of samples)*[M]*N*P. M, N, P are same as legacy measurement requirement. The table below is the legacy L1-RSRP measurement period for serving cell. M is number of samples which is pending on network configuration. M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise. N=8 which is about RX beam sweeping. P value is for SSB resource to be measured.

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement if UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of the SSB index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

K is defined by RAN1, which is KBM in RAN1 spec. According to RAN1 spec TS38.214, the UE reports a CSI report only if receiving at least KBM latest consecutive transmission occasion(s) for each of the CSI-RS resources. It can be seen that KBM is the the number of samples to be measured. In summary, both M and KBM are the number of samples. We do not need both. Since KBM is introduced by RAN1, and M is used for legacy measurement, M can be removed.

TS 38.214, clause 5.2.2.5 For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19' and with nroftimeinstance-r19 is configured, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least KBM latest consecutive transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource, and drops the report otherwise. The value of KBM is indicated by UE capability.

Proposal 1: For case 2, it is proposed that the measurement period is related with K(number of samples)*N*P, i.e. M is removed, since both M and K are the number of samples.

In RAN4 #114 meeting, it was agreed that if RSRP and beam ID are reported, RSRP accuracy and Beam ID prediction accuracy will be the KPIs. We think the successful rate for the correct prediction is also applied for the case that both RSRP and beam ID are reported. Some companies argue that with RSRP accuracy requirements, no need to apply beam ID prediction accuracy requirements. In our view, it depends on how to define the RSRP accuracy requirements. When the RSRP requirements are defined very tight, if UE pass the RSRP accuracy test, it is reasonable to assume that the beam prediction performance can also be guaranteed. In this case. No need to apply beam ID prediction accuracy. However, when the RSRP requirements are defined not very tight, e.g. with large margin, even if UE pass the RSRP accuracy test, the beam prediction performance cannot be guaranteed. In this case, beam ID prediction accuracy requirements need to be applied. Normally, a certain margin will be considered in the RSRP accuracy requirements. With above consdieration, it is preferred that the successful rate for the correct prediction is also applied for the case that both RSRP and beam ID are reported.

Proposal 2: for the case that both RSRP and beam ID are reported, it is proposed that both RSRP accuracy requirements and successful rate for the correct prediction are applied.

Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams Issue 2-3: Relative RSRP accuracy Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

One FFS for relative RSRP accuracy for case 1 is whether reported L1-RSRP can be measured RSRP. RAN1 agreed that for report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. It is clear that measured RSRP are not allowed to report.

RAN1 agreements in RAN1#120 Agreement For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion.

Proposal 3: for relative RSRP accuracy, it is proposed that the reported L1-RSRP cannot be measured RSRP, since RAN1 agreed that the RSRP of predicted beam(s)in the report of inference results is the predicted RSRP which is based on AI/ML output.

Another FFS is whether beam index n owns the largest reported value. As discussed above, RAN1 agree that only largest predicted RSRP is reported as absolute RSRP, other RSRP are reported as differential predicted RSRP with reference to the largest predicted RSRP. With the agreements, it is reasonable that beam index n owns the largest reported value.

Proposal 4: for relative RSRP accuracy, it is proposed that beam index n owns the largest reported value.

Based on above consideration, for case 1, the relative RSRP accuracy is proposed as (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value.

Proposal 5: for BM case 1, it is proposed that

- The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]

BM case 2 is temporal downlink beam prediction for Set A of beams based on the historic measurement results of Set B of beams. The model output is the L1-RSRP prediction of furture N time instances with K beams for each time instances. According to RAN1 agreements on the inference report for BM case2, RSRP #1 of time instance #1 is absolute RSRP; and the remaining RSRP are differential RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1 of time instance #1. Time instance #1 corresponds to the time instance indicated by the time instance indicator. Since multiple time instances are consdiered, the relative accuracy for BM case2 need to be updated to reflect different time instance. Based on above consideration, the relative accuracy for BM case2 is proposed as (predicted L1-RSRP of beam index i for time instance m - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i for time instance m - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value, 1<=m<=M where M is the number of time instance.

Proposal 6: for BM case 2, it is proposed that

- The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i for time instance m - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i for time instance m - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value among all the predicted beams. 1<=m<=M where M is the number of time instance

- Conclusion

This contribution provides discussion on RRM requirements for beam management. The proposals are:

Proposal 1: For case 2, it is proposed that the measurement period is related with K(number of samples)*N*P, i.e. M is removed, since both M and K are the number of samples.

Proposal 2: for the case that both RSRP and beam ID are reported, it is proposed that both RSRP accuracy requirements and successful rate for the correct prediction are applied.

Proposal 3: for relative RSRP accuracy, it is proposed that the reported L1-RSRP cannot be measured RSRP, since RAN1 agreed that the RSRP of predicted beam(s)in the report of inference results is the predicted RSRP which is based on AI/ML output.

Proposal 4: for relative RSRP accuracy, it is proposed that beam index n owns the largest reported value.

Proposal 5: for BM case 1, it is proposed that

- The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]

Proposal 6: for BM case 2, it is proposed that

- The relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i for time instance m - predicted L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i for time instance m - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value among all the predicted beams. 1<=m<=M where M is the number of time instance

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2514631, WF on [116bis][112] R19 AI for air interface, Qualcomm Incorporated

- R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm Incorporated

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface