# 3GPP TSG-RAN WG4 Meeting #117                             R4-2520458

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.11.8

Source: 	CMCC

Title: 			 Discussion on demodulation requirements for CSI prediction

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion core requirements for CSI prediction.

- Discussion

Issue 1-7: Doppler and MCS Agreement: Introduce tests for 20Hz Doppler with MCS [17or19] with 16Tx ports and 2Rx in FDD Introduce test for TDD with the details FFS

For simulation, RAN4 agreed following steps for alignment.

- Step 1: check SGCS1 before CSI feedback, ideal channel estimation.

- SGCS1 is calculated by comparing the SVD of model output (before feedback) with the SVD of the ground truth Raw channel，based on the ideal channel estimates

- Step 2:  check SGCS2 and SGCS3, ideal channel estimation

- SGCS-2: Compares the PMI feedback derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD.

- SGCS-3: Compares the PMI feedback derived based on Rel-18 eType II-Doppler CB with the model output against the PMI derived based on Rel-18 eType II-Doppler CB with the ground truth.

- Step 3: check SGCS1 and SGCS3，practical channel estimation

- γ1_AI/ML: AI/ML based CSI prediction is compared with Rel-15 Type I random PMI

- γ2_AI/ML: AI/ML based CSI prediction is compared with Rel-16 eType II CB (sample and hold)

In last meeting, it was agreed to introduce tests for 20Hz Doppler with MCS [17or19] with 16Tx ports and 2Rx in FDD. As for the MCS, MCS 13, 17, 19 are simulated. According to the simulation results summary [4], operation point for MCS13 with Doppler 20Hz is close to SNR 0dB, while high SNR is observed for high MCS. Among MCS 17 and 19, higher MCS is preferred for robust testing.

Proposal 1: for FDD, it is proposed to use MCS 19.

It was agreed to introduce test for TDD with the details FFS. For TDD, 4RX is more typical than 2RX. Propose to use 4RX to define test. And 20Hz Doppler can be used, which is same as that for FDD.

Proposal 2: for TDD, it is proposed to use 4RX

Proposal 3: for TDD, it is proposed to use 20Hz Doppler

For channel model for CSI prediction tests, it was agreed that TDL channel models will be used, FFS on whether CDL channel model is needed for generalization test. It is beneficial to use CDL channel model. However, legacy PMI tests are not based on CDL. And the feasibility for demodulation performance requirements with CDL channel model is under discussion in Rel-19 SI. Considering current situation, it is not feasible to consider CDL channel model in Rel-19. Using CDL channel model for CSI prediction test can be further dicsussed in later release based on the outcome of Rel-19 SI on SCM.

Proposal 4: For channel model for CSI prediction tests, the usage of CDL channel model for CSI prediction test can be further discussed in later release based on the progress on SCM.

Issue 1-8: Generalization Agreement: Test setup for the generalization will be further discussed based on the following options - Option 1: Introduce multiple tests with different MCS, which will lead to different SNR points. - Option 2: Introduce multiple tests with different channel models. The details of the channel models are FFS. - Option 3: the combination of option 1 and 2.

For AI/ML, one important topic is generalization. The goal of generalization is to verify whether the performance gain/minimum level of performance of AI/ML functionality/model can be achieved/maintained under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations.

For CSI prediction, from the perspective of generalization, multiple tests with different channel models is necessary. Following table summarize the agreements on demodulation for CSI prediction. It can be seen that different MCS results in different SNR point. From this point of view, multiple tests with different MCS leading to different SNR points also need to be considered.Taking above into account, for generalization, both multiple tests with different channel models and multiple tests with different MCS need to be considered.

Agreements summary on demodulation for CSI prediction Codebook to be used in the tests Agreement: - In Rel-19 only PMI requirements based on AI/ML CSI prediction will be defined - use Rel-18 eType II Doppler codebook for AI/ML based CSI predictions Requirement baseline Agreement: - Requirement to be introduced for Dopler domain basis N4=1 - N4>1 can be further discussed in the future if needed Test metric Requirements to be defined based on the ratio (same metric as for the current PMI reporting requirements in 38.101-4) - tue_AI/ML is 90 % of the maximum throughput obtained at SNRue_AI/ML using the precoders configured according to the UE reports, - trnd is the throughput measured at SNRue_AI/ML with random precoding. - trnd is obtained with Rel-15 type I single panel codebook - Note: SNRue_AI/ML is calculated in the same way as SNRue and SNRfollow1,follow2 in section 6.3 of 38.101-4 Requirement considerations Agreement: The performance of AI based CSI prediction should not be worse than R16 etypeII.

Proposal 5: for generalization, it is proposed to consider both multiple tests with different channel models and multiple tests with different MCS.

- Conclusion

This contribution provides on demodulation requirements for CSI prediction. The proposals are:

Proposal 1: for FDD, it is proposed to use MCS 19.

Proposal 2: for TDD, it is proposed to use 4RX

Proposal 3: for TDD, it is proposed to use 20Hz Doppler

Proposal 4: For channel model for CSI prediction tests, the usage of CDL channel model for CSI prediction test can be further discussed in later release based on the progress on SCM.

Proposal 5: for generalization, it is proposed to consider both multiple tests with different channel models and multiple tests with different MCS.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2514631, WF on [116bis][112] R19 AI for air interface, Qualcomm Incorporated

- R4-2514635, Summary on the CSI prediction simulation results, OPPO

- R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2502970, Baseline Simulation assumptions for AIML based CSI prediction

- R4-2505229, Updated simulation assumptions for AIML CSI prediction