# 3GPP TSG-RAN WG4 Meeting #117                              R4-2520460

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.14.3

Source: 	CMCC

Title: 			 Discussion on RRM performance requirements for A-IoT

Document for:	Discussion

- Introduction

In RAN #107 meeting, the revised WID on Solutions for Ambient IoT (Internet of Things) in NR was approved [1]. According to the WID, the objectives of RAN4 scope are duplicated as following.

- RAN4 scope: - Specify RF requirements for Ambient-IoT BS, device 1, and CW - RF requirements for Type 1-C Ambient-IoT BS - RF requirements for device 1 - RF requirements for CW - Specify RRM core requirements for device 1, if necessary - Study and develop OTA test methodology for A-IoT device 1 - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT. - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system - Use band n8 as an example band

In last meeting, there is discussion on RRM performance requirements for A-IoT device 1, and a WF was approved [2]. This contribution provides discussion on this topic.

- Discussion

- Issue 1-1-1: Test case for TD2R_min Agreement: - Test the performance of TD2R_min. And TD2R_min are tested with other RRM requirements. - Issue 1-1-2: How to define RRM test cases Agreement: - Define a single dedicated RRM test case - Transmit timing requirements, RA procedure and TD2R_min are tested in the same test - Issue 1-1-3: Test configuration/procedure Agreement: - In high level, take the test procedures/FRCs for sensitivity from RF session as baseline - CBRA is in use in the test

In last meeting, it was agreed to define a single dedicated RRM test case to verify RA procedure and the requirements related with timing. It was also agreed that CBRA is in use in the test. In our view, following procedures can be considered to define the test cases.

Step 1: TE transmits an R2D message carrying A-IoT Paging message

Step 2: device transmits an D2R message carrying Access Random ID (Msg1)

Step 3: device receives an R2D message carrying the same access random ID as transmitted by the device in Step 2 (MSG2). The R2D reception is no early than TD2R_min after Msg1 transmission

Step 4: device transmits an D2R message (MSG3)

Proposal 1: the test procedures for RRM test is proposed as following:

- Step 1: TE transmits an R2D message carrying A-IoT Paging message

- Step 2: device transmits an D2R message carrying Access Random ID (Msg1). The D2R transmission shall meet the transmit timing requirements

- Step 3: device receives an R2D message carrying the same access random ID as transmitted by the device in Step 2 (MSG2). The R2D reception is no early than TD2R_min after Msg1 transmission

- Step 4: device transmits an D2R message (MSG3). The D2R transmission shall meet the transmit timing requirements

- Conclusion

This contribution provides discussion on RRM performance requirements for A-IoT. The proposals are:

Proposal 1: the test procedures for RRM test is proposed as following:

- Step 1: TE transmits an R2D message carrying A-IoT Paging message

- Step 2: device transmits an D2R message carrying Access Random ID (Msg1). The D2R transmission shall meet the transmit timing requirements

- Step 3: device receives an R2D message carrying the same access random ID as transmitted by the device in Step 2 (MSG2). The R2D reception is no early than TD2R_min after Msg1 transmission

- Step 4: device transmits an D2R message (MSG3). The D2R transmission shall meet the transmit timing requirements

- Reference

- RP-250796, Revised WID: New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA Inc

- R4-2515061, Way Forward for [116bis][315] A-IoT_device, CMCC

- R4-2512134, WF on RRM for Ambient_IoT_Solutions, CMCC

- R4-2508275, WF on RRM requirements for Ambient_IoT_Solutions, CMCC

- R4-2504905, WF on RRM requirements for Ambient_IoT_Solutions, CMCC