# 3GPP TSG-RAN WG4 Meeting #117                              R4-2520461

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.13.3

Source: 	CMCC

Title: 			 Discussion on RRM performance requirements for SBFD

Document for:	Discussion

- Introduction

In RAN #104 meeting, the revised WID on SBFD was approved [1]. According to the WID, the objectives having RAN4 impact are duplicated as following.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In last meeting, there was discussion on RRM performance requirements, and a WF was approved [2]. This contribution provides discussion on this topic.

- Discussion

- Issue 1-1-1: Time offset for L1-SRS-RSRP measurements - Agreement - Specify the time offset conditions for L1-SRS-RSRP measurement accuracy requirements with cell phase error for [15kHz SCS and] 30kHz SCS and without cell phase error for other SCS

According to the discussion in last meeting, some companies mentioned that all existing commercial TDD NWs in FR1 are using 30kHz SCS, so there is no need to define accuracy requirements for 15kHz SCS. In our view, the requirements should be generic to all the SCS. And for L3-SRS-RSRP, the accuracy requirements are defined for TDD with 15KHz SCS. Similar approach can be used.

Table 10.1.22.1.1-1:  SRS-RSRP absolute accuracy in FR1

| Accuracy |  |  |  |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  | Extreme condition |  |  | SRS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB |  |  |  |  |  | dB |  | dBm / SCSSRS |  |  | dBm/BW Channel | dBm/BW Channel |
| SCSSRS (kHz) |  |  | SCSSRS (kHz) |  |  |  |  |  |  |  |  |  |
| 15 | 30 | 60 | 15 | 30 | 60 |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| 3 | 3.5 | 5 | 7.5 | 8 | 9.5 | 1 | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| 6.5 | 7 | 8.5 | 9.5 | 10 | 11.5 | 1 | NR_TDD_FR1_A, NR_TDD_FR1_C, NR_TDD_FR1_D, NR_TDD_FR1_E | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |

And there is no additional effort to define accuracy requirements for 15KHz SCS. Table 1 is the simulation results summary based on companies’ feedback.

Table 1 performance comparison between case 1 and case 2 based on companies’ simulation results,

(SNR=1dB, SRS symbol number = 1)

| FR | SCS (kHz) | SRS BW (PRBs) | Sample | CATT (R4-2417696) |  |  |  | HW (R4-2419148) |  |  |  | Samsung (R4-2419620) |  |  |  | Ericsson (R4-2419442) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | case 1 |  | case 2 |  | case 1 |  | case 2 |  | case 1 |  | case 2 |  | case 1 |  | case 2 |  |
|  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| FR1 | 15 | 24 | 1 | -0.98 | 0.41 | -0.61 | 0.71 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -0.77 | 0.2 | -0.41 | 0.54 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -0.71 | 0.11 | -0.35 | 0.48 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -0.83 | 0.13 | -0.5 | 0.48 | -0.98 | 0.07 | -0.54 | 0.42 | 1.2 | 1.2 | 0.7 |  | -0.14 | 0.74 | 0.03 | 0.76 |
|  |  |  | 2 | -0.7 | -0.01 | -0.36 | 0.33 |  |  |  |  | 1.1 | 1.1 | 0.6 |  |  |  |  |  |
|  |  |  | 3 | -0.63 | -0.06 | -0.29 | 0.26 | -0.84 | -0.23 | -0.39 | 0.15 | 1 | 1 | 0.5 |  |  |  |  |  |
|  | 30 | 24 | 1 | -1.58 | -0.07 | -0.81 | 0.57 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -1.38 | -0.26 | -0.61 | 0.34 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -1.27 | -0.37 | -0.55 | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -1.46 | -0.35 | -0.67 | 0.3 | -1.66 | -0.5 | -0.78 | 0.19 | 2.1 | 2.1 | 1 |  | -0.49 | 0.67 | -0.06 | 0.74 |
|  |  |  | 2 | -1.29 | -0.52 | -0.54 | 0.16 |  |  |  |  | 1.9 | 1.9 | 0.8 |  |  |  |  |  |
|  |  |  | 3 | -1.23 | -0.59 | -0.48 | 0.1 | -1.54 | -0.84 | -0.64 | -0.08 | 1.7 | 1.7 | 0.8 |  |  |  |  |  |
|  | 60 | 24 | 1 | -3.12 | -1.27 | -1.28 | 0.21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -2.82 | -1.53 | -1.03 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -2.66 | -1.62 | -0.93 | -0.1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -2.9 | -1.58 | -1.09 | -0.09 | -3.28 | -1.76 | -1.25 | -0.16 | 3.9 | 3.9 | 1.5 |  | -1.32 | 0.58 | -0.28 | 0.71 |
|  |  |  | 2 | -2.68 | -1.77 | -0.96 | -0.23 |  |  |  |  | 3.6 | 3.6 | 1.4 |  |  |  |  |  |
|  |  |  | 3 | -2.61 | -1.85 | -0.89 | -0.3 | -3.05 | -2.18 | -1.13 | -0.5 | 3.4 | 3.4 | 1.3 |  |  |  |  |  |
| FR2 | 60 | 24 | 1 | -2.36 | -0.66 | -0.71 | 0.63 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -2.07 | -0.93 | -0.5 | 0.43 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -1.98 | -1.04 | -0.44 | 0.36 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -2.21 | -1.04 | -0.6 | 0.38 | -2.59 | -1.22 | -0.7 | 0.3 | 3.1 | 3.1 | 0.9 |  | -0.93 | 0.62 | -0.02 | 0.74 |
|  |  |  | 2 | -2.04 | -1.2 | -0.47 | 0.23 |  |  |  |  | 2.7 | 2.7 | 0.7 |  |  |  |  |  |
|  |  |  | 3 | -1.97 | -1.27 | -0.39 | 0.18 | -2.33 | -1.56 | -0.56 | 0.02 | 2.6 | 2.6 | 0.6 |  |  |  |  |  |
|  | 120 | 24 | 1 | -5.58 | -3.12 | -1.07 | 0.35 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -5.22 | -3.48 | -0.86 | 0.16 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -5.06 | -3.62 | -0.76 | 0.07 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -5.37 | -3.56 | -0.93 | 0.09 | -6.01 | -3.6 | -1.05 | -0.05 | 6.8 | 6.8 | 1.3 |  | -2.71 | 0.52 | -0.2 | 0.73 |
|  |  |  | 2 | -5.07 | -3.82 | -0.77 | -0.07 |  |  |  |  | 6.2 | 6.2 | 1.1 |  |  |  |  |  |
|  |  |  | 3 | -4.96 | -3.92 | -0.73 | -0.15 | -5.56 | -4.18 | -0.92 | -0.3 | 6 | 6 | 1 |  |  |  |  |  |

It can be seen that whether there is performance gap between the case with cell phase error and the case without cell phase error is related with SCS. For 15KHz SCS and 30KHz SCS, the performance gap between case 1 and case 2 is small, less than 1dB. While with larger SCS, e.g. 120KHz, the performance gap between case 1 and case 2 is large.

Taking above into account, it is proposed to define accuracy requirements for 15KHz SCS with cell phase error.

Proposal 1: it is proposed to define accuracy requirements for 15KHz SCS. And the accuracy requirements for 15KHz SCS are defined with cell phase error.

Issue 2-1-1: Measurement accuracy for CSI-RS based L1 measurements - Background - Different cases for DUD - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD). - Agreement from RAN4#114bis - Note: The following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD - Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 - For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs - In the test case, not model the interfering UE. - If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other - Way forward - Define the relaxed accuracy for L1-RSRP and L1-SINR based on 24 RB by adding [TBD] relaxation on top of existing accuracy (based on 48 RB) - For case 3 (X1 < X3 and X2 < X3, (X1 + X2) >= X3; X3 = 48 RB), - for a UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB apply. - Option 2: accuracy requirements based on 24 RB apply. - for UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB apply

For case 3 (X1 < X3 and X2 < X3, (X1 + X2) >= X3; X3 = 48 RB), according to the core part discussion, for UE supporting simultaneous DL reception in 2 DL subbands, the measurement period is different pending on whether UE indicating CSI processing time scaled by 2. For UE indicating CSI processing time scaled by 2, it was agreed to add a lower bound of 8ms on top of existing measurement period, if the CSI-RS resource is across 2 DL subbands and UE indicates CSI processing time is scaled by 2. For UE not indicating CSI processing time scaled by 2, Lower bound is not needed and existing measurement period applies.

It can be sen that measurement period is relaxed for UE indicating CSI processing time scaled by 2, no need to relax the accuracy, i.e. existing accuracy requirements based on 48 RB apply. For UE not indicating CSI processing time scaled by 2, accuracy requirements based on 24 RB apply considering that UE may only measure 1 sub-band.

Proposal 2: For case 3 (X1 < X3 and X2 < X3, (X1 + X2) >= X3; X3 = 48 RB),

- for UE indicating CSI processing time scaled by 2

  - existing accuracy requirements based on 48 RB apply

- for UE not indicating CSI processing time scaled by 2

  - accuracy requirements based on 24 RB apply

- Conclusion

This contribution provides discussion on RRM performance requirements. The proposals are:

Proposal 1: it is proposed to define accuracy requirements for 15KHz SCS. And the accuracy requirements for 15KHz SCS are defined with cell phase error.

Proposal 2: For case 3 (X1 < X3 and X2 < X3, (X1 + X2) >= X3; X3 = 48 RB),

- for UE indicating CSI processing time scaled by 2

  - existing accuracy requirements based on 48 RB apply

- for UE not indicating CSI processing time scaled by 2

  - accuracy requirements based on 24 RB apply

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- R4-2515166, WF for [116bis][309] Rel-19 RRM performance_Part1, Moderator (Qualcomm)

- R4-2512133, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2502699, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, Apple

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2416919, LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

- R1-2410890, Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R4-2502633, Updated summary of simulation results for L1-SRS-RSRP, Huawei, HiSilicon

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2413900, WF on SBFD RRM, Huawei, HiSilicon

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R1-2501620, LS on L1 based UE-to-UE CLI measurement and reporting