3GPP TSG-RAN WG4 Meeting 117	R4-2520487

Dallas, USA, Nov. 17-21, 2025

Source: 	Xiaomi

Title: 	Discussion on core requirement and testing framework for AI beam management

Agenda Item:	6.11.4

Document for:	Discussion

- Introduction

In this contribution, we will discuss core part requirement and testing framework for AI based beam management.

- Discussion

      - 2.1 Measurement window

Issue 2-1: Measurement period for inference – case 2 Agreement: Measurement period for inference – case 2 K(number of samples)*[M]*N*P K as defined by RAN1

The agreement in 38.214 for K is as below:

For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19' and with nroftimeinstance-r19 is configured, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least 𝐾BM latest consecutive transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource, and drops the report otherwise. The value of 𝐾BM is indicated by UE capability.

For Beam Management Case 2, a scaling factor K_BM is introduced to adjust the measurement time for RSRP predictions, where K_BM is determined by UE capability, to adapt to different AI model requirements and UE implementations.

The parameter K_BM represents the minimum number of time samples required by the UE's internal AI model for inference, regardless of the number of SSB resources (M) configured in the resource set. It is a requirement on the time dimension.

The parameter M represents the number of SSB resources to be measured. The total measurement period must be sufficient to collect the required number of samples for each of the M resources.

Therefore, the determining factor for the number of consecutive transmission occasions needed is the maximum of these two independent parameters:

- It must be at least M to ensure all beams in the set are measured at least once.

- It must be at least K_BM to ensure the UE has the minimum required time samples for its prediction algorithm.

Hence, the term max(M, K_BM) should be used in the measurement period formula to correctly capture this requirement.

The proposed measurement period for FR2 in BM-case 2 is therefore defined as:

Measurement period TL1-RSRP_Measurement_Period_SSB for FR2 for BM-case 2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N*max(M, KBM))*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N* max(M, KBM))*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N* max(M, KBM))*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. KBM is up to AI BM Case-2 UE capability. |  |

Proposal 1: It is proposed to define the number of consecutive occasions for BM-case 2 as max(M, K_BM), where K_BM is the UE-capable minimum time samples, to ensure the measurement duration meets all requirements.

      - 2.2 Simplified Channel model

Issue 2-8: Test system setup Agreement: A multi-AoA IFF based test setup with 4 active cross-polarized probes placed in the single plane (xz plane). The relative angular separation between neighboring probes is 30º, 60º, and 60º, respectively.

The chamber testing for AI-based beam prediction faces a practical limitation in the number of probes, which is directly influenced by the number of Angles of Arrival (AOA) in the channel model. To address this, a simplification of the CDL models (which underpin UMi and Uma scenarios) is necessary.

Our analysis of a CDL-C model reveals that simplifying the channel by reducing clusters and AOAs (e.g., by merging clusters with small AOA offsets and removing low-power clusters) has a direct impact on the spatial distribution of L1-RSRP:

- The L1-RSRP of weaker TX beams decreases more significantly than that of stronger beams.

- The differential in L1-RSRP between different TX beams increases as the number of clusters and AOAs is reduced, which could artificially make it easier for an AI model to distinguish the top beams.

Link to Previous Agreement:

While this study uses CDL-C as an example, the CDL models are concrete instantiations of the UMi and Uma channel characteristics. Therefore, the phenomena observed here are directly relevant to the simplification of UMi and Uma models, as agreed in the previous meeting.

Proposal 2: For the simplified channel model, the cluster number should be reduced based on spatial angle offset and power threshold.

- Combine clusters if the AOA offset between them is smaller than a defined threshold.

- Remove clusters with power below a defined threshold.

The specific values for the angle offset threshold and power threshold should be determined in RAN4, balancing test complexity (number of probes) against the need to maintain a realistic spatial channel profile for accurate AI model evaluation.

      - 2.3 Test procedure

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

it’s agreed that the ground truth of the predicted RSRP is the ideal measurement of RSRP on the predicted TX beam.

Test procedure

Fig.2 test procedure

As shown in Fig.2, the main steps for test beam or L1-RSRP accuracy are as below:

- Step 1: TE configures resource sets for measurement of set B and prediction report for set A

- Step 2: UE measures for beams in set B in T1

- Step 3: UE performs beam prediction for set A

- Step 4: UE report predicted best beam(s) and/or L1-RSRP(s)

- Step 5: TE configures resource set for measurement of set A

- Step 6: UE measures for beams in set A in T2 under high SNR

- Step 7: UE report ground truth of L1-RSRP of set A and/or best beam index

- Step 8: TE compares predicted beam with ground truth based on different metric

We identified that there are several issues that will have impact on the test metric accuracy.

### 2.3.1 SNR impact

Same as SNR difference analysed for set B, there will be great SNR difference for 32 TX beams in set A either. If SNR for the worst TX beam is set as 10dB, then SNR for best TX beam will be too high. There may be some test issue. Suppose that if SNR of best several TX beams is set as 10dB, SNR for the worst 10 TX beam can be as low as -15dB. There will be great measurement error for the worst beam.

For set A, UE only needs to identify several best beam indexes and/or corresponding L1-RSRPs, UE doesn’t need to measure for all beams accurately. The reason is that since the L1-RSRP delta between best beam and worst beam is relatively large, the measured L1-RSRP for worst beam is still quite small, which will not exceed the value of L1-RSRP of best TX beams.

Therefore, for deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Observation 1: For deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Therefore, RAN4 needs to discuss how to set SNR condition for measurement of set A to derive ground truth.

Proposal 3: For deriving ground truth of set A, SNR level should satisfy that for Top-M beams, the measurement error is small. RAN4 to discuss how to decide M and how to set SNR.

### 2.3.2 Channel time variation impact

Since the beam prediction will be based on the measurement of set B at T1. Ground truth L1-RSRP will be measured at T2. There will be a time gap between T1 and T2. Due to doppler, L1-RSRP will also vary in time domain. L1-RSRP time domain variation will also have impact on ground truth.

For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation. While for BM case-2, UE will be assumed to move. Therefore, channel doppler can’t be set for 0. Doppler will depend on UE speed and UE trajectory model. The RSRP variation can be evaluated by different doppler and time gap. RSRP variation impact to BM case-2 can be considered in RSRP accuracy requirement.

Proposal 4: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 5: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

### 2.3.3 How to solve UE cheating issue

In legacy L1-RSRP test, ground truth is set by TE. Therefore, UE can honestly report the measured L1-RSRP results.

In current AI based BM test, UE will report both best predicted beam and ground truth value to TE. TE can only rely on the two reports from UE to decide whether UE can pass the test. It’s easy for UE to cheat if the configuration for set A at T1 and T2 are the same. For example, suppose that for both T1 and T2, the same resource is configured and beam is mapping to these resources with the same order. Then in T1, UE report the best predicted RS index is 7. And in T2, after measurement for all RSs, UE find out the best beam is in RS 6. However, UE can just report the best measured RS index with 7 to TE. since TE didn’t know which beam is the best, it will only compare two indexes from UE reported at T1 and T2. If the two index is the same, TE will assume that predicted best beam is correct. Therefore, RAN4 needs to discuss how to avoid the issue.

Observation 2: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it’s easy for UE to pass the test by cheating.

The issue can be solved by several possible options.

- Option 1: Configure different resource for set A in T1 and T2.

- Option 2: Configure same resource for set A in T1 and T2 while the mapping relation of RS index and beam index is different. For example, suppose the beam index and RS index mapping relation for T1 is shown in Fig.3(a). For T2, TE will re-transmit all beams in set A on all RSs. TE will change the mapping relationship between RS index and beam index as shown in Fig.3(b). In this way, UE can only honestly report the best measured beam in T2.

Fig.3(a) beam index and RS index mapping relation for set A in T1

Fig.3 (b) beam index and RS index mapping relation for set A for T2

Proposal 6: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth.

- Conclusion

Proposal 1: It is proposed to define the number of consecutive occasions for BM-case 2 as max(M, K_BM), where K_BM is the UE-capable minimum time samples, to ensure the measurement duration meets all requirements.

Proposal 2: For simplified CDL model, cluster number can be reduced from spatial angle offset and power aspect:

- Combine clusters if AOA offset between these clusters is smaller than a threshold

- Remove cluster with low power.

Observation 1: For deriving ground truth of set A, SNR level should satisfy that for at least Top-M beams, the measurement error is small. SNR didn’t need to guarantee that all beams in set A can be measured accurately.

Proposal 3: For deriving ground truth of set A, SNR level should satisfy that for Top-M beams, the measurement error is small. RAN4 to discuss how to decide M and how to set SNR.

Proposal 4: For BM case-1, channel doppler can set to 0 or a small value to guarantee that there is neglectable L1-RSRP variation.

Proposal 5: for BM case-2, channel doppler will depend on UE speed and UE trajectory. RSRP variation impact can be considered in RSRP accuracy requirement.

Observation 2: UE will report best predicted L1-RSRP/beam index at T1 and measured RSRP/beam index of best beam at T2 to TE, it's easy for UE to pass the test by cheating.

Proposal 6: RAN4 to discuss how to solve the UE cheating issue if UE report both predicted result and ground truth.

- Reference

[1] R1-2501595, FL summary 6 for AI/ML in beam management