# 3GPP TSG-RAN WG4 Meeting #117	R4-2520507

# Dallas, US, 17-21 Nov, 2025

Agenda Item:	8.2.5.1

Source: 	Xiaomi

Title: 	View on 6GR waveform

Document for:	Discussion

- Introduction

According to the endorsed 6G work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. System parameter is one of major agenda with following candidate scope:

In this contribution, our preliminary view on waveform are provided.

- Discussion

Last RAN4 meeting has following agreements:

#### Modulation Agreement - RAN4 evaluation work focus on the feasibility study and RF requirements impact - To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1 - Identify the main affected requirements for modulation evaluations - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions. - Align on the evaluation assumptions. - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation. - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed. - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1. - Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies. - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

The major work expected from RAN4 on modulation order/waveform is feasibility study with associated RF requirements impact e.g., MPR/Power boosting, Tx EVM, and applicable scenarios (DL/UL, and frequency ranges). Without sufficient input from RAN1 on basic waveform and modulation order structure, it’s hard for RAN4 to have efficient discussion.  Also, the evaluation work has dependency on RF requirements assumption and PA model used. Given the relevant discussion on RF core requirements and PA model is still on going, we suggest to postpone the evaluation work.

Proposal 1: Postpone modulation order evaluation until sufficient progress reached in RAN1/RAN4 e.g., start from Q2’ 26

The most relevant RF requirements for modulation order is Tx EVM and MPR; especially for Tx EVM both performance benefits and implementation feasibility needs to be considered.

When starting the evaluation work, RAN4 also needs to carefully consider the budget of Tx EVM and Rx EVM on high modulations.

Proposal 2: RAN4 evaluation focus on MPR and Tx EVM aspects by considering performance benefits and implementation feasibility.

- Conclusion

In this contribution, our preliminary view on waveform are provided.

Proposal 1: Postpone modulation order evaluation until sufficient progress reached in RAN1/RAN4 e.g., start from Q2’ 26

Proposal 2: RAN4 evaluation focus on MPR and Tx EVM aspects by considering performance benefits and implementation feasibility.

Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] R4-2514650,“WF on 6GR system parameter”, Feature Lead (Huawei)