# 3GPP TSG-RAN WG4 Meeting #117	R4-2520510

# Dallas, US, 17-22 Nov, 2025

Agenda Item:	8.8

Source: 	Xiaomi

Title: 	Overview for 6GR demodulation requirements

Document for:	Discussion

- Introduction

In the endorsed 6G RAN4 work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. Demod is one of the area with following candidate scope:

In this contribution, our preliminary view on demodulation area is provided.

- Discussion

In last RAN4 meeting, RAN4 agreed as general principle:

General performance requirement aspects - Agreement - In 6G SI, RAN4 will study how to strive to develop a demod/test performance requirement framework for the minimum performance to better reflect real field conditions than 5G, if necessary, by taking into consideration of the requirement/test coverage/alignment ablity and the test feasibility, complexity and cost/resources

Regarding demod performance requirements framework to better reflect real field condition, at least we can collect operators’ feedback on key system parameters to be closer to network deployment e.g., CHBW, SCS, and TDD DL-UL pattern.

In NR from UE perspective, RAN4 develop generic common test parameters for demodulation and CSI requirements definition which considering test coverage and also operators’ deployment options.

Proposal 1: RAN4 aims to establish a common test parameter which used as basis for RAN4 demod/CSI requirements introduction e.g. default CHBW, SCS, and TDD DL-UL pattern.

Proposal 2: RAN4 aims to collect operators’ feedback on key system parameters to better reflect real field conditions.

Traditionally, RAN4 work on demodulation area is strongly depending on physical layer channel design and related MIMO, TRP procedure. Without sufficient progress from RAN1, it’s hard to have efficient discussion on those area.

Proposal 3: Postpone RAN4 discussion on demodulation requirements related to physical layer channel and procedure design until sufficient progress reached in RAN1 i.e., no early than Q2’26.

RAN4 still can have some preliminary discussion based on 5G design in following area:

Proposal 4: RAN4 focus on following area in initial stage on 6GR demod area:

- Reference receiver assumption

- Interference modelling for SU-MIMO and MU-MIMO; intra-cell and inter-cell interference modelling (DL and UL)

- Unified channel model for DL and UL considering AI use cases, ISAC, NTN (NGSO motion, Mobile VSAT), HST

- Scalable requirements structure for different device types (CHBW, number of Rx)

Reference receiver assumption/Interference modelling

In NR, MMSE-IRC receiver was considered as baseline receiver in the beginning, and later R-ML receiver introduced for SU-MIMO scenario. For specific scenarios, new receiver type was also introduced in later release e.g. advanced receiver on handling multi-TRP transmission with different doppler/delay/power under HST-SFN scenario, CRS interference handling with/without NW assistant signalling under 5G/4G spectrum sharing/co-existence scenario, R-ML receiver for MU-MIMO with/without NW assistant signalling on co-scheduled UE.

Proposal 5: RAN4 evaluate candidate receiver types on following scenarios in both DL and UL

- Noise limited scenario

- SU-MIMO /MU-MIMO scenario

- Inter-cell interference scenario

- Spectrum sharing/co-existence between 6G and 5G/4G

- HST scenario

On interference scenarios, how to model interference from intra-cell and inter-cell need more evaluation. NR leverage LTE interference profiles to verify performance. However, some parameters from LTE require further update e.g. antenna assumption in gNB and UE side, Power class assumption, also new duplex mode (dynamic TDD, SBFD operation at gNB side).

On 6GR deployment, it’s worth to further evaluate the interference profile to reflect real deployment scenarios.

Proposal 6: RAN4 further evaluate interference profiles for intra-cell/interference cell scenarios to reflect real field conditions

- gNB and UE configuration e.g., power class, antenna configuration

- Homogenous and heterogenous scenarios

- Asynchronization TDD or dynamic TDD scenario

- Semi-static/Dynamic SBFD operation in gNB

Channel model

Another issue is related to channel model, NR channel model was derived based TDL channel model and specific channel model also introduced for NTN operation, HST scenario. RAN4 also evaluated spatial channel model based on different approaches as summarized in TR 38.753.

On 6G, it’s worth to further evaluate candidate channel model also considering new use cases i.e., AI, ISAC, NTN and new operating frequency.

Proposal 7: RAN4 evaluate candidate channel model for DL and UL considering new operating frequency, new use case including AI, ISAC, NTN, HST.

Scalable requirements structure for diverse device types

NR only focused on MBB (handheld UE) initial stage and specified requirements based on physical channel, duplex mode, number of Rx antennas (2Rx, 4Rx). RAN4 UE performance requirements were specified based on the principle with default configuration e.g., TDD pattern, default SCS and CHBW. This ideal was introduced to verify basic physical features based on aligned typical configuration which can ensure wide coverage with limited test case considering demodulation requirements were derived to verify BB performance which is agnostic to operating bands, SCS and channel bandwidth.

In NR later stage, special device types e.g., Redcap UE, XR with different capabilities in terms of number of RX antenna and BW were introduced. Specific new test cases were introduced for these new device types, also new dedicated test cases were introduced for new channel bandwidth e.g., 3MHz.

It’s worth to further study scalable requirements structure for diverse device type.

Proposal 8: RAN4 study scalable requirements structure for diverse device types especially different capabilities of number of Rx, CHBW and operating mode.

- RAN4 shall develop common test configuration for 6GR on UE demodulation and CSI.

- The common test configuration shall consider real deployment from operators meanwhile ensure sufficient test coverage and scalable requirements for different device type.

New test function

OLLA with link adaptation - Feature Lead notes - Option 1 discussion has been captured in Chairman and Ad-hoc minutes. Feature lead’s suggestion based on discussion is that companies can be encouraged to consider technical solutions on OLLA in the next meeting as several companies see value to study OLLA as part of 6G demod SI. - Proposals - Option 1: Study to include OLLA in ATP requirements. - Option 1A: Use proposed OLLA model from R4-2300703 as a starting point. - Option 1B: Study the feasibility to include OLLA in ATP requirements.

In NR RAN4 spent lots of effort on ATP test with feasibility and requirements alignment activity and observed the difficulty of OLLA algorithm alignment, and requirement alignment. And companies also questioned the test purpose of this OLLA with link adaptation just verified UE and BS performance? or purely UE performance? Any difference from UE receiver processing perspective with and without OLLA.

Observation on OLLA with link adaption: NR failed to introduce ATP test with OLLA due to technical challenge:

- OLLA belongs to BS implementation, hard to have aligned algorithm

- Failed to have aligned results, no feasible to introduce reasonable requirements

- Justification of OLLA in UE demod test not clear given it’s transparent to UE

Proposal 9: RAN4 further study the jointly test with both BS and UE for OLLA with link adaption

- Conclusion

In this contribution, overview on 6GR demod area are provided.

Overall sope:

Proposal 1: RAN4 aims to establish a common test parameter which used as basis for RAN4 demod/CSI requirements introduction e.g., default CHBW, SCS, and TDD DL-UL pattern.

Proposal 2: RAN4 aims to collect operators’ feedback on key system parameters to better reflect real field conditions.

Traditionally, RAN4 work on demodulation area is strongly depending on physical layer channel design and related MIMO, TRP procedure. Without sufficient progress from RAN1, it’s hard to have efficient discussion on those area.

Proposal 3: Postpone RAN4 discussion on demodulation requirements related to physical layer channel and procedure design until sufficient progress reached in RAN1 i.e., no early than Q2’26.

RAN4 still can have some preliminary discussion based on 5G design in following area:

Proposal 4: RAN4 focus on following area in initial stage on 6GR demod area:

- Reference receiver assumption

- Interference modelling for SU-MIMO and MU-MIMO; intra-cell and inter-cell interference modelling (DL and UL)

- Unified channel model for DL and UL considering AI use cases, ISAC, NTN (NGSO motion, Mobile VSAT), HST

- Scalable requirements structure for different device types (CHBW, number of Rx)

Receiver Type

Proposal 5: RAN4 evaluate candidate receiver types on following scenarios in both DL and UL

- Noise limited scenario

- SU-MIMO /MU-MIMO scenario

- Inter-cell interference scenario

- Spectrum sharing/co-existence between 6G and 5G/4G

- HST scenario

Interference profile

Proposal 6: RAN4 further evaluate interference profiles for intra-cell/interference cell scenarios to reflect real field conditions

- gNB and UE configuration e.g., power class, antenna configuration

- Homogenous and heterogenous scenarios

- Asynchronization TDD or dynamic TDD scenario

- Semi-static/Dynamic SBFD operation in gNB

Channel Model

Proposal 7: RAN4 evaluate candidate channel model for DL and UL considering new operating frequency, new use case including AI, ISAC, NTN, HST.

Scalable requirements structure for diverse device types

Proposal 8: RAN4 study scalable requirements structure for diverse device types especially different capabilities of number of Rx, CHBW and operating mode.

- RAN4 shall develop common test configuration for 6GR on UE demodulation and CSI.

- The common test configuration shall consider real deployment from operators meanwhile ensure sufficient test coverage and scalable requirements for different device type.

OLLA:

Observation on OLLA with link adaption: NR failed to introduce ATP test with OLLA due to technical challenge:

- OLLA belongs to BS implementation, hard to have aligned algorithm

- Failed to have aligned results, no feasible to introduce reasonable requirements

- Justification of OLLA in UE demod test not clear given it’s transparent to UE

Proposal 9: RAN4 further study the jointly test with both BS and UE for OLLA with link adaption

Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] R4-2515465, WF on [116bis][106] 6G Demod. Feature lead (MTK)