# 3GPP TSG-RAN WG4 Meeting #117	R4-2520510

# Prague, CZ, 13-17 Oct, 2025

Agenda Item:	8.4

Source: 	Xiaomi

Title: 	Overview for 6GR UE RF and general RF aspects

Document for:	Discussion

- Introduction

In the endorsed 6G RAN4 work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. Spectrum is one of the major area with following candidate scope:

In this contribution, our preliminary view on 6G UE RF and general RF topics with following area

- Single carrier output power

- Tx requirements

- Rx requirements

- Spectrum aggregation

- Discussion

  - Single carrier output power

2.1.1 MOP requirement framework

As discussed in the last RAN4 meeting, the MOP requirement framework for 6G is worth to be discussed. This issue can be separated to two fundamental sub-issues, how to grade the power level and how to map the power level to different functions.

How to grade the MOP levels

5G uses power class to grade the UE MOP power levels. Currently, the following power classes are defined/discussed.

| Power class (PC) | Maximum output power (MOP, dBm) | Tolerance (Db) | MOP + Tolerance (dBm) |
| --- | --- | --- | --- |
|  | 32 | Under discussion | Under discussion |
| PC1 | 31 | +2/-3 | 33/28 |
| PC1.5 | 29 | +2/-3 | 31/26 |
| PC2 | 26 | +2/-3 | 28/23 |
| PC3 | 23 | +2/-2 | 25/21 |

The step size between PC3/PC2/PC1.5 is 3 dB. As higher output power brings benefit to UL coverage, thus can save the network deployment cost and improve user experience, PA vendors are always trying to increase PA performance to make the UE MOP bigger. However, some of the capabilities are between the two power classes. For example, if a UE can transmit 28 dBm, it only can report PC2, the extra MOP capability is wasted. It’s necessary to use a more flexible MOP power level grading methodology to fully use the PA capability.

Observation 1: It’s necessary to use a more flexible MOP level grading methodology to fully use the PA capability.

The purpose of the flexible MOP level grading methodology is to remove the restrictions of standard. The step size needs to be decided first. Our understanding is 1 dB step size is sufficient because smaller step size can’t be accurately distinguished considering the PA capability and the test tolerance.

Proposal 1: 1 dB step size is used for the MOP level grading methodology.

When the step size is decided, the detail solution of MOP level grading methodology can be discussed. There’re several options for the new MOP power level grading methodology. Some of them are mentioned in the contributions in last meeting. There’re mainly 4 options on the table currently as followings.

Option 1: Remove the power class concept but define the nominal MOP level with 1 dB step size. Tolerance is still +2/-2(-3) dB.

To our understanding, this is the most straightforward method. NW can know the exact UE MOP capability and the tolerance is controlled in a reasonable level.

Option 2: Power class concept is kept with a coarser step size but allow UE to use power boosting to make finer step size possible. Tolerance is still +2/-2(-3) dB.

This option is proposed in R4-2513316. If the step size of power classes is still 3 dB and UE is allowed to power boost 1 or 2 dB, then the step size of final MOP level is 1 dB size which is the same with option 1. But option 2 seems more complicated thus we’re not in favor of it. The possible pros of option 2 may be the power class concept is kept but the necessity is doubtful. If there’s no other specific benefit of option 2, option 1 is a better choice.

Option 3: Power class concept is kept with large tolerance of +5/-2dB.

This option is proposed in R4-2513115. We’re not in favor of this option either because the nominal MOP capability and the tolerance is mixed in this proposal. The tolerance should include all of the differences compared with the target power. It may include the hardware/software non-ideal factors which finally is treated as power control tolerance for NW side. The power control tolerance has impacts to the system performance, such as the interference to other UEs/NWs, IoT level, etc. If the tolerance requirement is relaxed, UE can have two implementations. One is to increase the nominal power but keep the +2/-2 tolerance, the other is keep the same nominal power level but with worse tolerance performance. The latter one should be avoided but there’s no way to forbit it if option 3 is used. So we don’t think option 3 is agreeable.

Option 4: Removing the upper limit of the power class by changing the definition of Pcmax_H.

This option is mentioned in R4-2513021 but how to change the Pcmax_H is not clear. The contribution also mentioned that the solution of increase the power for some specific signals, it’s not clear if this solution is for the MOP requirement only. More details are needed for this solution, e.g., what’s the nominal output power definition, what’s the allowed tolerance, etc. It seems a solution more related to the power control not the MOP level definition.

Comparing the 4 options on the table, we’re more in favor of option 1.

Proposal 2: Option 1 is kept as a candidate solution to solve the MOP level grading issue.

Option 1: Remove the power class concept but define the nominal MOP level with 1 dB step size. Tolerance is still +2/-2(-3) dB.

How to define MOP for different features

For how to define MOP for different features, it’s more complicated and may need more thinking.

There’re 3 main working modes for single carrier as followings.

Single carrier without TxD and UL MIMO

Single carrier with TxD

Single carrier with UL MIMO

The MOP definition should cover all of the 3 features with clear and relatively simpler definition. For the UL MIMO, some companies already proposed that 1 layer, 2 layers should have the different capabilities.

From implementation point of view, the MOP definition should also cover all of the possible UE RF architectures. Figure 1 can be a reference architecture to be considered.

Figure 1: A possible UE RF architecture for MOP definition

Although the two PAs in the two RF chains have the power capacities of 30 dBm, the MOP capabilities for the two antennae are different because of the IL for the two RF chains are different. So the MOP for different features can be different.

According to the contributions in last RAN4 meeting, there may be two very different directions to define MOP capability. One is to define MOP for the feature as a whole, which is the legacy method used for 4G/5G. There’s another direction proposed in R4-2513251 that MOP is defined according to power capability of RF chain.

We’re not sure which direction is better. An in-depth discussion to consider all of impacted aspects to UE implementation and three working modes is needed. Some preliminary thinking of the two directions is provided in this contribution.

For the direction of defining MOP for the feature as a whole, it’s a straightforward direction because it’s already used for many years. One of the pros of this approach is that BS can know directly UE MOP capability for different features. But there’re also some cons already blamed by many companies that currently the power class for different features can be very complicated. Even there’s only one UE RF architecture, the MOP capabilities can be complicated and confusing. Taking Figure 1 as an example, the MOP capabilities for the three working modes are as followings.

Single carrier without TxD and UL MIMO: 27 dBm (max(27,25))

Single carrier with TxD: 28 dBm

Single carrier with UL MIMO: 1 layer: 27 dBm, 2 layers: 28 dBm

If the direction of RF chain MOP power capabilities is used, it also seems a more straightforward solution considering UE implementation. The complicity is that there should be some rules to map the RF capabilities to the different working modes. Still taking Figure 1 as an example, the MOP for the two RF chains is as followings,

MOP for ant 1: 27 dBm

MOP for ant 2: 25 dBm

Then the MOP capabilities for the three working modes are as followings.

Single carrier without TxD and UL MIMO: 27 dBm (max(ant1,ant2))

Single carrier with TxD: 28 dBm (min(ant1,ant2)+3)

Single carrier with UL MIMO: 1 layer: 27 dBm (max(ant1,ant2)), 2 layers: 28 dBm (min(ant1,ant2)+3)

The MOP definition of this approach is clear, and the MOP mapping according to the RF capabilities are also simple. However, if the mapping method should be described in the spec and if the description can be clear enough should be further discussed.

Proposal 3: The two following options are kept to further discuss how to define MOP for different features.

Option 1: MOP is defined according to the features/working modes.

Option 2: MOP is defined according to power capability of RF chain.

2.1.2 Pcmax simplification

Pcmax simplification is a rather complicated issue because it’s not only related to RAN4 RF requirements but also it may be related to RAN1 power control procedure. In NR, Pcmax, PHR, DPC are reported through MAC CE,

Figure 2: PHR MAC CE copied from TS 38.321

We’re not sure if Pcmax and DPC needs to be reported because PHR is already reported. But this is related to RAN1, more views from companies are needed.

For the equation,

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}

We’re still studying if there’s a good solution to solve the whole issue, but at least we think ∆TRxSRS shouldn’t be in the equation because the SRS IL is a static insertion loss which impacts maximum output power of SRS signal not the configured output power. So we propose to remove ∆TRxSRS from the Pcmax equation.

Proposal 4: Remove ∆TRxSRS from the Pcmax equation.

2.1.3 Default power class

For 6G, if default power class is needed or if this name should be continued to be used can be discussed. Default power class is used from 4G phase. At that stage, almost all of the UE supports PC3, only a small number of UE support PC2 as exceptions. So the default power class existing in 4G seems reasonable. However, in 6G, we’re not sure if the name of “default power class” is still appropriate. A similar concept like a lower limit of MOP is needed for the reference of NW deployment. For the lower limit, our preference is 23 dBm for FDD band, 26 dBm for TDD band.

Proposal 5: Revisit the name of “default power class” after MOP methodology discussion is clear.

Proposal 6: For FR1, the lower limit of MOP is 23 dBm for FDD band, 26 dBm for TDD band.

2.1.4 SAR/MPE solution

For SAR solution, we support the proposal to remove duty cycle solution in Pcmax clause, they’re too complicated when more and more MOP levels are defined.

Proposal 7: Remove duty cycle solution in Pcmax clause. SAR/MPE solution is left to implementation.

  - Tx requirements

UE Tx impairments

It was proposed in the WF that carrier leakage, image rejection, CIM3 and CIM5 can be revisited for 6G. There’re some proposals that high order modulation 256QAM needs improved performance of carrier leakage and image rejection up to -34 dBc. Although 256QAM is optional, we’re ok to improve the assumption of carrier leakage and image rejection. For CIM3 and CIM5, we see there’s little possibility to improve the performance.

Proposal 8: The following Tx impairments assumptions can be used for MPR/AMPR simulation.

Carrier leakage: -34 dBc

Image rejection: -34 dBc

CIM3: -60 dBc

CIM5: -70 dBc

Unwanted emission framework

For the general OOB boundary revisit, this is more likely related to the regulation requirement. If there’s no regulation problem to extend the OOB boundary, we’re supportive for this direction. But the impact to the implementation and UE RF requirements should be studied first.

Proposal 9: The regulation and the impact to UE RF requirements for OOB boundary change should be studied.

For the SEM requirement, we do see some difficulties for the first 1MHz requirement for full RB case and IMD3 performance for the 1RB case. If there’s no regulation issue, we’re also supportive to this study. One aspect needs to be pointed out is that SEM performance is related to SU study, which may involve the BB windowing performance and the EVM window length requirement. We estimate it’s a rather complicated and lengthy discussion. How to decouple the two discussion is an issue. One way can be that UE RF thread study the SEM requirement based on 5G SU, then revisit it after there’s conclusion for 6G.

Proposal 10: The regulation for SEM requirement should be surveyed.

Proposal 11: SEM requirement study uses 5G SU as the starting point. It can be revisited after there’s conclusion for 6G SU.

  - Rx requirements

Fundamental single carrier REFSENS methodology

For FR1, the following equation is used for 4G and 5G. We didn’t find good justification to change it.

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

There are two discussion points according to the last meeting’s WF.

Scalable REFSENS framework for different cases/features

Currently, there’re some framework for 2R -> 1R, 4R and 8R. We’re not sure if there’re other cases which need scalable REFSENS framework.

Proposal 12: The cases for the scalable REFSENS framework study should be clarified.

UL RB allocation framework

The UL RB allocation for FDD REFSENS seems indeed flexible. Some of them use full RB if there’s no Tx/Rx interference issues. Some of them uses RB restriction but the motivation for the RB restriction may be different for the bands. However, considering most of the REFSENS requirements for the bands are inherited from 4G. UE implementation, test platform for 6G will be continued to inherit 5G. If there’s a change for the UL RB allocation, much corresponding efforts will be needed for the product. So the benefit to change UL RB allocation should be clarified first to continue the study.

Proposal 13: The benefit to change the UL RB allocation for REFSENS should be clarified to continue the study.

RSD value

RSD is used to derive the REFSENS for the cases/features different with the baseline 1T/2R REFSENS requirements. Currently, RSD value accuracy is 0.1 dB which we think not necessary considering the impact to implementation and test tolerance. So we propose to round RSD value to 0.5 dB.

Proposal 14: If RSD is still defined, the RSD value is rounded to 0.5 dB.

CA MSD framework

For the CA MSD framework, we see two possible options for the improvement.

- Option 1: MSD requirement is kept with coarser MSD values.

    - 3 dB step size can be considered to save the discussion efforts.

    - Very large MSD CA will not have specific requirement, for example a roughly requirement like > 15 dB，or the CA request can’t be approved.

- Option 2: MSD requirement is removed.

    - If UE needs to report the interference/desense capability for specific configuration FFS.

The important decision should be if MSD requirement should be kept for CA. First the motivation MSD requirement can be discussed. From our understanding, if a requirement is defined, the motivation is mainly from system performance purpose. We’re not very sure about the system performance consideration of MSD requirement. Usually, CA is configured when the channel condition is good enough. In that scenario, Tx power is not in MOP level and Rx signal is not in REFSENS level either. The effort to improve performance in a scenario that is not very possibly to exist is questionable.

Proposal 15: RAN4 to discuss the possibility to remove CA MSD requirements from 3GPP spec.

REFSENS > 100MHz CBW

For REFSENS > 100MHz CBW, we don’t see the urgency because the maximum CBW is under discussion in system parameter thread. For 200MHz CBW, CA requirements can be a reference. If the single carrier REFSENS framework is decided, the large CBW REFSENS is not very difficult to be decided. It even can be discussed in WI phase.

Proposal 16: Large CBW REFSENS study is started after the general REFSENS discussion is clear.

  - Spectrum aggregation

On 6G spectrum aggregation, RAN4 needs to be deeply involved from RF/RRM feasibility persetcive to investigate potentail solutions on fully usage of operators’ spectrum with affordable cost from UE and gNB. Several directions can be considered in 6GR intial stage on flexcible spectrum usage:

- DL/UL spectrum decoupling

- Fragmented carrier

- CA via swicthing and uplink Tx swicthing

DL/UL decoupling

6GR will further extend spectrum to 7GHz or even 10GHz, how to efficient use spectrum to guranteen both coverage and high data rate request. By natually UE can utlize low bands spectrum for coverage (UL) and  high bands spectrum for DL data tranmission.  Decouple DL and UL spectrum is one potential research area for flexicble spectrum integration.

RAN4 needs to evaluate the operation restrcition and potentail reference architecture from both RF and RRM perspective, also the difference compared SUL operation for flexcible spectrum paring across low frequency to high frequency in different band group.

Fragmented spectrum

Operators’ spectrum maybe fragemented over same band or across different adjacent bands.

In Rel-19 RAN4 estiablished a study item to study using shared RF chain to receive multi non-contiguous carriers within same band.

The benefits of shared chain to receiver multi fragmented carriers as following:

- Save cost to receive multi-carriers with reduced RF/BB chain

- Support high order band combinations with more carriers with limited Tx/Rx chains

- Support high order of MIMO layers for same band combination

In Rel-19 fragmented carrier study, the restriction and performance impact were also anayzed with shared RF chain with two restrction realized:

- With shared RF chain, UE performance in in-gap interference rejection degraded as no sub-block analogue filter

- REFSENS degradation due Tx leakage with reduced attenuation on UL carrier frequency with wideband LPF under “single Rx chain mode”.

Based on RAN4 analyzed in Rel-19, some restriction maybe applied for single Rx mode:

- Power imbalance between wanted carriers

- In gap interference (power imbalance between wanted carrier and in-gap interference)

- Maximum frequency separation between carriers

Potential methods are discussed for capability signaling, CA fallback and architecture switching, and Triggering condition to enable ”One Rx RF chain” and summarized in TR 38.755.RAN4 also discussed related single Rx chain mode switching on/off mechanism with in-gap interference monitoring which may require potential RRM work.

For 6G, the fragmented carrier scenario can be further extended to inter-band cases as following:

Other WGs also discussed potential enhancement of spectrum aggregation e.g., single cell multi carrier or single cell multi-bands. RAN4 still needs to be deeply involved to study RF/RRM feasibility study incl. reference architecture, related scheduling conditions and capability design and the benefits compared to traditional CA framework.

Obseravtion on fragmented spectrum:

- Operators’ spectrum maybe fragemented over same band or across adjacent bands which bring implemenation difficulty and contraints

- It’s worth to study enhanced solutions on supported fragemented carriers with affordale cost and improved spectrum usage efficiency.

UL Tx switching/ CA via switching

NR introduced UL Tx switching and CA via switching to dynamic/semi-static switching Tx/Rx chains over different bands/carriers to efficient use the available Tx/Rx chain and flexible spectrum.

For UL Tx switching, different cases and scenarios were introduced under different release including CA UL switching between 1Tx-2Tx across 2 bands, 2Tx-2Tx UL switching between 2 bands, 1/2Tx-1/2Tx switching between 3/4 bands including SUL bands. In Rel-19 RAN4 maintenance, RAN4 also discussed 3Tx switching and NUL-SUL switching with non-zero switching time.

Under Rel-19, RAN4 also discussed and introduced LB CA via switching for SDL and FDD carrier due to RF front-end limitation and the candidate bands as show in below table.

Observation on UL Tx switching:

- NR developed UL Tx chain switching in several release to accommodate different cases/scenarios which is lack of harmonized scheme and solutions and make the feature fragmented over several release and difficulty for implementation.

- It’s worth to study unified solutions/schemes in 6G day1 to accommodate different scenarios/cases to simplify implementation and improve the performance

Rel-19 LB ca via switching only focus on FDD-SDL carrier with switching 2 carriers and Rel-20 will support LB CA via switching for FDD-SDL carrier with dual DL.

There is commonality between UL tx switching and carrier switching in terms of switching time, switching mechanism and potential RF/RRM impact basically the ideal was to share Tx/RF chain across closed operating frequency ranges within band group to achieve MIMO or frequency diversity gain.  It’s worth to study unified framework and switching mechanism for all above mention cases and new cases which on discussion in Rel-20/Rel-19 maintenance.

Observation on LB CA band combinations

- Due to RF front-end design challenge (antenna, filter etc.) , it’s difficult to support simultaneously Tx/Rx across closed/overlapped bands.

- It’s worth to study the restriction and optimized solution to support difficult band combinations in 6GR day e.g. LB and LB band combination.

Proposal 17: RAN4 shall be deeply involved in day 1 for flexible spectrum usage/spectrum aggregation study with tight cooperation with other WGs i.e., DL/UL decoupling, single cell multi carriers

Proposal 18: RAN4 work shall focus on RF/RRM feasibility study incl. reference architecture, related scheduling conditions and capability design and the benefits compared to traditional CA framework

Proposal 19: RAN4 initial study can be focused on following area:

- Fragmented spectrum (single cell multi carriers/BWPs)

  - Reference RF and BB architecture

  - RF/RRM feasibility and operation schemes between different operation modes

  - Leverage Rel-19 RAN4 fragmented carrier SI with extension to inter-band case

- CA via switching and Uplink Tx switching with unified framework and mechanism

  - Leverage NR design on LB CA via switching and uplink Tx switching with extension to more cases

- Conclusion

In this contribution, we provide some preliminary thinking on some potential directions for the general RF and UE RF discussion.

Single carrier output power

Observation 1: It’s necessary to use a more flexible MOP level grading methodology to fully use the PA capability.

Proposal 1: 1 dB step size is used for the MOP level grading methodology.

Proposal 2: Option 1 is kept as a candidate solution to solve the MOP level grading issue.

Option 1: Remove the power class concept but define the nominal MOP level with 1 dB step size. Tolerance is still +2/-2(-3) dB.

Proposal 3: The two following options are kept to further discuss how to define MOP for different features.

Option 1: MOP is defined according to the features/working modes.

Option 2: MOP is defined according to power capability of RF chain.

Proposal 4: Remove ∆TRxSRS from the Pcmax equation.

Proposal 5: Revisit the name of “default power class” after MOP methodology discussion is clear.

Proposal 6: For FR1, the lower limit of MOP is 23 dBm for FDD band, 26 dBm for TDD band.

Proposal 7: Remove duty cycle solution in Pcmax clause. SAR/MPE solution is left to implementation.

Tx requirements

- Proposal 8: The following Tx impairments assumptions can be used for MPR/AMPR simulation.

Carrier leakage: -34 dBc

Image rejection: -34 dBc

CIM3: -60 dBc

CIM5: -70 dBc

Proposal 9: The regulation and the impact to UE RF requirements for OOB boundary change should be studied.

Proposal 10: The regulation for SEM requirement should be surveyed.

Proposal 11: SEM requirement study uses 5G SU as the starting point. It can be revisited after there’s conclusion for 6G SU.

Rx requirements

Proposal 12: The cases for the scalable REFSENS framework study should be clarified.

Proposal 13: The benefit to change the UL RB allocation for REFSENS should be clarified to continue the study.

Proposal 14: If RSD is still defined, the RSD value is rounded to 0.5 dB.

Proposal 15: RAN4 to discuss the possibility to remove CA MSD requirements from 3GPP spec.

Proposal 16: Large CBW REFSENS study is started after the general REFSENS discussion is clear.

Spectrum aggregation

Obseravtion on fragmented spectrum:

- Operators’ spectrum maybe fragemented over same band or across adjacent bands which bring implemenation difficulty and contraints

- It’s worth to study enhanced solutions on supported fragemented carriers with affordale cost and improved spectrum usage efficiency.

Observation on UL Tx switching:

- NR developed UL Tx chain switching in several release to accommodate different cases/scenarios which is lack of harmonized scheme and solutions and make the feature fragmented over several release and difficulty for implementation.

- It’s worth to study unified solutions/schemes in 6G day1 to accommodate different scenarios/cases to simplify implementation and improve the performance

Observation on LB CA band combinations

- Due to RF front-end design challenge (antenna, filter etc.) , it’s difficult to support simultaneously Tx/Rx across closed/overlapped bands which make LB CA band combination hard to be deployed and implemented.

- It’s worth to study the restriction and optimized solution to support difficult band combinations in 6GR day e.g. LB and LB band combination.

Proposal 17: RAN4 shall be deeply involved in day 1 for flexible spectrum usage/spectrum aggregation study with tight cooperation with other WGs i.e., DL/UL decoupling, Fragmented carrier (single cell multi carriers), UL Tx switching/CA via switching

Proposal 18: RAN4 work shall focus on RF/RRM feasibility study incl. reference architecture, related scheduling conditions and capability design and the benefits compared to traditional CA framework

Proposal 19: RAN4 initial study can be focused on following area:

- Fragmented spectrum (single cell multi carriers/BWPs)

  - Reference RF and BB architecture

  - RF/RRM feasibility and operation schemes between different operation modes

  - Leverage Rel-19 RAN4 fragmented carrier SI with extension to inter-band case

- CA via switching and Uplink Tx switching with unified framework and mechanism

  - Leverage NR design on LB CA via switching and uplink Tx switching with extension to more cases

- References

[1] R4-2514642, “WF on [116bis][102] 6G general RF and UE RF”, Feature lead (Qualcomm), RAN4#116b

[2] R4-2514509, “Topic Summary for [116bis][102] 6G general RF and UE RF”, Feature lead (Qualcomm), RAN4#116b