3GPP TSG-RAN4 Meeting #117	R4-2520544

# Dallas, United States, 17th - 21st November, 2025

Agenda item:		6.11.6

Source:				Keysight Technologies UK Ltd

Title:					Simulation Results for AI-ML air interface Beam Management

Document for:		Discussion

# Introduction

In this contribution, simulation results are provided to evaluate the impact of the measurement error on AI-ML beam management. The simulation assumptions for AI/ML based beam management in agreement [1] are followed, including how to generate baseband error, RF error, reference model, test scenarios and test cases.

# System Level Simulations for dataset generation

The CDL Dense Urban Macro Channel model is configured to generate the baseline SLS (System Level Simulation) to generate our own dataset used in simulations. The simulation parameters considered in simulation results provided are:

- For scenario 1: a combination of Table 6.3.1-1 in 3GPP TR 38.843 [2] plus Table 6 in [1] customized for “spatial only” beam prediction.

- For scenario 2: a combination of Table 6.3.1-1 in 3GPP TR 38.843 [2] plus Table 7 in [1] customized for “spatial only” beam prediction.

Dataset size used for simulation was 120000 samples.

During RAN4#116bis, agreed way forward in [3] included the following statement:

Issue 2-8: Test system setup Agreement: A multi-AoA IFF based test setup with 4 active cross-polarized probes placed in the single plane (xz plane). The relative angular separation between neighboring probes is 30º, 60º, and 60º, respectively.

Proposal 1: RAN4 to work on a comparison of simulation results with latest system level simulation assumptions vs system level simulation assumptions considering test system setup implementation restrictions to quantify performance requirements.

# Simulation assumptions for baseband error

Baseband errors for simulation results provided are generated following below assumption:

- Generate error distribution separately for each individual TX-RX beam

- Model the baseband errors corresponding to Link Level Simulations considering NLOS Channel (TDL-C, DS=100ns) as Gaussian distributions and then apply the distributions to the corresponding Tx-Rx beam pair measurement in the SLS

  - Normal distribution of mean = -0.12 dB and sigma = 0.85 dB for TDL-C model for SNR = -3 dB proposed in [4] is used in the simulation of this contribution

# Simulation assumptions for RF error

RF errors are generated following below assumptions as indicated in [1]:

- Truncated Gaussian distribution under ±4.5 dB RF error (μ=0,  σ2=4.5) is used

  - Each TX beam is independent

  - 2Rx chains with independent RF error per chain (same distribution for each chain)

# Reference model

For better alignment of the performance between companies, the simulations are performed based on the reference model. The model description is shown in Fig. 1.

Fig. 1 Reference model description

# Performance metrics

The following two performance characteristics are included in simulation results provided as indicated in [1] Section 5:

KPI 1: Beam prediction accuracy

- The successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 0,1, 2, 3dB

KPI 2: Absolute RSRP accuracy

- 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ground truth L1-RSRP of the same beams

Note 1: the ‘ground truth’ underlined refers to:

- Ideal value without consideration of measurement error (for Case 1 defined in clause 8)

- Measurement result with consideration of measurement error (for Case 3 defined in clause 8)

Note 2:

90%-tile L1-RSRP difference = max (abs (95%-tile L1-RSRP), abs (5%-tile L1-RSRP))

# Simulation scenarios

The impact of measurement error in two scenarios of beam management prediction has been evaluated:

Scenario 1: narrow-to-narrow spatial beam prediction (CSI-RS to CSI-RS)

- Scenario 1: spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement, and the simulation assumption is in Table 1

Scenario 2: wide-to-narrow spatial beam prediction (SSB to CSI-RS)

- Scenario 2: Spatial domain prediction with 32 CSI-RS beams in Set A and 8 SSB beams in Set B, and the simulation assumption is in Table 2

Table 1 Beamforming characteristic of the BS pattern for System-Level simulation assumption in narrow-to-narrow scenario

| Parameter | Value |
| --- | --- |
| Beam pattern legend for Set A and Set B |  |
| Azimuth and elevation angle ranges | 32 beams with grid of 4 elevation angles from (100°to 150°) and 8 azimuth angles from (-60°to 60°) Note: here the 0 degree in Elevation refers to +Z axis |
| AoDs of Set A and Set B (°) | - Set A: - 4 elevation DFT beams: [106.25, 118.75, 131.25, 143.75] - 8 azimuth DFT beams: [-52.5, -37.5, -22.5, -7.5, 7.5, 22.5, 37.5, 52.5] - Set B [(azimuth, elevation)]: - [(-52.5, 143.75), (-37.5, 131.25), (-22.5, 118.75), (-7.5, 106.25), (7.5, 143.75), (22.5, 131.25), (37.5, 118.75), (52.5, 106.25) |

Table 2 Beamforming characteristic of the BS pattern for System-Level simulation assumption in wide -to-narrow scenario

| Parameter | Value |
| --- | --- |
| AoDs of Set A and Set B (°) | Set A (CSI-RS): - 4 elevation DFT beams: [106.25, 118.75, 131.25, 143.75] - 8 azimuth DFT beams: [-52.5, -37.5, -22.5, -7.5, 7.5, 22.5, 37.5, 52.5] Set B (SSB): - 2 elevation DFT beams: [112.5, 137.5] - 4 azimuth DFT beams: [-45, -15, 15, 45] Companies can use a subset of antenna elements to generate wider DFT beams for SSB. One of the options to generate setB beams is shown below. Other options to generate setB beams are not precluded. |
| BS Antenna Configuration and number of beams | For setA determination: Use the setup from 38.843 Table 6.3.1-1 Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Number of CSI-RS beams: 32 For setB determination: Use a subset of antenna elements, e.g., below. Antenna setup and port layouts at gNB: (2, 4, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Number of SSB beams: 8 |

During RAN4#116bis, agreed way forward in [3] included the following statement:

Issue 2-4: Draft CRs CR drafting: Introduce a new suffix for beam prediction requirements – X (applicable for all new requirements related to beam prediction) For TCI state switching, introduce a new clause 8.10.2X replicating the known TCI state condition clause for the beam prediction use case For L1-RSRP, introduce requirements also for SSB to SSB prediction for case 1 and case 2, reuse the same requirements as CSI-RS as applicable (for example only periodic resources) Introduce new clause on L1-RSRP measurements for prediction 9.5X, include SSB based prediction and CSI-RS based prediction as different subclauses For performance requirements (including test cases), consider separately SSB to SSB, SSB to CSI-RS and CSI-RS to CSI-RS prediction cases, including downselecting to even a single case.

Considering simulations done so far in RAN4, SSB to SSB could be down selected.

Proposal 2: Down select SSB to SSB for performance requirements.

# Simulation cases

The following two cases are simulated:

- Case 1: No error will be considered in training dataset, model input during inference and ground-truth.

- Fig. 2 Simulation procedure for Case 1 (No error considered)Case 3: Error will be considered in training dataset, model input during inference and ground-truth.

Fig. 3 Simulation procedure for Case 3 (Baseband error through LLS with TDL-C, DS=100ns NLOS channel modelled as a normal distribution as done in [4])

For Case 3 add the generated measurement errors into the ideal SLS dataset to derive the dataset with measurement errors. The measurement errors will be:

- Alt 1: Only baseband errors are included

- Alt 2: Only RF errors are included

- Alt 3: Both baseband errors and RF errors are included

# Simulation results

## All the simulation results are shown in tables 3 and 4:

Table 3. Simulation results for Scenario 1: spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement

|  |  | Case 1 | Case 3 (BB Error only) | Case 3 (RF Error only) | Case 3 (BB + RF Error) |
| --- | --- | --- | --- | --- | --- |
| KPI 1: Beam prediction accuracy | K=1, X=0 | 84.8 | 76.7 | 50.9 | 50.2 |
|  | K=1, X=1 | 95.7 | 89.2 | 59.3 | 58.3 |
|  | K=1, X=2 | 99.0 | 95.9 | 66.8 | 66.3 |
|  | K=1, X=3 | 99.7 | 98.3 | 73.9 | 73.1 |
|  | K=1, X=5 | 99.9 | 99.8 | 85.8 | 83.8 |
|  | K=2, X=0 | 97.8 | 94.2 | 75.5 | 75.6 |
|  | K=2, X=1 | 99.4 | 97.7 | 81.7 | 81.2 |
|  | K=2, X=2 | 99.8 | 99.1 | 86.4 | 86.1 |
|  | K=2, X=3 | 99.9 | 99.6 | 90.4 | 89.8 |
|  | K=3, X=0 | 99.5 | 98.3 | 87.4 | 86.6 |
|  | K=3, X=1 | 99.8 | 99.3 | 91.1 | 90.0 |
|  | K=3, X=2 | 99.9 | 99.7 | 93.9 | 92.9 |
|  | K=3, X=3 | 99.9 | 99.8 | 95.6 | 95.0 |
|  | K=4, X=0 | 99.8 | 99.4 | 93.2 | 92.5 |
|  | K=4, X=1 | 99.9 | 99.7 | 95.3 | 94.6 |
|  | K=4, X=2 | 99.9 | 99.8 | 96.5 | 96.2 |
|  | K=4, X=3 | 99.9 | 99.9 | 97.4 | 97.3 |
|  | K=5, X=0 | 99.9 | 99.7 | 95.7 | 95.4 |
|  | K=5, X=1 | 99.9 | 99.9 | 97.2 | 96.7 |
|  | K=5, X=2 | 99.9 | 99.9 | 98.0 | 97.6 |
|  | K=5, X=3 | 100.0 | 99.9 | 98.6 | 98.2 |
| KPI 2 RSRP accuracy(dB) | Top 1 | 1.5 | 2.5 | 7.7 | 7.7 |
|  | Top 3 | 1.5 | 2.6 | 7.6 | 7.7 |
|  | Top 5 | 1.5 | 2.6 | 7.6 | 7.7 |

Table 4. Simulation results for Scenario 2: Spatial domain prediction with 32 CSI-RS beams in Set A and 8 SSB beams in Set B

|  |  | Case 1 | Case 3 (BB Error only) |
| --- | --- | --- | --- |
| KPI 1: Beam prediction accuracy | K=1, X=0 | 82.6 | 61.4 |
|  | K=1, X=1 | 97.2 | 82.9 |
|  | K=1, X=2 | 98.9 | 93.4 |
|  | K=1, X=3 | 99.3 | 97.4 |
|  | K=2, X=0 | 96.1 | 84.9 |
|  | K=2, X=1 | 98.9 | 95.1 |
|  | K=2, X=2 | 99.4 | 98.0 |
|  | K=2, X=3 | 99.5 | 98.6 |
|  | K=3, X=0 | 98.4 | 92.8 |
|  | K=3, X=1 | 99.3 | 97.8 |
|  | K=3, X=2 | 99.5 | 98.7 |
|  | K=3, X=3 | 99.6 | 99.0 |
|  | K=4, X=0 | 99.2 | 96.8 |
|  | K=4, X=1 | 99.5 | 98.7 |
|  | K=4, X=2 | 99.6 | 99.0 |
|  | K=4, X=3 | 99.7 | 99.1 |
|  | K=5, X=0 | 99.4 | 98.1 |
|  | K=5, X=1 | 99.5 | 99.0 |
|  | K=5, X=2 | 99.6 | 99.2 |
|  | K=5, X=3 | 99.7 | 99.3 |
| KPI 2 RSRP accuracy(dB) | Top 1 | 1.4 | 2.5 |
|  | Top 3 | 1.3 | 2.6 |
|  | Top 5 | 1.3 | 2.6 |

Proposal 3: RAN4 to evaluate whether different UE RX beamforming setting could impact the performances of KPI 1 and KPI 2 and be the root cause of the span among simulations results from different companies.

# Comparison with other dataset

Performance has been also evaluated using vivo’s dataset for scenario 1 (Directory Listing: /ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/Beam/Datasets/R4_115). Comparison can be found in table 5.

Table 5. Simulation results for Scenario 1: spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement (Comparison Keysight’s and vivo’s datasets)

|  |  | Keysight’s dataset (Size 120000 samples) [training samples, validation samples, testing samples] = [90%, 5%, 5%] |  |  |  | Vivo’s dataset (size 120000 samples) [training samples, validation samples, testing samples] = [90%, 5%, 5%] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Case 1 | Case 3 (BB Error only) | Case 3 (RF Error only) | Case 3 (BB + RF Error) | Case 1 | Case 3 (BB Error only) | Case 3 (RF Error only) | Case 3 (BB + RF Error) |
| KPI 1: Beam prediction accuracy (%) | K=1, X=0 | 84.8 | 76.7 | 50.9 | 50.2 | 86.7 | 79.4 | 53.8 | 53.5 |
|  | K=1, X=1 | 95.7 | 89.2 | 59.3 | 58.3 | 93.9 | 88.6 | 61.7 | 60.6 |
|  | K=1, X=2 | 99.0 | 95.9 | 66.8 | 66.3 | 96.2 | 93.8 | 68.5 | 67.2 |
|  | K=1, X=3 | 99.7 | 98.3 | 73.9 | 73.1 | 97.3 | 96.0 | 74.8 | 73.8 |
|  | K=1, X=5 | 99.9 | 99.8 | 85.8 | 83.8 | 98.4 | 98.0 | 85.1 | 84.0 |
|  | K=2, X=0 | 97.8 | 94.2 | 75.5 | 75.6 | 96.0 | 94.2 | 77.4 | 76.2 |
|  | K=2, X=1 | 99.4 | 97.7 | 81.7 | 81.2 | 97.4 | 96.3 | 82.6 | 81.1 |
|  | K=2, X=2 | 99.8 | 99.1 | 86.4 | 86.1 | 98.1 | 97.6 | 86.3 | 85.4 |
|  | K=2, X=3 | 99.9 | 99.6 | 90.4 | 89.8 | 98.5 | 98.3 | 89.7 | 88.8 |
|  | K=3, X=0 | 99.5 | 98.3 | 87.4 | 86.6 | 98.2 | 97.2 | 86.1 | 85.6 |
|  | K=3, X=1 | 99.8 | 99.3 | 91.1 | 90.0 | 98.7 | 98.2 | 89.7 | 88.7 |
|  | K=3, X=2 | 99.9 | 99.7 | 93.9 | 92.9 | 99.0 | 98.8 | 92.1 | 91.6 |
|  | K=3, X=3 | 99.9 | 99.8 | 95.6 | 95.0 | 99.2 | 99.1 | 94.0 | 93.8 |
|  | K=4, X=0 | 99.8 | 99.4 | 93.2 | 92.5 | 98.8 | 98.3 | 91.0 | 90.8 |
|  | K=4, X=1 | 99.9 | 99.7 | 95.3 | 94.6 | 99.2 | 98.9 | 93.5 | 92.9 |
|  | K=4, X=2 | 99.9 | 99.8 | 96.5 | 96.2 | 99.4 | 99.1 | 94.8 | 94.6 |
|  | K=4, X=3 | 99.9 | 99.9 | 97.4 | 97.3 | 99.4 | 99.3 | 96.0 | 96.1 |
|  | K=5, X=0 | 99.9 | 99.7 | 95.7 | 95.4 | 99.2 | 98.8 | 93.9 | 93.8 |
|  | K=5, X=1 | 99.9 | 99.9 | 97.2 | 96.7 | 99.4 | 99.2 | 95.6 | 95.0 |
|  | K=5, X=2 | 99.9 | 99.9 | 98.0 | 97.6 | 99.5 | 99.4 | 96.4 | 96.2 |
|  | K=5, X=3 | 100.0 | 99.9 | 98.6 | 98.2 | 99.6 | 99.5 | 97.1 | 97.2 |
| KPI 2 RSRP accuracy(dB) | Top 1 | 1.5 | 2.5 | 7.7 | 7.7 | 2.3 | 3.6 | 8.6 | 8.3 |
|  | Top 3 | 1.5 | 2.6 | 7.6 | 7.7 | 2.5 | 3.7 | 8.5 | 8.6 |
|  | Top 5 | 1.5 | 2.6 | 7.6 | 7.7 | 2.6 | 3.7 | 8.3 | 8.7 |

Table 5 considers same error, same process, same inference model, same KPI calculation but different ideal RSRP (one coming from Keysight and the other coming from vivo)

# Conclusions

In this contribution, simulation results are provided to evaluate the impact of the measurement error on AI-ML beam management.

The following observations and proposals were made:

Proposal 1: RAN4 to work on a comparison of simulation results with latest system level simulation assumptions vs system level simulation assumptions considering test system setup implementation restrictions to quantify performance requirements.

Proposal 2: Down select SSB to SSB for performance requirements.

Proposal 3: RAN4 to evaluate whether different UE RX beamforming setting could impact the performances of KPI 1 and KPI 2 and be the root cause of the span among simulations results from different companies.

# References

- R4-2511797, “WF on updated simulation assumption for AI based beam management”, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT, 3GPP TSG-RAN WG4 Meeting # 116, August 2025

- TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”

- R4-2514631, “WF on [116bis][112] R19 AI for air interface”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116bis, October 2025

- R4-2514461, RRM Performance Requirements and Simulation Results of AI-ML BM, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #116bis, October 2025

- R4-2514639, Summary on beam management simulation results, vivo, 3GPP TSG-RAN WG4 Meeting #116bis, October 2025