3GPP TSG-RAN WG4 Meeting #117	R4-2520546

Dallas, USA 17th – 21st November 2025

Source: 	Nokia

Title: 					6 MHz A-MPR simulations and proposals

Agenda Item: 			7.1.5.2

Document for:	Approval

# 1	Introduction

We provide simulation results on 6 MHz CBW for additional emission requirements signaled via NS_06 and NS_15. We also provide A-MPR proposals for NS_06.

# 2	Simulation assumptions and scenario

## 2.1	Simulation parameters

- 6 MHz CBW with NRB = 30 as defined in [1]

- Power classes PC2 and PC3

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc

- NS_06 for n85 and n12 and NS_15 for n5

## 2.2	NS_06 mask

We use additional SEM in Table 1 from [1] for n85 and n12.

Table 1: Additional SEM for NS_06

| ΔfOOB (MHz) |  | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 6 | 10 | 15 |  |
| ± 0 – 0.1 | -13 | -15 | -15 | -18 | -20 | 30 kHz |
| ± 0.1 – 1 | -13 | -13 | -13 | -13 | -13 | 100 kHz |
| ± 1 – 5 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| ± 5 – 6 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| ± 6 – 10 |  | -25 | -25 |  |  |  |
| ± 10 – 11 |  |  |  | -25 |  |  |
| ± 11 – 15 |  |  |  |  |  |  |
| ± 15 – 20 |  |  |  |  | -25 |  |

## 2.3	NS_15 mask

We use the additional spurious mask from the table below for n5.

Table 6.5.3.3.20-1: Additional requirements "NS_15"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 851 ≤ f ≤ 859 | -53 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

# 3	Simulation results

## 3.1	NS_06

For NS_06 and PC3 no A-MPR is needed according to our simulations. For PC2 there is one allocation where the MPR is exceeded by 0.1 dB.

## 3.2	NS_15

In this section we provide examples of the NS_15 simulation results. In Fig. 1 we present the A-MPR triangles for QPSK modulation at Fc = 846 MHz, the upper edge of the operating band n5, for both PC2 and PC3. Complete sets of A-MPR triangles at this center frequency are provided in the Appendix.

PC3

PC2

Figure 1: Examples of the simulated A-MPR to satisfy the NS_15 additional spurious emission limit for 6 MHz channel at Fc = 846 MHz using PC2 (upper row) and PC3 (lower row).

Observation 1: According to our simulation results, A-MPR is needed for NS_15 for both PC2 and PC3.

# 4	A-MPR proposals

Currently no A-MPR is specified for PC3 NS_06. According to our simulation results this is also valid for 6 MHz CBW.

Proposal 1: RAN4 shall specify no A-MPR for PC3 NS_06 and 6 MHz CBW.

A-MPR is specified for PC2 NS_06 in the current specification for outer allocations and it is independent of CBW (see Table 6.2.3.32-1 from TS38.101-1 below). According to our results, the currently specified A-MPR is enough also for 6 MHz CBW.

Table 6.2.3.32-1: A-MPR for NS_06 (Power Class 2)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.0 |
|  | QPSK | ≤ 1.5 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3.0 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 3.5 |
|  | 16 QAM | ≤ 3.5 |
|  | 64 QAM | ≤ 4.0 |
|  | 256 QAM | ≤ 6.5 |

Proposal 2: For PC2 NS_06 RAN4 shall specify the same A-MPR for 6 MHz CBW as for other CBWs as specified in Table 6.2.3.32-1 in Ts 38.101-1.

# 5	Conclusions

We have provided simulation results on 6 MHz CBW and NS_06 and NS_15 for power classes PC2 and PC3. We have also provided A-MPR proposals for NS_06 based on our simulation results. We have made the following observations and proposals.

Observation 1: According to our simulation results, A-MPR is needed for NS_15 for both PC2 and PC3.

Proposal 1: RAN4 shall specify no A-MPR for PC3 NS_06 and 6 MHz CBW.

Proposal 2: For PC2 NS_06 RAN4 shall specify the same A-MPR for 6 MHz CBW as for other CBWs as specified in Table 6.2.3.32-1 in Ts 38.101-1.

# 6	References

[1] R4-2515084, Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 -- 6 MHz channel, Samsung, Nokia, Ericsson, Skyworks,Apple, Huawei, CATT, ZTE, Qualcomm

# 7	Appendix: NS_15 A-MPR triangles