3GPP TSG RAN4 Meeting #117							R4-2520550

Dallas, Texas US, 17th – 21st November 2025

Source:	Nokia

Title:	6GR System parameters - Channel Bandwidth

Agenda item:	8.2.3

Document for:	Approval

Introduction

Under 6G system parameters topic following items are to be discussed during the Q4 2025 [1].

Waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types.

At this meeting a sperate agenda have been set for the Channel Bandwidth including max/min CBW, FFT size, SCS, asymmetric CBW, spectrum utilization etc. which is discussed in this paper.

Discussion

Channel Bandwidth

Maximum Channel Bandwidth

Subcarrier spacing, maximum Channel bandwidth (CBW) and the required FFT size are connected to each other even though FFT size itself is an implementation issue. Table 2.1-1 below illustrates the SCS options according to the previous agreements, with a proposal for the maximum CBW (Channel Bandwidth) and the required FFT size (power of two) for different frequency ranges.

- Sub 6GHz [410 MHz-6.425 GHz]: For 6GR we propose to increase the maximum FFT size from 4096 to 8192, which allows the support of 100MHz CBW for FDD (with 15 kHz SCS) and 200MHz CBW (with 30 kHz SCS).

- Around 7GHz [6.425–8.4 GHz]: At RAN1#122bis, it was agreed to assume 30kHz SCS with a maximum channel bandwidth of 400 MHz at the network side for this frequency range. The remaining open issue is the minimum requirement for maximum UE CBW. This is covered by Section 2.1.3,

- Around 15 GHz: As for the around 7GHz, we think that 400MHz maximum channel bandwidth should be supported independently of the applicable SCS to be chosen.

- FR2-1: For 6G, we propose to increase the maximum FFT size from 4096 to 8192 and the maximum channel bandwidth to 800MHz correspondingly, which brings the maximum CBW size to the desired level of improvement compared to 5G NR.

Table 2.1-1: Subcarrier spacing and Maximum CBW Scenarios for 6G

| FR | Range | SCS (kHz) | Max CBW (MHz) | Required FFT size (assuming power of 2)** |
| --- | --- | --- | --- | --- |
| FR1 | Sub 6GHz (410 MHz – 6.425 GHz) | 15 (FDD) | 100 | 8192 |
|  |  | 30 (TDD) | 200 | 8192 |
|  | Around 7GHz (6.425 – 8.4 GHz) | 30 | 400 | 16384 |
| New FR(s) |  |  |  |  |
|  | Around 15GHz† (8.4 - 24.25 GHz) | 30* | 400 | 16384 |
|  |  | 60* |  | 8192 |
|  |  | 120* |  | 4096 |
| FR2-1 | 24.25 – 52.6 GHz | 120 | 800 | 8192 |

* Select one SCS per Range or sub-Range

** This is an implementation issue

† This range can be divided into several sub-Ranges, each with its own SCS.

For Sub 6GHz (410 MHz-6.425 GHz), support up to 200 MHz Channel bandwidth for TDD and up to 100 MHz Channel bandwidth for FDD, respectively.

For 15 GHz (8.4 to 24.25 GHz), support up to 400 MHz Channel bandwidth.

For FR2-1 (24.25 - 52.6 GHz), support up to 800 MHz Channel bandwidth.

How to enable UE to support 400 MHz bandwidth around 7 GHz using 30 kHz SCS

While 5G NR uses intra-band contiguous carrier aggregation for supporting contiguous spectrum of 400 MHz in FR1, 6GR should focus on optimizing a single wideband carrier for contiguous spectrum up to 400 MHz (for both Around 7 GHz and Around 15 GHz).

RAN1 #122bis made the following agreement regarding SCS and maximum CBW at network side

Agreement - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

In the following we discuss different options whether and how to enable UE to support 400 MHz BW.

- Alt. 1: a single 400 MHz carrier at least from scheduling perspective with 16k FFT operation and single RF chain supporting 400MHz

- Alt. 2: a single 400 MHz carrier at least from scheduling perspective with decomposed FFT operation (i.e. 2x8k FFT or 4x4k FFT) and single RF chain (i.e., single RF chain supporting 400MHz)

- Alt 3: a single carrier with UE CBW < 400 MHz operating in the cell with 400 MHz CBW at network side.

In addition to the basic alternatives (Alt. 1 – Alt. 3) there are also alternatives that either do not work at all or at least not as such in the considered scenario (400 MHz CBW at network side)

- Alt 4: This alternative is captured in FL summary as “[006r5] FL proposal/Option 2: A single carrier at least from scheduling perspective with 2 FFT operations and 2 RF chain (i.e., 2 RF chains and each support 200MHz)”. We believe this is not a complete singe carrier scheme. For example, when considering 6GR UL scenario, it is not possible to run DFT-s-OFDM signal using two independent processing chains.

- Alt 5: Intra-band CA with 2x200 MHz (or 4x100 MHz) does not fit to the given scenario. If the cell is configured with 400 MHz CBW, it’s not possible to use carrier aggregation.

On the need for 16k FFT

To understand the current NR design, TS 38.300 specifies that a maximum of 275 Resource Blocks (RBs) can be supported per cell or Bandwidth Part (BWP). Complementing this, RAN4 specifications defines the maximum configuration for 30 kHz SCS as 273 RBs for 100 MHz CBW (TS 38.101-1). Consequently, when operating with 30 kHz SCS and 400 MHz CBW, the system must accommodate up to 1100 RBs (derived from 4x275) and a total of 13200 Resource Elements (REs). To handle these 13200 REs, the smallest power-of-two FFT size required is 16384, commonly known as a 16k FFT.

How the UE supports the 400MHz CBW (i.e. using a single 16k FFT or multiple smaller sized FFTs) is up to UE implementation. Intra-band CA supporting aggregated bandwidth of 400 MHz can be seen as one reference scenario. It can be made using carrier specific FFTs, i.e. 4x 4k FFT (4x100 MHz) or 2x 8k FFT (2x200 MHz).

In the following we consider FFT implementation for Alt. 1 and Alt. 2 in more details.

Alt. 1: 16k FFT

Let us consider the inverse fast Fourier transform (IFFT) implementation, i.e., transmitter (Tx)-side OFDM processing. FFT implementations on the receiver (Rx) side can be realized in same manner. IFFTs of size 16383 may be realizing by using conventional IFFT factorizations, e.g., by first extending the IFFT of size N/4=4096 with three additional copies of the baseline IFFT (of size 4096) and then adding two so-called butterfly stages as illustrated in Figure 5. This approach increases the memory needed by the IFFT implementation by factor of four. The arithmetic increase is less significant as same arithmetic may be used for realizing all the baseline IFFTs and the combining butterfly arithmetic is otherwise quite simple.

Figure 5: High-level block diagram for decimation-in-time IFFT algorithm combining four IFFTs of size N/4= into IFFT of size N.

For traditional IFFTs, the pruning of the arithmetic, enabled by the fact that only some of the inputs are non-zero does not typically give high saving since the pruning effectively discards only some arithmetic in the very first butterfly stage of the IFFT of size N/4. Also, as the input samples are distributed into every fourth input bin of the IFFTs of size 4096 makes the utilization of sparse inputs challenging (e.g. when only few RBs out of the 1100 RBs are active).

In practice, additional pulse-shaping in terms of windowing or filtering is needed to mitigate the spectral leakage of the generated OFDM or DFT-s-OFDM waveforms.

Observation: (When compared to Intra-band contiguous CA with 2x8k FFT) the added complexity to support 16k FFT is relatively small.

Alt. 2: Decomposed processing:

So-called decomposed processing is an effective way for realizing OFDM waveforms by using small OFDM transforms. The basic principle in the Tx side is to divide the OFDM IFFT of size N into smaller transforms of size N/M as illustrated in Figure 5. The OFDM subband waveforms are generated using these smaller transform sizes and the CP of length NCP/M is inserted into resulting time-domain subband waveforms. Subband waveforms are then combined using either the time-domain or frequency-domain interpolation and combining. This approach is transparent to the baseline OFDM Rx processing with OFDM FFT size of N. In Rx side, same approach may be used by diving the OFDM waveform into narrower subbands by filter-bank processing and then taking the OFDM FFT of size N/M from these subband waveforms after the CP removal.

Figure 6: In transmitter-side decomposed processing, subband OFDM waveforms are generated by shorter IFFTs and these subband are combined with filter bank processing.

The filter-bank processing used by this approach may be configured to provide tight channel filtering for limiting the spectral leakage. In other words, same filtering as needed by the conventional Tx processing for reducing the spectral leakage may be used for combining the subband waveforms as well.

For supporting 400 MHz CBW with 1100 RBs, the OFDM IFFT of size 16384 may be divided into four OFDM IFFTs of size 4096, thereby enabling the utilization of the same IFFT module with maximum size of 4096 as needed for the NR. However, decomposed processing may be carried out by using decomposed size as small as 128 (since for this size the CP lengths are still integers). In this case, for partial-load scenarios, the OFDM processing needed by the non-active subbands may be easily halted thereby effectively reducing the power consumption.

The IFFT part of the DFT-s-OFDM waveform processing may also be realized by same approach, however, the forward transform cannot be partitioned into smaller parts by using this approach, i.e., conventional factorizations needs to be used for realizing the needed forward transforms.

Decomposed processing is an implementation friendly way to support 16k FFT

Compared to single FFT, decomposed processing can provide improved spectral properties for the processed signal with minor degradation in EVM performance.

Alt. 4: Operation with N separate FFTs and N RF chains

As said, in our understanding this is not a complete single-carrier scheme. At least for DFT-s-OFDM, it is intuitive to understand that DFT-s-OFDM using two (or more) separate IFFTs does not maintain the single-carrier properties of the transmitted signal. The same applies to UL transmission in general. For CP-OFDM, there will be discontinuity in the boundary of the subbands with very high impact to the EVM of the generated waveform. We estimate that Alt. 4 fails to meet the existing UE RF requirements defined for UL Tx. Based on that we propose to deprioritize Alt 4 for the 6GR UL scenario.

While Alt 4 has several issues in UL, it can be considered as one candidate solution for UE Rx (a single carrier at least from scheduling perspective). In order to have a contiguous resource grid, two (or more) sub-bands need to overlap in frequency. With this assumption the FFT processing for UE Rx might be quite similar to intra-band contiguous CA. However, Alt 4 is a new scenario from non-idealized point of view. There can be differences between sub-bands in frequency synchronization, time synchronization, phase characteristics, power characteristic. Etc. All these impact to Rx performance numbers. Hence, whether Alt 4 can be considered as a single carrier at least from scheduling perspective RAN4 work is needed to study the feasibility of DL operation with N separate FFTs and N RF chains.

Figure 6: Rx side realization for (almost) single carrier at least for scheduling perspective.

We believe that operation with N separate FFTs and N RF chains fails to meet the existing UE RF requirements defined for UL Tx

RAN4 studies are needed to study the feasibility of DL operation with N separate FFTs and N RF chains

Minimum requirement for max. UE CBW

The RAN1 #122bis agreement establishes a 400 MHz maximum Channel Bandwidth (CBW) and 30 kHz SCS at the network side, implying a 16k FFT scenario for 6GR base stations. This leads to two critical questions:

- Q1: Managing diverse UE capabilities, if the minimum requirement for maximum UE CBW is less than 400 MHz: how should different UE capabilities be managed within a 400 MHz cell?

- Q2: Minimum CBW requirement for UE: What is the minimum required maximum CBW for UEs operating in a 400 MHz cell?

Managing diverse UE capabilities:

In NR, all eMBB UEs support operation according to full BS CBW (i.e. up-to 100 MHz CBW for FR1) while Reduced Capability (RedCap) UEs operate with a narrow UE CBW (up to 20 MHz) within the wideband BS CBW. It has been seen as necessary to operate RedCap UEs not only around the CD-SSB but anywhere within the BS CBW. The related load balancing support was defined in Rel-18 with two separate feature groups covering scenarios where RRC configured BWP (and CBW) is outside the initial BWP comprising CD-SSB:

- FG 28-1: The UE needs NCD SSB if the CD-SSB is outside of the active DL BWP

- FG 28-1a: The UE can use the CD-SSB outside of the active BWP as it is anyway within its receiver BW and the UE can sync to it.

It is noted that the so-called load balancing scenario where CD-SSB is outside the UE’s CBW (or active BWP) is relevant also for 6GR, not only for lower-tier, narrowband UEs but also for UEs having (maximum) CBW smaller than 400 MHz. For that reason, there is a need for load balancing solution in 6GR to facilitate SSB measurements when CD-SSB is outside the UE’s CBW. In order to streamline the specs and UE capabilities compared to NR, 6GR should define only one (unified) load balancing solution, which should be a mandatory feature for all UEs from 6G day one.

Load balancing support (i.e. synchronization maintenance and measurements when CD-SSB is outside the UE’s CBW) is needed in the cases when UE CBW is smaller than 400 MHz.

Strive for a unified load balancing solution to facilitate SSB measurements when CD-SSB is outside the UE’s CBW.

Figure 7a below shows an example where a 400 MHz cell is configured with a 20 MHz cell-defining SSB (+ associated initial bandwidth part) at a predefined frequency location. It also shows the extreme CBW configuration options for 100 MHz and 200 MHz UEs, labeled as "low" and "high" and defined in a way that the CD-SSB overlaps the UE's CBW in frequency. Figure 7b shows the same example with one additional NCD-SSB to support load balancing

When comparing UE CBW size options (100 and 200 MHz), it can be noted that for 100 MHz CBW, less than 50% of the RBs of the 400 MHz carrier overlap the 100MHz UE CBWs that include the CD-SSB. This indicates that at least two NCD-SSBs would be needed to utilize all RBs of the 400 MHz carrier. At the same time, only one NCD-SSB would be enough for scenario with 200 MHz UEs. This indicates that the smaller is the UE CBW the more load balancing is needed, which increases the SSB overhead and the complexity.

Observation 2-x: The smaller is the UE CBW the more load balancing is needed increasing the complexity.

| - without load balancing | b) with load balancing based on one NCD-SSB |
| --- | --- |

Figure 7. UEs with different CBW capabilities on 400 MHz CBW at network side

Minimum CBW requirement for UE

Ideally, mandatory 400 MHz CBW support for UEs from the 6G Day 1 would be highly beneficial kHz for the appropriate bands.

- Spectrum: total amount of spectrum in around 7GHz range is almost 2GHz (i.e. 700 MHz in Upper 6 GHz, and 1275 MHz in above 7.125 GHz, respectively). This means 400 MHz CBW at network side may not be a corner case.

- Performance: support for a wider CBW is a performance issue: when compared to intra-band CA with the same aggregated bandwidth (e.g. 2x200 MHz), wideband carrier is better in all metrics – e.g. UL coverage, throughput, control channel overhead, complexity.

- UE performance when 400 MHz CBW is configured at network side: CBW size smaller than 400 MHz will reduce certain KPIs, e.g. peak data rate linearly by factor (UE CBW size)/400 MHz.

If it’s not possible to reach such an agreement, there should be only one, and sufficiently high minimum requirement for maximum CBW for UEs. This would ensure that 6GR UEs can deliver significantly improved user experience compared to 5G NR, without fragmenting the wideband carrier too much. One pragmatic compromise might be to define support for 400 MHz CBW as mandatory for UEs for DL only. In this scenario minimum requirement for the maximum UL CBW could be smaller, such as e.g. 200 MHz.

We advocate that all UEs should support at least 200 MHz CBW from day one. For example, higher tier eMBB and especially FWA UEs could be supporting up to 400MHz CBW. This means that regardless of the minimum requirement for maximum UE CBW, 6GR needs to support UE CBW of 400 MHz with 30 kHz SCS.

On the minimum requirement for maximum UE CBW - define only one minimum requirement for UEs for cells configured at the network side with 400 MHz CBW

- Consider if the one minimum requirement is defined separately for DL and UL

- FFS: Minimum requirement is 400 MHz or 200 MHz

Support UE CBW of 400 MHz with 30 kHz SCS, and single RF chain (per antenna port)

Channel Bandwidth between Maximum and Minimum

In NR the options between maximum and maximum channel bandwidth started out with an intentionally limited number of options and a clear objective to have certain step sizes. However, in the duration of NR more and more options were added which have complicated the adaptation of these since they originally were not intended to be introduced. Hence, a different methodology for options between maximum and maximum channel bandwidth is discussed further in relation to Irregular/Flexible channel bandwidth (Section 2.4.2).

Numerology

Table 2.1-1 above illustrates the SCS options according to the related agreement made in RAN1 #122 and RAN1 #122bis , with a proposal for the maximum CBW (Channel Bandwidth) and the required FFT size (power of two) for different frequency ranges. In the following we elaborate on the justifications for each choice of combination of SCS and maximum CBW sizes. A goal is to define SCS (for PxSCH and PxCCH) for each 6GR scenario as a cell and band-specific parameter.

FFT size

The FFT size itself as such is an implementation issue. However, choosing FFT sizes as powers of two enables faster, more energy-efficient, and easier-to-optimize implementations in both software and hardware. Furthermore, it provides a good FFT utilization for scenanrios with 2x ∙100 MHz. Finally it should be noted that even though the FFT size as such is an implementation issue, the 6GR scenario with the maximum number of RBs/cell (and the related maximum number of REs/cell) can be seen as a parameter indicative of the maximum FFT size for 6GR. For example, from RAN1 & RAN2 point of view, 275 RBs/cell is the maximum in NR. This corresponds to 3300 REs and can be made with 4k FFT. As larger maximum channel bandwidths for a given SCS is to be supported for 6GR, there is a need to increase the required FFT size for 6GR compared to the 4k FFT of NR, as can be seen from Table 2.3-1 above. Overall, we suggest adopting 8k FFT as baseline operation. But it should be noted that 16k FFT would be required in case 400MHz channel bandwidth should be supported also with 30kHz SCS.

8k FFT is a baseline for 6GR.

16k FFT needed to support 400 MHz CBW with 30 kHz for the appropriate bands.

SCS

As had been visible from the discussions during RAN1#122, clearly the discussions about SCS are specific for different ranges of carrier frequencies.

For Sub 6GHz (410 MHz - 6.425 GHz), we propose this range to remain aligned with current NR deployments using 15kHz SCS for FDD and 30kHz SCS for TDD aligned with the RAN1#122 agreement. This is reasonable given that MRSS requires that SCS is aligned with NR. We don’t see a need for any additional SCS to be supported for sub 6GHz.

For Around 7GHz (6.425 – 8.4 GHz), the RAN1#122bis decision was: “RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz.” We don’t see a need for any additional SCS to be supported for Around 7 GHz.

For Around 15 GHz (8.4 - 24.25 GHz), [30 60 120] kHz SCSs can be studied based on the RAN1 agreement cited above. We think that further evaluations are needed before making the decision on SCS for Around 15 GHz range. The studies on the applicable SCS should consider a possibility for further split of this frequency range in two sub ranges to align the operation with the Around 7GHz band for the lower part and FR 2-1 for the upper part of this frequency range.

For FR2-1 (24.25 - 52.6 GHz), the RAN1 agreed 120kHz SCS is aligned with current NR deployments in this range.

Consider SCS values shown in Table 2.3-1 as bold for RAN4 evaluations for Sub-6GHz, Around 7GHz and FR2-1.

Further studies in RAN1 for the frequency range around 15 GHz (8.4 - 24.25 GHz) will be needed on the applicable SCS including a potential further split of this frequency range to align the operation with the Around 7GHz band for the lower part and FR 2-1 for the upper part of this frequency range.

Spectrum Utilization

Spectrum Utilization

The maximum transmission bandwidth configuration for FR1 in TS 38.101-1 is shown in the following table, including the latest addition of 7 MHz channel bandwidth.

Note: 60 kHz scs may not be specified for 6GR.

| SCS (kHz) | 3 MHz | 5 MHz | 7 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 35 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | N/A | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Spectrum utilization of some NR bandwidths looks slightly more conservative than its next smaller and larger channel bandwidths, such 7 MHz and 45 MHz in 15 kHz SCS, for example. RAN4 needs to study 6GR SU with the aim of reducing the needed guard and should decide what is the appropriate SU value given that the spectrum is one of the costliest investment.

RAN4 should study the spectrum utilization for 6GR with the aim of improving it compared to 5G NR.

Minimum Guard Band

NR minimum guard bands are calculated using the equation, (CBW x 1000 (kHz) - RB value x SCS x 12) / 2 - SCS/2 as described in TR 38.817-01 clause 4.5.2.1. CBW is the channel bandwidth in MHz, RB value is the maximum transmission bandwidth presented above. SCS is the subcarrier spacing. The minimum guard bands for FR1 are shown in the following table and figure.

| SCS (kHz) | 3 MHz | 5 MHz | 7 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 142.5 | 242.5 | 342.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 505 | N/A | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | N/A | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

One issue we have observed in some NR network deployments is that the minimum guard band of UE specific channel bandwidth is larger than the minimum guard band of SIB1 channel bandwidth (i.e., the system bandwidth configured in BS), even though the UE specific channel bandwidth is smaller than the SIB1 channel bandwidth.

For example, the guard band for 30 MHz is larger than 40 MHz in the above table (where the line in the figure has negative slope). UE transmitting with full 40 MHz SIB1 channel bandwidth needs 552.5 kHz (for 15 kHz SCS) as minimum guard band to meet the emission requirement defined for the 40 MHz channel bandwidth. However, UE configured with UE specific channel bandwidth of 30 MHz aligned at the edge of the 40 MHz SIB1 channel bandwidth require a wider guard band of 592.5 kHz and therefore the emission requirements at the edge of the channel (i.e., SIB1 carrier bandwidth) is not guaranteed in the current standards.

To resolve this issue one network had to tweak the centre frequency of the 40 MHz BS channel (i.e., SIB1 carrier bandwidth) by shifting 40 kHz to secure an enough guard band for the UE specific channel bandwidth placed at the edge. The exceptional BS channel raster was introduced to NR band n28 in the legacy BS specifications.

This characteristic is against UE radio implementations and is an artifact coming out of the above equation. Technically, the guard band to meet emission requirement such as SEM and spurious emissions should be smaller if the transmission bandwidth is smaller. Such a tweak to the channel raster should never be needed. A monotonically increasing guard band with channel bandwidth will solve this issue. However, spectrum being one of the costliest resources should not be wasted on guard band, thus an effort should be made to reduce the required guard band.

A monotonically increasing guard band with channel bandwidth can resolve deployment issues caused due to different UE specific channel bandwidth and system bandwidth. This, however, wastes spectrum.

Figure 3: RAN1 agreement on maximum channel bandwidth during RAN1#122bis

In NR, minimum guard band for a given transmission bandwidth configuration is currently defined as a table, even though an equation for calculating the guard band is present. NR was specified for only a limited set of channel bandwidths, which were enhanced gradually over time with addition of regular and irregular channel bandwidths. This led to lot of repeated work and optionality issues. In 6G, our expectation is that all of the channel bandwidths needed will be specified from the start to avoid same repeated work as for NR. Therefore, for 6G, the granularity at which the channel bandwidths should be specified has to be studied and defined during the normative phase. Given, the maximum channel bandwidth can be as large as 400 MHz, as captured in the agreement from RAN1, it is sensible to have a unified framework from the start in order to define the maximum transmission bandwidth and minimum guard band.

Spectrum utilization is a key parameter that plays a role in determining the maximum transmission bandwidth and minimum guard band. Nevertheless, it is currently not obvious from the specifications, and one has to manually calculate what is the actual utilization for a given configuration. Given spectrum is being one of the costliest resources, it is sensible to have its utilization at the center stage and being made as an independent parameter from which other parameters are derived. Further, it will be helpful to have different utilization for different SCS and channel bandwidth ranges.

Use spectrum utilization as an independent parameter together with the maximum transmission bandwidth to derive the minimum guard bands. Study how to utilize it for different channel bandwidth ranges and numerology.

Irregular/Flexible channel bandwidth

Background for Flexible BW

In 5G the applicable FR1 channel bandwidths are listed in tables in 38.104 for BS and 38.101-1 for UE (it should be noted RAN4 starts to define requirements also on 6MHz channel bandwidth in Rel-20 NR). In principle all channel bandwidths are mandatory for a UE unless indicated by note 4 that the channel bandwidth is optional. In practice all UEs may not support all channel bandwidths but from 3GPP point of view those UEs are not compliant to specification.

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n1 | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
| n26 | 15 | 34 | 5 | 10 | 15 | 20 | 25 | 3 |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |
| n90 | 15 |  | 54 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2.4.1-1: Channel bandwidths for each NR band defined in TS 38.101-1 for UE.

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
| n1 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
| n26 | 15 | 3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |  |  |  |  |
| n90 | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |

Table 2.4.1-2: BS channel bandwidths and SCS per operating band in FR1 defined in TS 38.104.

In first 5G release, Rel-15 the list of possible channel bandwidths was not as comprehensive as it is now. For UE, 3, 35, 45, 70 and 90 MHz channel bandwidths were not specified at all. For BS, 70 and 90 MHz bandwidths were part of the initial release but 3, 35 and 45 bandwidths were not. However, 70 MHz and 90 MHz were added by CAT-F correction CRs already during later phase of Rel-15. Addition of 90 MHz bandwidth required also some RAN2 changes to signalling in Rel-15.

Then 3, 35 and 45 MHz had dedicated WIs to add the support. Adding 3 MHz in Rel-18 was very complicated as it required even RAN1 changes due to PBCH and PDCCH redesign (=puncturing support) and RAN4 needed to define the new sync raster entries. 3 MHz is still optional even in Rel-19 for some frequency bands.

In Rel-19 RAN4 has a basket WI called “Adding channel bandwidth(s) support to existing NR bands and CA/ENDC combinations in REL-19” to add missing channel bandwidths that have become relevant to some operators to an existing band. These channel bandwidths are listed in WID for FR1: {3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz, 25 MHz, 30 MHz, 35 MHz, 40 MHz, 45 MHz, 50 MHz, 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz}. Note that 3 MHz is only possible for bands below 1 GHz. Furthermore, in Rel-19 there was also a dedicated WI to add support for 7 MHz for both UE and BS “NR_FR1_7MHz_BW”. There is also ongoing Rel-20 WI to add support for 6 MHz for both UE and BS.

In addition to above work which is visible in UE and BS specifications there has been a lot of effort done under a study item on how to handle irregular channel bandwidths which are not multiple of 5 MHz and are smaller than 20 MHz. These are typically spectrum chunks that have become available when previous radio systems such as TV are not using the spectrum anymore (see also table 2.4-1 from TR 38.844). This study is captured in TR 38.844 [2].

| Band (s) | Channel Bandwidth(s) |
| --- | --- |
| n5 | 7, 11 MHz |
| n12, n85 | 6, 12 MHz |
| n26 | 7 MHz |
| n28 | 13 MHz |
| n29 | 6, 11 MHz |

Figure 2.4-1: Summary of operators’ input for irregular channel bandwidth

From this TR 38.844 we can find a figure 8-1 below. This figure shows one way to achieve irregular channel bandwidth without major specification changes. In this case BS operates with irregular CBW (13 MHz in figure 2.4-1) and UEs operate as normal UE using legacy bandwidths (10 MHz in figure 2.4-2).

Figure 2.4-2: Irregular channel bandwidth

Overlapping CA: the irregular bandwidth is handled by two overlapping component carriers (CCs) with NR channel bandwidth defined in Rel-17. It is network responsibility to prevent collisions between the different component carriers.

Overlapping UE channel BW from network perspective: network supports the irregular bandwidth (either by a single carrier or by two overlapping carriers) while each UE operates in an existing lower regular NR channel bandwidth

Flexible bandwidth for 6G

As we have discussed in previous chapter it is a major effort to add new channel bandwidths to specification after the initial release. This is natural as it requires UE and BS re-designs and updates on multiple specifications in RAN1, RAN2 and RAN4.

Therefore, it would be very beneficial if all channel bandwidths between minimum and maximum channel bandwidth were possible from specification perspective and those would be mandatory for an eMBB UE from day 1 of 6G. The channel bandwidth granularity could increase with the increasing CBW size. Support for the new CBW sizes with increased granularity should be a mandatory requirement for all 6GR UEs for entire channel bandwidth range for 6G i.e. from 3 MHz to 200/400 MHz for the new native 6G bands in 6 – 7 GHz. In case of LPWA UEs the similar flexibility would be applied but the maximum channel bandwidth would be narrower than for eMBB UE.

It is acknowledged that the maximum bandwidths in question are very wide which would mean that if granularity is 1 MHz then there would be 198/398 possible channel bandwidths and if granularity is 1RB then number of possible channel bandwidths is further increased. This potentially means that testing efforts and design efforts would increase although the way the specifications are written can possibly be do so that these issues are mitigated. Nevertheless, it may not be possible to have full flexibility due to increase on testing and design work on channel bandwidths for full 3 – 200/400 MHz channel bandwidth range. If flexibility needs to be constrained, then the channel bandwidths could be split into ranges which have different granularity. One possibility is presented below. Furthermore, in this table we have presented a way to go from desired SU to number of Nrb and size of a guardband for particular channel bandwidth. Note that SU numbers in table as just examples not actual proposals for 6GR.

Table 2.4.2-1: Different bandwidth granularity ranges

|  | Range 1 | Range2 | Range 3 | Range 4 | Range 5 |
| --- | --- | --- | --- | --- | --- |
| SU | 90% | 95% | 99% | 99% | 99% |
| Nrb | floor(SU*CBW/180kHz) |  |  |  |  |
| GB | (CBW - Nrb*180)/2 |  |  |  |  |
| Channel bandwidths | 3, 5 | 5 < CBW ≤ 20 MHz | 20 < CBW ≤ 100 MHz | 100 < CBW ≤ 200 MHz | 200 < CBW ≤ 400 MHz |
| Granularity | N/A | 1 MHz | 5 MHz | 10 MHz | 20 MHz |
| Optional vs. mandatory | mandatory | mandatory | mandatory | mandatory | Mandatory/optional |

*) Aligned with minimum CBW size in Table 2.2-1.

Range 1 and 2 in Table 2.4.2-1 i.e. 3,5,6-20 MHz are the original LTE channel bandwidths and represent the irregular channel bandwidth cases in 5G where smallest granularity is needed. These bands are typically if not entirely, all sub 1 GHz FDD bands and have been re-farmed from other radio systems such as 2G, TV or GSM-R services. Having reasonably high granularity for the narrowband scenarios would enable optimal utilization of the most valuable frequency assets without the added complexity and reduced performance introduced by the overlapping CA approach.

Range 3 in Table 2.4.2-1 i.e. 20 -- 50 MHz channel bandwidths are marked with 5 MHz granularity. These bandwidths are typically normal MNO spectrum and have been sold or licenced with minimum of 5 MHz chunks. Another more flexible option would have Range 3 to go up to 100 MHz. The difference to 5G even with more flexible option 2 is not as big as with range 2 if one compares with tables 2.4.1-1 and 2.4.1-2. 5G specs are however lacking 55, 65, 75, 85 and 95 MHz channel bandwidths compared to 6G proposal in table 2.4.2-1. Maybe bigger difference to 5G would be that all channel bandwidths in table 2.4.2-1 should be mandatory from day 1. One can say that there can be irregular bandwidth options also above 20 MHz CBW. However, these are relatively rare cases (c.f. <20 MHz CBW). And on the other hand, as the CBW size increases the relative size of uncovered portion reduces (e.g. 30/33 MHz =91% vs. 18/15 MHz= 83%)

Range 4 in Table 2.4.2-1 would go beyond current 5G specs having channel bandwidth of more than 100 MHz and up to 200 MHz. Possible granularity could be 10 MHz. This is important range as the intent is to specify atleast 200 MHz as minimum maximum channel bandwidth to the UE at least for native 6G bands around 6-7 GHz which makes 6G UE more capable compared to 5G UE.

Range 5 in Table 2.4.2-1 reflects the discussions on maximum possible FR1 channel bandwidth for 6G. In this context 400 MHz is often quoted as maximum. Note that in RAN1#122bis RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz.

RAN4 to discuss how to make channel bandwidth definition more flexible and future proof in 6GR compared to NR

Conclusion

In this contribution we present our view on 6GR General RF and UE RF. The following observations have been made:

Proposal 1: For Sub 6GHz (410 MHz-6.425 GHz), support up to 200 MHz Channel bandwidth for TDD and up to 100 MHz Channel bandwidth for FDD, respectively.

Proposal 2: For 15 GHz (8.4 to 24.25 GHz), support up to 400 MHz Channel bandwidth.

Proposal 3: For FR2-1 (24.25 - 52.6 GHz), support up to 800 MHz Channel bandwidth.

Observation 1: Decomposed processing is an implementation friendly way to support 16k FFT

Observation 2: Compared to single FFT, decomposed processing can provide improved spectral properties for the processed signal with minor degradation in EVM performance.

Observation 3: We believe that operation with N separate FFTs and N RF chains fails to meet the existing UE RF requirements defined for UL Tx

Observation 4: RAN4 studies are needed to study the feasibility of DL operation with N separate FFTs and N RF chains

Observation 5: Load balancing support (i.e. synchronization maintenance and measurements when CD-SSB is outside the UE’s CBW) is needed in the cases when UE CBW is smaller than 400 MHz.

Observation 6: Strive for a unified load balancing solution to facilitate SSB measurements when CD-SSB is outside the UE’s CBW.

Proposal 4: On the minimum requirement for maximum UE CBW - define only one minimum requirement for UEs for cells configured at the network side with 400 MHz CBW

Proposal 5: Support UE CBW of 400 MHz with 30 kHz SCS, and single RF chain (per antenna port)

Proposal 6: 8k FFT is a baseline for 6GR.

Proposal 7: 16k FFT needed to support 400 MHz CBW with 30 kHz for the appropriate bands.

Proposal 8: Consider SCS values shown in Table 2.3-1 as bold for RAN4 evaluations for Sub-6GHz, Around 7GHz and FR2-1.

Observation 7: Further studies in RAN1 for the frequency range around 15 GHz (8.4 - 24.25 GHz) will be needed on the applicable SCS including a potential further split of this frequency range to align the operation with the Around 7GHz band for the lower part and FR 2-1 for the upper part of this frequency range.

Proposal 9: RAN4 should study the spectrum utilization for 6GR with the aim of improving it compared to 5G NR.

Observation 8: A monotonically increasing guard band with channel bandwidth can resolve deployment issues caused due to different UE specific channel bandwidth and system bandwidth. This, however, wastes spectrum.

Proposal 10: Use spectrum utilization as an independent parameter together with the maximum transmission bandwidth to derive the minimum guard bands. Study how to utilize it for different channel bandwidth ranges and numerology.

Proposal 11: RAN4 to discuss how to make channel bandwidth definition more flexible and future proof in 6GR compared to NR

- 4	References

R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4#116, RAN4 Chair