3GPP TSG RAN4 Meeting #117							R4-2520551

Dallas, Texas US, 17th – 21st November 2025

Source:	Nokia

Title:	6GR System parameters - Device types

Agenda item:	8.2.5

Document for:	Approval

Introduction

Under 6G system parameters topic following items are to be discussed during the Q4 2025 [1].

Waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types.

At this meeting a sperate agenda have been set for the Device types discussion which is discussed in this paper.

WF from last meeting

Target of RAN4 study on device types differentiation in 6GR is to determine whether detailed RF and BB implementation feasibility and constrains would lead to distinct requirements for different device types. In that context RAN4 assumes specifically number of antennas, CBW, power class and supported modulation order per frequency range for four device types: IoT, Wearable, Smartphone and FWA devices, as agreed in [2]:

- RAN#116 Agreement - The following aspects could be further studied - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range. - Study on the NW impact should also be considered - Study how to better support variety devices from RAN4 perspective - E.g. differentiate devices, use cases, features - Other aspects are not precluded - Collaborate on the framework with other WGs: - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

Discussion

Device types

The 6GR SID objectives related to device types are collectively aiming to achieve an integrated solution in 6GR that supports a framework for device types that is extensible and efficient, and one that overcomes issues faced in previous generations.

For a scalable design for diverse device types, the coordination needs to specifically consider how to characterize 6G devices to determine whether they differ significantly enough to justify defining few distinct device types. For instance, in 5G, eMBB devices were originally (Rel-15) required to support by default 4Rx in most bands, while RedCap devices were originally (Rel-17) only required to support 2Rx or 1Rx (Rel-18). Such characteristics impact both the initial access and RF/PHY characteristics, coverage planning for deployments and UE configuration/capabilities. However, both: 5G eMBB and 5G RedCap devices support default initial NR access scheme, and some versions of 5G eMBB and 5G RedCap devices can even support the same number of RF chains (Rel-15 eMBB device with fallback to 2Rx for power savings and Rel-17 RedCap device) or power class (PC3). On the other hand, highly performing devices (CPEs) became a unique use case with distinct RF and power characteristics, that stand out from all the other devices.

To guide a proper and scalable device classification, a debate on diverse 6G device types that took place in RAN#109 considered several parameters that will be of significance in context of the potential device types categorization in 6G [3].The guideline includes endorsement of RAN1#122 agreements on RF and Baseband bandwidth, and further directs RAN1 and RAN4 to continue studying both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective.

Observation 1: RAN1 and RAN4 continue progress on the smallest maximum supported RF and BB bandwidth, which will provide a useful basis for RAN for assessing physical layer implications for diverse device types.

Channel bandwidth is one of the key parameters, whose relevance spans both RAN1 and RAN4, therefore necessitating coordination between the two WGs. The need for coordination on the parameters will be further emerging across several areas driven by the generic goal of designing the 6G radio (e.g. CBW, waveform design, FFT, synchronization raster, modulation schemes, initial access procedures etc.). However, for the purpose of advancing the understanding and decision-making within RAN specifically for device types, we propose focusing the discussion on three RAN4-relevant parameters at this stage:

RAN4 to study and provide input to RAN on how the three factors: number of Rx antennas, number of Tx antennas and Maximum Output Power need to differ for 6G eMBB, FWA, Wearable and massive IoT devices.

Number of Rx and Tx

5G was envisioned with two receivers as was the case with LTE. Currently four Rx is mandatory for only a selected bands with frequency of operation higher than 2GHz, n7, n38, n41, n48, n77, n78, n79, and n104 to name a few. However, with the advent of higher frequency bands, foldable form factors and new device technologies, the number of receivers that can fit in the eMBB device category has increased. Thus, the default number of receivers envisioned for 6GR might need to be higher than two/four for some scenarios. Further, with the upcoming spectrum allocation in upper 6GHz range, it would be hard to maintain desired coverage with a minimum of two/four receivers only. Thus, for these new bands a higher number of receivers might be a straightforward need, asking a higher baseline for eMBB devices (e.g. six receivers). Given the larger footprint and power budget for the FWA devices, it might be even beneficial to have a possibility to have more receivers for FWA devices. However, the FWA devices can accommodate larger antennas, which are not feasible for IoT or Wearable devices. Wearables and IoT devices have strict size and hardware limitations, limiting receiver antennas count.

Similarly, there are practical constraints for TX chains. Currently 5G standard support TX chains up to 8TX. However, RAN4 has specified requirements only up to 4TX. Work items are made with an assumption that UEs are supporting up to 3TX and FWA type of devices can support up to 4TX. For 6G, a wide range of frequency bands that is considered to meet the ambitious 6G design goals, envisions the introduction of a new bands in 7GHz. With its introduction of  distinct requirements on RF, antenna design and baseband processing might be imposed. This is: not all devices will be able to support common upper limit of receivers or transmitters due to hardware constraints and/or size limitations.

Therefore, we suggest discussing minimum required values for Rx and Tx chains per candidate 6G frequency ranges, with the following table as a starting point when defining RAN4 receiver and transmitter requirements:

| Band Device Type | Low ~1GHz |  | Mid ~3.5GHz |  | High ~7GHz |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rx | Tx | Rx | Tx | Rx | Tx |
| IoT | 1 | 1 | N/A | N/A | N/A | N/A |
| Wearable | 1 | 1 | 2 | 1 | N/A | N/A |
| Smartphone | 2 | 1 | 4 | 2 | 6 | 2 |
| FWA | 4 | 2 | 8 | 2 | 8 | 2 |

Consider four receivers as mandatory baseline for bands re-farmed from 5G for eMBB UE for RAN4 requirement work.

Consider six receivers as mandatory baseline for new 6GR bands for eMBB UE for RAN4 requirement work.

Consider eight receivers as mandatory baseline for new 6GR bands for FWA UE for RAN4 requirement work.

Consider eMBB UE and FWA UE devices support two TX chains as mandatory baseline for RAN4 requirement work.

Default power class

Power Class is another key parameter across releases that has been implicitly (e.g. for smartphone) or explicitly determining a UE type (e.g. HPUE, CPE, FWA). RAN4 is using the parameter to set RF requirements. Typically, eMBB and FWA devices that require and are able to achieve higher throughputs, trend towards increasing Power Class values. For instance, originally default Power Class in 5G for eMBB device: 23dBm trended towards 26dBm for FWA devices in Rel-18. We observe also there has been some generalization tendency in RAN4 to apply higher Power Classes across diverse device types in 5G. However, on the contrary RedCap devices, that were originally tied to Power Class 23dBm (Rel-17),  got traction of further simplifications to a default Power Class 20dBm in Rel-18.

Similarly to consideration of the number of RF chains, the wide range of frequency bands envisioned for 6G and the current ambiguity on transmit power differentiation or generalization across devices, will impose further challenges These challenges affect the ability to recognize actual support of a device capability. Without studies on the need to differentiate it will be difficult to assess whether a device conforms to the expected performance requirements or deviates.

Therefore, we suggest discussing minimum required values for Maximum Output Power per candidate 6G frequency ranges, with the following table as a starting point:

|  | Default power class Output Power [dBm] |
| --- | --- |
| IoT | 23 |
| Wearable |  |
| Smartphone | 26 29 (2x26 dBm) |
| FWA |  |

RAN4 to study how Maximum Output Power needs to differentiate for 6G eMBB, FWA, Wearable and massive IoT devices by candidate 6G frequency ranges.

Conclusion

In this contribution we present our view on 6GR General RF and UE RF. The following observations have been made:

Proposal 1: RAN4 to study and provide input to RAN on how the three factors: number of Rx antennas, number of Tx antennas and Maximum Output Power need to differ for 6G eMBB, FWA, Wearable and massive IoT devices.

Proposal 2: Consider four receivers as mandatory baseline for bands re-farmed from 5G for eMBB UE for RAN4 requirement work.

Proposal 3: Consider six receivers as mandatory baseline for new 6GR bands for eMBB UE for RAN4 requirement work.

Proposal 4: Consider eight receivers as mandatory baseline for new 6GR bands for FWA UE for RAN4 requirement work.

Proposal 5: Consider eMBB UE and FWA UE devices support two TX chains as mandatory baseline for RAN4 requirement work.

Proposal 6: RAN4 to study how Maximum Output Power needs to differentiate for 6G eMBB, FWA, Wearable and massive IoT devices by candidate 6G frequency ranges.

- 4	References

R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4#116, RAN4 Chair

R4-2514641, WF for 6GR system parameter, RAN4#116, FL Huawei, HiSilicon

RP-252873, Summary of 6G Diverse Device Types (including Minimum UE Bandwidth), RAN#109, Moderator (Apple