3GPP TSG-RAN WG4 Meeting # 117		R4-2520584

Dallas, TX, USA, November 17 – 21, 2025

Agenda item:	10.2.3

Source:	Apple

Title:	On SUL+NUL UL Tx switching with non-zero switching time

WI/SI:	NR_MC_enh-Core

Release:	Rel-18

Document for:	Discussion

# 1	Introduction and Scope

During RAN4 meeting #116 in Bangalore, India, an agreement was made on how to introduce non-zero switching time for SUL 1T +NUL 1T and NR 1T + NR 1T CA into the existing 3GPP specifications. As captured in the topic WF [1] summary below, few approaches were considered and it was decided to alow companies to study the approaches and check with RAN2 which approach could be used.

In this discussion paper, we will share our views on how to introduce SUL 1T +NUL 1T and NR 1T + NR 1T CA feature in the current specifications.

# Discussion

NUL+SUL carrier switching feature was introduced in Rel-15 with 0µS of switching time between the two carriers.

As captured below (From 3GPP TS38.306 V19.0.0), Rel-18 UL Tx switching capability is signalled band combination list. The switching period for UL Tx switching is non-zero and can be any value from the set (35µS, 140µS, 210µS).

Observation 1:	The existing UL Tx feature already supports 1Tx-1Tx non-zero switching through ULTxSwitchingBandPair-r18, ULTxSwitchingBandPair-v1840.

Therefore, we non-zero  SUL 1T +NUL 1T and NR 1T + NR 1T can be enabled using the current Rel-18 UL Tx switching feature, via a separate BC list. That is, the UE would not report SUL+NUL BC in legacy BC list. Only NWs implementing the Rel-18 signaling can comprehend that UE supports 1T+1T NUL+SUL via non-zero switching time. Pure legacy NW who does not understand this BC would not regard the UE supports SUL+NUL BC.

In addition, in the latest 3GPP specifications (TS38.101-1 V19.3.1), Table 5.2C-1 which defines the operating band combinations for SUL in FR1, has Note 1 as defined below:

NOTE 1:   If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is according to switchingPeriodFor1T-r18 parameter within UE capability IE ULTxSwitchingBandPair-r18 or ULTxSwitchingBandPair-v1840if reported, otherwise 0 us.

Based on the arguments above, we can propose the following:

Proposal 1:	To enable non-zero SUL 1T +NUL 1T and NR 1T + NR 1T switching in older Releases, we support Approach 1 which consists of extending current Release 18 UL Tx switching to support this feature.

# 3	Conclusions

Observation 1:	The existing UL Tx feature already supports 1Tx-1Tx non-zero switching through ULTxSwitchingBandPair-r18, ULTxSwitchingBandPair-v1840.

Proposal 1:	To enable non-zero SUL 1T +NUL 1T and NR 1T + NR 1T switching in older Releases, we support Approach 1 which consists of extending current Release 18 UL Tx switching to support this feature.

# 4	References

R4-2511775, “WF on SUL NUL none zero switching period”, OPPO, Xiaomi, MediaTek, 3GPP TSG-RAN WG4 Meeting #116, Aug 25th – 29th, 2025