# 3GPP TSG-RAN WG4 Meeting #117	                                                     R4-2520613                                                     Dallas, USA, Nov. 17-21, 2025

Source:	Apple

Title:	CSI reporting requirement and testing framework for CSI prediction

Agenda item:	6.11.3

Document for:	Discussion

Introduction

This paper discusses the CSI reporting requirement framework for CSI prediction. In this paper, we address the performance monitoring for AI/ML-based CSI prediction use cases.

Discussion

Performance monitoring

The WI on AI/ML-based CSI prediction use case is specifying the reporting of performance. First, the delay and accuracy of performance monitoring metric is discussed in RAN4#115 [1].

Agreement: Monitoring reporting delay and accuracy: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed. CSI prediction: - CSI prediction, the details are subject to RAN1 decision - RAN4 will further study the feasibility to specify the corresponding delay and/or accuracy requirements for UE sided monitoring

In RAN4 agreement, monitoring reporting is under RAN1’s scope. RAN1#120-bis and RAN1#121 have made the agreements as follows.

RAN1#120-bis Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric.

In RAN1 agreement, SGCS is agreed the metric of CSI prediction using UE-side model for performance metric type 3. In Type 3, the UE computes the performance metrics, reports them to the network, which then determines whether to initiate functionality fallback to legacy CSI reporting.

RAN1#121 Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, - one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - wideband frequency granularity - only for one prediction instance configured by NW - per layer, and the total number of layers is the same as the reported RI in the associated inference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’ - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codepoint is used to indicate the value range of (0 0.3] - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1 Agreement For CSI prediction using UE-side model, support single UCI part with the following report format (i.e., CSI field mapping order) among a set of SGCSs. SGCS1 #1. SGCS1 #2. …. SGCS1 #v. SGCS2 #1. SGCS2 #2. …. SGCS2 #v. - SGCS1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI, - SGCS2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCSi #k is the SGCS of k-th layer of i-th SGCS where k ={1, …., v}, and i ={1, 2}. - v is the value of the reported RI in the associated inference report.

During RAN4#116 the following agreements were reached:

Agreement:

RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction

FSS which requirements are to be introduced.

Issue 1-2: Requirement baseline for monitoring

Agreement:

Introduce the following requirements for CSI performance monitoring:

- Reporting delay

  - FFS how the delay is defined

- Reporting accuracy

  - FFS on whether accuracy requirement can be defined/checked

- Mapping table (38.1.3.3 section 10)– To be checked whether RAN1 captures or RAN4.

- If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this

During RAN4#116bis the following agreements were reached:

Issue 1-1: Requirements for performance monitoring

Agreement:

No performance monitoring delay requirements are specified in RAN4 for CSI prediction.

Issue 1-7: Doppler and MCS

Agreement:

Introduce tests for 20Hz Doppler with MCS [17or19] with 16Tx ports and 2Rx in FDD

Introduce test for TDD with the details FFS

Issue 1-8: Generalization

Agreement:

Test setup for the generalization will be further discussed based on the following options

- Option 1: Introduce multiple tests with different MCS, which will lead to different SNR points.

- Option 2: Introduce multiple tests with different channel models. The details of the channel models are FFS.

- Option 3: the combination of option 1 and 2.

It is FFS on how to quantify the generalization performance.

Issue 1-9: Activation

Agreement:

No activation requirement in RAN4 spec for CSI prediction will be introduced.

- Note: it is a general understanding that UE has to be ready to start measurements for inference after sending RRC reconfiguration complete. No details will be specified in RAN4.

Based on the RAN1#121 agreement, the requirements for CSI prediction using UE-side models and reporting contents of UE-assisted performance monitoring are defined.

For CSI prediction using UE-side models, the agreement specifies that two types of SGCS values are calculated and reported. The first type, SGCS1, is based on the predicted CSI and the ground truth CSI. The second type, SGCS2, is based on the ground truth CSI and the non-predicted CSI corresponding to the latest CSI-RS transmission occasion not later than the CSI reference resource of the inference reporting instance.

However, for the network to rely on these predictions, it must continuously monitor the prediction quality and decide when to trust the reported CSI or revert to legacy CSI reporting. Today there is no standardized, verifiable way to ensure that a UE-reported similarity metric truly reflects the accuracy of its prediction. Without clearly defined accuracy requirements and corresponding test procedures, a malfunctioning or mis-implemented predictor can silently degrade link adaptation, scheduling, or mobility performance while still presenting apparently healthy CSI reports to the network.

Issues with Only SGCS1
SGCS1 measures the similarity between the UE’s predicted CSI and the UE’s own ground-truth CSI, i.e., CSI derived from standard CSI-RS measurements on the configured reference resource. Although SGCS1 provides a direct view of prediction quality in principle, the network cannot directly observe how the UE determines its ground truth or performs the similarity computation. This creates room for errors or even manipulation, for example, the UE could inadvertently use outdated channel samples, or, in a worst case, use an internally simulated (“genie”) channel, producing a high SGCS1 even when the actual prediction is inaccurate. If RAN4 were to rely only on SGCS1, it would leave operators exposed to silent failures, mis-implementation, or intentional misreporting that could compromise overall system performance.

Dual-Metric Safeguard with SGCS1 and SGCS2
The standardized introduction of SGCS2, defined as the similarity between the same ground truth and the most recent non-predicted CSI measurement, provides a critical second layer of protection. Under realistic, dynamic channel conditions, a functioning AI/ML predictor should consistently yield SGCS1 values significantly higher than SGCS2, reflecting the genuine performance gain of prediction over legacy CSI. Conformance and network monitoring can therefore look not only at absolute SGCS1 thresholds but also at the relationship between SGCS1 and SGCS2. Any large inconsistency or unexpected correlation (e.g., SGCS1 not appreciably exceeding SGCS2) flags potential mis-reporting or degraded predictor performance.

In the CSI prediction use case, the UE runs an on-device AI/ML model to estimate future or missing CSI and reports both the predicted CSI and a performance monitoring metric, typically the SGCS.  The purpose of SGCS is to allow the network (or test equipment) to monitor whether the model’s predictions remain accurate over time and to trigger fallback or model deactivation if performance degrades.

However, the core issue is that the test equipment has no access to the ground truth SGCS.

Therefore, there is no direct way to verify whether a reported SGCS truly reflects the model’s real accuracy.
A malfunctioning, outdated, or misconfigured model might still report high SGCS values if the UE computes it incorrectly or with outdated measurements.
To address this, RAN4 can introduce a trend-based validation approach rather than relying on absolute SGCS thresholds.
The key idea is to progressively vary channel predictability, for example, by changing Doppler, delay spread, or fading dynamics and observe whether the UE’s reported SGCS values respond consistently to these variations.
Under controlled conditions:

- when the channel becomes more stable or correlated, SGCS should increase,

- when the channel becomes faster fading or more random, SGCS should decrease.

If the UE’s reported SGCS follows this expected trend, it indicates that the internal monitoring mechanism behaves realistically, even if the test equipment cannot verify the exact similarity calculation.

It is challenging to apply a trend-based verification method for SGCS reporting because current 5G test equipment does not support dynamically changing channel conditions, other than basic SNR variation used in RRM tests. Introducing new fading playback or time-varying channel models at this late stage of 5G would require TE upgrades and new test modes, which is neither practical nor aligned with existing conducted test methodology. Since dynamic channel evolution cannot be emulated in RAN4 conformance today, any verification method must operate under static or quasi-static RF conditions.

RAN4 is to define a reporting-accuracy requirement for AI/ML performance monitoring based on the consistency of SGCS values under static test conditions. The UE shall repeatedly calculate and report SGCS in an unchanged radio environment, and the TE shall verify that at least 90 % of reported values stay within a bounded tolerance range. This statistical consistency method avoids the need for dynamic channels or ground-truth CSI while ensuring that UE-reported SGCS is stable, repeatable, and suitable for use in AI/ML life-cycle management.

Building on this verification capability, RAN4 should define two complementary performance requirements for real time monitoring and LCM . First, an absolute SGCS1 floor ensures that the predictor’s overall accuracy remains above a minimum acceptable level. For example, within a sliding window of 20–40 reports, the 5th–10th percentile of SGCS1 should exceed a target T_abs (value for further study). Second, a dual-metric margin ensures that prediction provides measurable gain over legacy CSI: within the same window at least 95 % of reports should satisfy SGCS1 – SGCS2 ≥ Δ_min (about one to two quantization codepoints or roughly 0.03–0.06 in linear SGCS). These requirements capture both absolute accuracy and relative improvement.

To provide operational stability, RAN4 can further require persistence and hysteresis before triggering lifecycle management (LCM) actions such as model fallback. For example, the UE would only be deemed non-compliant if either the absolute floor or dual-metric margin is violated for two consecutive windows, and the alarm would clear only after two good windows. Additional refinements: per-layer checks tied to the reported rank indicator, and mobility-specific threshold tables: would adapt the requirements to realistic Doppler conditions. Together, TE-based SGCS verification and these layered performance thresholds give operators a trusted, standardized basis for deciding when to continue, switch, or retrain AI/ML CSI prediction models.

In AI/ML-based CSI prediction, continuous performance monitoring is essential to determine when a model remains reliable and when it should be switched, retrained, or deactivated. The UE reports similarity metrics such as SGCS1(predicted vs. ground truth CSI) and SGCS2 (non-predicted vs. ground truth CSI), which together indicate both the model’s absolute accuracy and its gain over legacy CSI reporting. However, current specifications do not yet define how these reported metrics translate into standardized Life-Cycle Management (LCM) triggers. Without clear thresholds and persistence rules, networks may react too late, or too aggressively to temporary metric fluctuations. To ensure consistent behavior across vendors and predictable system operation, RAN4 should define performance KPI thresholds and trend-based rules that govern when LCM operations such as model switching or deactivation are triggered.

Even though the network ultimately decides how to act on the reported KPIs, RAN4 still needs to standardize minimum performance thresholds so that AI/ML-based CSI prediction behaves consistently across vendors, devices, and deployments. If no common thresholds are defined, each vendor could interpret “acceptable model performance” differently one might keep a degraded model active, another might switch or retrain early, and a third might never deactivate the model at all. This would make AI/ML prediction impossible to certify, test, or benchmark in a multi-vendor environment, because there would be no shared reference for when a model is considered compliant or has failed.

Defining thresholds does not take control away from the network, it only establishes the boundary between “standard-compliant” and “non-compliant” model behavior. The network still decides the action (fallback, retrain, switch model, disable AI, etc.), but the trigger conditions must be harmonized by RAN4 so the industry can guarantee that a deployed AI model meets a minimum reliability level. Without standardized LCM criteria, there is no way to ensure interoperability, no common basis for field testing, and no assurance that AI-assisted CSI is ever better than legacy CSI reporting.

RAN4 shall define minimum performance thresholds for LCM of AI/ML-based CSI prediction using SGCS1 and SGCS2, to ensure interoperable and certifiable model behavior across vendors. While the network remains free to decide which corrective action to take (e.g., switch, retrain, deactivate), the trigger conditions for when a model is considered compliant or degraded must be standardized rather than left to vendor-specific interpretation

- Absolute accuracy: SGCS1 shall remain above a defined minimum floor (T_abs) within a sliding monitoring window.

- Relative gain: SGCS1 – SGCS2 ≥ Δ_min to guarantee the AI model continues to outperform legacy CSI reporting.

- Persistence / hysteresis: LCM actions are triggered only after consecutive window violations, and cleared only after consecutive compliant windows.

- Mobility profiles: Thresholds (T_abs, Δ_min) may be Doppler-class specific (e.g., pedestrian vs. high mobility).

Conclusion

In this contribution, we have discussed the RRM impact of AI/ML-based CSI prediction. The proposals are presented as follows.

- RAN4 is to define a reporting-accuracy requirement for AI/ML performance monitoring based on the consistency of SGCS values under static test conditions. The UE shall repeatedly calculate and report SGCS in an unchanged radio environment, and the TE shall verify that at least 90 % of reported values stay within a bounded tolerance range. This statistical consistency method avoids the need for dynamic channels or ground-truth CSI while ensuring that UE-reported SGCS is stable, repeatable, and suitable for use in AI/ML life-cycle management.

RAN4 shall define minimum performance thresholds for LCM of AI/ML-based CSI prediction using SGCS1 and SGCS2, to ensure interoperable and certifiable model behavior across vendors. While the network remains free to decide which corrective action to take (e.g., switch, retrain, deactivate), the trigger conditions for when a model is considered compliant or degraded must be standardized rather than left to vendor-specific interpretation

- Absolute accuracy: SGCS1 shall remain above a defined minimum floor (T_abs) within a sliding monitoring window.

- Relative gain: SGCS1 – SGCS2 ≥ Δ_min to guarantee the AI model continues to outperform legacy CSI reporting.

- Persistence / hysteresis: LCM actions are triggered only after consecutive window violations, and cleared only after consecutive compliant windows.

- Mobility profiles: Thresholds (T_abs, Δ_min) may be Doppler-class specific (e.g., pedestrian vs. high mobility).

- References

- 3GPP TSG-RAN WG4 Meeting #116, R4-2511890, “WF on AI/ML for air interface for [116][131],” Qualcomm Incorporated, Bengaluru, India, 25 – 29 Aug 2025, Agenda item 7.17.1.