# 3GPP TSG-RAN WG4 Meeting #117	R4-2520629

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.11.8

Source:	Apple

Title:	Simulation Results for CSI Prediction

Release:	Rel-19

Document for:	Information

# 1. Introduction

In RAN4#116bis CSI reporting requirements for CSI prediction was discussed and way forward [1] was agreed. The simulation assumptions for requirements with CSI prediction was discussed and way forward [2] was agreed. In this contribution we present simulation results for PMI reporting requirements.

# 2. Simulation Results

Based on simulation assumption agreed in [2] we present results for Step 3 below.

The presented results are with model is trained with mixed dataset of 20, 50Hz Doppler.

Channel model and antenna correlation: TDLA-30 XP-Med

Observation window: 5/5ms

Prediction window: 1/5ms/5ms

FDD

SCS/CBW: 15KHz/ 10MHz

Sub-band size: 4

Antenna config 16x2

Intermediate metrics

The SGCS for prediction are presented for SNR closest to the SNR @ 90% max TP with CSI prediction.

Table 1: Intermediate metrics for alignment - FDD

| MCS | SNR | SGCS2 |  | SGCS3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Layer 1 | Layer 2 | Layer 1 | Layer 2 |
| 17 | 8 | 0.86 | 0.76 | 0.89 | 0.81 |
| 19 | 10 | 0.87 | 0.77 | 0.89 | 0.83 |

Test metrics

The agreed test metrics and additional metrics to assess performance gain over sample and hold with eType II codebook are presented below.

Table 2: Test metrics for alignment - FDD

| MCS | SNR_AIML SNR @90% Max TP with AIML | Gamma1 TP ratio of AIML vs Rand @ SNR_AIML | Gamma2 TP ratio of AIML vs eTypeII @ SNR_AIML | SNR_eTypeII SNR @90% Max TP with eTypeII | Gamma3 TP ratio of eTypeII vs Rand @ SNR_eTypeII |
| --- | --- | --- | --- | --- | --- |
| 17 | 7.9 | 3.71 | 1.27 | 12.0 | 2.21 |
| 19 | 9.8 | 3.30 | 1.27 | 14.3 | 2.11 |

For FDD with 2RX there is > 20% gain over sample and hold with eType II.

- For FDD with 2RX the gain with CSI prediction over eType II is > 20%.

TDD

SCS/CBW: 30KHz/ 40MHz

Sub-band size: 8

Antenna config 16x4

Intermediate metrics

The SGCS for prediction are presented for SNR closest to the SNR @ 90% max TP with CSI prediction.

Table 3: Intermediate metrics for alignment - TDD

| MCS | SNR | SGCS2 |  | SGCS3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Layer 1 | Layer 2 | Layer 1 | Layer 2 |
| 17 | 8 | 0.77 | 0.58 | 0.76 | 0.58 |
| 19 | 10 | 0.77 | 0.59 | 0.77 | 0.59 |

Test metrics

The agreed test metrics and additional metrics to assess performance gain over sample and hold with eType II codebook are presented below.

Table 4: Test metrics for alignment - TDD

| MCS | SNR_AIML SNR @90% Max TP with AIML | Gamma1 TP ratio of AIML vs Rand @ SNR_AIML | Gamma2 TP ratio of AIML vs eTypeII @ SNR_AIML | SNR_eTypeII SNR @90% Max TP with eTypeII | Gamma3 TP ratio of eTypeII vs Rand @ SNR_eTypeII |
| --- | --- | --- | --- | --- | --- |
| 17 | 4.4 | 3.87 | 1.14 | 5.9 | 2.71 |
| 19 | 5.9 | 3.13 | 1.13 | 7.4 | 2.59 |

For TDD with 4RX there is > 10% gain over sample and hold with eType II.

- For TDD with 2RX the gain with CSI prediction over eType II is > 10%.

# 3. Conclusion

In this contribution we present simulation results for PMI reporting requirements. Our observations are captured below:

- For FDD with 2RX the gain with CSI prediction over eType II is > 20%.

- For TDD with 2RX the gain with CSI prediction over eType II is > 10%.

# Reference

- R4-2514631, “WF on [116bis][112] R19 AI for air interface”, Qualcomm

- R4-2514634, “WF on updated simulation assumption for AI based CSI prediction”, OPPO, …