# 3GPP TSG-RAN WG4 Meeting #117	R4-2520635

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.19.4

Source:	Apple

Title:	On UE Demodulation Requirements for NTN Phase 3

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

The UE demodulation requirements for Rel-19 NTN Phase 3 WI were discussed in RAN4#116bis and way forward [1] was agreed. In this contribution we present simulation results for the agreed test cases for defining requirements.

# 3. Simulation Results

For the test cases agreed in RAN4#116bis in [1] alignment and impairment results are provided.

RedCap 1Tx-1Rx, 10MHz/15kHz, FDD/HD-FDD

Table 1: Simulation results for RedCap with 1RX

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| 1 | MCS4, 52PRB | NTN-TDLA100-200 | 16 | 1x1 | 2.2 | 4.2 |
| 2 | MCS4, 52PRB | NTN-TDLC5-200 | 32 | 1x1 | 0.8 | 2.8 |
| 3 | MCS13, 52PRB | NTN-TDLC5-200 | 16 | 1x1 | 8.3 | 10.3 |

eRedCap 1Tx-1Rx/2RX, 10MHz/15kHz, FDD/HD-FDD

Without reduced baseband bandwidth capability

Table 2: Simulation Results for eRedCap - without reduced BW capability

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| 1 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x1 | 2.2 | 4.2 |
| 2 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x1 | 8.3 | 10.3 |
| 3 | MCS4, 52 | NTN-TDLA100-200 | 16 | 1x2 | Reuse from Rel-17 |  |
| 4 | MCS13, 40 | NTN-TDLC5-200 | 16 | 1x2 | 4.8 | 6.8 |

With reduced baseband bandwidth capability

Table 3: Simulation Results for eRedCap - with reduced BW capability

| Test number | MCS/PRB | Channel model | HARQ process | Antenna configuration | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Alignment | Impairment |
| 1 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x1 | 2.5 | 4.5 |
| 2 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x1 | 7.7 | 9.7 |
| 3 | MCS4, 25 | NTN-TDLA100-200 | 16 | 1x2 | -1.6 | 0.4 |
| 4 | MCS13, 25 | NTN-TDLC5-200 | 16 | 1x2 | 4.3 | 6.3 |

# 3. Conclusion

In this paper, we present simulation results for UE demodulation requirements for NTN Phase 3 WI.

# Reference

- R4-2515038, “Way Forward for [116bis][311] Rel-19 Demodulation_Part1”, Samsung