3GPP TSG-RAN WG4 Meeting #117                                       	                      R4-2520645

# Dallas TX, USA, November 17 – November 21, 2025

Agenda item:	4.6.1.3

Source:	Apple Inc.

Title:	Discussion on Release Independence for 6Rx UEs

Release:	Rel-19

Document for:	Discussion

# Introduction

A new WID on RF enhancements for NR FR1/FR2 and EN-DC has been created for Release 19. The WID covers three main areas: High power UE (HPUE) for CA in terrestrial network (TN), power boosting and/or MPR reduction, and 6Rx for handheld and FWA UEs. The main objectives for 6Rx, as highlighted in the WID [1], are captured below:

Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding

  - For this discussion paper we focus on our view on release independence of the feature.

# Discussion

Companies have discussed in the past how to specify applicability rules for the core requirements phase. More specifically, how to indicate that a UE support 6 MIMO layers with 6Rx and how to capture in the specifications that only FWA devices are supporting this feature. During last RAN Plenary it was agreed that RAN2 specifications would recognize, in writing, this outcome of the discussion.

Observation #1: RAN Plenary has confirmed that 6 MIMO layers is a FWA feature, which gets explicitly captured in the text of RAN2 specifications.

In addition, also during the last RAN Plenary, it was discussed the issue of Release Independence. As per the 3GPP specification 38.307, Requirements on User Equipments (UEs) supporting a Release-Independent Frequency Band, is the spec for the release independence issues and maintained by RAN4. By its name, the release independence should be primarily used for frequency band and band combinations.

Observation #2: TS 38.307 is primarily used to define release-independent band and band combinations. The large majority of the release independent features are spectrum-related, on topics like NR operating frequency bands, channel bandwidths, carrier aggregation configurations, and power classes, among others.

In terms of spectrum related features, there is a plethora of use cases that comprise the large majority of TS 38.307. Among others, we can find:

- NR Operating Frequency Bands, which include power classes and RF requirements.

- Supported Channel Bandwidths, which are specified per duplex mode (FDD, TDD, SDL, SUL)

- Carrier Aggregation (CA) Configurations, which cover both intra-band and inter-band CA.

- Power Classes, which details requirements related to UE power levels for different bands and modes.

On the other hand, there are some non-spectrum related release-independent features, which are generic baseband requirements for the already specified features in legacy releases based on RAN4 consensus, such as the following release independent performance requirements for features already specified in NR Rel-15:

- Rel-15 RF requirements for 4Rx UEs

- Rel-15 Transparent Tx Diversity

- Rel-15 UE demodulation and CSI requirements for MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference

- Rel-17 PDSCH absolute physical layer throughput requirements with link adaptation

- Rel-17 RF requirements for 8Rx UE

- Rel-17 Demodulation requirements for 8Rx UE

The main principle is that the newly defined requirements for legacy features can be release independence. In other words, previous non-spectrum related release independence cases are focused on the new requirements only without additional signalling or new features included.

Proposal #1: RAN4 to confirm 6Rx UE as a Rel-19 feature only, and do not consider release independence.

# Conclusions

Our observations and proposals are summarized below:

  - Observation #1: The implementation of 6Rx UE is quite involved in terms of UE architecture and modem implementation in comparison to 4Rx.

Observation #2: 6Rx with 4 and 6 MIMO layer UE feature go hand in hand with SRS requirements, the latter not been deemed needed to be release independent during RAN4#115.

Proposal #1: Do not define 6Rx UE with 4 and 6 MIMO layers as a release independent feature.

# References

- RP-240720 “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, by RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024