3GPP TSG-RAN4 Meeting  #117	R4-2520666

Dallas, US. 17th – 21st, November 2025

Agenda item:	8.9

Source:	Apple

Title:	On spectrum sharing for 6GR

Document for:	Discussion

# 1	Introduction

#### Issue 1-1-1: Migration scheme

- Agreements

- RAN4 focus on MRSS between 6GR and NR until further RANP guidance

- RAN4 focus on inter-RAT mobility between 6GR and NR

#### Issue 1-1-2: Scenarios for MRSS between 6GR and NR

- Agreements

- For operating frequency range or band for MRSS between 6GR and NR

- Start RAN4 6G-5G MRSS study in FR1

- Whether to consider 6G-5G MRSS in FR2-1 is further discussed

The discussion on 5G-6G spectrum sharing was triggered in RAN4#116bis meeting. A WF on [116bis][108] 6G spectrum sharing was agreed with the following high level agreement for RAN4 further study.

This contribution will share our views on the open issue based on these agreements.

# 2	Discussion

In [1], it was also agreed to take the following aspects for MRSS study

- System parameters

- RF requirements

- RRM requirements

- Interference handling

## 2.1 system parameters

RAN4#116bis concluded the table below for the parameters to be impacted/potentially impacted by MRSS. In this section, we further share our view for each parameter, e.g. how MRSS will impact the mentioned system parameter in terms of parameter design, implementation and deployment etc.

| System parameters | Whether impacted by MRSS |
| --- | --- |
| Numerology | Maybe impacted |
| Channel raster | Maybe impacted |
| Sync raster | Maybe impacted |
| Waveform | FFS |
| Modulation | FFS |
| Channel bandwidth | FFS |

Numerology:

At first for the basic numerology design for 6GR, we think it should not be impacted by MRSS, e.g. the basic numerology design shall consider the system needs for 6GR and be able to support efficient deployment of 6GR. When it comes to a MRSS bands, using the same set of subcarrier spacings can greatly simplifies the MRSS design. This also allows a very dynamic sharing of resources between 5G and 6G following the short-term traffic verification and avoiding any guard bands between 5G and 6G. In this regard, it is proposed that same single numerologies that have been deployed shall be considered for 5G-6G MRSS band.

Proposal 1: It is proposed that the same single numerologies that have been deployed in 5G shall be used for each band for MRSS.

Channel raster:

Channel raster was introduced for each band based on deployment situation in 5G. e.g. for FDD bands and several small legacy TDD bands, 100MHz channel raster was defined. While for new 5G TDD bands, only SCS based channel raster was defined. Whatever type of channel raster, it is just a calculation and selection towards a subset of the global raster to be used for test setting. Our preference would be to simply the channel raster in 6G. Since 100MHz is used for some of the bands below 3000MHz, the raster points for these bands need to be kept. RAN4 could discuss which of the following options is agreeable

Option 1: Only use global raster, e.g. 5kHz for Bands <3000MHz and 15kHz for bands >3000MHz

Option 2: Define unified channel raster (e.g. 5kHz).

Proposal 2: RAN4 discuss which of the following options is considered for channel raster

Option 1: Use global raster, e.g. 5kHz for Bands <3000MHz and 15kHz for bands >3000MHz

Option 2: Define unified channel raster (e.g. 5kHz).

Sync raster:

RAN4 assessment on raster design is critical to ensure efficient MRSS operation in terms of fast cell search and achieving UE power saving goal. In 5G the raster is designed assuming at least one sync raster is available for the UE wherever the UE channel is located. Such kind of design can provide maximum flexibility on network deployment while it will unnecessarily increase the UE initial cell search time. As indicated in [3], only a few percent of the defined raster are used in commercial network. In 6G we think it’s deserving to reconsider what is the best trade-off between network deployment and UE initial cell search time since the latter one is also one of the key performances for operator. How can raster design help to improve initial access performance? Should we reduce the raster for MRSS only to a subset of the already defined set? Should we couple raster design to minimum channel bandwidth? Overall, we think 5G principle on raster design should not be automatically agreed for 6G before we answer these questions. Also, there are divergence in RAN1 discussion on whether to use common signal or not for MRSS. One view is to use common SSB design for 5G and 6G for MRSS band. While another view is to use the same 6G SSB design regardless of whether MRSS is used or not to avoid multiple “modes” which will complicating the 6G design. in regard of the potential conclusion in RAN1 design on common signal we also need to consider the design principle on how to place 5G and 6G sync raster, e.g. FDM or TDM.

Proposal 3: RAN4 discussion on sync raster should consider several high-level points as starting point,

- How can raster design help to improve initial access performance?

- Should we reduce the raster for MRSS only to a subset of the already defined set?

- Should we couple raster design to minimum channel bandwidth?

- Design principle on how to place 5G and 6G sync raster, e.g. FDM or TDM, pending RAN1 progress on common signal design.

Modulation

Higher order modulation may be defined for 6GR, e.g. uniform 1024 QAM in uplink and uniform 4096QAM in downlink. While higher order modulation means higher PAPR which may put additional requirement for equipment or more power back off is needed to support higher order modulation. For spectrum migration based on legacy MSR, the performance impact needs to be evaluated for uniform modulation and constellation sharping modulation.

Proposal 4: For spectrum migration based on legacy MSR, the DL performance impact needs to be evaluated for uniform modulation and constellation sharping modulation if higher order modulation is to be used for 6G RBs.

Channel bandwidth

Thanks to the ultra-lean design, 6G - 5G MRSS demand is clear and efficient for FR1 bands migration.  By RM patterns around the few NR always-on signals such as SSBs or SIB1, the spectral efficiency penalty of NR carrier can be pretty low. Table 2.1-1 gives an example calculation for the configuration of SSB (20RBs) periodicity =20ms, SIB1(48RB) periodicity=40ms. For channel bandwidth >=10MHz, the overhead is only a few percent. While from the calculation, it also indicates that the overhead is not negligible for smaller channel bandwidth below 5MHz. In this regard, it would make sense to focus on large channel bandwidth (e.g.>5MHz) for MRSS.

Table 2.1-1MRSS SSB+SIB1 overhead for FR1: SSB (20RBs) periodicity =20ms, SIB1(48RB) periodicity=40ms

Proposal 5: it is proposed to focus on large channel bandwidth (e.g. >5MHz) for 5G-6G MRSS.

## 2.2 RF requirement

For MRSS carrier, the base station needs to transmit both 5G signal and 6G signal. Harmonized requirements between 5G and 6G will benefit such BS implementation when developing 6G and 5G-6G MSR specification.

For UE, since it is scheduled by network and just need to follow the resource configuration change either for 5G or 6G, there seems not much impact on UE requirement. If single SCS is considered across 5G and 6G shared carrier, the factors causing power change will only include RB number change and frequency location change. But overall, the requirement can be the same for SA mode.

Observation 1: Compatible RF requirement between 5G and 6G will benefit BS implementation supporting 5G-6G MRSS, which can be considered when developing 6G and 5G-6G MSR specification.

Observation 2: UE either support 5G or 6G in MRSS. So, it is supposed that no RF requirements impact due to MRSS and it just need to follow normal RF requirements for SA mode.

## 2.3 RRM requirement

Depending on how MRSS signal is designed as well as the migration scenarios to be addressed there could be different implication for RAN4 RRM scope. E.g. The SSB design is fundamental to initial access, cell detection, and synchronization in 5G NR. As also mentioned in section 2.2, there could be different discussion direction on SSB design in RAN1, including using sharing/not sharing signal channel between 5G and 6G, sharing scheme (FDM, TDM and both FDM and TDM) etc. RAN4 may need to study MRSS based measurement and handover performance optimization. Since these aspects are highly related to RAN1 design, we prefer to delay the discussion in this area until there is clear progress in RAN1/2.

Proposal 6: Delay the discussion on MRSS based RRM requirement until RAN1/2 have sufficient progress.

## 2.4 interference handling

Another potential aspect is co-existence between 6GR and 5G.  RAN4 introduced UL 7.5kHz shifting solution (shown in figure 2.3-1) in 5G-4G DSS due to different waveform used for LTE and NR mode respectively. For 6G-5G MRSS, such mechanism is not needed given the same waveform will be used.

Table 2.3-1 UL 7.5kHz shifting for DSS

Proposal 7: 7.5kHz shifting is not needed for 6G-5G MRSS.

## 2.5 x-WG coordination

Timely coordination is essential between RAN1/4 WGs to ensure a successful and robust MRSS design. RAN1 will start MRSS discussion in 2026 Feb meeting and operating bands/carriers is thought to be one of the high-level aspects that will impact on MRSS design [4]. In the last RAN4 meeting, it was agreed to prioritize FR1 band for MRSS study and, overhead wise it would be meaningful to limit the bandwidth to be >=5MHz. We suggest sending LS to RAN1 on the conclusion reached in RAN4, e.g. frequency bands and channel bandwidth (if consensus is reached in this meeting).

Proposal 8: It is proposed to send LS to RAN1 about RAN4 conclusion on frequency range priority and observation on channel bandwidth for MRSS.

# 3 	Conclusion

This paper discussed MRSS impact on RAN4 scope. The following observations and proposals are concluded.

Proposal 1: It is proposed that the same single numerologies that have been deployed in 5G shall be used for each band for MRSS.

Proposal 2: RAN4 discuss which of the following options is considered for channel raster

- Option 1: Use global raster for channel setting during test, e.g. 5kHz for Bands <3000MHz and 15kHz for bands >3000MHz

- Option 2: Define unified channel raster (e.g. 5kHz) for channel setting during test.

Proposal 3: RAN4 discussion on sync raster should consider several high-level points as starting point,

- How can raster design help to improve initial access performance?

- Should we reduce the raster for MRSS only to a subset of the already defined set?

- Should we couple raster design to minimum channel bandwidth?

- Design principle on how to place 5G and 6G sync raster, e.g. FDM or TDM, pending RAN1 progress on common signal design.

Proposal 4: For spectrum migration based on legacy MSR, the DL performance impact needs to be evaluated for uniform modulation and constellation sharping modulation if higher order modulation is to be used for 6G RBs.

Proposal 5: it is proposed to focus on large channel bandwidth (e.g. >5MHz) for 5G-6G MRSS.

Observation 1: Compatible RF requirement between 5G and 6G will benefit BS implementation supporting 5G-6G MRSS, which can be considered when developing 6G and 5G-6G MSR specification.

Observation 2: UE either support 5G or 6G in MRSS. So, it is supposed that no RF requirements impact due to MRSS and it just need to follow normal RF requirements for SA mode.

Proposal 6: Delay the discussion on MRSS based RRM requirement until RAN1/2 have sufficient progress.

Proposal 7: 7.5kHz shifting is not needed for 6G-5G MRSS.

Proposal 8: It is proposed to send LS to RAN1 about RAN4 conclusion on frequency range priority and observation on channel bandwidth for MRSS.

# 4	References

R4‑2514646, WF for [116bis][108] 6G spectrum sharing, CMCC, RAN4#116bis

R4-2513032, On spectrum sharing for 6GR, Apple, RAN4#116bis

R4-2513075, Views on 6G system parameters, RAN4#116bis, MediaTek

R1-2507985, FL summary#1 on overview of 6GR air interface, Moderator (NTT DOCOMO)