3GPP TSG-RAN4 Meeting #117	R4-2520688

# Dallas, U.S.A, 17th – 21st November, 2025

Source:	LG Electronics

Title: 		On Ku-band VSAT Half-Duplex RF requirements with FR1-Numerology

Agenda Item:	7.6.4.1

Document for: Discussion and Approval

# Introduction

The key discussion point in “On Ku-band VSAT Half-Duplex RF requirements with FR1-Numerology” addresses the HD-FDD requirements issue and VSAT with small antennas (e.g., 20×20 cm). [1,2].

# Discussion on UE RF requirements for HD-FDD for Ku band with FR1 numerology

## Sub-topic 1-1 : Issues for HD-FDD requirements.

Issue 1-1-1: Antenna type assumption for HD-FDD VSAT

The Ku-band spectrum can be divided into two main segments — Downlink and Uplink — with the overall band covering both.

- The Downlink band (10.7–12.75 GHz) has a center frequency of 11.725 GHz and a total bandwidth of 2.05 GHz, resulting in a fractional bandwidth of about 17.5 %.  Easy Antenna implementation.

- The Uplink band (13.75–14.5 GHz) is narrower, with a center frequency of 14.125 GHz and 750 MHz of total bandwidth, corresponding to a fractional bandwidth of approximately 5.3 %. Very Easy Antenna implementation.

- When combining the entire Ku-band range (10.7–14.5 GHz), the center frequency becomes 12.6 GHz and the total bandwidth expands to 3.8 GHz, yielding a fractional bandwidth of around 30.2 %. Difficult Antenna implementation.

Observation1. In a half-duplex system, a single-feed antenna with a 30% bandwidth is high in difficulty, as it requires complex multi-resonance tuning.  In contrast, a dual-feed dual-band antenna with about 20% and 5% bandwidths is of moderate difficulty, since each band can be optimized separately but still needs Tx/Rx isolation and filtering

Proposal 1. Both single-feed and dual-feed topologies can be used and other architecture are neither excluded. From the above mentioned two approach the dual-feed topology is preferred because it does not require an RF switch, offering advantages in cost, ant. design simplicity and elimination of switching time delay.

Issue 1-1-3: Transmit ON/OFF time mask for HD-FDD

Observation 2: Capability to switch between the Rx and Tx modes is the only added functionality that HD-FDD operation adds on top of the FDD operation. Timing requirement for this switch may be necessary unless this is already indirectly addressed with other already existing RF requirements.

Observation 3: There is no technical reason why already defined Ku- and Ka-band FDD RF-requirements could not be directly applied also for HD-FDD operation in these bands, while it may be necessary to make additional clarifications into 38.101-5 to avoid any ambiguities linked with the applicability and testing of the requirements for both FDD and HD-FDD operation.

Observation 4: For FR1 numerology the 38.211 Physical channels and modulation specification allows 13.02us transition time for transition from Tx to Rx and vice versa [2]. For FR2 numerology the 38.211 Physical channels and modulation specification allows 7.01us transition time for transition from Tx to Rx and vice versa [2].

Proposal 2 Consider existing Transmit ON/OFF time mask requirement to be sufficient to ensure that HD-FDD device can also protect its own receiver from Tx self-interference and hence there is no need to introduce new RF requirements for RX-TX or TX-RX switching.

Issue 1-1-5 & 1-1-6: EIRP/EIS requirements for HD-FDD

Observation 5.  Based on the above Antenna type analysis in Issue 1-1-1 HD-FDD VSAT may exhibit up to about 0.5 dB better EIRP/EIS compared to FDD solution. This is mainly due to architecture changes in RX and TX RF front-end components and avoidance of interference from own transmitter.

Proposal 3. The observed possible 0.5 dB better performance in EIRP/EIS for HD-FDD vs. FDD is considered attractive, but not as mandatory modification to the minimum requirement. Therefore, it is considered reasonable to either maintain the existing EIRP/EIS requirement or consider 0.5dB improvement to the EIRP/EIS specifications when compared to the full-duplex configuration, as both options remain within the measurement tolerance.

## Sub-topic 1-2 : Issues for VSAT with Small Antennas

Issue 1-2-2: Assumptions for VSAT with small antenna

In array design, the element spacing (d) critically determines the grating-lobe behavior, beamforming efficiency, beam scan loss, and achievable scan range.  Using a λ/2 spacing (approximately 11.9 mm) at the 12.6 GHz center frequency, the element spacing translates to ~0.42λ for the downlink band (10.7 GHz) and ~0.58λ for the uplink band (14.5 GHz).

Observation 6.  Based on the Ku-band operating range (10.7–14.5 GHz), the element spacing corresponding to 0.5 λ at the center frequency (12.6 GHz) is approximately 11.9 mm, which theoretically represents the most ideal spacing to achieve balanced beamforming efficiency, grating-lobe suppression, and scan performance across the band. Under this condition, within a compact 20x20cm array aperture, a maximum configuration of approximately 17 × 17 elements can be accommodated, ensuring effective utilization of the available physical aperture while maintaining stable beamforming characteristics over the full Ku-band.

Proposal 4. Adopt 0.5 λ (11.9 mm) spacing at 12.6 GHz for Ku-band as working assumption for analysis of 200 mm × 200 mm aperture and enabling a 17 × 17 array with stable beamforming performance.

Observation 7.  From the NTN VSAT Type 6, the number of antenna elements is reduced from 45 × 45 to 17 × 17, resulting in approximately 10log10((17×17)/(45×45)) ≈ -8.5 dB reduction in beamforming gain.  Similarly, the number of PA elements is reduced from 45 × 45 to 17 × 17, leading to an additional ~8.5 dB reduction. Consequently, EISREFSENS_50MHz degrades from –122 dBm to –113.5 dBm, and the minimum EIRP decreases from 60 dBm to approximately 43 dBm.

Proposal 5. It is proposed to use EISREFSENS_50MHz ≤ –113.5 dBm and Min EIRP = 43 dBm for LEO, as the starting point for discussions on a compact 20x20cm array aperture.

Observation 8. Although the theoretical value of n in a Cosⁿ(θ) type attenuation model is 1, the scan loss increases as the number of elements grows. In addition, Beamforming efficiency also decreases as the number of elements increases, and performance variation must be considered depending on manufacturing quality.  The reduction in the number of elements from 2,025 (45 × 45) to 289 (17 × 17) is expected to result in an array gain loss that is approximately 1–2 dB smaller. [2].

Proposal 6. Taking into account all system-level considerations mentioned in Observation 7 and combining them with observation 8, the proposed targets for a compact 20x20cm array aperture becomes EISREFSENS_50MHz ≤–115.5 dBm and Min EIRP = 45 dBm for LEO operation.

Observation 9. Free Space Path Loss LEO 600 and 1200 @ Elevation angle 30 degree is around 176 dBm and 181 dBm.  Therefore, Link Margin = (EIRP – Pathloss) – (EISREFSENS) can be calculated as (Assume Min EIRP = 45 dBm)

|  | EIRP of mobile VSAT (dBm) | Pathloss (dB) | EIS_REFSENS (dBm) | Link Margin (dB) |
| --- | --- | --- | --- | --- |
| LEO 600 w/ 30o | 45 | 176 | -127 ~ -136 | -4 ~ 5 |
| LEO 1200 w/ 30o | 45 | 181 | -127 ~ -136 | -9~ 0 |

Where EIS_REFSENS (dBm) = EISREFSENS_50M -4+ ΔFR1_REFSENS (G-FR1-NTN-A1-4 (Note 1))

EISREFSENS_50M = -120 ~ -129 for LEO

|  | EIRP of mobile VSAT (dBm) | Pathloss (dB) | EIS_REFSENS (dBm) | Link Margin (dB) |
| --- | --- | --- | --- | --- |
| LEO 600 w/ 30o | 45 | 176 | -133.8 ~ -142.8 | 2.8 ~ 11.8 |
| LEO 1200 w/ 30o | 45 | 181 | -133.8 ~ -142.8 | -2.2 ~ 6.8 |

Where EIS_REFSENS (dBm) = EISREFSENS_50M -10.8+ ΔFR1_REFSENS (G-FR1-NTN-A1-2 (Note 1))

EISREFSENS_50M = -120 ~ -129 for LEO

Proposal 7. Based on the UpLink Link Margin analysis, it appears that LEO communication is feasible using a 20 by 20 VSAT configuration at least up to 50MHz CBW.

Issue 1-2-3: ACLR and ACS for the new VSAT with small antenna

In this analysis we compare the radiation patterns of the electronically steered antennas used in the co-existence analysis [3] with the radiation pattern of potentially smaller (15x15 elements) electronically steered antenna and analyze the power that is emitted or received by the VSAT to understand if additional co-existence analysis is needed or if existing results can be used to justify the reuse of VSAT ACLR and ACS values.

The analysis is only done by tilting the beam in -plane. Analysis is done in single plane to make it easier to visualize the results and compare them. While the beam can be tilted in two-dimensions ( and ) to create 3D antenna pattern in real applications we believe that this analysis also describes the functionality and characteristics 3D case due to symmetrical design of the panel(s) that have been used in the co-existence simulations.

The antenna parameters used in the analysis are shown in table below and are in line with values used in co-existence simulations. It’s to be noted that these parameters are quite ideal, although some implementation margin is included, and should not be directly used as reference when defining the VSAT RF requirements.

Table 1 Antenna model parameters for low-cost implementation

| Matrix size / Elements | N=15x15= 225 | N=30x30= 900 | N=45x45= 2025 | N=78x78= 6084 |
| --- | --- | --- | --- | --- |
| Roll-off model cosn(θ) | n= 1.5 | n= 1.5 | n= 1.5 | n= 1.5 |
| Beam gain model alog2(N) | a= 3.0 | a= 3.0 | a= 3.0 | a= 3.0 |
| Element gain (dB) with IM | 3.5 | 3.5 | 3.5 | 3.5 |
| Peak gain (dB) = 0deg | 27 | 33 | 36.6 | 41.3 |
| Peak gain (dB) = 60deg | 21.8 | 27.7 | 31.2 | 36 |

When PA output power of 6.9 and 9.1dBm is used per element the corresponding TRP and Peak EIRP are shown in table below and these are also in line with the Table 6a.2.2.2-1of [3].

Corresponding 2D radiation patterns for different beam steering angles () are shown in Figure 1.

Table 2 Antenna model parameters for low-cost implementation

| Matrix size / Elements | N=15x15= 225 | N=30x30= 900 | N=45x45= 2025 | N=78x78= 6084 |
| --- | --- | --- | --- | --- |
| PA output power | 6.9 | 6.9 | 6.9 | 9.1 |
| TRP (dBm) | 30.4 | 36.4 | 40.0 | 46.9 |
| Peak power = 0deg (dBm EIRP) | 57.4 | 69.4 | 76.4 | 88.2 |
| Peak power = 60deg (dBm EIRP) | 52.7 | 64.8 | 71.9 | 83.6 |

Figure 1.  -axis radiation patterns with different electrical  tilt angles

Figure 4. Radiation pattern comparison compares the radiation patterns of 45x45 and 78x78 matrix with smaller 15x15 element matrix.

Figure 2. Emissions when integrated over 5-degree window and comparison of 78x78 to 15x15 and 45x45 to 15x15 0-degree patterns shows the radiation patterns when integrated using 5-degree wide window to reduce the impact of notches when comparing patterns with one another. Figure also shows the comparison for pattern with 0-degree beam direction.

Figure 3. Comparison of all beam directions shows the delta between the larger and smaller matrix for all beam directions [-70:10:70]. Positive delta indicates that emission power with larger matrix is higher than with smaller one.

Figure 2. TX emissions when integrated over 5-degree window and comparison of 78x78 to 15x15 and 45x45 to 15x15 with 0-degree beam direction

Figure 3. Comparison of TX emissions for all beam directions

Figure 4. Relative antenna gains when integrated over 5-degree window and comparison of 78x78 to 15x15 and 45x45 to 15x15 with 0-degree beam directions shows the relative antenna gains (Beam peak= 0dB) when integrated using 5-degree wide window to reduce the impact of notches when comparing patterns with one another. Figure also shows the comparison for pattern with 0-degree beam direction. These plots can be used to estimate the delta between the antenna matrixes in spatial isolation that improves the receiver capability to tolerate interference arriving from other direction than beam peak.

Figure 5. Comparison of relative antenna gains for all beam directions shows the delta between the larger and smaller matrix for all beam directions [-70:10:70]. Negative delta indicates that relative gain delta with larger matrix is higher than with smaller one i.e. there’s higher isolation towards interference from other than beam peak direction.

Figure 4. Relative antenna gains when integrated over 5-degree window and comparison of 78x78 to 15x15 and 45x45 to 15x15 with 0-degree beam direction

Figure 5. Comparison of relative antenna gains for all beam directions

Following observations and proposals can be made:

Observation 10: The radiated power of the 15x15 element matrix to any of the directions is always smaller that the radiated power of larger matrixes (e.g. 45x45 and 78x78) when PA output power per element is kept the same i.e. the TRP is a function of the antenna elements.

Observation 11: The minimum delta between the 15x15 to 78x78 and 15x15 to 45x45 is 13.7dB and 6.3dB respectively indicating that emission power of 15x15 antenna matrix is always significantly lower than emissions of 45x45 and 78x78 matrixes. 15x15 element matrix emissions reach the same level of emissions as larger matrixes, when conducted power per element is 6.9+6.3= 13.2dBm for 45x45 and 6.9+13.7= 20.6dBm in case of 78x78 element matrix is used as reference.

Observation 12: Due to reciprocity of the antennas (similar radiation characteristics for TX and RX) the interference power from TN network to smaller NTN VSAT (with 20x20cm form factor) receiver is lower than with larger matrixes (e.g. 45x45 and 78x78). In the case that interfering signal arrives from the direction of the side-lobe it is possible that VSAT with smaller antenna does not enjoy similar spatial isolation improvement as the larger matrixes as gain delta between the main lobe (own signal) and side-lobe(s) is smaller, however the impact of this to VSAT ACS depends a lot on the geometry on the network and interference statistics. In addition, the ACS for Ku-Band VSAT is limited by the implementation as concluded also during Rel-19 Ku-band co-existence analysis.

Proposal 8: Based on observations 10 to 11 the interference from smaller NTN VSAT (with 20x20cm form factor) to TN network is always lower than with larger matrixes (e.g. 45x45 and 78x78). Considering also the observation 12 there seems to be no need to repeat the co-existence simulations for NTN VSAT to TN interference nor for TN to NTN VSAT interference and it is proposed that existing Ku-band FDD ACLR and ACS requirements are reused for the VSAT with smaller antenna aperture size like 20x20cm.

# Conclusions

Sub-topic 1-1 : Issues for HD-FDD requirements.

Issue 1-1-1: Antenna type assumption for HD-FDD VSAT

Proposal 1. Both single-feed and dual-feed topologies can be used and other architecture are neither excluded. From the above mentioned two approach the dual-feed topology is preferred because it does not require an RF switch, offering advantages in cost, ant. design simplicity and elimination of switching time delay.

Issue 1-1-3: Transmit ON/OFF time mask for HD-FDD

Proposal 2 Consider existing Transmit ON/OFF time mask requirement to be sufficient to ensure that HD-FDD device can also protect its own receiver from Tx self-interference and hence there is no need to introduce new RF requirements for RX-TX or TX-RX switching.

Issue 1-1-5 & 1-1-6: EIRP/EIS requirements for HD-FDD

Proposal 3. The observed possible 0.5 dB better performance in EIRP/EIS for HD-FDD vs. FDD is considered attractive, but not as mandatory modification to the minimum requirement. Therefore, it is considered reasonable to either maintain the existing EIRP/EIS requirement or consider 0.5dB improvement to the EIRP/EIS specifications when compared to the full-duplex configuration, as both options remain within the measurement tolerance.

Sub-topic 1-2 : Issues for VSAT with Small Antennas

Issue 1-2-2: Assumptions for VSAT with small antenna

Proposal 4. Adopt 0.5 λ (11.9 mm) spacing at 12.6 GHz for Ku-band as working assumption for analysis of 200 mm × 200 mm aperture and enabling a 17 × 17 array with stable beamforming performance.

Proposal 5. It is proposed to use EISREFSENS_50MHz ≤ –113.5 dBm and Min EIRP = 43 dBm for LEO, as the starting point for discussions on a compact 20x20cm array aperture.

Proposal 6. Taking into account all system-level considerations mentioned in Observation 7 and combining them with observation 8, the proposed targets for a compact 20x20cm array aperture becomes EISREFSENS_50MHz ≤–115.5 dBm and Min EIRP = 45 dBm for LEO operation.

Link Margin = (EIRP – Pathloss) – (EISREFSENS) can be calculated for 50MHz CBW as follows:

|  | EIRP of mobile VSAT (dBm) | Pathloss (dB) | EIS_REFSENS (dBm) | Link Margin (dB) |
| --- | --- | --- | --- | --- |
| LEO 600 w/ 30o | 45 | 176 | -127 ~ -136 | -4 ~ +5 |
| LEO 1200 w/ 30o | 45 | 181 | -127 ~ -136 | -9~ 0 |

Where EIS_REFSENS (dBm) = EISREFSENS_50M -4+ ΔFR1_REFSENS (G-FR1-NTN-A1-4 (Note 1))

EISREFSENS_50M = -120 ~ -129 for LEO

|  | EIRP of mobile VSAT (dBm) | Pathloss (dB) | EIS_REFSENS (dBm) | Link Margin (dB) |
| --- | --- | --- | --- | --- |
| LEO 600 w/ 30o | 45 | 176 | -133.8 ~ -142.8 | +2.8 ~ +11.8 |
| LEO 1200 w/ 30o | 45 | 181 | -133.8 ~ -142.8 | -2.2 ~ +6.8 |

Where EIS_REFSENS (dBm) = EISREFSENS_50M -10.8+ ΔFR1_REFSENS (G-FR1-NTN-A1-2 (Note 1))

EISREFSENS_50M = -120 ~ -129 for LEO

Proposal 7. Based on the Uplink Link Margin analysis, it appears that LEO uplink communication is feasible using a 20x20cm VSAT configuration at least up to 50MHz CBW and it is proposed to use the EISREFSENS_50MHz ≤ ––115.5 dBm and Min EIRP = 45 dBm as starting points in defining the requirements for VSAT with smaller antenna aperture size of ~20x20cm.

Issue 1-2-3: ACLR and ACS for the new VSAT with small antenna

Proposal 8: Based on observations 10 to 11 the interference from smaller NTN VSAT (with 20x20cm form factor) to TN network is always lower than with larger matrixes (e.g. 45x45 and 78x78). Considering also the observation 12 there seems to be no need to repeat the co-existence simulations for NTN VSAT to TN interference nor for TN to NTN VSAT interference and it is proposed that existing Ku-band FDD ACLR and ACS requirements are reused for the VSAT with smaller antenna aperture size of ~20x20cm.

# References

[1] R4-2515082 Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2, 3GPP TSG-RAN4 Meeting #116 bis

[2] R4-2513711 On Ku-band Half-Duplex RF requirements with FR1-Numerology, 3GPP TSG-RAN4 Meeting #116 bis

[3] R4-2508774, Ku band co-existence simulation assumptions (running document), Ericsson, TSG-RAN WG4 Meeting #115, May 2025