3GPP TSG-RAN WG4 Meeting #117	R4-2520705

Dallas, USA, Nov 17th – 21st, 2025

Source:	Nokia

Title:	On General Aspects of AI/ML for NR Air Interface Phase 1

Agenda item:	6.11.2

Document for:	Discussion

Introduction

In RAN4#116 and RAN4#116bis meetings, RAN4 continued the discussion on the general aspects of AI/ML based on the objectives agreed in RAN4#107, WI ‎‎[1].

In this paper, we share our further views on the generalization related aspects, mainly for UE sided model cases:

Discussion

Generalization related aspects

The following Agreements were reached in RAN4#115:

Agreement: The high level and general guidelines for the generalization test are provided as follow. The exact decisions are to be made case by case. - Study and, if feasible, define requirements for each AI/ML functionality - Define one test per UE capability as a minimum - FFS >1 test per UE capability - Define a minimum set of test configurations (including NW sided conditions if any) as mandatory for testing of AI/ML-enabled Feature Potential areas to consider for generalization testing: - gNB array parameters - port layouts, array size, antenna virtualization - propagation conditions - Deployment scenarios - Carrier frequencies - Speeds - Indoor/outdoor - Bandwidths - SNR

Furthermore, on the issue of static and non-static scenarios for testing, following agreement was reached in RAN4#114bis meeting:

Agreement: - Static scenarios are assumed by default. Non-static scenarios are introduced on a use-case specific basis if needed for testing. - FFS whether a mixed datset can be created for testing generalization, and whether such a mixed dataset would be a static or non-static scenario

Generalization - criteria and test method

The generalization capability of a functionality can impact the performance of the functionality, for example if the functionality is not generalized enough across a set of scenarios, this will result in frequent switch of the functionality resulting in the degradation of the performance.

Less generalized functionalities across a set of scenarios can result in frequent switching of functionality/configurations resulting in performance degradation.

To mitigate the problems caused by less generalized functionalities, it is important to ensure that the functionality is generalized enough. Ensuring the generalization aspects of functionality across different scenarios and conditions is not straightforward. At the outset, we need the scenarios and conditions that needs to be identified and aligned for each use case. The following agreement was reached in RAN4#116bis meeting for CSI prediction use case [2].

Agreement: Test setup for the generalization will be further discussed based on the following options - Option 1: Introduce multiple tests with different MCS, which will lead to different SNR points. - Option 2: Introduce multiple tests with different channel models. The details of the channel models are FFS. - Option 3: the combination of option 1 and 2. It is FFS on how to quantify the generalization performance.

In order to address the FFS regarding the quantification of generalization performance of AI/ML-based functionalities, we propose a new method, in which the aim is to verify if a given AI/ML functionality fulfils the agreed generalization criteria or not. Such a test can be used to verify/check whether an AI/ML functionality (developed and validated for a specific baseline setup) generalizes well to other scenario(s) and/or configuration(s) of interest.

How to test Generalization?

The underlying principle of the Generalization criteria is to evaluate the performance of an AI/ML functionality across multiple test setups beyond the baseline setup. For instance, for the sake of illustration, a binary verdict can be determined, that is, “1” if the performance requirements are fulfilled for a scenario/test setup, and “0” otherwise. Then, the average can be a high-level indicator of the generalization of the functionality under the test.

To enable effective evaluation of the generalization aspects of an AIML enabled functionality, it is important to:

- Identify and align the relevant scenarios and configurations. RAN4 to strive to only select a limited number of scenarios.

- Identify target values of performance requirements depending on the scenarios and use cases.

- Establish a test method to evaluate the functionality across all identified setups.

Such a method, coupled with the proposed Generalization criteria, will provide RAN4 with a standardized mechanism to evaluate and compare the generalization performance of AI/ML-based functionalities (e.g., BM, CSI Feedback) across multiple deployment conditions.

RAN4 should consider generalization testing covering following aspects:

- Identify and align the relevant scenarios and configurations. RAN4 to strive to only select a limited number of scenarios.

- Identify target values of performance requirements depending on the scenarios and use cases.

- Establish a test method to evaluate the functionality across all identified setups.

Conclusion

In this contribution, we addressed the following topics mainly for UE sided model cases:

- Generalization related aspects

Following Observation and Proposal were made in this paper:

Observation 1: Less generalized functionalities across a set of scenarios can result in frequent switching of functionality/configurations resulting in performance degradation.

Proposal 1: RAN4 should consider generalization testing covering following aspects:

- Identify and align the relevant scenarios and configurations. RAN4 to strive to only select a limited number of scenarios.

- Identify target values of performance requirements depending on the scenarios and use cases.

- Establish a test method to evaluate the functionality across all identified setups.

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#107, March 12-14, 2025, Incheon, South Korea.

- R4-2514631, “WF on requirements for AIML air interface part 1”, Qualcomm Incorporated, RAN4#116bis, Prague, Czech Republic, 13th ‒ 17th October, 2025.