3GPP TSG-RAN WG4 Meeting #117	R4-2520710

Dallas, USA, November 17th –  21st, 2025

Source:	Nokia

Title:	6G AI/ML General Aspects

Agenda item:	8.8.1

Document for:	Discussion

Introduction

During RAN4#116bis meeting, RAN4 started discussing 6G AI topics. Agreements achieved during RAN4#116bis meeting are summarized in the WF ‎[1]. The following agreements were reached on the study scope and the timeline of RAN4 6G AI study:

Agreement: - In 6G AI/ML study, the detailed RAN4 scope shall include - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… - RAN4-driven use cases: RAN4 shall identify, select and study only those AI/ML use cases that fall within RAN4 core responsibility. The detailed principles are captured under 3-1. - Other WG-driven use cases: RAN4 shall assist other working groups in these use cases by studying RAN4 impacts with RAN4 analysis and evaluation relevant to implementation and performance. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures: RAN4 can study relevant RAN4 requirements for LCM procedures. If the use cases are identified by other WG, RAN4 related study should be based on the agreements and/or requests by other WG. The exact LCM context will be separately discussed and decided in the responding WGs. - RAN4 may study the feasibility and testability of the AI/ML functionalities. The study may include different aspects. It will be further decided how to manage the discussion under different agenda items, e.g. AI or testability.

Agreement: - RAN4 timeline for Rel-20 6G AI/ML study - Stage 1: Q4'25 and Q1'26 - To clarify study scope for RAN4 AI/ML framework - To select the top-prioritized RAN4-led AI/ML use cases, with the detailed evaluation plan concluded for Stage-2 - Stage 2: Q2’26 to Q2’27 - To analyse and conclude RAN4 AI/ML framework (including RAN4 aspects for LCM procedures and other RAN4 specific issues) - To study comprehensively the RAN4-led use case(s) selected in Stage 1. - Other new use case(s) are not precluded depending on the consensus and the limited RAN4 bandwidth - Based on the agreement and/or the request from other WG, RAN4 will study the selected other WG-driven use cases with RAN4 analysis and evaluation. RAN4 related bandwidth limitation should be taken into consideration.

In this paper, we provide our further views on the general aspects related to RAN4 6G AI/ML. Specifically we discuss following topics:

- Post deployment enhancements

- AI/ML impacts on the measurement framework

Our views on RAN4 6G AI/ML use cases are discussed in our accompanying paper [2].

- Discussion

Post-deployment enhancements

In the last RAN1 meeting (RAN1#122) [3] the following agreement was reached regarding the feature selection for the study in 6G.

Agreement For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following: - Definition of each (sub-)use case, including at least AI/ML model input/output - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results - Assumption on training types, e.g., - offline training, online training/finetuning - Label construction (if applicable), including whether/how to obtain label data for model training - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model - Collaboration/interaction between UE and NW, e.g., - no collaboration/interaction - UE/Network collaboration targeting at separate or joint ML operation - High level potential specification impact

As it is mentioned in the above agreement that online training/finetuning are part of training assumptions for 6G, it becomes clear that more frequent updates in AI/ML functionalities will occur, and these updates may impact the behaviour of AI/ML functionalities in a significant manner. In the existing 5G-A, conventional (pre-deployment) conformance testing methods, the UE-side AI/ML enabled functionality configurations are tested against the set of KPIs for different requirements and test scenarios. However, due to more frequent (over-the-air) changes/updates in the implementation versions of the AI/ML features combined with the radio environmental conditions, the performance behaviour of the AI/ML functionalities can differ from what was tested earlier in lab conditions with previous implementation versions. Hence, the introduction of any testing/validation approach which can be applied in post-deployment, i.e., after initial performance certification in the lab, would be beneficial for 6GR AI/ML use cases.

In 5G-A Rel-19, it was agreed to use the performance monitoring framework to handle post-deployment testing. It was sufficient for Rel. 19 use cases since the scope was only AI/ML use cases with offline training. Now in 6GR the assumption on training types has widened. As can be seen from the RAN1#122 agreement [3], assumptions explicitly include new continuous learning methods such as online training/finetuning, dynamic label construction. This requires more than observation and monitoring, especially when the UE-side AI/ML enabled functionality has the capability of online training/finetuning.

Apart from the regular functionality updates and UE feedback reports, these cases require more from both the UE and the NW to ensure that the AI/ML functionalities are performing as expected throughout the updated functionality life cycle, e.g., exploration and exploitation phases due to online training.

For 6G AI/ML use cases, we can envisage a pre- and post- deployment testing and validation workflow as illustrated in Figure 1.

Figure 1: Post deployment testing and validation, as envisaged for 6GR UE-side AI/ML enabled features and use cases.

The figure is divided into 3 phases as described below.

- As a part of the pre-existing standard conformance tests, the AI/ML enabled functionality in the device is tested and conformed. This is the legacy way. Once this is done, the devices are deployed in the field.

Phases marked 2 and 3 in the Figure 1 is what we refer to as the Post Deployment Workflow. These phases 2 and 3 can happen in tandem based on the nature of update to the AIML enabled functionality.

- In-field post deployment validation:

  - Proactively, conduct the test of the update in the field and based on the results enable the AIML enabled functionality.

  - Reactively, enable the updated AIML enabled functionality and monitor it for the performance degradation and take suitable actions.

- In-lab post deployment testing: Where the devices re-conformed in the lab for the specific Features or Feature Groups before enabling the updated functionality in the field.

Now with the kind of new 6G features that different working groups are considering, and the complexity of testing AI/ML enabled functionalities involving online training and finetuning, the post deployment testing & validation procedures should be robust enough to cater to all kind of in-the-lab and in-the-field scenarios.

New aspects related to post-deployment testing to be considered in 6G are:

- Significant update to the in-lab test framework, test requirements, conformance testing and certification [RAN4].

- A new framework for in-field post-deployment validation [RAN2, RAN4]

- Updates to ML management procedures required to support in-field validation of AI/ML functionalities [RAN2]

To achieve this, we propose a new RAN4 study on post-deployment testing and validation and its enhancement in tight collaboration with RAN2, to ensure a robust post-deployment testing and validation framework and its requirements are in place for 6G use cases from Day 1. In [4], Nokia has provided views regarding the need to design in-field post-deployment validation mechanisms.

RAN4 to further discuss and consider studying post-deployment testing and validation enhancements in 6G in collaboration with RAN2 from early phases of the 6GR SI.

AI/ML impacts on the Measurement Framework

The application of AI/ML to RAN use cases has several advantages such as measurement reduction, improved and timely decisions for operations such as handover, radio link management and beam switching etc. However, these use cases have also added predicted measurements in the reports in addition to the physically performed measurements. There are several open points regarding these reports which may contain either predicted measurements or physically performed measurements or (a combination of) both. This issue requires further RAN4 attention since different types of measurements and predictions might have different delay and accuracy requirements.

Accuracy of predicted measurements

During previous study/work items on beam management and AI/ML Mobility, it has been observed that UE may report to the network either the predicted measurements or the physically performed measurements.

In 6G, for the physically performed measurements report, the common understanding is to follow the Non-AI/ML measurement accuracy requirements for both L1-RSRP and L3-RSRP measurements. However, for predicted RSRP accuracy requirements, it has not been yet  agreed to follow the same requirements as of the measured RSRP accuracy. The predicted RSRP accuracy may differ from the physically performed measurement accuracy due to the following reasons:

- The AI/ML model would be likely trained on a dataset that consists of physically performed measurements from the field or the synthetic data. Therefore, the physically performed measurement would be the upper bound for the performance of the AI/ML model in case of best predictions. Hence, their accuracy would be in general less than the physically performed measurements.

- Although in case of skipped measurements (e.g., Temporal domain Case-B), where skipped measurements may be replaced by predicted measurements,   lower accuracy in prediction might not be justified. The argument would be that other non-AI/ML based techniques may also result in similar lack of accuracy while applying measurement reduction. Therefore, the additional complexity of AI/ML based solutions may not be justified. However, in case of future predictions (such as Temporal domain Case-A prediction of AI/ML BM/Mobility use cases) depending on the duration of prediction window  reduced accuracy would be expected and it is understandable.

The above-described loss of accuracy in reported predicted measurements may impact Network sided decisions and therefore it should be studied thoroughly in RAN4. RAN4 should consider the following aspects in order to study any relaxation in predicted measurement reports:

- Is it feasible to consider different accuracy classes for different reports?

  - More accurate reports for physically performed measurements

  - Less accurate reports for predicted measurements

- How may NW use more and less accurate reports?

- What level of relaxation can be allowed for predicted measurement reports without impacting overall performance?

- How to ensure that predicted measurements and physically performed measurements, having different level of accuracy, can be distinguished on the network side? Network may need to be indicated about different types of reports as listed below:

  - Reports based only on physically performed measurements

  - Reports based on physically performed measurements mixed with predicted results

  - Reports based only on predicted results

- How to mitigate, using network-controlled mechanisms, any potential negative impact on the overall performance due to loss of accuracy in the reports?

- Whether the UE is allowed to send predication with lower accuracy?

Depending on the use-case, different levels of prediction accuracy may be required at the network and in some scenarios lower accuracy of predicted reports may have adverse effects on Network decisions for operations such as handover and beam switching etc.

Potential improvement of prediction accuracy

As discussed in the above section, one of the reasons of loss of accuracy in predicted measurements is the training dataset consisting of physically performed measurements containing measurement errors and/or synthetic data as well as the use of physically performed measurement as input to AI/ML during inference phase. These errored measurements become the upper bound for the prediction performance. During the study on AI/ML BM use case, it was observed that if an improvement in prediction accuracy is desired, then a further enhancement in physically performed measurement accuracy requirements would be needed. However, RAN4 didn’t really discuss how much relaxation in prediction accuracy may be acceptable in order to ensure correct Network sided decisions and how to improve prediction accuracy if the required accuracy is not achieved. RAN4 may consider the following aspects to improve prediction accuracy:

- Enhancement in physically performed measurement accuracy requirements used in AI/ML systems.

- Preference to cluster-based approach wherever possible to guarantee minimum required performance

- Capabilities of UE supporting improved accuracy for physically performed measurements

Since physically performed measurements would be used for initial training of AI/ML functionalities before deployment as well for in-field fine-tuning and retraining, therefore, they become the upper bound of the prediction performance.

Stricter requirements for physically performed measurements, prioritization of cluster-based approaches and support of UEs capable of improved accuracy for physically performed measurements are some of the potential ways to improve the predicted measurement accuracy.

Impact on reporting delay requirements

As discussed in Section 2.1.2.1, we may have measurements with lower accuracy when using AI/ML-based measurement prediction. In case different classes of measurements are defined based on their accuracy, RAN4 needs to study whether and how to define different reporting delay requirements for such measurements. The following aspects can be considered by RAN4 when studying the impact on reporting delay requirements:

- Enhanced reporting delay requirements for all measurement classes (i.e., with lower and higher accuracies)

- Enhanced reporting delay requirements for measurements with lower accuracy compared to measurements with higher accuracy

Reporting delay requirements may differ for different types of measurement classes with lower or higher accuracies.

Impact on measurement gaps configuration

During measurement gaps, the UE must temporarily stop communicating with the serving cell to measure, e.g., other frequencies. AI/ML techniques can help reduce the number of measurements a UE needs to perform. Hence, the impact on measurements gap will be significant. Below we highlight a few. RAN4 needs to study whether and how to define different requirements for such measurement gap configurations. The following aspects can be considered by RAN4 when studying the impact on measurement gap configurations:

- Reduction in Measurement Gaps Configurations: With the help of AIML predictions, less measurement gaps configurations will be required. One of the factors that may impact measurement gap configuration is the prediction horizon of AIML model.

- Dynamic measurements gap configuration: Instead of static, periodic gaps, NW could use AI/ML-assisted predictions to configure gaps dynamically, only when necessary.

- Simplifications of Measurement Gap Patterns (Table 9.1.2-1, TS38.133: Gap Pattern Configurations):

  - Depending on AIML capability on UE side, measurement gap pattern might be configured accordingly.

  - Measurement Gap length (MGL) and Measurement Gap Repetition Period (MGRP) might be impacted as well.

  - Table 9.1.2-1 might be extended or reduced depending on AIML capabilities.

For AI/ML based predictions, RAN4 to study the impacts on measurement framework. More precisely, RAN4 to study the following aspects:

- Accuracy of predicted measurements

- How to mitigate adverse impacts of less accurate predicted measurements

- Impact on reporting delay requirements

- Impact on measurement gaps configuration

Conclusion

In this contribution, we share our views on general AI/ML aspects and framework in 6G. Specifically, we cover following topics:

- Post-deployment enhancements

- AI/ML impacts on the Measurement Framework

The following Observations and Proposals were made.

Observation 1: New aspects related to post-deployment testing to be considered in 6G are:

- Significant update to the in-lab test framework, test requirements, conformance testing and certification [RAN4].

- A new framework for in-field post-deployment validation [RAN2, RAN4]

- Updates to ML management procedures required to support in-field validation of AI/ML functionalities [RAN2]

Proposal 1: RAN4 to further discuss and consider studying post-deployment testing and validation enhancements in 6G in collaboration with RAN2 from early phases of the 6GR SI.

Observation 2: Depending on the use-case, different levels of prediction accuracy may be required at the network and in some scenarios lower accuracy of predicted reports may have adverse effects on Network decisions for operations such as handover and beam switching etc.

Observation 3: Since physically performed measurements would be used for initial training of AI/ML functionalities before deployment as well for in-field fine-tuning and retraining, therefore, they become the upper bound of the prediction performance.

Observation 4: Stricter requirements for physically performed measurements, prioritization of cluster-based approaches and support of UEs capable of improved accuracy for physically performed measurements are some of the potential ways to improve the predicted measurement accuracy.

Observation 5: Reporting delay requirements may differ for different types of measurement classes with lower or higher accuracies.

Proposal 2: For AI/ML based predictions, RAN4 to study the impacts on measurement framework. More precisely, RAN4 to study the following aspects:

- Accuracy of predicted measurements

- How to mitigate adverse impacts of less accurate predicted measurements

- Impact on reporting delay requirements

- Impact on measurement gaps configuration

- References

- R4-2514637, WF on 6G AI, RAN4#116bis, Samsung, RAN4#116bis, Prague, Czech Republic, 13th ‒ 17th October, 2025.

- R4-2520711, AI/ML use cases in 6G, Nokia, RAN4#117 meeting, Dallas, USA, 17th – 21st November 2025.

- R1-2506456, Moderator summary #2 on AI/ML for 6GR, Samsung, 3GPP TSG RAN WG1 #122, Bengaluru, Aug 25th – 29th, 2025.

- R2-2508355, “On Management and Model Transfer for AI/ML, and Data Collection”, Nokia, RAN2#132 meeting, Dallas, USA, 17th – 21st November 2025.