3GPP TSG-RAN WG4 Meeting #117	R4-2520711

Dallas, USA, November 17th – 21st, 2025

Source:	Nokia

Title:	AI/ML use cases in 6G

Agenda item:	8.8.2

Document for:	Discussion

Introduction

During RAN4#116bis meeting, RAN4 started discussing 6G AI topics. Agreements achieved during RAN4#116 meeting are summarized in the WF ‎[1]. The following agreements were reached on the study scope and the timeline of RAN4 6G AI study:

Agreement: - In 6G AI/ML study, the detailed RAN4 scope shall include - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… - RAN4-driven use cases: RAN4 shall identify, select and study only those AI/ML use cases that fall within RAN4 core responsibility. The detailed principles are captured under 3-1. - Other WG-driven use cases: RAN4 shall assist other working groups in these use cases by studying RAN4 impacts with RAN4 analysis and evaluation relevant to implementation and performance. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures: RAN4 can study relevant RAN4 requirements for LCM procedures. If the use cases are identified by other WG, RAN4 related study should be based on the agreements and/or requests by other WG. The exact LCM context will be separately discussed and decided in the responding WGs. - RAN4 may study the feasibility and testability of the AI/ML functionalities. The study may include different aspects. It will be further decided how to manage the discussion under different agenda items, e.g. AI or testability.

Agreement: - RAN4 timeline for Rel-20 6G AI/ML study - Stage 1: Q4'25 and Q1'26 - To clarify study scope for RAN4 AI/ML framework - To select the top-prioritized RAN4-led AI/ML use cases, with the detailed evaluation plan concluded for Stage-2 - Stage 2: Q2’26 to Q2’27 - To analyse and conclude RAN4 AI/ML framework (including RAN4 aspects for LCM procedures and other RAN4 specific issues) - To study comprehensively the RAN4-led use case(s) selected in Stage 1. - Other new use case(s) are not precluded depending on the consensus and the limited RAN4 bandwidth - Based on the agreement and/or the request from other WG, RAN4 will study the selected other WG-driven use cases with RAN4 analysis and evaluation. RAN4 related bandwidth limitation should be taken into consideration.

In this paper, we provide our further views on the following main topics related to RAN4 6G AI/ML use cases:

- Use cases related to RF

- Use cases related to Demod

- Use cases related to RRM

Our views on general aspects and RAN4 AI/ML framework are discussed in our accompanying paper [2].

- Discussion

Use cases related to RF

The following agreement was reached in RAN4#116bis meeting regarding use cases for AI-nonlinearity compensation.

Agreement: - The scope of potential RAN4-led AI use case, AI-DPD/DPoD, will be further discussed in the next meeting. The candidate sub-use cases include - UL: - UE: AI-DPD - NW: AI-DPoD - DL: - NW: AI-DPD - UE: AI-DPoD - To facilitate the 6G AI scope discussion, the interested companies are encouraged to timely provide more analysis, including the performance benefit, spec impact, the assumption behind their analysis, on the specific sub-use case(s).

Here we provide our view regarding AI based DPD for UL.

AI based DPD for UL

For UL traffic, AI-based nonlinearity correction can be achieved using digital predistortion (DPD) done using UE sided AI/ML models. Performance improvements may be expected in comparison to non-AI based solution.

OFDM systems suffer from high peak to average power ratio which cause the power amplifier (PA) to introduce nonlinear distortions to the transmitted signal. To counteract this effect, the PA operates with power backoff, which reduces PA efficiency, the output power, and ultimately, the cell coverage. Traditionally, PA nonlinearity has been handled by imposing TX emission requirements. In order to meet those requirements, the transmitter may apply a digital-predistortion (DPD) mechanism that pre-compensates the PA nonlinearity (typically also using a high-resolution feedback receiver), and/or it may limit the maximum transmit power.

Since (1) the traditional DPD hardware implementation cost is very high, and (2) the power constraint naturally limits the cell coverage, there is a question of whether these emissions can be addressed differently, and/or whether the emission requirements could subsequently be relaxed.

AI-based methods have shown potential to handle the PA nonlinearity e.g. as shown in Figure 1. below where an AI-based DPD is applied to the transmitted signal before said signal is passed to the PA.

Figure 1 : BLER when AI DPD is applied at TX. Carrier Frequency=3.5GHz, UE Speed=0, Delay Spread=100ns, 64QAM, TX EVM~13%. Trainable Parameters =418K.

UE-sided AI-based solutions compensation show potential for coping with PA nonlinearity.

The following table captures the view on the AI-based DPD use case.

|  | AI-based DPD |
| --- | --- |
| Model input | Time domain samples before pre-distortion |
| Model output | Time domain samples after pre-distortion |
| Label | Time domain samples |
| Training types | Offline training Online training/finetune2 |
| KPI | BLER, EVM, MPR |
| Benchmark | No DPD |
| Model location for inference | UE-sided model |
| Collaboration/interaction between UE and NW | Similar to UE-sided model as NR |
| Potential specification impact | 1. RAN4 requirements, e.g. EVM 2. Tx power determination 3. Signaling/ procedure related to LCM for UE-sided model |

RAN4 to consider studying AI-based-DPD (UE sided model) as a RAN4-driven use case.

Use cases related to Demod

AI-based SRS IL compensation

According to the details provided in [1] and [3], the AI-based SRS Insertion Loss (IL) compensation use case is proposed to resolve the IL imbalance issue. This issue was identified in RAN4 in Rel-18, where an LS [4] was sent to RAN1 to describe the SRS IL imbalance issue in application to 8Rx UE architectures. RAN1 evaluated performance losses in case SRS IL is not compensated at the UE and/or gNB sides. However, no agreements were made to introduce RAN1 solutions to resolve the issue. In Rel-19, the SRS IL imbalance issue was part of the objectives of the NR_ENDC_RF_Ph4 WI [5].

The main specification impact expected from the AI-based SRS IL compensation use case proposed in [3] is the UE reporting of SRS IL range. As quoted in the agreements below made in the RAN4#114bis meeting [6], potential solutions for the SRS IL imbalance issue were discussed in RAN4 in Rel-19 NR_ENDC_RF_Ph4 WI, including the SRS IL imbalance UE assistance and reporting.

Sub-topic 4-2: SRS IL imbalance issue solutions Way forward - RAN4 to further discuss the candidate SRS IL imbalance solution framework based on agreements for issues 4-2-1 and 4-2-2. - Companies are encouraged to provide views on additional aspects including - SRS output power testing - Enhancements to PCmax equations to improve SRS IL compensation Issue 4-2-1: SRS IL imbalance UE self-compensation Agreement: - RAN4 assumptions on UE behavior for Rel-19 SRS IL handling: - For SRS transmissions Rel-19 UE is expected to perform self-compensation of SRS IL up to its configured maximum output power - FFS if any additional clarifications on behavior are needed - No RAN1/4 specification impacts (i.e. this behavior is aligned with existing specifications) Issue 4-2-2: SRS IL imbalance UE assistance and reporting Agreement: - The following two possible general solutions may be considered for further consideration - Solution 1: Introduce UE capability indication on whether UE performs SRS IL self-compensation - FFS on the detailed solution including clarification on self-compensation and whether static or dynamic indication will be needed - Solution 2: Extend Type 3 SRS PHR reporting to carriers with PUSCH subject to RAN1 confirmation on feasibility - Further check whether solution 2 is in the scope or not - Considerations of any solutions should be based on system level throughput evaluations, and implementation complexity

In Rel-19 NR_ENDC_RF_Ph4 WI, RAN4 discussed solutions like SRS IL imbalance UE self-compensation and SRS IL imbalance UE assistance and reporting.

As we are at the beginning of the 6G study, solutions for SRS IL compensation can be discussed as part of 6G UE RF in RAN4 and/or 6G SRS in RAN1, especially if the SRS IL imbalance issue will be found more relevant for 6G systems. If non-AI solutions for the SRS IL compensation in 6G will be discussed and agreed in RAN4/RAN1, then we believe that there is no need to rely on AI/ML for SRS IL compensation in 6G.

If non-AI solutions for SRS IL compensation will be discussed and agreed in 6G in RAN4/RAN1, then there is no need to rely on AI/ML for SRS IL compensation.

Use cases related to RRM

The following agreement were reached in RAN4#116bis meeting:

Agreement: - Additional principle/method for AI-RRM use case selection - Whether the proposed AI-RRM use case should be driven by RAN4: - Existing 5G-A use cases included in RAN2 and RAN1 WIs should be precluded in RAN4 6G study. - Coordination with RAN1 and RAN2 when potential AI-RRM use cases are agreed in RAN4 to avoid overlapping scope. Agreement: - The scope of potential RAN4-led AI-RRM use case(s) will be further discussed in the next meeting.

Based on above agreements, we provide our view on the Table provided in the agreed WF [1]:

On RRM use cases, we provide our view in the Table below:

| Index | use cases | Model location (Training method) | Proposed by companies | Expected benefits (e.g., RS reduction, MG reduction etc.) | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- | --- |
|  | L1 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Spatial domain (inter-cell) - Time and spatial domain (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c), Nokia ((b, d, e), as RAN1-led study), Apple (c, d, e) | 1b and 1e: Multi domains (time and spatial) predictions allow a small subset of observations predict the rest with high reliability. Overhead reduction and better performance. 1d: This is an intercell extension of rel.19 intracell spatial domain beam prediction use case. The advantages are measurement reduction better performance by selecting the best beams in intercell scenario. | Some of these use cases are being discussed in RAN1 as a potential RAN1-led studies. We support L1 level beam predictions (b, d and e) as RAN1-led use cases. | Impact on prediction accuracy (accuracy requirements), reporting delay requirements. Potential impacts on known conditions for TCI states. Impacts on ML Management and performance monitoring should also be considered. |
|  | L3 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Time and freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) - Freq. domain prediction for N carriers single cell | NW and UE sided model (Offline training) | OPPO (a, b, c, e), Apple (c,d, e), CMCC (e), ZTE (e), Samsung (e, f), Nokia (a, b, c, d, e) | For 2a, the expected benefit would be timely and accurate mobility decision considering the quality and reliability of potential target beams. For 2b, the additional benefit would the RS reduction. For 2c, the benefit is low-latency inter-cell beam prediction using partial measurements, reducing RS overhead. For 2d, the additional benefit on top of 2a would be MG reduction, optimization of Cfra resources for handovers. For 2e, extension of frequency domain prediction to non-collocated cells. | RAN4 needs to check/coordinate with RAN2 for any potential conflict with RAN2 6G AI studies. | Impact on prediction accuracy (accuracy requirements), reporting delay requirements and measurement gaps configuration and scheduling restrictions. Impacts on ML Management and performance monitoring should also be considered. |
|  | L3 cell measurement: - Time domain (intra-cell) - Freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c) Apple (c), Nokia (c) | For (3c), similarly to 5GA AI/ML for mobility study goals, measurement effort reduction and handover performance improvement are expected. An additional benefit would be MG reduction. | (3a) and (3b) are already covered in the 5GA RAN2 AI/ML for mobility use cases. For (3c), RAN4 needs to check/coordinate with RAN2 for any potential conflict with RAN2 6G AI studies. | Impact on prediction accuracy (accuracy requirements), reporting delay requirements and measurement gaps configuration and scheduling restrictions. Impacts on ML Management and performance monitoring should also be considered. |
| 7a (7+8 combined) | Spatial domain RX beam sweeping reduction (L1/L3) | UE sided model | Apple (L1 for scheduling restriction and delay reductions), vivo Qualcomm, Apple, CMCC, Xiaomi, Nokia | Can reduce the number of RX beam measurements required to select the optimal RX beam for each TX beam | RAN4 may need to check/coordinate with RAN1 for any potential overlapping in AIML 6G study. | Potential impacts on measurements requirement and accuracy requirements of Tx-Rx beam pair prediction. Potential impacts on known conditions for TCI states. Impacts on ML Management and performance monitoring should also be considered. |

Multi-Domain L3 level Prediction for Measurement-Gap Reduction

Use case 4, multi-domain L3 level Prediction for Measurement-Gap Reduction, is already covered in sub-use cases listed in use case 2 for L3 beam level measurements. Therefore, there is no need to consider a separate use case for multi-domain L3 level predictions.

Multi-domain L3 level predictions can be considered in respective sub-use cases of use case 2. There is no need to study a separate case for multi-domain L3 level predictions.

Spatial Domain RX Beam Sweeping Reduction

Two use-cases were specified in Rel-19 for beam prediction: BM-Case1 (Spatial-Domain DL Tx Beam Prediction) and BM-Case2 (Temporal-Domain DL Tx Beam Prediction). In both use-cases, Set B identifies the set of beams for which measurements are used as input to the AI/ML model, and Set A identifies the set of beams for predictions obtained as output of the AI/ML model.

Rel-19 AIML beam management may present some limitations for 6G:

- Based on the Rel-19 assumptions for model input and following legacy BM principles, each TX beam in Set B should be measured multiple times to allow the UE to determine the RX beam. For identifying the optimal RX beam requires a complete sweep across all RX beams of the UE, resulting in a substantial number of measurements for each TX beam in Set B.

- Under the Rel-19 assumptions for model output and the newly introduced UE capability (R4-2514620), the RX beam corresponding to a UE-predicted TX beam in Set A is considered known if the TX beam is not QCL Type D to a known TCI or to a TX beam in Set B. To support this new UE capability in Rel-19, the known RX beam may be based on Tx-Rx beam pair prediction. However, further study and discussion may be needed because Top-1 Tx-Rx beam pair prediction may not be fully accurate. When using Tx-Rx beam pair prediction additional measurements - fewer than a full sweep of all RX beams - might be necessary to identify the best predicted RX beam corresponding to the best predicted TX beam.

Possible advantages of Tx-Rx beam pair prediction:

- can reduce the number of RX beam measurements required to select the optimal RX beam for each TX beam when measuring TX beams in Set B.

- can facilitate the support of the newly introduced Rel19 UE capability for AIML BM (UE capability related to known RX beam corresponding to the predicted TX beam).

- can be built on top of the Rel-19 DL Tx Beam prediction, following the same RAN4 assumptions and considering limited changes in RAN4 requirements.

RAN4 to consider studying Tx-Rx beam pair prediction (use case 7a) as a potential extension of DL Tx beam prediction to avoid some of the limitations in Rel-19 DL Tx beam prediction.

Conclusion

In this contribution, we share our views on RAN4 6G AI/ML use cases. Specifically, we cover following topics:

- Use cases related to RF

- Use cases related to Demod

- Use cases related to RRM

The following Observations and Proposals were made:

Observation 1: UE-sided AI-based solutions compensation show potential for coping with PA nonlinearity.

Proposal 1: RAN4 to consider studying AI-based-DPD (UE sided model) as a RAN4-driven use case.

Observation 2: In Rel-19 NR_ENDC_RF_Ph4 WI, RAN4 discussed solutions like SRS IL imbalance UE self-compensation and SRS IL imbalance UE assistance and reporting.

Proposal 2: If non-AI solutions for SRS IL compensation will be discussed and agreed in 6G in RAN4/RAN1, then there is no need to rely on AI/ML for SRS IL compensation.

Proposal 3: On RRM use cases, we provide our view in the Table below:

| Index | use cases | Model location (Training method) | Proposed by companies | Expected benefits (e.g., RS reduction, MG reduction etc.) | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- | --- |
|  | L1 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Spatial domain (inter-cell) - Time and spatial domain (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c), Nokia ((b, d, e), as RAN1-led study), Apple (c, d, e) | 1b and 1e: Multi domains (time and spatial) predictions allow a small subset of observations predict the rest with high reliability. Overhead reduction and better performance. 1d: This is an intercell extension of rel.19 intracell spatial domain beam prediction use case. The advantages are measurement reduction better performance by selecting the best beams in intercell scenario. | Some of these use cases are being discussed in RAN1 as a potential RAN1-led studies. We support L1 level beam predictions (b, d and e) as RAN1-led use cases. | Impact on prediction accuracy (accuracy requirements), reporting delay requirements. Potential impacts on known conditions for TCI states. Impacts on ML Management and performance monitoring should also be considered. |
|  | L3 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Time and freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) - Freq. domain prediction for N carriers single cell | NW and UE sided model (Offline training) | OPPO (a, b, c, e), Apple (c,d, e), CMCC (e), ZTE (e), Samsung (e, f), Nokia (a, b, c, d, e) | For 2a, the expected benefit would be timely and accurate mobility decision considering the quality and reliability of potential target beams. For 2b, the additional benefit would the RS reduction. For 2c, the benefit is low-latency inter-cell beam prediction using partial measurements, reducing RS overhead. For 2d, the additional benefit on top of 2a would be MG reduction, optimization of Cfra resources for handovers. For 2e, extension of frequency domain prediction to non-collocated cells. | RAN4 needs to check/coordinate with RAN2 for any potential conflict with RAN2 6G AI studies. | Impact on prediction accuracy (accuracy requirements), reporting delay requirements and measurement gaps configuration and scheduling restrictions. Impacts on ML Management and performance monitoring should also be considered. |
|  | L3 cell measurement: - Time domain (intra-cell) - Freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) | NW and UE sided model (Offline training) | OPPO (a, b, c) Apple (c), Nokia (c) | For (3c), similarly to 5GA AI/ML for mobility study goals, measurement effort reduction and handover performance improvement are expected. An additional benefit would be MG reduction. | (3a) and (3b) are already covered in the 5GA RAN2 AI/ML for mobility use cases. For (3c), RAN4 needs to check/coordinate with RAN2 for any potential conflict with RAN2 6G AI studies. | Impact on prediction accuracy (accuracy requirements), reporting delay requirements and measurement gaps configuration and scheduling restrictions. Impacts on ML Management and performance monitoring should also be considered. |
| 7a (7+8 combined) | Spatial domain RX beam sweeping reduction (L1/L3) | UE sided model | Apple (L1 for scheduling restriction and delay reductions), vivo Qualcomm, Apple, CMCC, Xiaomi, Nokia | Can reduce the number of RX beam measurements required to select the optimal RX beam for each TX beam | RAN4 may need to check/coordinate with RAN1 for any potential overlapping in AIML 6G study. | Potential impacts on measurements requirement and accuracy requirements of Tx-Rx beam pair prediction. Potential impacts on known conditions for TCI states. Impacts on ML Management and performance monitoring should also be considered. |

Proposal 4: Multi-domain L3 level predictions can be considered in respective sub-use cases of use case 2. There is no need to study a separate case for multi-domain L3 level predictions.

Proposal 5: RAN4 to consider studying Tx-Rx beam pair prediction (use case 7a) as a potential extension of DL Tx beam prediction to avoid some of the limitations in Rel-19 DL Tx beam prediction.

- References

- R4-2514637, WF on 6G AI, RAN4#116bis, Samsung, RAN4#116bis, Prague, Czech Republic, 13th ‒ 17th October, 2025.

- R4-2520710, 6G AI/ML General Aspects, Nokia, RAN4#117 meeting, Dallas, USA, 17th – 21st November 2025

- R4-2513310, Consideration on RAN4 AI study for 6G, RAN4#116bis, Huawei, HiSilicon

- R4-2303519, LS on the UE SRS IL imbalance issue, RAN4#106

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103, Huawei

- R4-2505101, WF on 6Rx RF requirements and SRS IL imbalance, RAN4#114bis, Intel Corporation