# 3GPP TSG-RAN WG4 Meeting #117	R4-2520734

# Dallas, US, 17th – 21st November, 2025

Source: 	vivo

Title: 	Discussion on RF requirements for support of less than 5 MHz channel bandwidth for (e)RedCap UE

Agenda Item:	7.4.3.1

Document for:	Approval

Introduction

In RAN#109, WID for NR_RRM_Ph6 [1] was approved [1]. One objective of this WID is to specify RF requirements for support of less than 5 MHz channel bandwidth, considering example bands: n100, n106, n26, n28 and n85. In RAN4#116bis, RAN4 started the discussion on this topic and the WF was captured in [2]. In this contribution, we discuss the remaining issues on support of 3MHz for (e)RedCap UE.

Discussion

From our perspective, there isn’t much work to do for the RF requirements part for introduction of 3MHz for (e)RedCap UEs. Practically most of requirements defined for 3MHz non-RedCap in band n100, n106, n26, n28 and n85 can be inherited for (e)RedCap UEs.

In the last meeting, almost all the RF requirements are clear for the support of 3MHz for (e)RedCap and the draft CR captured all the agreements are in [3], which was postponed.

However, some general issues are still FFS:

Release independent and UE capability Issue 4-1: Whether 3MHz UE CBW should be optional or mandatory in Rel-20 for (e)Redcap FFS whether 3MHz should be optional or mandatory in Rel-20 for (e)Redcap Issue 4-2: Release independent of 3MHz UE CBW for (e)Redcap It is agreed 3MHz UE CBW is release independent from Rel-18 for (e)RedCap UE. Issue 4-3: UE capability FFS in next meeting what UE capabilities of (e)Redcap need to be updated to support 3MHz and sent LS to RAN2.

We propose 3MHz UE CBW should be optional for (e)RedCap based on two primary factors: Firstly, the Rel-20 features under discussion show little compelling evidence of near-term widespread commercial deployment, weakening the argument for mandatory inclusion. Secondly, the established precedent within the NR specifications clearly shows that 3MHz bandwidth support is treated as optional for non-RedCap UEs across multiple bands (n26, n28, n31, n72, n85, n100). The single exception is band n106, where its extremely limited total bandwidth (5MHz) necessitates mandatory support. Thus, we prefer to consider 3MHz optional for (e)RedCap.

Proposal 1: It is suggested 3MHz to be optional in Rel-20 for (e)RedCap.

Regarding UE capability, we think UE capabilities of (e)RedCap do not need to be updated to support 3MHz. However, we think the UE capabilities of 3MHz need to be updated. All the UE capabilities of 3MHz were not applicable for (e)RedCap, now this restriction should be removed with the introduction of 3MHz for (e)RedCap.

| support12PRB-CORESET0-GSCN-41637-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block located at GSCN 41637. A UE supporting this feature shall also indicate support of support3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15 kHz SCS only. This feature is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 [2]. NOTE: The UE supporting this FG supports configuration of 12 PRB BWP operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. | UE | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |
| support12PRB-CORESET0-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block that is located according to Table 5.4.3.1-2 in TS 38.101-1 [2]. A UE supporting this feature shall also indicate support of support3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15kHz SCS only. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 12 PRB BWP operation. | Band | No | FDD only | FR1 only |  |
| support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only. It applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2], this feature shall be indicated for the band. | Band | No | FDD only | FR1 only |  |
| support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | Band | No | FDD only | FR1 only |  |

Proposal 2: RAN2 should be informed that UE capabilities related to 3MHz needs to be updated by removing the restriction:

‘This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18.’

Conclusion

This contribution discusses RF impacts for support of 3MHz for (e)RedCap. The following proposals are made:

Proposal 1: It is suggested 3MHz to be optional in Rel-20 for (e)RedCap.

Proposal 2: RAN2 should be informed that UE capabilities related to 3MHz needs to be updated by removing the restriction:

‘This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18.’

- References

[1] RP-252955, New WID: NR RRM enh Phase 6, moderator (Apple), RAN#109.

[2] R4-2515101, WF for [116bis][323] NR_less_than_5MHz_RedCap_UERF, Sony, RAN4#116bis.

[3] R4-2515100, draftCR for 38.101-1 _Adding 3MHz for e_RedCap UE, Ericsson, RAN4#116bis.